(C) Copyright 1994, 1995 IBM Corporation, All Rights Reserved

(C) Copyright 1994, Lanworks Technologies, Inc., All Rights Reserved

This file is a modified softcopy of the book BootWare User Guide,

Multiprotocol Edition from Lanworks Technologies, 2425 Mississauga

Ave., Ontario, Canada L4W 4Y6 and is to be used as a guide for

implementing RPL on the IBM 4695 terminal.

Please note that the files referred to in this document as being on

the BootWare Utility diskette can be found on the 469X Supplemental

Drivers diskette (V1.21 or later) in the 4695\RPL subdirectory.

This file is divided into 4 major sections addressing IBM RPL, Novell

Netware RPL, bootp for TCP/IP RPL, and unique message information for

the IBM 4695 POS terminal.

Users should be aware that the LAN NODE id referred to in descriptions

below is hard programmed into the planar board.  When a planar board

is replaced, the LAN NODE id of the terminal will change, requiring

updating of server files if the old LAN NODE id is in these files.

IBM RPL

-------

RPL Boot Protocol Overview

--------------------------

RPL (Remote Program Load) is a protocol originally defined by IBM

which allows workstations to start up by downloading files from a file

server.

This code supports the multicast option of the RPL architecture.

OS/2 Lan Server installation Instructions

-----------------------------------------

BootWare for RPL is tested on IBM LAN Server 3.0.  Users of later

versions of LAN Server should see the utility disk for additional

information.

Refer to the OS/2 LAN Server documentation on setting up the

remoteboot service.  Please refer to OS/2 LAN Server Network

Administrator Reference Volume 3:  Network Administrator Tasks Chapter

13 and 14.

Sample installation files can be found on the BootWare Utility Disk.

Users are strongly recommended to examine the configuration

examples.

Note: RPL and RIPL refer to the same remoteboot protocol in this

chapter.

Since the 4695 Ethernet adapter is not supported 'natively' by OS/2

LAN Server, a map of the process to setup the server to support

RPL is:

1) Setup RPL service for OS/2 LAN Server per the documentation.

Select as a device any IBM NDIS Ethernet adapter.

2) Stop the remote boot service and backup the files RPL.MAP and

NDISDD.PRO.

3) Edit RPL.MAP to define entries for the 4695 by adding the following

two lines:

yyyyyyyyyyyy DOSXXX.CNF 3 10 A IBMLAN$ DOS~IBM4695 ~ ~ ,,, Z R_IBM4695 ~ ~

yyyyyyyyyyyy OS220XXX.CNF 3 10 A ~ OS2~2.0~IBM4695 ~ ~ , , , ~ R_20_IBM4695 ~ ~

Replace XXX in the lines above with a description of your own choosing.

4) In the IBMLAN\RPL directory create the files DOSXXX.CNF and

OS2220XXX.CNF by copying existing files of this type and modifying

the file such that the path of PROTOCOL.INI points to

\IBMLAN\RPL\DOS\IBM4695.

Change the name of the NDIS driver to IBMENI.DOS (for DOS) and

IBMENI.OS2 (for OS2)

5) Create the subdirectory IBMLAN\RPL\DOS\IBM4695.  Create a valid

DOS PROTOCOL.INI for the 4695 and put in this subdirectory.

6) Copy the IBMENI.DOS driver into this subdirectory.

7) Create the subdirectory IBMLAN\RPL\IBMCOM\IBM4695.  Place a vaild

OS/2 PROTOCOL.INI file for the 4695 in this subdirectory.  Copy

an existing CONFIG.20 file from another subdirectory in

IBMLAN\RPL\IBMCOM and edit it so that it points to the correct

OS/2 driver (IBMENI.OS2).

8) Copy the IBMENI.OS2 driver to the IBMLAN\RPL\IBMCOM\MACS

subdirectory.

9) For OS/2 remoteboot, add the following statement to the end of the

file IBMLAN\RPL\NDISDD.PRO

IBMENI.DOS    IBM4695

10) Start the RPL service again.  When creating the new RPL

workstation, the R_20_IBM4695 and R_DOS_IBM4695 should appear as

valid selections in the list for server record ID.

Memory Optimization

-------------------

Under LAN Server 3.0, memory overhead is associated with DOS

Remoteboot.  For this reason, a remoteboot workstation has less

conventional memory available for application use.  However, it is

still possible to load the redirector high by using the /HIM and /UMB

parameters for the DOS LAN Requester.  Please refer to LAN Server 3.0

Network Administrator Reference Volume 2:  Performance Tuning Chapter

4 for further information.

NETWARE

-------

BootWare for NetWare is a software that will allow a 4695 to load DOS,

network drivers and other files directly from a NetWare file server,

without requiring a boot diskette in the workstation.

Features of BootWare for Netware include:

- Supports all versions of NetWare 2.x, 3.x, and 4.x, including

Portable NetWare;

- Allows selection of different boot images for each workstation;

- Handles multiple file servers on the network;

- Boot management utilities provided;

- Does not require any RPL NLM's to be loaded on the server;

Contained within this section is information on the following topics:

- how to create a boot diskette;

- how to use IMGGEN to create a single boot image file from a boot

diskette;

- what happens when you transfer from the image file to SYS:LOGIN;

- what you'll need to do in order to use BootWare on networks with

multiple file servers;

- creating additional boot image files with IMGGEN;

- creating BOOTCONF.SYS, an index file for boot images;

- how to avoid problems with batch files in SYS:LOGIN;

- error messages - describes all of the BootWare error messages and

what steps you can take to resolve the errors;

- problems and solutions - describes an assortment of common problems

users of BootWare have reported, and how these problems can be

solved;

- Using Multiple Protocol Stacks provides helpful information for

those using BootWare with packet drivers or ODI drivers with more

than one protocol stack

- Using BootWare with Microsoft Windows - provides some important

information for Windows users

- Notes for Older NetWare Versions - gives some important information

for NetWare 386 v3.0, NetWare v2.0a, and NetWare ELS users

Network Requirements

--------------------

BootWare supports all versions of NetWare from Advanced NetWare v2.0

up to and including NetWare 4.x, in single or multiple server

environments.

Ethernet frame types supported are 802.3, 802.2, or Ethernet II

(type-8137) versions.  802.3 Ethernet frame types have been the Novell

default up until NetWare 3.12 and NetWare 4.0, when 802.2 became the

default.  The Ethernet II versions support installations where

non-NetWare hosts share the Ethernet media with NetWare servers.

Unlike RPL-style PROMs, BootWare for NetWare communicates using

native NetWare protocols and does not require any NLMs or VAPs to

be loaded at the server to support remote booting. This ensures

complete compatibility with all versions of NetWare.

Creating a Boot Image

---------------------

Before you can use BootWare in your workstation, you must first create

a regular boot diskette, the same as you would if you were to boot

from a floppy drive.

The next step involves taking a snapshot of the boot diskette,

capturing its contents and creating an image file which will reside on

the file server's hard disk.

Step 1: Format the diskette

Tip:  If you are using high density media and not using much of the

disk's storage space, consider formatting the disk as a lower density

disk; this will reduce memory requirements later when you want to edit

the disk's image with the MAPGEN utility.

Step 2: Copy needed files onto boot diskette

Copy all required device drivers, NetWare DOS Workstation

files, and any other files you would normally place on a boot

diskette.

ODI Driver Note:  Be sure to copy the Novell RPLODI.COM file onto your

boot diskette, and to modify your .BAT file so that RPLODI is loaded

immediately after LSL.

Your AUTOEXEC.BAT file on the boot disk should look similar to the

following:

LSL

RPLODI     <---must be used for RPL to work successfully

IBMENIO    <---the 4695 Ethernet ODI driver

IPXODI

NETX   (or VLM)

LOGIN

When creating boot image files, it is necessary to ensure that there

are sufficient copies of DOS available for each workstation using

BootWare so as not to infringe upon copyright laws.

Step 3: Run IMGGEN

IMGGEN is a utility supplied in the \4695\RPL\NETWARE subdirectory which

creates a boot image file from the contents of a boot diskette.

(IMGGEN is similar to the Novell DOSGEN Utility, and creates

compatible image files.)

The following steps will copy the contents of a boot diskette to the

SYS:LOGIN directory on a file server.  Note that these steps may be

completed on any workstation on the network that has a floppy drive.

- Place the boot diskette in any drive of your workstation,

normally drive A:.

- Log on to the network and change to the directory where

you have installed the BootWare Utility files.

- At the prompt, use the IMGGEN utility to create the boot

file.

- Type:

IMGGEN A: <Enter>

IMGGEN will create a boot image file called NET$DOS.SYS in the

SYS:LOGIN directory of your current server using the files contained

on the boot diskette in drive A:.  This file is now your default boot

file.  The 4695 BootWare RPL code will use this file to load DOS, the

network drivers, and other information so that the workstation will

boot correctly from the network.

Check to make sure that the NET$DOS.SYS file exists in the SYS:LOGIN

directory.

Step 4:

The batch file that loads NETX.EXE (or VLM.EXE) must be copied from

the floppy disk to the LOGIN directory.  If you don't do this, you'll

get a Batch file missing message as NETX or VLM transfers from drive

A:  to SYS:LOGIN but does not find the batch file that was running on

drive A:.

While IMGGEN creates the image file it will check the batch files to

determine when NETX is loaded.  You will be asked if you want to copy

the batch file to the LOGIN directory.

If you see a Batch File Missing message when your workstation is

booted, you may also need to copy AUTOEXEC.BAT into the default

directory that is specified by the user's LOGIN script, or follow

other steps listed in the upcoming section Getting Rid of Batch File

Missing Messages.

Store the boot diskette you just generated in a safe place as it may

be required for use at a later date.  The workstation can now be

booted using BootWare.

Contrary to instructions provided by Novell, do NOT use the Novell

RPLFIX utility.  In some cases, running RPLFIX can even cause BootWare

to hang!

Step 5:

In order for multiple workstations to boot using this file, the file

must be flagged as Shareable.  When IMGGEN creates an image file it

will automatically flag the file as shareable.  If you need to

manually set it (example, if you created the image with DOSGEN) use

the FLAG command:

FLAG NET$DOS.SYS S <Enter>

Step 6:

If your network has more than one file server, copy NET$DOS.SYS and

AUTOEXEC.BAT to each server's SYS:LOGIN directory.

You now have created a single boot image file that any workstation

with BootWare installed can load and boot from the network.  If those

workstations on your LAN using BootWare require different boot

diskettes (ie.  different driver configuration, different LAN

adapters, different CONFIG.SYS options), you'll need to create

more boot image files in the SYS:LOGIN directory.  Please refer to the

upcoming section, Creating Multiple Boot Images for instructions on

how to do this.

Using BootWare With Multiple Servers

------------------------------------

BootWare does essentially the same thing the NetWare drivers do in

order to find a network file server and establish a connection.

BootWare broadcasts a special message which more than one file server

may respond to.  BootWare will attempt to boot from whichever server

responds first.

Because of the random nature of finding a particular server (it

depends on how busy a server is, and how fast it is), it is absolutely

necessary to copy all boot image files, AUTOEXEC and other .BAT files,

and BOOTCONF.SYS files across all file servers in your internetwork.

Failure to do so may cause BootWare to issue an Error opening boot

file message every now and then as it connects to a server that

doesn't contain the required boot image files.

Creating Multiple Boot Images

-----------------------------

Although many similarly-configured workstations can boot from the same

boot image file, it is more likely that more than one boot image file

will be required to suit the differing needs of the workstations on

your network.

Supporting multiple boot images requires an understanding of three

important issues:

1. Creating the BOOTCONF.SYS Boot Index File

2. Using Unique .BAT File Names in Boot Images

3. Using IMGGEN to Create Different Boot Images

Step 1:Creating the BOOTCONF.SYS Boot Index File

BootWare is capable of supporting more than one boot image by means of

a special boot index file called BOOTCONF.SYS.

The BOOTCONF.SYS file is an index of all the boot image file names and

the network address and node address of each workstation that must

access those files.  This file must be placed in the SYS:LOGIN

directory.

The BOOTCONF.SYS file may be created using any ASCII text editor.

Each line of BOOTCONF.SYS should be entered using the following

format:

0X<network>,<node>=<image file name>

such that if the Network number is 1, the Node number is 00001B091519,

and the Boot image filename is WRKSTN2.SYS, then the appropriate

syntax is:

0X1,0000C0B091519=WRKSTN2.SYS

Note that the first character of a BOOTCONF.SYS entry is a ZERO, not

the letter O!  Also, Ethernet node addresses are 12 digits long, you

don't need to include the leading zeroes.

Entries of this format must be generated for each workstation that

needs a boot file other than the default NET$DOS.SYS.  BootWare scans

the contents of BOOTCONF.SYS during workstation boot, and if no entry

is found which matches the workstation's network and node address, the

default boot image file NET$DOS.SYS will be used.

To find your workstation's network and node address, use the

NetWare USERLIST command with the /A option:

USERLIST /A

The user name marked with the * lists your workstation's network

number and node address.  BootWare also writes the LAN address to

the CRT and POS displays when RPL is enabled on a 4695.

You may also put comments on each line entry in BOOTCONF.SYS, as long

as there is at least one space separating the boot image filename and

the comment.  In general, it is a good idea to keep comments

reasonably short, as BootWare must scan through them; lots of comments

will add delays to your workstation's boot procedure.

If there are multiple file servers on your network, copies of the boot

image files must exist on all servers.  This is a requirement since a

workstation can initially attach to any file server.  If your network

has multiple file servers, BOOTCONF.SYS and all batch files must also

be copied to all file servers.

Step 2: Using Unique .BAT File Names in Boot Images

When a server contains more than one boot image file in the SYS:LOGIN

directory, care must be taken to avoid conflicts with .BAT files.

When a workstation executes NETX or VLM during its boot sequence, the

remainder of the currently running .BAT file is executed from the

SYS:LOGIN directory, not from the boot image.

If all of the AUTOEXEC.BAT files in your boot image files are

identical, you will not encounter any problems.  Your boot images can

continue using normal AUTOEXEC.BAT files without any changes.

However, if your AUTOEXEC.BAT files differ from boot disk to boot

disk, you need to have each boot image file use a uniquely- named

.BAT file. Otherwise, you will experience problems when your boot

images start running the wrong AUTOEXEC.BAT in SYS:LOGIN when they

execute NETX or VLM.

For all versions of NETX v2.12 and newer, and for all versions of

the NetWare DOS Requester, NETX (or VLM.EXE) will transfer to

exactly the same batch filename as was being executed in the image

file.

The best way to take advantage of this feature is to have your

AUTOEXEC.BAT in the DOS image invoke another .BAT file, ie.,

IMAGE1.BAT.  You will also need to copy this .BAT file into SYS:LOGIN.

When NETX (or VLM) is loaded from IMAGE1.BAT in the image file, it

will continue executing from IMAGE1.BAT in SYS:LOGIN.  Each image disk

can have its own batch filename and thus avoid conflicts in SYS:LOGIN.

Our testing shows that this method will only work with DOS 3.2 and

newer.

Step 3: Using IMGGEN to Create Different Boot Images

The following procedure describes the steps necessary to create

multiple boot image files:

- Create a floppy boot diskette as per the earlier

instructions.

- Create a boot image file by typing:

IMGGEN <floppy drive:> <image filename>

IMGGEN A: DOS63.SYS <Enter>

Pick a filename which will help you remember something about the

contents of the image file, for example DOS63.SYS.  If you don't

specify a file extension, .SYS is the default, and we strongly

recommend sticking with the .SYS default.

The customized boot image files must be flagged as Shareable.  At the

DOS prompt, type:

FLAG <filename> <S> <Enter>

This process can be repeated to create boot image files for all the

workstations on the network.

Keep track of which workstations are to use which boot image files,

as you will now need to enter this information in the BOOTCONF.SYS

file.

Transferring from the Boot Image to SYS:LOGIN

---------------------------------------------

For those who are curious, the following text describes the

interactions between NETX (or VLM) and BootWare, and how it affects

.BAT file execution.  Others may skip this section.  When you execute

NETX from your image disk, you are normally doing so from within a

batch file called AUTOEXEC.BAT.  DOS sees AUTOEXEC.BAT as a text file

containing command lines of variable length.  After reading a command

line from the AUTOEXEC.BAT file, DOS remembers the starting position

(or file offset) of the next line in the file.  After completing

execution of the current command line, execution in the AUTOEXEC.BAT

file continues at the previously retained offset.

When you execute NETX (or VLM) from the image disk batch file, NETX

senses that BootWare was used to boot the workstation, and tells

BootWare to clean up.  After BootWare does some housekeeping, and

removes its self from memory, the image files are closed and drive A:

is restored.  NETX (or VLM) then transfers control to the same batch

file in SYS:LOGIN, at the same point where it left off in image disk.

In this guide, this is what we call the NETX Transfer Point.

Understanding this transfer point is important in solving problems

with missing batch files, and in supporting more than one boot image

file in the SYS:LOGIN directory.

Getting rid of Batch File Missing messages

------------------------------------------

There are a number of reasons why you might get this message when

booting with RPL.  Perhaps you forgot to copy your .BAT file to

SYS:LOGIN.  Remember that once you run NETX or VLM, execution

transfers to the exact position in the SAME .BAT file in SYS:LOGIN.

You may also need to copy your .BAT file to the default directory

which is assigned by your LOGIN script.

If the LOGIN command was the last line of your .BAT file in

SYS:LOGIN, there is a simple solution to the Batch File Missing

message.

Most text files end with the ASCII End Of File character (EOF, or

1Ah). Each command line within your AUTOEXEC.BAT file ends with a

Carriage Return (CR, or 0Dh) and Line Feed (LF, or 0Ah) character

combination. You can eliminate the Batch file missing message by

removing the CR, LF characters after the LOGIN command, but before

the EOF character.

If your text editor will not allow you to do this, run the BATCRLF

utility (provided on the BootWare utility disk) In the following

example, we show how BATCRLF can be used to remove the CR,LF from the

LOGIN command line in the .BAT file GREENPC.BAT:

BATCRLF GREENPC.BAT

If the message still persists, or your .BAT file has additional

command lines after the LOGIN command, the error message may be caused

by not having sufficient rights in SYS:LOGIN to continue reading the

.BAT file.  No explicit rights to SYS:LOGIN are granted to users when

they are created.  A solution to this is to grant Read, Open, and

Search rights to the group EVERYONE for SYS:LOGIN using the Novell

SYSCON utility.

Another cause of the Batch file missing message is a user's login

script which changes the mapping of the first network drive from the

SYS:LOGIN to some other directory.  You can avoid this problem by

leaving the first network drive mapped to SYS:LOGIN throughout the

entire LOGIN script.  The LOGIN script supports a command called EXIT,

which can be used to run a .BAT file upon exiting from the LOGIN

script.  If you are running the LOGIN command from a .BAT file in

SYS:LOGIN, and there are further command lines in this file, what

happens when you EXIT from your login script with a .BAT file

specified?  Curiously enough, the remainder of the SYS:LOGIN .BAT file

will be processed, and only then will your EXIT .BAT file be

processed.

Utility files

-------------

This chapter provides information on the following BootWare for

NetWare utilities:

- MAPGEN, a boot image file viewing and editing tool

- UNRPLFIX, a tool which removes the RPLFIX modification from boot

image files

- BATCRLF, a tool which may help avoid Batch file missing messages

- CHKID, a tool which allows conditional .BAT file execution based on

workstation node address

- CHKID.SYS, a device driver for CONFIG.SYS, allows conditional device

driver loading

MAPGEN

------

MAPGEN is a utility for Novell networks which gives the network

administrator the ability to examine or edit the contents of DOS boot

image files generated by the IMGGEN, or DOSGEN utility.

MAPGEN maps a named boot image file to a specified local workstation's

floppy drive.  Having mapped an image file to a drive letter, the

network administrator can examine or modify the contents of the image

file just as easily as working with the original boot image diskette.

Without MAPGEN, viewing or modifying boot images is a tedious

procedure.  First, the boot image diskette must be located and

inserted into the workstation's floppy drive.  If changes are made to

the contents of the disk, the IMGGEN program must be run to refresh

the contents of the boot image file on the network file server.

MAPGEN is easy to use; it follows the syntax of the NetWare MAP

command.  Any local workstation floppy drive can be overlaid with a

MAPGEN image drive.  When the MAPGEN drive is removed, normal

operation is restored to the local drive.

Benefits

- Contents of boot image files can be viewed/modified directly.

- Allows for easy support of multiple boot images

- IMGGEN images can be maintained by users of diskless or remote

workstations

Using the MAPGEN Utility

------------------------

The file MAPGEN.COM should be copied from the BootWare Utility

Diskette onto the SYS:LOGIN directory of all file servers which will

require it.

Before running MAPGEN, you must also make sure that you have a copy of

COMMAND.COM in your current directory, or in your search drive

mappings.

In order to map a boot image file to a local floppy drive, do the

following:

- Set the default directory to SYS:LOGIN.

- Determine the name of the boot image file that requires

viewing or modification. This can be done by examining the

contents of the   BOOTCONF.SYS file.

- Issue the following MAPGEN command:

MAPGEN <local_drive_letter>:=<image file_name> [/option]

- where option is one of the following:

A - use EMS memory

E - use XMS memory

C - use conventional memory

If no option is specified MAPGEN first tries XMS, then EMS, and then

conventional memory to find enough available memory to store the

entire boot image file.

For example, to use MAPGEN to examine or change a boot image file

called NET$DOS.SYS on your workstation's drive A:  you must type the

following command:

MAPGEN A:=NET$DOS.SYS

Note:  It is important to note that there may be an upward

compatibility difficulty between earlier versions of DOS and

newer DOS versions of the IMGGEN image files, ie.  if the

workstation is booted using DOS version 3.3 and you attempt

to use MAPGEN to access a boot image file created using DOS

version 5.0, a warning message will be displayed.

At this point, you may change the current drive to A:  and begin to

view or edit the contents of the boot image file as required.  DOS

commands and text editors can be used to change or modify the files.

When you have completed working with the boot image file contents, it

is recommended that the current or default directory be changed back

to SYS:LOGIN.  Type:

EXIT <Enter>

If you have made changes to the image, MAPGEN will prompt you to see

if you want to save the changes to the existing image.

MAPGEN Error Messages

MAPGEN error: Invalid command syntax.

- MAPGEN command is not being issued in the proper format.  See

Using the MAPGEN utility for proper command syntax.

MAPGEN error: Network shell is not loaded.

- MAPGEN command has been issued on a workstation without first

having loaded the network shell (NETX or VLM).  Load the

appropriate network shell and run MAPGEN again.

MAPGEN error: cannot overlay a hard drive.

- MAPGEN can only overlay floppy drives.

MAPGEN error: cannot overlay B: in a single-floppy system.

- On a workstation with a single-floppy drive, the boot image

file may only be mapped to drive A:.

MAPGEN error: COMMAND.COM must be in current directory or search

drive.

- MAPGEN could not find a copy of COMMAND.COM in the current

directory or in your search drive mappings.

MAPGEN warning: DOS on image file is newer than what is currently

running.

- Make sure the version you booted your workstation with is at

least as new as the version of DOS within the image file.

MAPGEN error: drive letter <driveletter>: is not a local drive.

- MAPGEN operates only by mapping the boot image file to a local

workstation floppy drive; re-issue the command using only one of

the local floppy drives of the workstation.

MAPGEN error: error reading image file.

MAPGEN error: error writing image file.

MAPGEN error: error opening image file.

- MAPGEN has encountered an error opening the image file for READ

or WRITE access.  Ensure that the proper image file is being

addressed, or recreate the boot image using the IMGGEN command

and retry.

MAPGEN error: MAPGEN currently supports images up to 2.88M

- MAPGEN only supports the following 5.25" formats:

160K/360K/1.2M and the following 3.5" formats:

720KB/1.44MB/2.88MB.

MAPGEN error: not enough available memory.

- In order to read or modify the boot image file, a copy of the

file must be loaded into the workstation's memory.  The minimum

available free RAM for this purpose is 160KB, and varies with the

size of the boot image file.  Remove any

terminate-and-stay-resident software or increase the memory

capacity of the workstation.

MAPGEN warning: file is flagged as READ ONLY

- MAPGEN will not write to the file as it has been flagged READ

ONLY.  The file must be flagged READ/WRITE in order to update the

file contents.

MAPGEN warning: insufficient rights in this directory.

- MAPGEN cannot access the image file as you do not have

sufficient rights to OPEN and READ files in the directory.

MAPGEN warning: no WRITE privileges in this directory.

- MAPGEN cannot update any changes that may be made to the image

file as you have no WRITE privileges in the current directory.

MAPGEN warning: No EMS installed

MAPGEN warning: Requires EMS 3.2 or newer

MAPGEN warning: not enough EMS memory available

- These messages indicate MAPGEN has problems with your EMS

memory interface; MAPGEN will not use EMS memory in these cases.

MAPGEN warning: EMS error code: xx

- An error was encountered while writing or reading EMS memory.

MAPGEN warning: No XMS installed

MAPGEN warning: Requires XMS 2.0 or newer

MAPGEN warning: not enough XMS memory available

- These messages indicate MAPGEN has problems with your XMS

memory interface; MAPGEN will not use XMS memory in these cases.

MAPGEN warning: XMS error code nn

- An error was encountered while writing or reading XMS memory.

MAPGEN warning: COMSPEC not set, MAPGEN may not execute properly.

- If the COMSPEC environment variable is not set, you may

encounter problems running MAPGEN under some versions of DOS,

particularly DRDOS 6.0.

UNRPLFIX

--------

UNRPLFIX is used in cases when BootWare is used with boot image files

that have been unnecessarily RPLFIXed.

RPLFIX is a Novell utility which alters a boot image file in order to

provide DOS 5.0 compatibility to older Novell software and older boot

ROMs.

In some cases, a RPLFIXed boot image will cause BootWare to hang

after the message appears on the screen.  If your boot image is

RPLFIXed then you should remove the RPLFIX modification by running

UNRPLFIX.

For example, to UNRPLFIX a boot image called LABDOS6.SYS, the

following command line should be typed:

UNRPLFIX LABDOS6.SYS

If the boot image was not previously RPLFIXed, then an error message

will be displayed; otherwise, UNRPLFIX will indicate that the RPLFIX

modification was successfully removed.

BATCRLF

-------

BATCRLF is a utility which may help in eliminating the Batch file

missing message which may occur after LOGIN.  BATCRLF is meant to be

used in cases when the .BAT file contains the LOGIN command as the

last command in the .BAT file.  It removes the trailing carriage

return and line feed characters from the .BAT file, which tells DOS

not to terminate processing of the .BAT file at this command, rather

than continuing after LOGIN and not being able to find the .BAT file

(which serves no purpose at that point).

BATCRLF is used as follows:

BATCRLF BATfilename

where BATfilename is the name of the .BAT file which executes the

LOGIN command.

If BATCRLF finds either no LOGIN command within the .BAT file, or

that LOGIN is not the last command within the .BAT file, an error

message is issued, otherwise BATCRLF will indicate that it has

successfully processed the .BAT file.

CHKID

-----

The CHKID utility allows you to control the execution of a batch file

relative to the node address of a specific workstation on the network.

You may want to use CHKID if you've created a boot image file that is

used by several workstations and you want to execute different .BAT

file instructions for one or more of these workstations.

The CHKID command can be issued within a batch file (ie. AUTOEXEC.BAT)

with a either a node address, a range of addresses, or any combination

(up to 128 characters on the command line).  CHKID will check the

network to determine if the present workstation address falls in the

range of the addresses specified on the command line.  If valid, the

DOS ERRORLEVEL is set to 0, otherwise the ERRORLEVEL is set to 1.

Based on the ERRORLEVEL value, the flow of the batch file may be

redirected as required by the workstation.

The file CHKID.COM should be copied from the BootWare Utility

Diskette onto the SYS:LOGIN directory of all file servers which

will require it.

The following is the required usage and command syntax for CHKID.

It will be displayed on the workstation when you type:

CHKID <Enter>

Usage: ChkID id

id-id2

ie. ChkID 24-2C,61,A0-BF

where id is in hexadecimal format

sets ERRORLEVEL to 0 if matched

There is a batch file provided on the BootWare Utility Diskette

called AUTOID.BAT. This file gives a sample structure for an

AUTOEXEC.BAT-type batch file using the CHKID command.

echo off

ipx

netx

rem Now we're running in SYS:LOGIN

chkid 00608CE903D3

if errorlevel == 1 goto chk2

rem Insert commands here for the 1st address

goto end

:chk2

chkid 020701042573

if errorlevel == 1 goto default

rem Insert commands here for the 2nd address

goto end

:default

rem Insert commands here for all other addresses

:end

If you have never used the batch file ERRORLEVEL command before, it is

important to know that you must test for the highest value of

ERRORLEVEL first.  If you test for ERRORLEVEL == 0, you will match

BOTH conditions.  The correct method is to test for ERRORLEVEL == 1,

and to branch based on this condition.

CHKID Error Messages

- Network interface (IPX) not found.

- The Novell transport layer (IPX) was not loaded. CHKID

will not function without IPX, as it determines the workstation's

node address via IPX.

CHKID.SYS

---------

The CHKID.SYS utility allows you to control the execution of commands

in the CONFIG.SYS file relative to the node address of a specific work

station on the network.

Use CHKID.SYS if you have created a boot image file that is used by

several work stations and you want to execute different commands in

the CONFIG.SYS file on one or more of these work stations.

CHKID.SYS compares a list of node address on the command line with the

node address of the local workstation.  If the local address matches

an address in the command line, the next line in the CONFIG.SYS file

will be executed.  If the addresses don't match then the next line is

not executed.  Addresses on the command line may include a specific

address, ranges of addresses or both.

The file CHKID.SYS should be copied from the BootWare Utility Diskette

in to the boot image file which will require it.  (It must be in the

image file, not in SYS:LOGIN.)

The following is the required usage and command syntax for CHKID.SYS:

device=chkid.sys [v] [x] id id1-id2

There are two optional parameters, v and x. The v parameter will

display verbose messages of the status of chkid while it is executing.

This is only used during testing of a system using chkid.

The x parameter reverses the condition of the

address checking.  Normally the next line of the CONFIG.SYS will

only be executed if the addresses match, if you include the x

then the next CONFIG.SYS line will not be executed for the

address that matched.

Examples:

- If your workstation's node address was C0F9B544 and

CONFIG.SYS included:

device=chkid.sys C0F9B544

device=himem.sys

- HIMEM.SYS will be loaded only on the workstation with

address C0F9B544

- If you changed CONFIG.SYS to:

device=chkid.sys x C0F9B544

device=himem.sys

- HIMEM.SYS will NOT be loaded on the workstation with

node address C0F9B544, but will be loaded at all

other addresses.

NetWare Error Messages

----------------------

This section describes all of BootWare for NetWare's generic error

messages.

- Not enough memory (min. 512KB).

The workstation requires at least 512KB of RAM.  Add more RAM as

required.

- The Network Interface Board is not working.

BootWare had problems initializing the network adapter.  The problem

is usually caused by a mismatch between the adapter's actual

configuration and the configuration expected by BootWare.  Check for

a configuration problem, or for a conflict with other options.

- MSD: no reply

This is not an error message, but merely an indication that BootWare

could not locate the optional BootWare/MSD module on your network.

BootWare/MSD is an optional software package used to specify which

file server a given workstation on your network should boot from.

- File server could not be found.

Either the file server is down, or not responding to connection

requests.  Check that your workstation is connected to an active

cable segment.

- Error opening boot file.

The main reason why this message could appear is that you have more

than one file server on your network, and some of them don't have

the needed boot files in their SYS:LOGIN directories.

It is also possible that the file exists, but it is open and locked

by another station.  Check that the file exists and is flagged as

shareable.  Use the MONITOR command (or NLM) on the file server

console to check if the file is in use by another connection.

Another reason could be that you've run DOSGEN to generate your boot

image file (NET$DOS.SYS or equivalent), but you generated this file

in the SYS:SYSTEM directory.  All boot image files need to be in the

SYS:LOGIN directory.

- Error reading boot diskette file.

An error occurred while reading the boot image file.  This problem

is usually caused by a hardware-related difficulty that caused the

read to fail after many retries, and earlier error messages.  If

other messages were displayed, refer to their explanation for

suggestions on what the problem could be.

- Rx<n>, Tx <n>

These messages may appear during the boot process on a heavily

congested network.  It is normal to get several of these messages in

a row, but if many of them appear consecutively, it usually

indicates a network problem.

Rx1 indicates that a packet was received whose contents BootWare

didn't understand.

Rx2 indicates that BootWare timed out while waiting for a packet.

Reasons could be:

- cabling problems (terminal not connected);

- server not online;

- Ethernet frame type doesn't match what the server is expecting;

- With some ODI drivers, you may see Rx2 messages if you have

forgotten to load RPLODI before loading the ODI driver; please

check that RPLODI.COM is being loaded.

- <, >

The < and > messages appear only after IPX.COM (or your

board-specific ODI driver) has loaded, but before NETX or VLM has

completed execution.  The > indicates that IPX had problems

transmitting data on the network, while the < indicates that

there was a problem with receiving data from IPX.  Since BootWare

has managed to get this far in the boot sequence, it is usually a

configuration mismatch between your LAN adapter and IPX which is

causing the problem.

There are several reasons why the > message could appear; either you

have loaded the wrong IPX (perhaps for a different LAN adapter), or

the IPX you have loaded does not match your network adapter's

configuration.

If the < message appears repeatedly and you are on an Ethernet

workstation, it is likely that the Ethernet standard IPX is

configured for is incorrect (ie.  802.2 instead of 802.3, or

Ethernet II).

Questions and Answers

----------------------

This section contains information in a question and

answer format that may help you if you should have a problem using

BootWare for NetWare.

Q: Hanging; after Jumping to Boot Record -

Immediately after the Jumping to Boot Record message,

my workstation just hangs.

A: A likely cause for this problem may be that you have used

RPLFIX on the image.

Q: When I boot with BootWare, my workstation hangs, but when

I boot from the same floppy used to generate my boot image,

everything boots normally. What could be causing this?

A: When you boot from a floppy, you may be loading some program which,

during initialization, resets or corrupts the LAN adapter's memory

region while searching for memory, or hardware options.  If your

LAN adapter is disturbed at this point, it doesn't really matter,

because you haven't connected to the network yet.  When booting

with a boot ROM, however, when your LAN adapter is reset, it

destroys the connection with the file server, and may even cause

your workstation to hang.  It's usually possible to reconfigure

software or hardware options to avoid this problem.

For example, when using most memory managers, take care to

specifically EXCLUDE the memory region(s) occupied by your LAN

adapter, otherwise your memory manager may corrupt these regions

during initialization and cause BootWare to hang.

Q: When I run DOSGEN or MAPGEN, I get an error message

saying file in use during OPEN. This also happens when I update

BOOTCONF.SYS.

A: When a Boot ROM hangs for some reason after the message

Jumping to Boot Record, but before NETX or VLM has executed, the

boot image file (and possibly BOOTCONF.SYS) remains open, and

cannot be updated.  The files will be automatically closed within

15 minutes when the file server terminates the workstation's

inactive connection, but may also be closed by manually clearing

the connection using the MONITOR console command (or NLM) at the

file server or remote console.

Q: When I boot, the LOGIN directory appears as drive A: and my RAM

disk is gone.

A: You need to use a program called MKDRIVEA.COM, which

can be found on the BootWare Utility Diskette.  The file

MKDRIVEA.DOC describes how and when to use MKDRIVEA.COM.

Q: Even though I set my COMSPEC correctly in my LOGIN script, DOS

still has problems reloading COMMAND.COM when I exit an

application.

A: Setting COMSPEC from within your LOGIN script doesn't always

work; we suggest you set COMSPEC from a .BAT file that is called by

the LOGIN script's EXIT command.

Q: I'm trying to support two different protocol stacks on my

workstation (TCP/IP and IPX), but I'm having all kinds of problems.

A: Various drivers exist today which will allow you to

support both IPX and TCP/IP (or other) protocol stacks within your

workstation.  There are some tricks to getting these drivers

working.  Please see the subsection, Using BootWare with Multiple

Protocol Stacks, below.

Multiple Protocol Stacks or Frame Types

---------------------------------------

The 4695 version of BootWare supports 802.3, 802.2, or Ethernet_II

frame types.

It is possible to use almost any frame types required by the

workstation, providing some conditions are met:

The BootWare frame type must be supported at the file server.  This

may mean adding support for an additional frame type at the file

server.  The example below shows how a file server driver

can support both 802.2 and 802.3 frame types:

load IBMENIO port=300 int=a frame=ETHERNET_802.2 name=first

bind ipx to first net=1234

load IBMENIO port=300 int=a frame=ETHERNET_802.3 name=second

bind ipx to second net=1235

Most server drivers are re-entrant; only one instance of the driver

actually exists in the file server, with two instances of data, one

for each frame type, with a negligible impact on file server

resources.

No changes need to be made to the workstation's NET.CFG

file in most cases, as long as the following rules are observed:

a) The first (or default) frame type in the Link Driver

section is used by RPLODI, and must be supported at the file

server. This frame type does not have to be the same as that

supported by BootWare; it could be different.

b) If multiple frame types are specified in the Link Driver

section, and IPX is to be bound to a frame type other than the

first frame type, then a line needs to be added which ties in IPX

to the specific frame type. For example, to force IPX to use 802.2

the following statement would suffice:

Protocol IPX E0 ETHERNET_802.2

BootWare will communicate with the file server using its specific

frame type until the ODI driver in the image file loads; at that

point the ODI driver will take over and will communicate with the

server using its specified frame type.

Using Packet Drivers

---------------------

Most packet drivers load before IPX, and this is what causes problems

with BootWare, as BootWare assumes control of the LAN adapter until it

senses IPX is loaded.

One solution is to create a RAM disk using extended or EMS memory,

copy the packet drivers and IPX over to the RAM disk, and to execute

these programs from the RAM disk instead of from the DOS image file.

Another solution that is available on some packet drivers is to

postpone their initialization until the IPX layer is installed.

Using BootWare with Microsoft Windows 3.1 and Netware

-----------------------------------------------------

When loading Windows 3.1 in 386 enhanced mode, it is common to get an

"Invalid path for EMM386" error message.  If this happens, add the

"/y=[path]" switch to the EMM386 entry in CONFIG.SYS.  An example

would be:

device = emm386.exe /y=x:\dos\emm386.exe

where x:\dos\ is the directory on the network where EMM386 resides.

Both problems occur because BootWare generates a virtual A:  disk

while loading files in CONFIG.SYS and AUTOEXEC.BAT.  This virtual A:

disk is destroyed once NETX is loaded.  However, both EMM386 and

WINA20 handler remember that they are loaded from the A:  drive.  When

Window starts, it goes to the non- existent A:  drive to search for

EMM386 or WINA20.

Formatting floppy disks under Windows

--------------------------------------

When you boot with a boot ROM, Windows will not allow you to format a

floppy in drive A:; if you do, the message Cannot format the MS-DOS

startup drive will appear.  As a preventative measure, Windows will

not allow you to format a disk in the drive you booted from.

The only way you can format a floppy disk is in a DOS box.  If you

want to format a bootable floppy with system tracks, you will need

a floppy with a copy of DOS on it. The format program will prompt

you to insert this disk so it can copy the system tracks from it.

BootWare and Windows Swapfiles

------------------------------

If you do not have a local hard drive, we recommend that the swapfile

feature is disabled; this will reduce the time it takes for Windows to

load, and will reduce network traffic when Windows is running.  Do not

disable swapfiles unless you have enough memory in your workstation;

if you will be running one or two DOS windows, then 4MB of memory

should be sufficient; heavy Windows users will require at least 8MB of

memory.

If you have a local hard drive, the set the swapfile feature to use

the local hard drive instead of the network.

Notes For Older NetWare Versions

--------------------------------

If you are using NetWare v3.0, or v2.0a please see the information

contained in BWVIEW on the BootWare utility disk.

TCPIP/BOOTP

------------

bootp is the RPL protocol used in TCP/IP networks.  It is defined

in RFCs 951, 1048, 1497, and 1533.  (RFCs are a collection of Internet

standards.)

Within the bootp request frame, the 4695 implements the following

information in the vendor information area of the packet:

- magic cookie field:  99.130.83.99 (as recommended by RFC-1497)

- Reserved field (with a tag of 128): IbMpOs4964

In order to boot the workstation, the BootWare ROM first sends a

broadcast to all the servers on the network to see if one of them is

running a BOOTP daemon and has the required information for the

workstation (IP address and boot image file name).  When the

workstation receives a response from a server it will then know its IP

address, the name of the boot image file and the IP address of the

server that has the image file.

The workstation then sends a request to the server to transfer a copy

of the boot image file.  This image file is saved in the workstation's

memory.  When the end of the file is reached BootWare will clear the

workstation's screen and turn control over to the file that was just

downloaded.

Normally the boot image file will then set itself up as a drive A:

RAM disk and reset the workstation.  When the workstation is reset

this time there is a bootable disk in the new drive A:  (the RAM disk)

and the workstation will boot from this disk just as it would from a

floppy disk.

After the workstation has fully booted and connected to the network a

utility program (FREEMEM) can be used to remove the boot RAM disk (and

restore drive A:  to the local floppy drive, if there is one) and

release all the memory that was used to boot the workstation.

Setting up the Server

---------------------

Before a workstation can use BootWare for remote booting you need to

setup a server to provide the workstation with the information it

needs.  If you have multiple servers on your network, only one of them

needs to be setup for remote booting.  This server needs two daemons

installed, a BOOTP daemon and a TFTP daemon.  The BOOTP daemon

provides the workstation with its IP address and the name of the image

file to boot.  The TFTP (Trivial File Transfer Protocol) daemon is

used during the transfer of the boot file to the workstation.

BOOTP Daemon

------------

The BOOTP daemon may have been included with the TCP/IP package for

your server; check your documentation to see if it is.  If you do not

have the daemon, contact the provider of your server software to see

if it available.

Installing the daemon will depend on your operating system; follow

the documentation that was included with your daemon.

If you are using a Unix server the following instructions may

apply:

- The daemon should be located in the /ETC directory.  To run

the daemon in the background add an entry for it in the

/ETC/INETD.CONF file.

The BOOTP daemon uses a file (normally called BOOTPTAB) which

contains information for all workstations. This file must contain

at least the IP address and boot image file name for each

workstation.

TFTP Daemon

-----------

The boot image file is transferred to the workstation using

TFTP (Trivial File Transfer Protocol). The TFTP daemon should be

included with your TCP/IP package, but may be disabled; see your

TCP/IP documentation for information.

When setting up TFTP you should create a directory for all of the

image files. The suggested directory name is /TFTPBOOT

Setting up the Workstation

--------------------------

Before you can use BootWare in your workstation, you must first create

a regular boot diskette, the same as you would if you were to boot

from a floppy drive.

The next step involves taking a snapshot of the boot diskette,

capturing its contents and creating an image file which will reside on

the file server's hard disk.

Step 1: Creating a boot diskette

- Format the diskette

If you have not already done so, you need to format a

diskette as a DOS system disk, using the command:

FORMAT A:/S

Once you have formatted the floppy, it is a good idea to

run the CHKDSK command (provided with DOS), to make sure that the

diskette does indeed contain the required system files.

BootWare is able to work with any size floppy disk as the

source for the image file, but it is recommended that you use the

smallest sized disk to reduce the amount of memory that is needed

to boot the workstation.

Step 2: Copy needed files onto boot diskette

Now copy all the files to the disk that are needed to

connect the workstation to the network.  This must include a

CONFIG.SYS and any device drivers and an AUTOEXEC.BAT to load the

required network files.  If you are mapping network directories to a

local drive the AUTOEXEC.BAT file on the floppy disk should transfer

control to a batch file on the network drive.

Note: If you are using a memory manager see "Using Memory

managers" as some restrictions may apply.

Here is an example AUTOEXEC.BAT file using FTP Software

Inc.'s PC/TCP software to map the /PUBLIC directory, on the server

called TEST, to local drive E:

prompt $p$g

lsl

IBMENIO

odipkt

ethdrv

idrive

idmnt unix test /public e: pcnfs nobody

e:

init

At the end of this AUTOEXEC file control is transferred to another

batch file (INIT.BAT) located on the network drive.  This second batch

file should then change the DOS COMSPEC variable so that DOS will be

able to find a copy of COMMAND.COM after the image is released from

memory.  The FREEMEM program is then executed to release all the

memory used during booting and to restore drive A:  back as a local

drive (FREEMEM must be run from a network drive)

set comspec = e:\dos\command.com

freemem

Following the FREEMEM program you can have any other commands needed

to start the workstation.

Once the boot diskette is created, boot the workstation using this

disk to confirm that it is working correctly. When you are

confident the boot diskette is correct you can then use it to

create the image file.

Step 3: Creating a boot image file

IMAGEGEN is a utility supplied on the BootWare Utility disk which

creates a boot image file from the contents of a floppy boot diskette.

The following steps will copy the contents of a boot diskette to a

directory on a file server. These steps may be completed on any

workstation on the network that has a floppy drive.

- Place the boot diskette in any drive of your workstation,

normally drive A:.

- Log on to the network and change directories so the

directory for the boot images is your current directory (normally

the /TFTPBOOT directory).

- At the DOS prompt, use the IMAGEGEN utility to create the

boot file. IMAGEGEN requires two parameters; a local drive letter

and an image file name. For example, to create an image of the disk

in drive A: and call it station1.img, type:

imagegen a: station1.img

IMAGEGEN normally creates an extended memory image file; if the

workstation does not have extended memory, include the /c option to

create a conventional memory image.  This image file can also be

created on a hard disk in a workstation that is not connected to the

network, then transferred to the server on a floppy disk.

In order for workstations to boot using this file, the file must

have world-readable permission.  You can use the Unix command

chmod to change the file permissions, example:

chmod o=r station1.img

Store the boot diskette you just generated in a safe place as it may

be required for use at a later date.  The workstation can now be

booted using BootWare.

You now have created a single boot image file that any workstation

with BootWare installed can load and boot from the network (assuming

the workstations are using the same type of LAN adapter).  If there

are workstations on your LAN using BootWare that require different

boot configurations (ie.  different LAN adapters, different CONFIG.SYS

options), you'll need to create more boot image files.

You can now reboot the workstation to make sure the image file

works.

Address resolution

------------------

There are two options for BOOTP on the 4695 RPL screen.

This determines how BootWare will get the IP address for this

workstation.

BOOTP

-----

Select BOOTP if your are using a BOOTP daemon to provide

the IP address to the workstation.  This selection assumes that the

image file is on the same server as the BOOTP daemon.

BOOTP/ARP

---------

Select BOOTP/ARP if your are using a BOOTP daemon, but the image file

is on a different server.  After receiving the reply from the BOOTP

daemon the workstation will do an ARP (Ping) to find the server with

the image file. In order to use this option you must be able to

specify a server IP address for your workstation in your BOOTPTAB

file.

TCPIP Utilities

---------------

IMAGEGEN

--------

IMAGEGEN is used to create a boot image file from a floppy disk.

Before you can use IMAGEGEN you must first create a floppy diskette

that will boot DOS and connect the workstation to the network.

The command line syntax for IMAGEGEN is:

imagegen [/w|/c] <floppy drive letter> <image file name>

By default IMAGEGEN will create an extended memory image file (this

allows images to to 2.88MB).  To create a conventional memory image

(for use on workstations without extended memory) use the /c option.

A conventional memory image is limited to approximately 400KB.

IMAGEGEN can also create an image file that can be written to by

using the /w option.  This will create a RAM disk for drive A: while

the workstation boots.  This RAM disk will remain in memory until you

run FREEMEM.  (If you don't run FREEMEM, the RAM disk will remain in

memory and can be used by the workstation.)

IMAGEGEN can create image files with subdirectories. All that is

required is to have the subdirectories on the source floppy disk.

For example to create a boot image file called

STATION1.IMG from the floppy in drive A: the command

line is:

imagegen a: station1.img

To create the same image, and be able to write to the RAM disk, the

command line is:

imagegen /w a: station1.img

Once the boot image file is created it should be copied to the

image file directory on the network. The file must have world-read

permission.

FREEMEM

-------

FREEMEM releases all the memory that was used to transfer the boot

image file from the server after the workstation has booted.  It also

changes drive A:  back to the local floppy drive (drive A:  is used as

a RAM disk while the workstation is booting from the image file).

Notes:

- Before you can use FREEMEM you must change your DOS

COMSPEC variable to a DOS directory on the network, otherwise you

will get an error when DOS tries to reload COMMAND.COM after

FREEMEM terminates

- FREEMEM can not be included in the image file, it must be

executed from a network drive (or local hard drive)

- After using FREEMEM you should have the exact same amount

of memory available that you have when you boot from the floppy

disk you used to create the image file

MAPGEN

------

MAPGEN is a utility which gives you the ability to examine or edit the

contents of DOS boot image files generated by the IMAGEGEN utility.

MAPGEN maps a named IMAGEGEN image file to a specified local

workstation's floppy drive.  Having mapped an image file to a drive

letter, you can examine or modify the contents of the image file just

as easily as working with the original image diskette.

Without MAPGEN, viewing or modifying an image file is a tedious

procedure.  First, the image diskette must be located and inserted

into the workstation's floppy drive.  Then if changes are made to the

contents of the disk, the IMAGEGEN program must be run to refresh the

contents of the boot image file on the network file server.

MAPGEN is easy to use; any local workstation floppy drive can be

overlaid with a MAPGEN image drive.  When the MAPGEN drive is removed,

normal operation is restored to the local drive.

Benefits

- Contents of boot image files can be viewed/modified directly

- Allows for easy support of multiple boot images

- Boot images can be maintained by users of diskless or remote

workstations

Using the MAPGEN Utility

The file MAPGEN.COM should be copied from the BootWare Utility

Diskette onto a publicly accessible directory on the network.

Before running MAPGEN, you must also make sure that you have a copy

of COMMAND.COM in your current directory, or in your search path.

The syntax for the MAPGEN program is:

MAPGEN <local drive letter>:=<boot image file name> [/option]

where option is one of the following:

A - use EMS memory

E - use XMS memory

C - use conventional memory

If no option is specified MAPGEN first tries XMS, then EMS, and then

conventional memory to find enough available memory to store the

entire boot image file.

Before using MAPGEN you should first change your default directory to

the directory that contains your boot image files.

For example, to use MAPGEN to examine or change a boot image file

called station1.img on your workstation's drive A:  you must type the

following command:

mapgen a:=station1.img

At this point, you may change the current drive to A: and begin to

view or edit the contents of the boot image file as required. DOS

commands and text editors can be used to change or modify the

files.

When you have completed working with the boot image file contents,

it is recommended that the current or default directory be reset to

your boot image file directory. Then type:

EXIT

If you have made changes to the data, MAPGEN will prompt you and ask

if you want to save the changes to the image.

BPPATCH

-------

The BPPATCH program is used to replace parameters from the BOOTP reply

packet into text files.  This allows one common image file to be used by

several workstations, with the parameters unique to each workstation (like

the IP address) being specified in the BOOTPTAB file on the server and

replaced in the image file when the workstation boots.

See the BPPATCH.DOC file on the utility diskette for details on the use

of BPPATCH.

BOOTP problems

--------------

Boot screen messages:

Searching for BOOTP server (tries x)

If the workstation has stopped at this prompt and the

retry count is increasing it means that no reply has been received

from the BOOTP dmon on the server. Things to check:

-  A proper connection to the file server exists.  Can you

connect to the server if you boot from a floppy disk?

-  Your server is running a BOOTP daemon.  You may want to

try running the daemon in standalone mode to see if it is getting

the request from the workstation.  Running the daemon in

standalone mode will require removing it from the INETD.CONF file

and starting it with the -s option.  The daemon should also have

a debug mode where it will display information on the screen for

each request it gets and replies to.

Transferring boot image file (x-y)

The two numbers in the brackets, x and y, indicate retries and

packet number respectfully.  During a normal transfer the first

number, the retries, should remain at 1 and the second number, the

packet number, should increase.

If the retry count is increasing, but the packet number is not, it

indicates the workstation is not receiving any packets from the TFTP

daemon.  If the packet number remains at 1 make sure your server is

running TFTP.

If both the retry count and packet number are increasing it

indicates the transfer is progressing correctly, but the workstation

is not receiving packets as fast as it wants; this is is usually

caused by a busy network.

File transfer error messages:

Most file transfer errors originate at the server.  Some common file

transfer errors are:

- File transfer error:

If all you see is "File transfer error:"  with no description, it

indicates the workstation received an error packet, but the sending

host did not indicate what the error was.

- File transfer error: File not found

This message comes from the TFTP daemon on the server.  It means the

TFTP daemon could not find the file the workstation requested.  The

full file name is displayed on the workstation screen, make sure

this file name is correct and that the file does exist.

- File transfer error: Access violation

This message comes from the TFTP daemon on the server.  It usually

indicates that the boot image file does not have world read

permission.

You can also get this message if you are running the TFTP daemon in

secure mode.

- File transfer error: Image file too large

While downloading the boot image file from the server the image file

has filled all the available conventional memory in the workstation.

The only way to correct this is to decrease the size of the image

file, or use an extended memory image; see IMAGEGEN.

- Error: Not enough extended memory for image

While downloading the boot image file from the server the image file

has filled all the available extended memory in the workstation.

The only way to correct this is to decrease the size of the image

file, or add more memory in the workstation.

- File transfer error: No image file name specified, system halted

The workstation received a response from the BOOTP daemon, but there

was no boot image file name specified in the response.  Check the

BOOTPTAB file to make sure the entry for the workstation is correct.

Using Memory Managers

---------------------

Currently the only supported memory manager is EMM386 included with

DOS.  QEMM, 386Max and other memory managers will give error messages

when you try and load them when the boot image file is in extended

memory.

EMM386

------

If you are using EMM386 there is an additional command line option

you must include, /y.

The /y option allows you to specify a path for EMM386 after the

workstation has booted.  If you try and run Microsoft Windows after

booting from the network you will get an Invalid path for EMM386.

This is because Windows needs access to the EMM386 file, and since the

workstation was booted from the network the original file is no longer

available.  Using the /y option you can specify a network path so

Windows can access EMM386 from the network.  Example:

device=emm386.exe /y=e:\dos\emm386.exe

HIMEM.SYS

If you are using MSDOS 6.2 there is an option you must use with

HIMEM.SYS to disable the extended memory RAM test.  The option is

TESTMEM:OFF.  Your HIMEM.SYS command line should be:

device=himem.sys /TESTMEM:OFF

BootWare/IBM4695 Display Definitions

------------------------------------

This is a description of error messages displayed on 2x20 displays on

a 4695.  The general format of a message is:

XXYZZ  + any appropriate text

where the definition of XXYZZ is:

XX = [ 10 | NW | RP | TC ]

(10 = configuration/intialization)

Y  = severity code: 1 = operational status

2 = non-fatal error

3 = fatal error

ZZ = event code

Configuration, status, and non-fatal error messages will appear on the

screen for 2 seconds; Fatal Error messages will stay posted

indefinately.  In the case of incrementing display parameters (such as

download block number) the display will be updated once per second

with the current value, instead of posting each changed value on the

display.

4. Configuration/Initialization Messages (xx=10)

Code     Description        Parameter

10101    CodeRev            N.NN

10102    LAN Node ID        XXXXXXXXXXXX

10103    BootProtocol       00 = NW802.3

01 = NW802.2

02 = NWEth_2

03 = RPL

04 = BOOTP

05 = BOOTP/ARP

10104   Initialized OK

10105   Ethernet card improperly installed or not connected

10106   Can't init LAN adapter, check configuration

10107   Ethernet card not found at expected address

5. IBM RPL unique messages

Code      Description                       Parameter      Notes

----------------------------------------------------------------------

RP301     Error clearing buffer overflow                   (Fatal)

RP201     Transmit error                                   (non-fatal)

RP101     Find Frame Count                    nnnn         (status)

RP102     File Request Count                  nnnn         (status)

RP103     Data Sequence #                     nnnn         (status)

6. Netware unique messages

Code     Description                       Parameter       Notes

----------------------------------------------------------------------

NW301    Error clearing buffer overflow                    (Fatal)

NW302    Server Not Found                                  (Fatal)

NW201    Malformed Receive pkt                             (non-fatal)

NW202    Receive Timeout                                   (non-fatal)

NW203    Image file open failed            imagefilename   (non-fatal)

NW204    MSD is unavailable                MSD servername  (non-fatal)

NW101    MSD: no reply                                     (status)

NW102    MSD: reply received               MSDservername   (status)

NW103    Found server                      servername      (status)

NW104    Opening boot image                imagefilename   (status)

NW105    Jumping to boot record                            (status)

7. TCP/IP Messages

Code     Description                       Parameter       Notes

----------------------------------------------------------------------

TC301    Error clearing buffer overflow                    (Fatal)

TC302    no file name                                      (Fatal)

TC303    imagefile too large                               (Fatal)

TC304    not enough extended memory                        (Fatal)

TC305    file transfer error                               (Fatal)

TC306    TFTP timeout                                      (Fatal)

TC201    checksum error                                    (non-fatal)

TC101    Search For Server Count           xxx             (status)

TC102    Host IP address (decimal)         xxx.xxx.xxx.x   (status)

TC103    Our IP address (decimal)          xxx.xxx.xxx.x   (status)

TC104    ARP Ping Count                    xxx             (status)

TC105    Transfer Count                    xxxx            (status)

TC106    File Name                         filename        (status)

TC107    Jumping to boot record                            (status)