111	VBE/AF Proposal

VESA®

Video Electronics Standards Association

2150 North First Street, Suite 440	Phone: (408) 435-0333

San Jose, CA 95131-2029	FAX: (408) 435-8225

VESA BIOS Extension/Accelerator Functions (VBE/AF)

Version: 1.0P
Document Revision: .12P

Revision Date: date  \* mergeformat µ01/13/95§

IMPORTANT NOTICE: This is a draft proposal from the VESA Software Standards Committee. This proposal has not yet been ratified by the VESA general membership and is subject to change.

Purpose

To add a module to the popular VESA BIOS Extension (VBE) interface that allows software and drivers to access the accelerator features present in most graphics hardware.

Summary

This document will add a new function to the VBE standard BIOS that will provide access to accelerated graphics hardware. Some of the functions supported include hardware cursors, off-screen sprites, drawing and other acceleration functions.

VBE/AF is essentially a hardware-independent device interface at the very lowest level. It is not meant to replace higher-level API, but it will allow those APIs and applications that want direct hardware support to access hardware in a standardized way.

Intellectual Property

The proposals and standards developed and adopted by the Video Electronics Standards Association (VESA) are intended to promote uniformity and economies of scale in the video electronics industry for the benefit of both the industry and end users of video electronic products.  VESA cannot ensure that the adoption of a standard; the use of a method described in a standard; or the making, using, or selling of a product in compliance with the standard does not infringe upon the intellectual property rights (including patents, trademarks and copyrights) of others.  VESA therefore makes no warranties, express or implied, that products conforming to a VESA standard do not infringe the intellectual property rights of others, and accepts no liability direct, indirect, or consequential, for any such infringement.

symbol 211 \f "Symbol"µ§Copyright 1995 - Video Electronics Standards Association.  Duplication of this document within VESA member companies for review purposes is permitted.  All other rights reserved.

Trademarks

All trademarks used in this document are property of their respective owners.
VESA		Video Electronics Standards Association.

Patents

VESA proposal and standards documents are adopted by the Video Electronics Standards Association without regard to whether their adoption may involve patents on articles, materials, or processes.  Such adoption does not assume any liability to any patent owner, nor does it assume any obligation whatever to parties adopting the proposal or standards document.

Support for this Specification

If you have a product which incorporates VBE/AF. you should ask the company that manufactured your product for assistance.  If you are a manufacturer, VESA can assist you with any clarification you may require.  All comments or reported errors should be submitted in writing to VESA using one of the following mechanisms:

America Online:  Go To Keyword... VESA.

FAX: 408-435-8225

Mail to:	Video Electronics Standards Association
		Suite 440 - 2150 North First Street,
		San Jose, CA  95131-2029

- VBE/AF Workgroup Members

- Any industry standard requires input from many sources.  The people listed below were members of the VBE/AF workgroup which was responsible for combining all of the industry input into this proposal:

- Initial Review Group:

- Chairman: Tom Ryan, SciTech Software	916-894-8400; tomryan@attmail.com

- Kendall Bennett, SciTech Software

- Brian Decker, S3

- Rod Dewell, Excalibur

- Kevin Gillett, SMOS Systems

- Brad Haakenson, Cirrus Logic

- Don Pannell, Sierra Semiconductor

- Raluca Iovan, Phoenix Technologies

- Rebecca Nolan, Chips & Technologies, Inc.

- David Penley, S3

- This specification is also being review by a wide range of application developers to ensure that it fully meets their needs.

- Document Revision History

- 01/10/95-0.10P - First draft.

- 01/11/95-0.11P - Lots of changes. Removed most redundancies with VBE 2.0 spec.

- 01/13/95-0.12P - Changed introduction, added Q&A, spell check, uploaded to AOL.

- Work to be done

- symbol 183 \f "Symbol" \s 12µ§ Continue to refine function calls

- symbol 183 \f "Symbol" \s 12µ§ Select device context buffer synchronization method (p. 21)

- symbol 183 \f "Symbol" \s 12µ§ Define color formats for SetForeColor (p. 22)

- symbol 183 \f "Symbol" \s 12µ§ Define DrawLine function (p. 27)

- symbol 183 \f "Symbol" \s 12µ§ Expand the hardware cursor functions to include hardware sprites.

- symbol 183 \f "Symbol" \s 12µ§ Add function to check for hardware panning (wraparound) capabilities

- symbol 183 \f "Symbol" \s 12µ§ Generate application examples

- Table of Contents

- toc \o "1-3" µIntellectual Property	ii

- Trademarks	ii

- Patents	ii

- Support for this Specification	ii

- VBE/AF Workgroup Members	iii

- Document Revision History	iii

- Work to be done	iii

- Table of Contents	iv

- 1. Introduction	1

- 1.1. Scope of the VBE/AF Standard	1

- 1.2. Backgrounder	3

- 2. VBE/AF Overview	4

- 2.1. VBE/AF  Features	4

- 2.2. VBE/AF Affected Devices	4

- 3. VBE/AF Mode Numbers	5

- 4. VBE/AF Functions	7

- 4.1. VBE/AF Return Status	7

- 4.2. VBE/AF Coordinate System	8

- 4.2. Function 00h - Return VBE/AF Controller Information	9

- 4.3. Function 01h - Return VBE Mode Information	16

- 4.4. Function 02h  - Set VBE Mode	17

- 4.5. Function 03h - Return current VBE Mode	17

- 4.6. Function 04h  - Save/Restore state	17

- 4.7. Function 05h - Display Window Control	17

- 4.8. Function 06h - Set/Get Logical Scan Line Length	17

- 4.9. Function 07h - Set/Get Display Start	17

- 4.10. Function 08h - Set/Get DAC Palette Format	17

- 4.11. Function 09h - Set/Get Palette Data	17

- 4.12. Function 0Ah - Return VBE Protected Mode Interface	17

- 4.13. Function 0Bh - Return VBE/AF Device Context Buffer	18

- 4.14. Accelerator Function - SetForeColor	22

- 4.15. Accelerator Function - SetBackColor	23

- 4.16. Accelerator Function - SetClipRect	24

- 4.17. Accelerator Function - DrawScan	25

- 4.18. Accelerator Function - DrawScanList	25

- 4.19. Accelerator Function - DrawRect	26

- 4.20. Accelerator Function - DrawLine	27

- 4.21. Accelerator Function - DrawPattScan	28

- 4.22. Accelerator Function - BitBlt	29

- 4.23. Accelerator Function - TransBitBlt	30

- 4.24. Accelerator Function - MonoBitBlt	32

- 4.25. Accelerator Function - SetCursor	33

- 4.26. Accelerator Function - SetCursorPos	34

- 4.27. Accelerator Function - SetCursorColor	34

- 4.28. Accelerator Function - ShowCursor	34

- 4.29. Accelerator Function - WaitTillIdle	35

- 4.30. Accelerator Function - EnableDirectAccess	35

- 4.31. Accelerator Function - DisableDirectAccess	35

- Appendix 1 - VBE/AF Quick Reference	36

- Appendix 2 - VBE/AF Data Structures	37

- Appendix 3 - VBE/AF Implementation Considerations	38

- A3.1. Certification Requirements	38

- A3.1.1. VAFTest Utility	38

- A3.1.2. VAFPerf Utility	38

- Appendix 4 - Application Programming Considerations	39

- Appendix 5 - Related Documents	40

- Appendix 5 - Questions & Answers	41

- §

- 1.	Introduction

- This document contains the VESA BIOS Extension Accelerator Functions (VBE/AF) specification for standard software access to high performance accelerated graphics display controllers that support resolutions, color depths, and frame buffer organizations beyond the VGA hardware standard.  It is intended for use by both applications programmers and system software developers.

- System software developers may use this document to supplement the System and INT 10h ROM BIOS functions to provide the VBE/AF services.  Application developers can use this document as a guide to programming all VBE/AF compatible devices.

- To understand the VBE/AF specification, some knowledge of 80386 assembly language and the VGA hardware registers may be required.  However,  the services described in this specification may be called from any high-level programming language that provides a mechanism for generating software interrupts with the 80386 registers set to user-specified values.

- In this specification, 'VBE/AF' is synonymous with 'VBE Accelerator Functions version 1.0'.

- 1.1.	Scope of the VBE/AF Standard

- The primary purpose of the VESA VBE/AF is to provide standard software support for the many unique implementations of high performance Accelerated Super VGA (SVGA) graphics controllers on the PC platform.  This is to provide software applications and Operating System device drivers a standard mechanism to access these accelerated functions in supported devices.

- The VBE/AF specification is not intended to replace high performance OEM specific accelerated device drivers for operating systems like Windows or OS/2, but to provide the operating systems and application developers the ability to build generic accelerated drivers that will work across a wide variety of hardware devices.  This will help to raise the baseline of support for non-OEM specific drivers from dumb framebuffer access to high performance accelerated access.  It is also intended to provide Video Game developers on the PC standardized access to accelerated display hardware without requiring any OEM specific display driver, raising the baseline of support for Video Games on the PC from dumb framebuffer access to high performance accelerated access.

- VBE/AF is also intended to compliment the set of services provided by  the VBE Core Functions Version 2.0 (or VBE 2.0), and will require that a VBE 2.0 or later interface be available. VBE/AF will extend upon some of the VBE 2.0 services, such as detecting the underlying hardware and initializing accelerated display modes in a method that is backwards compatible with an unaccelerated VBE 2.0 implementation.  When a VBE/AF accelerator mode is active, the standard VBE 2.0 functions calls should continue to function as normal (such as display start address programming, palette programming and bank switching).

- VBE/AF services provide access to a standard set of accelerator functions provided by the display controller, and report the availability and details of all supported functions to the application as necessary.  VBE/AF services also provide information on how to set up and maintain the state that the VBE/AF device driver needs for correct operation (such as accessing I/O and memory mapped hardware registers, and maintaining a display device context for the VBE/AF device driver containing necessary configuration information). If a hardware device does not support a particular accelerated function, such as line drawing, support for this function can be mapped out and the application will need to implement it in software. However if a hardware device supports a particular function, it must be supported by the VBE/AF (if the hardware has line drawing, it must be provided). Optional functions are only provided for the cases where the hardware does not support a particular feature, not where a VBE/AF implemetor decided to be lazy. For most present day hardware, all the VBE/AF functions should be implemented.

- VBE/AF services are also designed to correctly arbitrate between accelerated device access and dumb framebuffer device access for the installed display device.  This allows application code to take advantage of accelerated functions in certain areas, or perform direct software rendering into banked or linear framebuffer video memory.  This does however assume that a VBE/AF device provides both accelerated access and dumb framebuffer access in a manner compatible with the VBE 2.0 interface (i.e.: a 64k banked framebuffer and/or a flat linear framebuffer).

- For reasons of space and speed efficiency, the VBE/AF accelerator functions are only provided through a set of high performance, relocateable 32 bit protected mode routines. All 32 bit protected mode routines can be relocated directly into the applications own code space and are executed by passing all parameters in 386 extended registers and making a near call to the specific routine. 16 bit real mode and 16 bit protected mode is not supported by the VBE/AF specification.

- In order for the VBE/AF standard to be acceptable and used by application developers, it must be fast and flexible.  Application developers must be able to use the functions to implement high performance, complex accelerated graphics primitives by building upon the accelerated building blocks provided by the VBE/AF. For this reason, the VBE/AF functions have been boiled down into the smallest possible set of accelerator functions that can be implemented as efficiently as possible. When an application calls the VBE/AF device driver routines, it will already have performed any kind of application specific special casing necessary. The VBE/AF functions are also broken down into functions that change the current state of the accelerator hardware, and those that actually render primitives. Thus if you need to draw 1000 rectangles or lines of the same color, you only need to change the color and mix once, and then blast out the 1000 rectangle or lines as efficiently as possible.

- Since graphics controller services on the PC are typically implemented in ROM, the VBE services are defined so that they should be implemented within the standard VGA ROM.  When ROM implementations of VBE are not possible, or when field software upgrades to the onboard ROM are necessary, the VBE implementation may be also offered as a device driver or DOS Terminate & Stay Resident (TSR) program.

- 1.2.	Backgrounder

- Since the introduction of Microsoft Windows 3.0, PC based video hardware has quickly developed from the original IBM VGA standard to incredibly high performance hardware that supports pixel resolutions and depths way beyond the original IBM VGA standard.  In order to be able to handle the massive bandwidth that Graphics User Interface (or GUI) operating systems like Windows require in these extended resolutions, these devices have also incorporated a number of extensions for off-loading the burden of graphics rendering from the CPU to the graphics hardware device.  This advanced hardware acceleration (or GUI acceleration) has now become the defacto industry standard, and any new SuperVGA device developed today at least contain some sort of GUI hardware acceleration.

- However, several serious problems face a software developer who intends to take advantage of these "GUI Accelerator" devices.  Because there is no standard hardware implementation, the developer is faced with widely disparate hardware accelerator architectures.  Lacking a common software interface, designing applications for these environments is costly and technically difficult.  Except for applications or Operating Systems supported by OEM-specific display drivers, very few software packages can take advantage of the power and capabilities of GUI hardware accelerator products.

- The VBE Core Functions standard was originally conceived to enable the development of applications that wished to take advantage of display resolutions and color depths beyond the VGA definition.  The new VBE/AF services are intended to extended the VBE Core Function services to provide standardized access to high performance GUI hardware accelerator functions.

- 2.	VBE/AF Overview

- This chapter outlines the various features and limitations of the VBE/AF standard.

- 2.1.	VBE/AF  Features

- symbol 183 \f "Symbol" \s 10 \hµ§	Standard application interface to GUI Accelerator Devices.

- symbol 183 \f "Symbol" \s 10 \hµ§	Extension of VBE Core Functions Version 2.0.

- symbol 183 \f "Symbol" \s 10 \hµ§	Support for most common GUI Accelerator Functions.

- symbol 183 \f "Symbol" \s 10 \hµ§	Support for pixel depths from 8 bit to 32 bits per pixel.

- symbol 183 \f "Symbol" \s 10 \hµ§	Support for basic mix operations (REPLACE, OR, AND, XOR).

- symbol 183 \f "Symbol" \s 10 \hµ§	Support for offscreen memory management for bitmap storage.

- symbol 183 \f "Symbol" \s 10 \hµ§	High performance 32 bit protected mode interface.

- symbol 183 \f "Symbol" \s 10 \hµ§	All parameters passed in registers for speed.

- symbol 183 \f "Symbol" \s 10 \hµ§	Device context buffer used to maintain state of device.

- 2.2.	VBE/AF Affected Devices

- All SuperVGA GUI accelerator devices in the IBM PC platform are suitable for use within the VBE/AF standard to enable access to the device by VBE/AF-compliant applications.  Note however that the VBE/AF does require that an application be able to use the accelerator or direct framebuffer access for rendering graphics primitives.  This means a device like the IBM 8514/A that does not provide a direct framebuffer access cannot be supported by the VBE/AF.  In general, if a device can support VBE 2.0 it can also support the VBE/AF functions.

- 3.	VBE/AF Mode Numbers

- The VBE/AF uses the VBE 2.0 Set SuperVGA Video Mode (function 4F02h) for initializing the accelerated video modes. The VBE/AF specification extends the video mode numbering scheme used by VBE 2.0 in the following manner (for more information refer to the VBE 2.0 documentation):

- The format of VBE/AF mode numbers is as follows:

- D0-D8		= 	Mode number

- If D8 == 1, this is a VESA defined mode

- D9-D12	= 	Reserved by VESA for future expansion (= 0)

- D13		= 	Accelerator Access Select

- If D13 == 0, Unaccelerated video mode

- If D13 == 1, Accelerated video mode

- D14		= 	Linear/Flat Frame Buffer Select
				If D14 == 0, Use VGA Frame Buffer

- If D14 == 1, Use Linear/Flat Frame Buffer

- D15		= 	Preserve Display Memory Select

- If D15 == 0, Clear display memory

- If D14 == 1, Preserve display memory

- Thus, VBE/AF accelerated mode numbers are the similar to their VBE 2.0 counterparts, but have bit 13 set to 1 to initialize the accelerator hardware for access.  Modes may only be set if the mode exists in the Accelerated Video Mode List returned in Function 4F00h.  Because a hardware device may support a difference set of video modes depending upon whether the GUI Accelerator is active or not, the VBE/AF returns a separate list of accelerated video modes in the information block returned by Function 4F00h.

- Note: The VBE/AF does not define specific mode numbers for specific device resolutions, but allow the OEM to define modes for all resolution supported by their hardware.  This allows OEM's to include support for non-standard high resolution video modes such as 1152x768 or some other such variation.  VBE/AF applications should follow the guidelines in Appendix ?? - Application Programming Considerations - for setting a desired mode.

- Note:  Mode 81FFh is a special mode introduced in VBE 2.0 designed to preserve the current memory contents and to give access to the entire video memory.  This mode must still be supported when an accelerated video mode is active, enabling the state of the accelerated video memory to be saved if desired.

- VBE/AF Implementation Note: Although the mode numbers defined for the unaccelerated and the accelerated video modes may be different, it would be wise to define the supported accelerated video modes as the same number as the unaccelerated version but with bit D13 set to a 1. Hence the function 4F01h can use the same mode information blocks for both the accelerated and non-accelerated functions, and simply modify a few of the fields depending on whether the mode is accelerated or not.  Thus mode 101h could be defined for 640x480x256 unaccelerated and mode 2101h defined for 640x480x256 accelerated. This will save considerable space in a BIOS implemented and make conversion of a VBE 2.0 BIOS to support VBE/AF relatively straightforward.

- 4.	VBE/AF Functions

- This chapter describes in detail each of the functions defined by the VBE/AF standard.  VBE/AF device access is initialized by using the INT 10h interrupt vector, passing arguments in the 80x86 registers.  The INT 10h interrupt handler first determines if a VBE/AF function has been requested, and if so, processes that request.  Otherwise control is passed to the standard VGA BIOS for completion. The INT 10h mechanism is used to initialized a VBE/AF display mode, relocate the 32 bit accelerator functions from the VBE/AF implementation into the application program and create the appropriate device context buffer used by the accelerator functions.  Then all access to the VBE/AF accelerator functions is performed by making direct calls to the relocated 32 bit device driver functions with all parameters loaded in the required registers.  When a VBE/AF accelerator mode is active, the standard VBE 2.0 functions calls should continue to function as normal, including the 32 bit protected mode versions.

- All VBE/AF INT 10h functions are called with the AH register set to 4Fh to distinguish them from the standard VGA BIOS functions.  The AL register is used to indicate which VBE/AF function is to be performed.

- All VBE/AF accelerator functions are called directly after the code has been relocated, and require that the accelerator device context buffer be passed in the ES:EDI registers to all functions. The stack should also be a valid 32 bit protected mode stack, and should be large enough to allow the accelerator functions to use up to 1024 bytes of temporary stack space if necessary.  The values of EAX,EBX,ECX,EDX and ESI are determined. VBE/AF accelerator functions do not return any status codes.

- When the VBE/AF accelerator functions are called, the current I/O permission bit map must allow access to the I/O ports that the VBE/AF functions may need to access.  Also a VBE/AF accelerator function may need to directly access memory mapped registers in physical memory.  Access to these physical memory locations is provided by the application by creating selectors to the memory areas that the device may need, and storing the values of these selectors in the accelerator device context buffer.  When the VBE/AF accelerator functions needs to access memory mapped registers, it will load the appropriate selector from the device context buffer temporarily, and will restore the value of that selector before returning to the application.  The locations of these I/O ports and memory mapped register areas can be found in the Sub-table (Ports and Memory) returned by VBE/AF Function 0Bh.

- 4.1.	VBE/AF Return Status

- The AX register is used to indicate the completion status upon return from VBE/AF INT 10h functions.  If VBE support for the specified function is available, the 4Fh value passed in the AH register on entry is returned in the AL register.  If the VBE function completed successfully, 00h is returned in the AH register.  Otherwise the AH register is set to indicate the nature of the failure.

- VBE RETURN STATUS

- AL ==	4Fh:	Function is supported

- AL !=	4Fh:	Function is not supported

- AH ==	00h:	Function call successful

- AH ==	01h:	Function call failed

- AH ==	02h:	Software supports this function, but the hardware does not

- AH ==	03h:	Function call invalid in current video mode

- Note: Applications should treat any non-zero value in the AH register as a general failure condition as later versions of the VBE/AF may define additional error codes.

- 4.2.	VBE/AF Coordinate System

- All the VBE/AF accelerated functions take coordinates in a global framebuffer coordinate system. This coordinate system starts with (0,0) at the start of framebuffer memory and increments the X coordinate for every pixel and Y coordinate for every scanline.  If the logical scanline width is set to 1024 bytes, then the coordinate (0,1) will be rendering into the byte at location 1024 from the start of framebuffer memory.

- It is up to the application to impose any other logical coordinate system on top of the VBE/AF device driver routines, such as handling viewport mapping etc. Also if the application is doing double buffered rendering and wished to render into the second physical display page, it must offset the Y coordinate of all rendering operations by the starting Y coordinate of the second physical display page (which also means that it must align the second page on a scanline boundary for correct operation).

- Also note that all coordinates are passed to the VBE/AF routines in the full 32 bit registers, although modern accelerator hardware never exceeds +-32k pixels for coordinate information.  They are passed in 32 bit registers for simplicity only, and all VBE/AF routine will ignore the top 16 (or more) bits of the passed values when programming the hardware.  This means that application software can either load the full 32 bit register with a coordinate value, or simply load the low 16 bits in the 16 bit register if that is more convenient for the application.

- 4.2.	Function 00h - Return VBE/AF Controller Information

- This required function returns the capabilities of the display controller, the revision level of the VBE/AF implementation, and vendor specific information to assist in supporting all display controllers in the field.  The semantics of this function are identical to those in the VBE 2.0 specification, and extended to include information about the VBE/AF accelerator device. The full specification of Function 00h has been reproduced from the VBE 2.0 specs with the extension information documented (and highlighted with bold type).

- The purpose of this function is to provide information to the calling program about the general capabilities of the installed VBE software and hardware. This function fills an information block structure at the address specified by the caller. The VbeInfoBlock information block size is 256 bytes for VBE 1.x, and 512 bytes for VBE 2.0 and VBE/AF.

- Input:		AX	= 4F00h	Return VBE Controller Information

- ES:DI	= 		Pointer to buffer in which to place 
					VbeInfoBlock structure

- (VbeSignature should be set to 'VBE2' when

- function is called to indicate VBE 2.0 and 							VBE/AF information is desired and the information

- block is 512 bytes in size.)

- Output:	AX	= 		VBE Return Status

- Note: All other registers are preserved.

- The information block has the following structure:

- VbeInfoBlock struc

- VbeSignature			db	'VESA'	 	; VBE Signature

- VbeVersion			dw	0200h		; VBE Version

- OemStringPtr			dd	?		; Pointer to OEM String

- Capabilities			db	4 dup (?)	; Capabilities of graphics cont.

- VideoModePtr			dd	?		; Pointer to Video Mode List

- TotalMemory			dw	?		; Number of 64kb memory blocks

- ; Added for VBE 2.0

- OemSoftwareRev		dw	?		; VBE implementation Software revision

- OemVendorNamePtr		dd	?		; Pointer to Vendor Name String

- OemProductNamePtr		dd	?		; Pointer to Product Name String

- OemProductRevPtr		dd	?		; Pointer to Product Revision String

- AccelVbeVersion		dw	0100h		; VBE/AF Version

- AccelVideoModePtr		dd	?		; Pointer to Accelerated Mode List

- Reserved			db	214 dup (?)	; Reserved for VBE implementation scratch

- ;   area

- OemData			db	256 dup (?)	; Data Area for OEM Strings

- VbeInfoBlock ends

- Note: All data in this structure is subject to change by the VBE implementation when VBE Function 00h is called.  Therefore, it should not be used by the application to store data of any kind.

- Description of the VbeInfoBlock structure fields:

- The VbeSignature field is filled with the ASCII characters 'VESA' by the VBE implementation.  VBE 2.0 applications should preset this field with the ASCII characters 'VBE2' to indicate to the VBE implementation that the VBE 2.0 extended information is desired, and the VbeInfoBlock is 512 bytes in size.  Upon return from VBE Function 00h, this field should always be set to 'VESA' by the VBE implementation.

- The VbeVersion is a BCD value which specifies what level of the VBE standard is implemented in the software. The higher byte specifies the major version number. The lower byte specifies the minor version number.  For the VBE/AF version implemented, please see the AccelVbeVersion field.

- Note:  The BCD value for VBE 2.0 is 0200h and the BCD value for VBE 1.2 is 0102h. In the past we have had some applications misinterpreting these BCD values.  For example, BCD 0102h was interpreted as 1.02, which is incorrect.

- The OemStringPtr is a Real Mode far pointer to a null terminated OEM-defined string. This string may be used to identify the graphics controller chip or OEM product family for hardware specific display drivers. There are no restrictions on the format of the string.  This pointer may point into either ROM or RAM, depending on the specific implementation.  VBE 2.0 BIOS implementations must place this string in the OemData area within the VbeInfoBlock if 'VBE2' is preset in the VbeSignature field on entry to Function 00h.  This makes it possible to convert the RealMode address to an offset within the VbeInfoBlock for Protected mode applications.

- Note: The length of the OEMString is not defined, but for space considerations, we recommend a string length of less than 256 bytes.

- The Capabilities field indicates the support of specific features in the graphics environment. The bits are defined as follows:

- D0	= 0 		DAC is fixed width, with 6 bits per primary color
  	= 1		DAC width is switchable to 8 bits per primary color 
D1	= 0		Controller is VGA compatible
  	= 1		Controller is not VGA compatible  
D2	= 0		Normal RAMDAC operation
	= 1		When programming large blocks of information to the RAMDAC
			use blank bit in Function 09h. i.e. RAMDAC recommends
			programming during blank period only. 
D3	= 0		Controller does not support VBE/AF extensions
  	= 1		Controller does support VBE/AF extensions 
D4	= 0		No need to call EnableDirectAccess to access framebuffer
	= 1		Must call EnableDirectAccess to access framebuffer
D5	= 0		Controller does not have a hardware cursor
	= 1		Controller has a hardware cursor
D6	= 0 		Controller does not have hardware clipping
	= 1 		Controller has hardware clipping
D7	= 0		Controller does not support transparent BitBlt
	= 1		Controller supports transparent BitBlt
D8-31	= Reserved

- BIOS Implementation Note: The DAC must always be restored to 6 bits per primary as default upon a mode set.  If the DAC has been switched to 8 bits per primary, the mode set must restore the DAC to 6 bits per primary to ensure the application developer that he does not have to reset it.

- Application Developer's Note:  If a DAC is switchable, you can assume that the DAC will be restored to 6 bits per primary upon a mode set.  For an application to use a DAC the application program is responsible for setting the DAC to 8 bits per primary mode using Function 08h.

- VGA compatibility is defined as supporting all standard IBM VGA modes, fonts and I/O ports; however, VGA compatibility doesn't guarantee that all modes which can be set are VGA compatible, or that the 8x14 font is available.

- The need for D2 = 1 "program the RAMDAC using the blank bit in Function 09h" is for older style RAMDAC's where programming the RAM values during display time causes a "snow-like" effect on the screen.  Newer style RAMDAC's don't have this limitation and can easily be programmed at any time, but older RAMDAC's require the that they be blanked so as not to display the snow while values change during display time..  This bit informs the software that they should make the function call with 80h rather than 00h to ensure the minimization of the "snow-like" effect.

- Bit D3 is used to determine whether the VBE/AF extensions are available to an application. If this bit is set to 0, VBE/AF is not available (the default for VBE 2.0). If this bit is set to 1, then the extended VBE/AF information is present in the VbeInfoBlock structure, and the extended VBE/AF INT 10h functions are available.

- Bit D4 is used to indicate if the hardware requires specific arbitration routines to be called to enable and disable direct video memory access.  If this bit is set to 1, then an application program must ensure that it calls the EnableDirectAccess and DisableDirectAccess accelerator functions before performing any direct access to the video framebuffer. If this bit is set to 0, then all the application needs to do is call the WaitTillIdle function to ensure that the accelerator has finished performing the last accelerated rendering function (hardware that require arbitration will implicitly perform a WaitTillIdle function in the EnableDirectAccess function).

- Bit D5 is used to indicate if the hardware supports a Hardware mouse cursor in accelerated modes, or if the cursor should be rendered in software.  Generally hardware cursor support is the norm for accelerated video cards, and may be required by some software.

- Bit D6 is used to indicate if the hardware clipping of accelerated primitives is supported, or if clipping should be performed in software.  If hardware clipping is available, it is assumed that clipping is available for accelerated access to the entire framebuffer including the designated OffScreen memory areas.  If necessary the VBE/AF should restrict the offscreen memory area if the hardware clipping region is restricted.

- Bit D7 is used to indicate if hardware transparent BitBlt's are supported, or if transparent blt'ing should be performed in software.  Transparent BitBlt support may be required by certain game related applications in order to function correctly or at least to function with reasonable performance.

- The VideoModePtr points to a list of  mode numbers for all  display modes supported by the VBE implementation.  Each mode number occupies one word (16 bits). The list of mode numbers is terminated by a -1 (0FFFFh).  The mode numbers in this list represent all of the potentially supported modes by the display controller.  Please refer to Chapter 3 for a description of VESA VBE mode numbers.  VBE 2.0 BIOS implementations must place this mode list in the Reserved area in the VbeInfoBlock or have it statically stored within the VBE implementation if 'VBE2' is preset in the VbeSignature field on entry to Function 00h.

- Note: It is the application's responsibility to verify the actual availability of any mode returned by this function through the Return VBE Mode Information (VBE Function 01h) call. Some of the returned modes may not be available due to the actual amount of memory physically installed on the display board or due to the capabilities of the attached monitor.

- Note: If a VideoModeList is found to contain no entries (starts with 0FFFFh), it can be assumed that the VBE implementation is a "stub" implementation where only Function 00h is supported for diagnostic or "Plug and Play" reasons.  These stub implementations are not VBE 2.0 compliant and should only be implemented in cases where no space is available to implement the whole VBE.

- The TotalMemory field indicates the maximum amount of memory physically installed and available to the frame buffer in 64KB units.  (e.g. 256KB = 4, 512KB = 8)  Not all video modes can address all this memory, see the ModeInfoBlock for detailed information about the addressable memory for a given mode.

- The OemSoftwareRev field is a BCD value which specifies the OEM revision level of the VBE software.  The higher byte specifies the major version number. The lower byte specifies the minor version number.  This field can be used to identify the OEM's VBE software release.  This field is only filled in when 'VBE2' is preset in the VbeSignature field on entry to Function 00h.

- The OemVendorNamePtr is a pointer to  a null-terminated string containing the name of the vendor who produced the display controller board product.  (This string may be contained in the VbeInfoBlock or the VBE implementation.)  This field is only filled in when 'VBE2' is preset in the VbeSignature field on entry to Function 00h.  (Note:  the length of the strings OemProductRev, OemProductName and OemVendorName (including terminators) summed, must fit within a 256 byte buffer; this is to allow for return in the OemData field if necessary.)

- The OemProductNamePtr is a pointer to  a null-terminated string containing the product name of the display controller board.  (This string may be contained in the VbeInfoBlock or the VBE implementation.)  This field is only filled in when 'VBE2' is preset in the VbeSignature field on entry to Function 00h.  (Note:  the length of the strings OemProductRev, OemProductName and OemVendorName (including terminators) summed, must fit within a 256 byte buffer; this is to allow for return in the OemData field if necessary.)

- The OemProductRevPtr is a pointer to a null-terminated string containing the revision or manufacturing level of the display controller board product.  (This string may be contained in the VbeInfoBlock or the VBE implementation.)  This field can be used to determine which production revision of the display controller board is installed.  This field is only filled in when 'VBE2' is preset in the VbeSignature field on entry to Function 00h.  (Note:  the length of the strings OemProductRev, OemProductName and OemVendorName (including terminators) summed, must fit within a 256 byte buffer; this is to allow for return in the OemData field if necessary.)

- The AccelVbeVersion is a BCD value which specifies what level of the VBE/AF standard is implemented in the software. The higher byte specifies the major version number. The lower byte specifies the minor version number.  For VBE/AF 1.0 this value is 100h.

- The AccelVideoModePtr points to a list of  mode numbers for all accelerated display modes supported by the VBE/AF implementation.  Each mode number occupies one word (16 bits). The list of mode numbers is terminated by a -1 (0FFFFh).  The mode numbers in this list represent all of the potentially supported modes by the display controller.  The list of available accelerated display resolutions may not be the same as those provided with unaccelerated access, and mode numbers stored in here will all have bit D13 set to 1.  Please refer to Chapter 3 for a description of VESA VBE/AF mode numbers.

- Note: It is the application's responsibility to verify the actual availability of any mode returned by this function through the Return VBE Mode Information (VBE/AF Function 01h) call. Some of the returned modes may not be available due to the actual amount of memory physically installed on the display board or due to the capabilities of the attached monitor.

- The Reserved field is a space reserved for dynamically building the VideoModeList if necessary if the VideoModeList is not statically stored within the VBE implementation.  This field should not be used for anything else, and may be reassigned in the future.  Application software should not assume that information in this field is valid.

- The OemData field is a 256 byte data area that is used to return OEM information returned by VBE Function 00h when 'VBE2' is preset in the VbeSignature field.  The OemVendorName string, OemProductName string and OemProductRev string are copied into this area by the VBE implementation.  This area will only be used by VBE implementations 2.0 and above when 'VBE2' is preset in the VbeSignature field.

- 4.3.	Function 01h - Return VBE Mode Information

- This required function returns extended information about a specific VBE/AF display mode from the mode list returned by VBE/AF Function 00h.  This function is identical to the VBE 2.0 function, except that an VBE/AF application will pass in an accelerated video mode number (21xxh) rather than a normal VBE 2.0 mode number (1xxh).

- VBE/AF Note: The OffScreenMemOffset and OffScreenMemSize fields are used by VBE 2.0 to determine how much offscreen memory is available for the application to use for caching bitmap and font data in video memory.  However the VBE/AF controller will fill in appropriate MaxX and MaxY values in the device context buffer when it is initalised.  These values indicate the entire physical expanse of video memory currently available (including offscreen memory regions to the right and below the default video page), and the controller will have allocated enough memory for storing the hardware cursor image above the last Y coordinate (this where all controllers that I know of can store it).

- This is a much cleaner solution for VBE/AF since applications will find it much easier to work in (x,y) coordinates rather than linear memory buffer offsets, and more clearly defines just what is region of the video memory the application is allowed to render into.

- 4.4.	Function 02h  - Set VBE Mode

- Identical the VBE 2.0 function, except that a VBE/AF application will pass in a VBE/AF mode number (21xxh) rather than a VBE 2.0 mode number (1xxh).  After the mode has been set, it is assumed that the video hardware accelerator has been reset to the default state and is ready to accept drawing commands.

- 4.5.	Function 03h - Return current VBE Mode

- Identical the VBE 2.0 function.

- 4.6.	Function 04h  - Save/Restore state

- Identical to VBE 2.0 function

- 4.7.	Function 05h - Display Window Control

- Identical to VBE 2.0 function, except that if bit D9 of the ModeAttributes field is set to 1, an application must ensure that EnableDirectAccess is called before the bank switching routines are called. Generally the VBE/AF application will call the direct 32 bit protected mode bank switch function, and not the INT 10h version.

- 4.8.	Function 06h - Set/Get Logical Scan Line Length

- Identical to VBE 2.0 function.

- 4.9.	Function 07h - Set/Get Display Start

- Identical to VBE 2.0 function. Generally the VBE/AF application will call the direct 32 bit protected mode display start routine.

- 4.10.	Function 08h - Set/Get DAC Palette Format

- Identical to VBE 2.0 function.

- 4.11.	Function 09h - Set/Get Palette Data

- Identical to VBE 2.0 function.  Generall the VBE/AF application will call the direct 32 bit protected mode palette programming routine.

- 4.12.	Function 0Ah - Return VBE Protected Mode Interface

- Identical to VBE 2.0 function.

- 4.13.	Function 0Bh - Return VBE/AF Device Context Buffer

- This required function call returns the size of the device specific context buffer, and initializes this buffer for the specific hardware device. This buffer also contains pointers to the 32 bit protected mode entry points for the device driver functions, which is copied directly into the provided device context buffer by the VBE/AF. After the application has initialized the buffer and copied it to protected mode memory, the vectors to the routines should be updated to point to the newly relocated routines in the device context buffer.  This function also provides information on the I/O port locations and memory mapped register locations that the device driver functions need access to, and device specific state information used by the device driver routines.

- Input:		AX	= 4F0Bh	Return VBE/AF Device Context Buffer
		BL	= 00h	Return Device Context Buffer length

- BL	= 01h	Initialize Device Context Buffer

- ES:DI  =	Real Mode address of DC Buffer

- Output:		AX	= 	Status
		CX	=	Length of  DC buffer (if BL = 00h)

- The format of the accelerator functions Device Context buffer is as follows:

- DevCtxBuffer struc

- MaxX			dw	?		; Maximum framebuffer X coordinate

- MaxY			dw	?		; Maximum framebuffer Y coordinate

- SetForeColor		dd	?		; Set the foreground color/mix

- SetBackColor		dd	?		; Set the background color/mix

- SetClipRect		dd	?		; Set hardware clipping rectangle

- DrawScan		dd	?		; Draw a single scanline

- DrawScanList		dd	?		; Draw a list of scanlines

- DrawRect		dd	?		; Draw a solid rectangle

- DrawLine		dd	?		; Draw a solid line

- DrawPattScan		dd	?		; Draw a patterned scanline

- BitBlt			dd	?		; Screen/Screen BitBlt

- TransBitBlt		dd	?		; Transparent Screen/Screen BitBlt

- MonoBitBlt		dd	?		; Monochrome expansion Scr/Scr BitBlt

- SetCursor		dd	?		; Download hardware cursor image

- SetCursorPos		dd	?		; Set hardware cursor position

- SetCursorColor	dd	?		; Set hardware cursor color

- ShowCursor		dd	?		; Show/Hide the hardware cursor

- WaitTillIdle		dd	?		; Waits till engine is finished

- EnableDirectAccess	dd	?		; Enable direct framebuffer access

- DisableDirectAccess	dd	?		; Disable direct framebuffer access

- BankSwitchCB	dd	?		; Pointer to bank switch callback

- VidMemSel		dw	?		; Selector to video memory area

- IOPortsOff		dw	?		; Offset of I/O port table in buffer

- IOMemoryOff		dw	?		; Offset of I/O memory table in buffer

- IOMemSel1		dw	?		; Selector to first I/O memory area

- IOMemSel2		dw	?		; Selector to second I/O memory area

- IOMemSel3		dw	?		; Selector to third I/O memory area

- IOMemSel4		dw	?		; Selector to fourth I/O memory area

- ; All info below is variable in length and pointed to by the vectors above

- DeviceStateInfo	db	? dup (?)	; Device specific state buffer

- IOPortsTable		dw	? dup (?)	; I/O port access table (WORDs)

- IOMemoryTable	dd	? dup (?)	; I/O memory access table (DWORDs)

- 32BitCodeBuffer	db	? dup (?)	; All 32 bit code resides in here also

- DevCtxBuffer ends

- The format of the IOPortsTable is:

- Port, Port, ... , Port, Terminate Port List with FFFFh.

- Example 1. For Port/Index combination 3D4/5h and 3CE/F the table would look like this:	03DEh,03DFh,03CEh,03CFh,FFFFh

- The format of the IOMemoryTable is:

- Location, Length, ... ,Location, Length, Terminate list with FFFFFFFFh.

- Example 1. For Memory locations B1000-B3FFFh (length = 2FFFh) the table would look like this:
	000B1000h, 00002FFFh, FFFFFFFFh

- The MaxX and MaxY fields are initialized by the VBE/AF to contains the maximum X and Y framebuffer coordinates.  All valid coordinate values range from 0 to MaxX and 0 to MaxY inclusive.  The clipping rectangle set by the SetClipRect function must fall within these values (SetClipRect will not check that the arguments are within range!).  The VBE/AF device will restrict the range of valid coordinates stored in MaxX and MaxY in order to protected special offscreen memory areas that it requires (such as hardware cursor storage areas etc.).  Although the hardware may be able to clip coordinates outside of these values, many VBE/AF devices have a restricted coordinate range, so user applications should never pass values to accelerator functions that are outside of these specified maximum values.

- Each of the accelerator entry points in the Device Context Buffer above is filled in initially by Function 0Bh subfunction 01h with offsets to the locations of each routine in the Device Context Buffer from the start of the buffer.  All of the 32 bit accelerator functions are copied into the device context buffer pointed to by ES:DI (which will be a real mode memory address, not protected mode!).  Once the application has called Function 0Bh sunfunction 01h to get the address of the Device Context Buffer, it must then copy the buffer from real mode memory into the applications own flat model memory space (which also copies all the accelerator functions at the same time).  The application must then modify the pointers to all of the routines to point to the full 32 bit linear address locations of the accelerated driver functions.

- Thus if the SetForeColor vector is set to 0000003Fh by the Function 0Bh, when the application has relocated the Device Context Buffer it should the replace the value of this vector with the offset of the buffer in application memory added with 3Fh. For instance in 32 bit C code:

- void InitDevCtxBuffer()

- {

- DevCtxBuffer *dc = CopyDevCtxFromRealMode();

- dc->SetForeColor = dc->SetForeColor + (uint)dc;

- dc->SetBackColor = dc->SetBackColor + (uint)dc;

- }

- Note that if any of the functions are not supported by the hardware (such as hardware clipping, hardware cursor and transparent BitBlt support), then the offsets to these routines will be set to 0.

- After relocation to flat model memory and fixups have been applied to the device driver function pointers, the application must then create protected mode selectors to any necessary I/O mapped register regions. The base address and length of the memory regions to be accessed are defined in the IOMemoryTable which is pointed to by IOMemoryOff and is located directly in the device context buffer.  The base addresses of these locations are specified as physical memory addresses, and may be located anywhere in physical memory (including well above the 1Mb memory mark).  These addresses are then used by the application to create selectors to the physical memory regions (using DPMI calls or whatever calls the environment provides), which then stores these selectors in the IOMemSel1, IOMemSel2 etc. variables in the device context buffer.  Currently there is room for up to 4 separate memory mapped register selectors to be stored in the device context buffer, which should be ample for all VBE/AF device drivers (if there is a perceived need for more we should increase this number, but the selectors _must_ be in a standard location so that the VBE/AF functions can get to them easily, and they must be initialized by the calling application). Note that not all four selectors need to be created, only those required by the device driver (which may be none or in most cases only one).

- The last thing that the VBE/AF application must do is to create a selector to the framebuffer memory area and store this selector in the VidMemSel field of the device context buffer.  If the framebuffer memory is banked through the normal 64Kb VBE 2.0 bank switching mechanism, the application must also store a pointer to a 32 bit bank switching callback function in the BankSwitchCB field, or store 0 in this field if the framebuffer is a 32 bit linear framebuffer.

- This bank switching callback may be called by the accelerator routines if it needs to directly access the video memory, and will be passed the new bank value in the EDX register, and should preserve the value of all registers across the call (including the EDX register). This may simply be pointed directly to the relocated 32 bit VBE 2.0 bank switching routine, or more likely to point at a special application specific bank switch routine, which may internally cache the value of the current bank for optimization purposes.

- Note:. All device driver accelerator functions end with a near RET (as opposed to FAR RET) to call the code after it has been relocated into the code space accessible by the default CS register (for most DOS extenders this simply means copying into the default data segment, as all data is executable).

- VBE/AF Impementors Note: The DeviceStateInfo area of the Device Context Buffer is provided for the VBE/AF implementation to retain state information about the current rendering context expected by the application. Information about the current foreground and background color's and mixes should be stored in here. The device driver may also wish to store information about the current video mode within this area such as the current pixel depth, resolution and logical scanline width.

- EDITORS NOTE: Actually, this brings up an issue that should be resolved. The device driver routines will probably need to know certain information about the state of the hardware such as the logical scanline width etc., which may be changed by some of the other VBE 2.0 functions. Caching this information in the device context buffer rather than requiring it to be obtained from the hardware will simplify the resulting code, but we have problems with the device driver getting out of sync with the hardware. We have two possible solutions to this:

- 1. If the application wishes to use a different logical scanline width, it should change the width _before_ call Function 0Bh to initialize the buffer. If the width is changed at some point in time, the buffer will need to be re-initialized.

- 2. We can provide a ReSync accelerator routine that will update the internal state of the DeviceContext buffer given the current hardware state.

- 4.14.	Accelerator Function - SetForeColor

- This required function sets the current foreground color and mix operation for subsequent accelerated rendering primitives.

- Input:		ES:EDI =	Device Context Buffer

- EAX =	New foreground color
		BL =	New foregound mix

- = 00h	REPLACE mode

- = 01h	XOR mode

- = 02h	OR mode

- = 03h	AND mode

- The format of the pixel color passed in EAX is dependent on the currently active video mode, and will be in the same format expected by the current framebuffer mode (check the DirectColorInfo in the ModeInfoBlock for the format).

- EDITORS NOTE: Should we set the format of the colors to a standard format, or should be set it to the format used by the framebuffer. How does most vendors hardware handle color setting in 15/16/24/32 bits per pixel modes? The SetForeColor routine could munge the data into the correct format if we want to standardize on the color formats passed, or the munging can be done by the application (which will need to do munging anyway if they intend to render directly into video memory).

- VBE/AF Impementors Note: The VBE/AF implementation should cache the value of the foreground color and mix in the DeviceStateInfo area of the DeviceContextBuffer so that it can be correctly reset back to the value set in the last call to this routine if another device driver routine needs to change the current hardware color or mix mode (for example during BitBlt operations).

- 4.15.	Accelerator Function - SetBackColor

- This required function sets the current background color and mix operation for subsequent accelerated rendering primitives.

- Input:		ES:EDI =	Device Context Buffer

- EAX =	New background color
		BL =	New background mix

- = 00h	REPLACE mode

- = 01h	XOR mode

- = 02h	OR mode

- = 03h	AND mode

- The format of the pixel color passed in EAX is dependent on the currently active video mode, and will be in the same format expected by the current framebuffer mode (check the DirectColorInfo in the ModeInfoBlock for the format).

- VBE/AF Impementors Note: The VBE/AF implementation should cache the value of the background color and mix in the DeviceStateInfo area of the DeviceContextBuffer so that it can be correctly reset back to the value set in the last call to this routine if another device driver routine needs to change the current hardware color or mix mode (for example during BitBlt operations).

- 4.16.	Accelerator Function - SetClipRect

- This optional function sets the current hardware clipping rectangle for subsequent accelerated rendering primitives. All subsequent accelerated output will then be clipped to this clipping rectangle.

- Input:		ES:EDI =	Device Context Buffer

- EAX =	Left clip coordinate
		EBX =	Top clip coordinate

- ECX =	Right clip coordinate

- EDX =	Bottom clip coordinate

- 4.17.	Accelerator Function - DrawScan

- This required function renders a solid scanline at the specified location in the currently active foreground color and mix.

- Input:		ES:EDI =	Device Context Buffer

- EAX =	X1 coordinate of scan
		EBX =	Y coordinate of scan

- ECX =	X2 coordinate of scan

- This routine will render a scanline from X1 to X2 at the specified Y coordinate, without rendering the pixel at the largest X coordinate. For example if X1 < X2, it renders pixels in the interval [X1,X2).  This routine will work correctly even if X2 < X1, swapping the values if required in either software or hardware. This function will result in no output if X1 == X2.

- VBE/AF Impementors Note: This function is required, even though the hardware may not directly support a scanline rendering primitive.  In this case this routine should render the scanline using the fastest possible method for that particular hardware device. For most accelerated hardware without scanline rendering, the fastest method of doing this is by rendering a single pixel high rectangle (rectangles are usually highly optimized).

- 4.18.	Accelerator Function - DrawScanList

- This required function renders a solid scanline at the specified location in the currently active foreground color and mix.

- Input:		ES:EDI =	Device Context Buffer

- EAX =	Y coordinate of first scan
		EBX =	Length of scanlist

- DS:ESI =	Pointer to scanline list X coords

- This routine will render a list of scanlines starting at the specified Y coordinate.  The X1 and X2 coordinates for each of the scanlines are passed in a contiguous array of WORDS's pointed to by DS:ESI, which contains 'length' coordinate pairs.  All scanlines are rendered without rendering the pixel at the largest X coordinate. For example if X1 < X2, it renders pixels in the interval [X1,X2).  This routine will work correctly even if X2 < X1, swapping the values if required in either software or hardware.  If a scanline in the list has X1 == X2, then this scanline will be skipped.

- VBE/AF Impementors Note: This function is required, even though the hardware may not directly support a scanline rendering primitive.  In this case this routine should render the scanline using the fastest possible method for that particular hardware device. For most accelerated hardware without scanline rendering, the fastest method of doing this is by rendering single pixel high rectangles (rectangles are usually highly optimized).

- 4.19.	Accelerator Function - DrawRect

- This required function renders a solid rectangle at the specified location in the currently active foreground color and mix.

- Input:		ES:EDI =	Device Context Buffer

- EAX =	Left rectangle coordinate
		EBX =	Top rectangle coordinate

- ECX =	Right rectangle coordinate

- EDX =	Bottom rectangle coordinate

- This routine will render a rectangle from (Left, Top) to (Right, Bottom), without rendering the pixel at the rightmost X coordinate or the scanline at the bottommost Y coordinate. Thus only pixels in the interval [Left, Right) and scanlines in the interval [Top, Bottom) will be rendered.  This function will result in no output if Left == Right or Top == Bottom, and is undefined if Right < Left or Bottom < Top.

- 4.20.	Accelerator Function - DrawLine

- This required function renders a solid line given the set of bresenham parameters in the currently active foreground color and mix.

- Input:		ES:EDI =	Device Context Buffer

- EAX =	??
		EBX =	??

- ECX =	??

- EDX =	??

- ESI =	??

- This routine renders a line given the set of bresenham parameters. The last pixel in the line always drawn by this routine.

- EDITORS NOTE: This is a sticky one. We need to define the input parameters to this function to be a generic set of bresenham parameters for rendering a line, rather than taking a set of (X,Y) coordinates. It is important that we can pass the bresenham parameters directly, so that lines can be rendering with endpoints that do not land exactly on a pixel boundary (such as when a line is clipped in software, or is passed through a 2D or 3D floating point transformation).

- I have not currently delved enough into the various hardware line drawing functions to determine a proper generic set of input parameters. Can someone with extensive knowledge in the field please supply a proposal for how this function should be defined?

- 4.21.	Accelerator Function - DrawPattScan

- This optional function renders a patterned scanline at the specified location in the currently active foreground color and mix and background color and mix.

- Input:		ES:EDI =	Device Context Buffer

- EAX =	X1 coordinate of scan
		EBX =	Y coordinate of scan

- ECX =	X2 coordinate of scan

- DL =	8 bit stipple pattern

- This routine will render a scanline from X1 to X2 at the specified Y coordinate, without rendering the pixel at the largest X coordinate. For example if X1 < X2, it renders pixels in the interval [X1,X2).  This routine will work correctly even if X2 < X1, swapping the values if required in either software or hardware. This function will result in no output if X1 == X2.

- This function also applies the specified 8 bit stipple pattern to the scanline, where the pattern is X coordinate aligned with the left edge of the display. Thus the bit in the stipple pattern that applies to a specific pixel in the scanline is determine by AND'ing the pixel's X coordinate with 8. Hence pixel 0 corresponds to bit 0, pixel 1 = bit 1, ... pixel 8 = bit 0 etc. Where a bit is 1 in the stipple pattern, the pixel is drawn and where it is 0, the pixel remains untouched.  It is the responsibility of the calling application to rotate the stipple pattern before calling this routine if it is desire that the pattern be aligned to a different starting coordinate (such as with Windows Bitmaps and setting the bitmap origin).

- VBE/AF Impementors Note: The width of the stipple pattern is set to 8 bits, which is the most common value supported by all hardware. If the hardware supports a 16 bit or 32 bit stipple pattern, it is the responsibility of this routine to replicate the bits in the stipple to either 16 or 32 bits in width.

- EDITORS NOTE: We could perhaps define the stipple prior to calling this routine, with a separate function such as SetForeColor, however generally when rendering most stippled primitives, the stipple changes for every scanline, so the current solution may be the best one.

- 4.22.	Accelerator Function - BitBlt

- This required function copies a rectangular region of video memory from one location to another.

- Input:		ES:EDI =	Device Context Buffer

- DS:ESI =	Pointer to BitBltParamBlock

- BL =	Mix operation with dest data

- = 00h	REPLACE mode

- = 01h	XOR mode

- = 02h	OR mode

- = 03h	AND mode

- The format of the BitBltParamBlock is as follows:

- BitBltParamBlock struc

- SrcLeft			dw	?	; Left coordinate of source rect

- SrcTop			dw	?	; Top coordinate of source rect

- SrcRight			dw	?	; Right coordinate of source rect

- SrcBottom			dw	?	; Bottom coordinate of source rect

- DstLeft			dw	?	; Left coordinate of destination  rect

- DstTop			dw	?	; Top coordinate of destination  rect

- IncX				db	?	; 1 to increment X, 0 to decrement

- IncY				db	?	; 1 to increment Y, 0 to decrement

- BitBltParamBlock ends

- This routine will copy a rectangle of video memory from (SrcLeft, SrcTop, SrcRight, SrcBottom) to (DstLeft, DstTop) without copying the pixel at the rightmost X coordinate or the scanline at the bottommost Y coordinate. Thus only pixels in the interval [SrcLeft, SrcRight) and scanlines in the interval [SrcTop, SrcBottom) will be copied.  This function will result in no output if SrcLeft == SrcRight or SrcTop == SrcBottom, and is undefined if SrcRight < SrcLeft or SrcBottom < SrcTop.

- This routine will also correctly handle cases of overlapping regions only if the application passes in the appropriate values for IncX and IncY. Please refer to the application notes at the end for details on how to compute the values of IncX and IncY to be passed to build a generic BitBlt routine. Overlap handling is not directly handled by this function as this simplifies the device driver code, and most high level API's will already have determined this information before the low level BitBlt code needs to be called.

- EDITORS NOTE: For the moment I have also passed in a desired mix code to be used when combining the source data with the destination data. However if this is complicated to implement on all hardware platforms (it shouldn't be) then we can simply re-define this function to always do a BitBlt in REPLACE mode.

- 4.23.	Accelerator Function - TransBitBlt

- This optional function copies a rectangular region of video memory from one location to another.  This function also takes a value for a color to be represented as transparent during the BitBlt operation.

- Input:		ES:EDI =	Device Context Buffer

- DS:ESI =	Pointer to BitBltParamBlock

- BL =	Mix operation with dest data

- = 00h	REPLACE mode

- = 01h	XOR mode

- = 02h	OR mode

- = 03h	AND mode

- ECX =	Transparent color value

- The format of the BitBltParamBlock is as follows:

- BitBltParamBlock struc

- SrcLeft			dw	?	; Left coordinate of source rect

- SrcTop			dw	?	; Top coordinate of source rect

- SrcRight			dw	?	; Right coortdinate of source rect

- SrcBottom			dw	?	; Bottom coordinate of source rect

- DstLeft			dw	?	; Left coordinate of destination  rect

- DstTop			dw	?	; Top coordinate of destination  rect

- IncX				db	?	; 1 to increment X, 0 to decrement

- IncY				db	?	; 1 to increment Y, 0 to decrement

- BitBltParamBlock ends

- This routine will copy a rectangle of video memory from (SrcLeft, SrcTop, SrcRight, SrcBottom) to (DstLeft, DstTop) without copying the pixel at the rightmost X coordinate or the scanline at the bottommost Y coordinate. Thus only pixels in the interval [SrcLeft, SrcRight) and scanlines in the interval [SrcTop, SrcBottom) will be copied.  This function will result in no output if SrcLeft == SrcRight or SrcTop == SrcBottom, and is undefined if SrcRight < SrcLeft or SrcBottom < SrcTop. Source pixels that are the same color as the passed in 'transparent' color value will not be copied, and the destination pixel will remain untouched. This is primarily aimed at fast arcade style game animation systems.

- This routine will also correctly handle cases of overlapping regions only if the application passes in the appropriate values for IncX and IncY. Please refer to the application notes at the end for details on how to compute the values of IncX and IncY to be passed to build a generic BitBlt routine. Overlap handling is not directly handled by this function as this simplifies the device driver code, and most high level API's will already have determined this information before the low level BitBlt code needs to be called.

- EDITORS NOTE: For the moment I have also passed in a desired mix code to be used when combining the source data with the destination data. However if this is complicated to implement on all hardware platforms (it shouldn't be) then we can simply re-define this function to always do a BitBlt in REPLACE mode.

- We also need to consider how the transparency is to be handled. It may be simpler and supportable on a wider variety of hardware if we simply state that pixels with a value of 0 in the source bitmap will be considered transparent, rather than an arbitrary color. It is very easy for the application to put the bitmaps in the correct format, and it is more important that the function be supported on as wide a variety of hardware as possible.

- Any input on this?

- 4.24.	Accelerator Function - MonoBitBlt

- This required function copies a rectangular region of video memory from one location to another, and performs monochrome expansion on the data from a single plane of source video memory.

- Input:		ES:EDI =	Device Context Buffer

- DS:ESI =	Pointer to BitBltParamBlock

- BL =	Mix operation with dest data

- = 00h	REPLACE mode

- = 01h	XOR mode

- = 02h	OR mode

- = 03h	AND mode

- BH =	Plane to read from (0-bitDepth).

- The format of the BitBltParamBlock is as follows:

- BitBltParamBlock struc

- SrcLeft			dw	?	; Left coordinate of source rect

- SrcTop			dw	?	; Top coordinate of source rect

- SrcRight			dw	?	; Right coortdinate of source rect

- SrcBottom			dw	?	; Bottom coordinate of source rect

- DstLeft			dw	?	; Left coordinate of destination  rect

- DstTop			dw	?	; Top coordinate of destination  rect

- IncX				db	1	; 1 for monochrome BitBlt

- IncY				db	1	; 1 for monochrome BitBlt

- BitBltParamBlock ends

- This routine will copy a rectangle of video memory from (SrcLeft, SrcTop, SrcRight, SrcBottom) to (DstLeft, DstTop) without copying the pixel at the rightmost X coordinate or the scanline at the bottommost Y coordinate. Thus only pixels in the interval [SrcLeft, SrcRight) and scanlines in the interval [SrcTop, SrcBottom) will be copied.  This function will result in no output if SrcLeft == SrcRight or SrcTop == SrcBottom, and is undefined if SrcRight < SrcLeft or SrcBottom < SrcTop. IncX and IncY

- It is up to the application program to store the data in the correct planes in video memory for performing the monochrome expansion BitBlt. Since there is no VBE/AF hardware mechanism for doing this, it is up to the application program to render the data directly to the framebuffer in the correct format the desired location in video memory.

- EDITORS NOTE: How do we define the plane to be used for the source of the monochrome BitBlt? Do we define it as being from 0 to the maximum bits per pixel depth (as it is now) or some other value? How is this defined in terms of bits in the pixels; does 0 = the MSB or the LSB of the bits in the pixel definition?)

- 4.25.	Accelerator Function - SetCursor

- This optional function downloads the specified cursor definition from the application into the hardware cursor. The cursor data is passed by the application in a format that is compatible with the Windows 3.1 cursor file format.

- Input:		ES:EDI =	Device Context Buffer

- DS =	Pointer to applications default data segment

- ESI =	Pointer to 32x32 Cursor Data

- This function is used to download the cursor data for the hardware cursor definition by the application program, which is passed to the VBE/AF by the application in the following format (compatible with the Windows 3.1 cursor file format):

- typedef struct {

- ulong       xorMask[32];

- ulong       andMask[32];

- } VBEAFcursor;

- 4.26.	Accelerator Function - SetCursorPos

- This optional function sets the location of the hardware cursor in display coordinates.

- Input:		ES:EDI =	Device Context Buffer

- EAX =	X coordinate of cursor

- EBX = 	Y coordinate of cursor

- CL = 	X cursor hotspot value

- CH =	Y cursor hotspot value

- This function takes the X and Y coordinates of the new cursor location, and also the X and Y coordinates of the hotspot value in the cursor definition image.  This function will place the cursor so that the hotspot of the cursor image is located at the specified (X,Y) location, and will correctly handle special cases where the cursor definition needs to be located off the top or left edges of the display screen (such as when X=0, Y=0 and HotX,HotY > 0).

- VBE/AF Impementors Note: It is up to the VBE/AF implementation to ensure that the cursor location is adjusted correctly so that it can display a cursor starting at a location that is off the top or left edges of the display. An application program will simply be passing mouse coordinate information and will not be able to compensate for this.

- 4.27.	Accelerator Function - SetCursorColor

- This optional function sets the current hardware cursor color.

- Input:		ES:EDI =	Device Context Buffer

- AL =	8 bit index, or Red component

- AH =	Green component

- BL =	Blue component

- In 8 bit color index modes, the color index to use for the cursor color is passed in the AL register. However in all other DirectColor modes (15/16/24 and 32 bits per pixel modes) the color values are passed in as individual 8 bit Red, Green and Blue components.

- 4.28.	Accelerator Function - ShowCursor

- This optional function unconditionally either show or hides the hardware cursor.

- Input:		ES:EDI =	Device Context Buffer

- AL =	1 to show, 0 to hide

- 4.29.	Accelerator Function - WaitTillIdle

- This required function pauses execution until the hardware accelerator has completed all currently queued rendering operations.

- Input:		ES:EDI =	Device Context Buffer

- The primary purpose of this function is to provide the application with the ability to ensure all rendering is complete, before either directly accessing the video memory, or when it needs to swap display pages when doing double buffering.

- 4.30.	Accelerator Function - EnableDirectAccess

- This optional function disables the accelerator and turns on direct framebuffer access.

- Input:		ES:EDI =	Device Context Buffer

- The primary purpose of this function is to correctly arbitrate video memory access between the accelerator and the users application. Some hardware devices need the state of the hardware to be changed in order to enable direct access to the framebuffer, so this function must be called directly accessing video memory.  A bit in the ModeAttributes field of the ModeInfoBlock tells the application whether this function must be used.

- 4.31.	Accelerator Function - DisableDirectAccess

- This optional function disables direct framebuffer access and turns on the hardware accelerator.

- Input:		ES:EDI =	Device Context Buffer

- The primary purpose of this function is to correctly arbitrate video memory access between the accelerator and the users application. Some hardware devices need the state of the hardware to be changed in order to enable direct access to the framebuffer, so this function must be called before calling any accelerator functions after calling EnableDirectAccess.  A bit in the ModeAttributes field of the ModeInfoBlock tells the application whether this function must be used.

- Appendix 1 - VBE/AF Quick Reference

- Needs to be completed when specs are ready for ratification.

- Appendix 2 - VBE/AF Data Structures

- Needs to be completed when specs are ready for ratification.

- Appendix 3 - VBE/AF Implementation Considerations

- To be completed as problems are isolated and resolved. This should try to document the considerations that a VBE/AF implementor will need to look at when doing an implementation for their hardware.

- A3.1.	Certification Requirements

- The VESA VBE/AF specification as well as the VBE 2.0 specification are subject to certification process before a product can be marked as being VBE/AF compliant. This needs to expanded with more details as time goes on, but it is imperative with VBE/AF that such a certification process exists so that applications written to use the VBE/AF will be assured of a solid foundation upon which to build high performance applications.

- A3.1.1.	VAFTest Utility

- A Test program will be developed which must be passed in order to claim your system is VBE/AF Compliant.

- A3.1.2.	VAFPerf Utility

- A performance profiling program will be developed which will be used to provide an official measure of the performance of a particular VBE/AF implementation. It will measure the performance of all VBE/AF functions and will provide users with a 'VBE/AF Rating' which can be used to compare and contrast the speed of different VBE/AF based video cards.

- Appendix 4 - Application Programming Considerations

- This section needs to be padded out with information about programming using the VBE/AF specifications, and sample code should be provided that shows how to do the following things:

- symbol 183 \f "Symbol" \s 10 \hµ§	Initialize and accelerated video mode.

- symbol 183 \f "Symbol" \s 10 \hµ§	Setup the device context buffer correctly for the device.

- symbol 183 \f "Symbol" \s 10 \hµ§	Fill a rectangle.

- symbol 183 \f "Symbol" \s 10 \hµ§	Draw a line.

- symbol 183 \f "Symbol" \s 10 \hµ§	Perform a screen to screen BitBlt.

- symbol 183 \f "Symbol" \s 10 \hµ§	Perform a screen to screen transparent BitBlt.

- symbol 183 \f "Symbol" \s 10 \hµ§	Perform a screen to screen expansion BitBlt.

- symbol 183 \f "Symbol" \s 10 \hµ§	Correctly arbitrate accelerated rendering and software rendering.

- Appendix 5 - Related Documents

- symbol 183 \f "Symbol" \s 10 \hµ§	VGA Reference Manual(s)

- symbol 183 \f "Symbol" \s 10 \hµ§	GUI Accelerator controller data sheets

- symbol 183 \f "Symbol" \s 10 \hµ§	VBE Core Functions Version 2.0 Standard

- Appendix 5 - Questions & Answers

- Windows drivers already provide acceleration, why is another specification needed?

- The VBE/AF specification is not intended to replace high performance, chip-specific accelerated device drivers for operating systems like Windows or OS/2, but to provide the operating systems and application developers the ability to build a generic, plug & play, accelerated driver that will work across a wide variety of hardware devices. It is also intended to provide entertainment software developers on the PC the ability to write a single accelerated driver that works on just about any chip, raising the baseline of support for computer games on the PC from simple framebuffer access to high performance accelerated access.

- Isn’t a BIOS slow? Why is this a BIOS specification?

- The fact that VBE/AF can be implemented in a ROM BIOS allows for true “plug and play” operation. When VBE/AF is implemented in ROM, the user will not have to load any additional drivers in order to take advantage of the acceleration features of their graphics card. For reasons of space and speed efficiency, the VBE/AF accelerator functions are only provided through a set of high performance, relocateable 32 bit protected mode routines. All 32 bit protected mode routines can be relocated directly into the applications own code space, therefore execute at full speed. A program only has to access the ROM once to get the actual routines.

- We are already out of ROM space, how will we fit VBE/AF into ROM?

- The VESA Software Standards Committee (SSC) is working on a simple proposal to expand available video ROM space, at very little cost and with full compatibility with the PC architecture. There are also proposals to remove several legacy features and mode tables from the video ROM that are seldom, if ever, used and to put that legacy functionality into a TSR. Due to the fact that VBE/AI only provides access to hardware, it will require a relatively small amount of ROM space.

- Is this specification only useful for DOS applications?

- No. This specification defines how to access acceleration features at the very lowest level of abstraction. As such, any software that runs on the x86 platform can conceivably use this information. This includes DOS applications, games and “plug and play” operating system drivers for environments such as Windows, OS/2, Unix and others. This specification can be used by any application or operating system that needs access to high performance accelerator features. It will be especially useful for entertainment, virtual reality and other applications that exert high demands on the graphics subsystem and thus need direct access to accelerator functions.

- Is this just another software layer to slow things down?

- No. VBE/AF was designed to do three things: 1.) Let the application software or operating system know what features are available in hardware. 2.) Provide the code to enable those features and 3.) Get out of the way and let the software write directly to the hardware. This is contrasted with a higher level API that does many complex functions in software. With VBE/AF, a programmer could choose to use one of those higher level APIs that would in turn call VBE/AF for device support, or they can write directly to the VBE/AF specification using their own proprietary routines and libraries.

- Why have you implemented only a small set of  functions?

- The VBE/AF functions have been boiled down into the smallest possible set of accelerator functions that can be implemented as efficiently as possible and are the most commonly needed by application software. When an application calls the VBE/AF device driver routines, it will already have performed any kind of application specific special casing necessary. The VBE/AF functions are also broken down into functions that change the current state of the accelerator hardware, and those that actually render primitives. Thus if you need to draw 1000 rectangles or lines of the same color, you only need to change the color and mix once, and then blast out the 1000 rectangle or lines as efficiently as possible.

- Will any additional hardware be necessary to support VBE