IEEE-1284 Parallel Port Device Detection code. Made in 1998 by Craig Hart.

Note: This code is released as freeware; use/modify it as you will. No

warranties of any kind are made or implied.

This code is written for Turbo Pascal (should work in almost any version;

tested on TP 7). It detects and displays the information from Plug and Play

(IEEE-1284) compliant Parallel Port devices. Up until now, learning to program

IEEE-1284 has been almost imposible unless you spent money to purchase the

standard. I hope to change all that with this code, which is easily read (all

in one file) and commented for clarity.

This code is based on the IEEE-1284 routines in Linux, originally written by

Phil Blundell, Carsten Gross and Jose Renau. Please look at the Linux source

code if you wish to check my sources. The Pascal implementation is entirely my

own work, and is, IMHO, much clearer to understand since it's not scattered

across multiple source files, nor full of cryptic C style logical operators.

This routine (IMHO) does _exactly_ what Windows 95 does when detecting PnP

Parallel devices. You do NOT need an EPP or ECP printer port for this code to

function. Any old printer port will do. Obviously, you must have an IEEE-1284

printer or other device attached! This code was tested with a HP Laserjet 5L

printer, however most current HP printers as well as many printers (from other

vendors) designed with Windows 95 in mind also respond to this program.

This routine is currently hard coded to I/O port 378, which is LPT1 is the

majority of computers. Since full source code is released, it is left as an

exercise to the reader to re-write it for other ports, if required.

*important* This code does NOT work under Windows95, nor most other forms of

Multitasking. This is because the timing loops are very critical - a difference

of even 10 milliseconds is enough to ruin your day :-). This code works well

with Win95 booted to safe mode, command prompt only (but not under restart

computer in DOS mode!!); and any version of DOS, both with and without memory

managers. If you added operating system specific calls to give the code 100%

CPU time this code would then probably work. Another exercise for the reader.

You may contact me by Email if you wish to discuss this program or any other

hardware programming issue (yes, I'm a hardware nut!): chart@hyperlink.net.au

or my internet website: http://hyperlink.net.au/~chart

I am also active in Fidonet DR_DEBUG, HDCONF, PASCAL, 80xxx and other technical

and programming echo's, and may be mailed there.

Craig Hart

02-02-98