The Mxxxx assemblers provided support for the Intel Assembly Language.

A flag is provided ( -I ) to switch the assembler into Intel compatibility mode.

If the assembler enconters the $MACROFILE directive the switch to Intel compatibility mode will be done automatically.

The DEBUG option will perform the same function as the Intel assembler in that the Symbols will

be palced at the head of the .hex output file, separated from the Hex code by a '$' on a new line.

In intel compatibility mode

the '$' prefix on directives is recognised.

the non C-type operators are recognised

Intel	C	Meaning

MOD	%	Modulus

SHL	<<	Left shift

SHR	>>	Right Shift

EQ	==	Equal

LT	<	Less than

LE	<=	Less than or equal

GT	>	Greater than

GE	>=	Greter than or equal

NE	!=	Not equal

AND	&	Bit AND

OR	|	Bit OR

XOR	^	Bit Exlusive-OR

Macro parameters need a '@' charcter within the macro definition to itentify the parameter name

Intel Macro

MAC1	MACRO	PARAM1

IF PARAM1 EQ 0

ORG 100H

ELSE

ORG PARAM1

ENDIF

ENDM

Mxxxx Macro

MAC1	MACRO	PARAM1

IF @PARAM1 EQ 0

ORG 100H

ELSE

ORG @PARAM1

ENDIF

ENDM

The Intel Assembler allows the use of EQU's for Ports aswell as setting constants

the Mxxxx assembers do not permit this, if Port equates have been used eg.

OUTPORT	EQU	P2

then the textual replacement DEFINE directive may be used

DEFINE	OUTPORT	P2

this also may be used for register EQU's although it should not be used as

a replacement for EQU's.

DSEG and CSEG are predefined directives and therefore if macros of the same

name have been defined, a "too many ENDM statements" error will be displayed.

a modified version of RSEG.MAC is availiable.

ORIGIN.MAC should work without any modification.

RSEG.MAC

;; Default DATE & CODE segment setup and support

;; for PAGE3 switching

DSEG

ORG 20H		;initial condition start data above registers

SEG PAGE3

ORG 300H	;initial condition start at beginning of page 3

P3ADDR	SET $

CSEG

ORG 0	   	;start code at location 0

P3SEG	MACRO

SEG PAGE3	;creat new segment PAGE3

ORG P3ADDR

ENDM