+-----------------------------------------+

|                                         |

|  *************************************  |

|  * The IBM-PC COMMUNICATIONS library *  |

|  *************************************  |

|                                         |

+-----------------------------------------+

An interrupt driven serial I/O lobrary

for Stand-Alone 8086 applicatons

using MICRO-C.

Copyright 1990-1994 Dave Dunfield

All rights reserved.

DK86 COMMUNICATIONS LIBRARY                                      Page: 1

1. IBM-PC COMMUNICATIONS Library

The  MICRO-C  WINDOWING  LIBRARY  is  a  set  of  functions  which

implement serial I/O with an 8250 type UART device, such as  is  used

on most PC serial cards.

The routines support  interrupt  driven  reception,  and  XON/XOFF

software flow control, as well as setting and monitoring  the  RS-232

control signals.

To install  these  function  into  the  8086  library,  place  the

COMM.ASM file in the library subdirectory  (usually  \MC\LIB86),  and

issue the following commands:

slib i=tiny  a=comm

slib i=small a=comm

1.1 Function Definitions

The  following  functions   are   available   in   the   IBM-PC

communications library:

CCLOSE                                                           CCLOSE

PROTOTYPE:

Cclose()

ARGUMENTS:

None

RETURN VALUE:

None

DESCRIPTION:

This function closes the communications port previously  opened

using "Copen". The system interrupt vectors and all other hooks to

the comm port are restored.

This function must be called before any program  using  "Copen"

terminates, otherwise the communications port will be left  in  an

indeterminate state.  If  this  is  not  done,  there  will  be  a

possibility of system crash if an interrupt is received  from  the

port after the program has terminated.

EXAMPLES:

Cclose();       /* Close comm port */

exit(0);        /* And terminate */

CGETC                                                             CGETC

PROTOTYPE:

int Cgetc()

ARGUMENTS:

None

RETURN VALUE:

Character read from the communications port

DESCRIPTION:

This function reads a single character from the  communications

port previously opened with "Copen". If no character is available,

"Cgetc" will wait for one.

EXAMPLES:

/* Get a string from the comm port */

while((c = cgetc()) != '\r')

*ptr++ = c;

*ptr = 0;

COPEN                                                             COPEN

PROTOTYPE:

int Copen(int port, int speed, int mode, int modem)

ARGUMENTS:

port    - Comm port to use (1, 2, 3 or 4)

speed   - Baudrate divisor to set

mode    - Communications parameters to set

modem   - Modem control lines to set

RETURN VALUE:

0       - Successful open

!0      - Requested comm port is not available

DESCRIPTION:

The "Copen" function opens a serial communications port on  the

IBM PC for  access.  An  independant  interrupt  handler  and  I/O

drivers  are  installed,  which  allow  high  speed  full   duplex

operation of the serial port with optional XON/XOFF  flow  control

of both receive and transmit streams.

Only one serial port may be accessed  at  a  time  using  these

functions,  If  "Copen"  is  called  more  than  once,   it   will

automatically close the last port before opening the new one.

The meaning of the "speed", "mode", and "modem"  parameters  if

documented in the "comm.h" header file.

An external "char" variable "Cflags" may be accessed to  enable

or disable transparency of the serial channel. When  "transparent"

is selected, XON/XOFF flow control is disabled, and  all  data  is

sent and received with no changes. When operating  in  this  mode,

you must insure that "Cgetc" is called frequently enough that  the

256 byte internal receive buffer will not overflow.

Since "Cflags" is used by the  interrupt  handler,  you  should

disable and enable interrupts around any accesses to it.

EXAMPLES:

#include comm.h     /* Get comm port defintions */

extern char Cflags;

/*

* Program to read & echo characters on the serial port

* in transparent mode. (Until ESCAPE char is received)

*/

main()

{

char c;

if(Copen(1, _2400, PAR_NO|DATA_8|STOP_1, SET_RTS|SET_DTR))

abort("Cannot access COM1");

disable();                  /* Disable interrupts */

Cflags |= TRANSPARENT;      /* Set transparency */

enable();                   /* Re-enable interrupts */

while((c = Cgetc()) != 0x1B)    /* Do until ESCAPE */

Cputc(c);

Cclose();                   /* Close the serial port */

}

CPUTC                                                             CPUTC

PROTOTYPE:

Cputc(char c)

ARGUMENTS:

c       - Character to write to communciation port

RETURN VALUE:

None

DESCRIPTION:

The  "Cputc"  function  writes  the  given  character  to   the

communcinations port previously opened by "Copen".

EXAMPLES:

while(*ptr)         /* Write a string to comm port */

Cputc(*ptr++);

CSIGNALS                                                       CSIGNALS

PROTOTYPE:

int Csignals()

ARGUMENTS:

None

RETURN VALUE:

The modem input signals read from the open comm port

DESCRIPTION:

This function reads the modem input signals (DSR, CD,  RI  etc)

from the serial communication port previously opened  by  "Copen",

and returns them as an integer value.

The meaning of the individual bits in  the  value  returned  by

"Csignals" is documented in the "comm.h" header file.

EXAMPLES:

if(!(Csignals() & DSR)) {

Cclose();

abort("Modem not ready"); }

CTESTC                                                           CTESTC

PROTOTYPE:

int Ctestc()

ARGUMENTS:

None

RETURN VALUE:

0-255   - Character read from comm port

-1      - No character available

DESCRIPTION:

This function tests for a  character  from  the  communications

port previously opened with "Copen", and returns that character if

one if found. If no character is available, "Ctestc" returns -1.

EXAMPLES:

if((c = Ctestc()) == -1) {

Cclose();

abort("No character available"); }

DK86 COMMUNICATIONS LIBRARY

TABLE OF CONTENTS

Page

1. IBM-PC COMMUNICATIONS Library                                       1

1.1 Function Definitions                                            1