This peace of a software was written after purchasing the HP7221C plotter

at Computer Swap Meeting as a bargin, when I discovered that it doesn't

understand the single 'H' of HPGL (this is why it was so cheap).

Anyway, after some additional effort, the end result is this software,

HPGL to Compact Binary Code Converter. It is written for IBM DOS compatible,

and was tested on my 16MHz SX, as well as 66MHz 486, with no problems

observed.

To use the software, you must have your plotter connected to com1 port,

set to LOCAL mode (if you forget about this, you will have to exit the

software by hitting ^Q and start the procedure from beginning).

Plotter rear panel settings should be set to Baud Rate 2400, parity OFF,

modem ON, duplex HALF.

To run the software, simply type PLOT, with optional file name.

If file name is not supplied, by pressing the <RET> key, menu with directory

will appear. You can navigate through menus by using UP, DOWN, PageUP,

PageDOWN keys and select the file or subdirectory by pressing <RET>.

After selecting the file to convert, you will have to answer to the question,

where the plot origin is (Centered, or at Lower left corner).

If your plotter was properly set, Plotter ID will be displayed, and plotting

will begin.

Programme reads the plot file and translate the HPGL commands into

format the HP7221 understands.

Most of the HPGL commands are supported: PU, PD, MA, MR, CI, AA, AR,

Labeling, etc.

However, it is good idea to use only basic HPGL commands (most CAD packages

can be set to use lines only instead of circles arcs and labels).

This sofware is provided AS IS. No responsibility for any kind of damage,

resulting from usage of this sofware can be accepted.

For your convinience, source file is also provided. If you think that

you can improve the programme, you are most welcome to do so.

In this case, I would appriciate, if you send me the improved source file.

If you have a problem, try to e_mail me at:

<bojan@icpdd.neca.nec.com.au>,  or: <bstajc@ozemail.com.au>

Have fun!

Bojan Stajcar

1/1 Halliday Street, 3149 Mt.Waverley, VIC

Australia