PROM RESIDENT ICB COMMUNICATION PROTOCOL

Goals

This protocol is an effort at providing a basic method for interprocessor

communication using the Arete ICB (Interprocessor Communication Bus). The

package includes mechanisms for command passing, board identification,

inter board monitor manipulation, booting the system from many sources

(modem, disk, tape, etc.), and support for diagnostics.

Mechanism

The Arete's architecture allows intelligent boards to communicate over a

proprietary bus, the ICB.  Any one board in the card cage can correspond

with any other via the master CPU which has special software to forward

message packets. The packets are transceived a byte at a time through the

ICB status registers with interrupt driven handshake (boards with no dual

port memory) or the packets may be written into the free ICB dual port

memory space.

Packet Command Format

Below is a graphic representation of the ICB command format.

|___|___|___|___|________________|_______|__________________|

|   |   |   |         |            |             |

|   |   |   |         |            |             |

|   |   |   |         |            |             data (variable)

|   |   |   |         |            data count (word)

|   |   |   |         information (long word)

|   |   |   source (byte)

|   |   destination (byte)

|   status (byte)

command (byte)

command - (see below)

status - (see below)

source - slot number of the board initiating the ICB command (0 - 1fh)

destination - slot number (also 0 - 1fh)

information - multi purpose (i.e. address, count, device #, etc.)

data count - number of bytes in the following data field

Packet ICB Commands

Upon power up or reset an iop should perform some self diagnostics, build it's

board information structure including self test status (this structure is

explained in the file "vreg.h"), and then poll the ICB for an interrupt from

the ICB master. The iop reads it's shifted slot address from the interrupt data

register resetting the interrupt (this address is used to place 'floating' dual

port ram on boards so equipped). At this time the ICB master will update

necessary fields in the vreg structure (icb int level, dtb id, etc.) followed

by a IVRUPDT (virtual register update) single byte command.  The iop signals

it's readiness to communicate over the ICB by acknowledging the IVRUPDT command.Below is a list of ICB packet commands and responses:

ISTSCMD

01  Self test status command is requested by the master upon completion of

prom resident power-on/reset self test. The data field is to contain the

board descriptor structure complete with self test status. The master

responds by echoing the packet back to the iop after updating fields in

the board descriptor (dtbid, xdtbid, etc.). Only boards without ICB

dual port ram need support this command.

IPUTCMD

02  This command includes a character in the information feild of the

header to be displayed on the console device. An acknowledge with

valid status is returned to the master upon completion.

IGETCMD

03  Read console device command will return a status plus a character

from it's [console keyboard] input queue, echoing the character back

to the console.

ILEDCMD

04  Instructs the destination to write it's status led's. A zero in

the info field is yellow, 1 green, 2 red, 3 no lamp. No acknowledge

is returned to the master.

ICONFCMD

05  Commands a board to send it's configuration (options) to the master.

Only boards without ICB dual port ram need support this command.

IWMEMCMD

06  The write memory command contains an address (information field)

followed by the code to be written in the data field. A status should be

returned.

IRMEMCMD

07  The read memory command contains an address (information field) and

a length (long word) in the data field. Respond by returning command,

valid status, and contents of the requested memory.

IJMPCMD

08  Is the jump (start execution at given address) command. The response

must be coded into the downloaded program.

IGTASCMD

09  Is the test and set instruction for the testing of the TAS bit in

the icb status register.  When the iop receives this instruction it

must read the TAS register, mask out all but the TAS bit and return the

remainder to the master in the information feild.

Note that the slave cpu has no TAS bit and therefore connot execute

this command.

ICTASCMD

0a  Is the clear TAS instruction.  The packet is returned to the

master when completed with status updated.

ICONCMD

0b  This command followed by an ascii string should be processed as

if the string were entered at the board's console device with the

possible output redirected over the ICB back to the master. Status

must be returned to the master when complete.

IWJCMD

0c  Write memory and jump command contains a destination address for the

code that follows. The jump address is derived from the code. The response

must be coded into the downloaded program.

Packet ICB Status

The status field in the ICB command structure should be initialized zero by

the master and updated when returned. Below are the valid status

ICMPLT

01   Task complete.

IBDEST

02   Illegal destination slot number.

IILLEG

03   Command not supported.

IPARAM

04   Parameter out of range.

IABRTD

05   Command aborted.

ICBBUSY

06   ICB is busy.

Single Byte ICB Commands

To avoid unnecessary processing overhead the following single byte ICB commands

have been created. The commands are passed through the ICB interrupt register.

A processor receiving a command must acknowledge by echoing the command back.

IMEMCMD

81  Informs the processor of a memory resident ICB command packet. The

bd_imcptr field in the virtual register structure points to the start

of the packet.

IDWNLD

82  Commands an iop to go into download mode.

IABORT

83  Abort current ICB command.

IVRUPDT

84  When the ICB master alters certain iop virtual register fields the i/o

processor must be informed of the update. The vreg fields are: bd_icbint,

bd_dtbid, and bd_xdtbid.

IDEVREQ

85  The reqfst field of the virtual register structure points to a

(struct) devq request.