.nf

.sp 4

.ce 3

GED: An Interactive Solid Modeling System

For

Vulnerability Assessments

.sp

.AU

Michael J. Muuss

Keith A. Applin

J. Robert Suckling

Gary S. Moss

Earl P. Weaver

Charles A. Stanley

.AI

US Army Ballistic Research Laboratory, ARRADCOM

Aberdeen Proving Ground

Maryland

21005

.AB

For many years the production and modification of target descriptions,

no matter what method used, has been a slow, labor-intensive procedure.

In an effort to improve the response time of target descriptions, the Ballistic

Research Laboratory (BRL) has developed an interactive solid modeling system.

This system is called the Graphics Editor (GED) and is designed to replace the

traditional manual method of production and modification of target descriptions.

GED presently applies to the combinatorial geometry (COMGEOM) solid

modeling technique.

Using GED, the target descriptions

are interactively viewed, modified, and constructed with immediate visual

feedback at each step.  The GED system virtually eliminates explicit numerical input

and opens a new dimension in the target description process.

.AE

.sp

.PP

.bp

.ce

I.  INTRODUCTION

.nh

.PP

As the Army's lead laboratory for vulnerability technology, the

Ballistic Research Laboratory (BRL) constantly performs vulnerability

analyses for a wide variety of military systems.  Vital to these

vulnerability studies are three dimensional computer models of the

physical characteristics of these systems.  These computer

models are generally called target descriptions.

Since the mid 1960's, BRL has used a solid modeling technique

called combinatorial geometry (COMGEOM) to model targets.

The COMGEOM technique uses logic operations to combine basic geometric

shapes or primitives to produce the three-dimensional target description.

The COMGEOM target descriptions are processed by

the Geometric Information

For Targets (GIFT)\u1,2\d

.FS

\u1\d Lawrence W. Bain, Mathew J. Reisinger, "The GIFT Code User Manual; Vol I,

Introduction and Input Requirements," BRL Report No. 1802, July 1975. (AD# B0060371)

.FE

.FS

\u2\d Gary G. Kuehl, Lawrence W. Bain, Mathew J. Reisinger, "The GIFT Code User

Manual; Vol II, the Output Options," ARRADCOM Technical Report No. ARBRL-TR-02189,

September 1979. (AD# A078364)

.FE

code for use in follow-on vulnerability

assessment codes.

.PP

Target descriptions are basically large collections of numerical data which traditionally

have been handled manually in a batch environment.  Hence, the

production and modification of target descriptions has been a slow, labor-intensive

process.  In 1980, BRL initiated an effort to improve the response

time of the target description process by applying interactive

computer graphics techniques.  As a result of this work unit, BRL has

developed the Graphics EDitor (GED), an interactive solid modeling

system based on the COMGEOM technique.

Using GED an analyst can view, build, and modify target descriptions

interactively by manipulating the graphical representation, thus

providing immediate visual feedback on a vector display device.

The GED system is designed to replace the manual method of production

and modification of target descriptions.

.PP

This report is intended to serve as  a user manual

for the GED system.

The process of viewing and editing a description using GED

is covered in detail.  The internal data structure is also covered, as

it is an important part in the overall design of the GED system.

.sp 2

.ce

II.  COMGEOM BACKGROUND

.PP

Since the GED system is presently based on the COMGEOM solid modeling

technique, a brief overview of the COMGEOM technique is required

to effectively use GED.

For more detailed information on the COMGEOM technique see references 1 and 2.

.PP

The COMGEOM technique utilizes two basic entities - a solid and a region.

A solid is defined as one of fifteen basic geometric shapes or

primitives.  Table I contains a list of these fifteen

solids.  The individual parameters of each solid define the solid's

location, size, and orientation.  A region is a combination of

one or more solids and is defined as the volume occupied

by the resulting combination of solids.

Solids are combined into regions using any of three logic

operations: union(OR), intersection(+), or difference(-).

The union of two solids is defined as the volume in either

of the solids.

The difference of two solids is defined as the volume of the first

solid minus the volume of the second solid.

The intersection of two solids is defined as the volume

common to both solids.

Figure 1 presents a graphical representation of these operations.

Any number of solids may be combined to produce a region.

As far as the COMGEOM technique is concerned, only a region can

represent a component of the target.

The solids are only building blocks to be combined into regions.

Since regions represent the components of the target system, they

are further identified by code numbers.

These code numbers either identify the region as a target component (nonzero item code)

or as air (nonzero air code).

Any volume not defined as a region is assumed to be "universal air" and

is given an air code of "01".

If it is necessary to distinguish between universal "01" air and any

other kind of air, then that volume must be defined as a region

and given an air code other than "01".

Normally, regions cannot occupy the same volume (overlap), but regions identified with

air codes can overlap with any region identified as a component

(i.e. one that has a nonzero item code).  Regions identified with different air codes

however, can not overlap.

.sp 2

.ce

III.   COMPUTER GRAPHICS

.PP

Computer graphics is one of the fastest growing fields in the computer industry.

Computer graphics has applications in many diverse areas, from electronic

games to medicine, from cartoons to the space industry.  Just

what is interactive computer graphics and why is it so versatile?

Human visual perception is quite keen and communications with a

computer are generally faster and easier with images or displays rather than with

numbers. Furthermore, by driving the display image with the computer,

one can actually communicate with the computer through the display itself.

This technology is called interactive computer graphics.  The

user converses with the computer through the display using

devices such as light pens, data tablets, function switches, and

control dials.  The response of the computer is immediately reflected

on the display.  Thus a fast channel of communication is available between

person and machine.

.sp 2

.ce

IV.  APPROACH TO THE PROBLEM

.PP

To speed up the model description process, a faster more efficient method of manipulating

the large masses of data involved in a target description is needed.

Interactive computer graphics seems to provide the capability of

handling such data in an efficient, relatively fast manner.

The key to the whole situation is SOFTWARE - the missing link

between the target description problem and a possible solution.

.PP

In view of this fact, existing software graphics

packages were evaluated with respect to their utility in the target description process.

Quite an exhaustive search of commercially available systems

was conducted and none were found which completely satisfied our requirements.

A study was then initiated to examine the feasibility of producing

the required graphics software in-house.

The result of this study was a preliminary version of GED which,

using existing hardware,

quite convincingly demonstrated the

feasibility of such an undertaking\u3\d.

.FS

\u3\d Michael John Muuss, Earl P. Weaver, "Interactive Construction

of COMGEOM Targets - A Feasibility Study", report to be published.

.FE

It was then decided to develop GED into a full production code

and to acquire the hardware necessary for full implementation

of the system.

The production level GED code has been used since January 1982 to

build target descriptions interactively at BRL.

.sp 2

.ce

V.  GED OVERVIEW

.PP

The main function of the GED system is to edit target descriptions

with immediate visual feedback.  Hence, GED produces images both suitable

for editing and capable of being displayed in real time.

When an object is displayed, the solids combined to model the

object are drawn.

No attempt is made to produce edge representations of regions or

to remove hidden lines.  At the present, such an undertaking would

take away from the "real time" displays now enjoyed.

.PP

The GED is a modular code written in the C programming language

which presently runs

under the BRL UNIX Networking System\u4\d.

.FS

\u4\d Michael John Muuss, "BRLNET - The Implementation of a Multi-processor

UNIX System", a final report of work done under ARO Project Number 1127,

August 27, 1980.

.FE

The workstations presently used are Vector General (VG) 3DI vector refresh systems

driven by DEC PDP-11/34 or PDP-11/70 computers.

Peripheral devices include a joystick, a pen and data tablet, function

switches, and control dials.  These peripheral devices and functions

of each are discussed in later sections.

.PP

Overall control of GED is maintained via the keyboard.  The keyboard

commands perform general housekeeping functions and allow one

to control what is displayed, create/modify combinations, list

solid parameters, etc.

The keyboard commands are discussed in a later section.

.sp 2

.ce

VI.  GED INTERNAL DATA STRUCTURE

.PP

One of the critical aspects of a graphics software package is its internal data structure.

Since the target description problem deals with such large volumes of data,

the importance of the data structure here is emphasized.

Thus it is felt that an introduction to the internal data structure of GED is

important for any potential user.

.PP

The GED internal data structure is hierarchical in nature with

each node or position in the hierarchy being occupied by an OBJECT.

An OBJECT is the GED basic data unit and is defined as either a SOLID

or a COMBINATION.  A SOLID is one of the generalized COMGEOM primitive

types while a COMBINATION is a group of objects.  Each member object

of a combination has a transformation associated with it.

Any object not at the top of an hierarchy is referenced by (is a member of)

a higher combination and each such reference has an associated transformation.  The

bottom object of every hierarchy path is a solid.  This hierarchical data structure

allows actual subsystems of a target to be grouped together and edited

as a unit without worrying about individual parts.

To display any object, each path in the object's hierarchy is traveled

to its bottom position where a solid is located.

The transformations are accumulated for each path traversed and then

applied to the solid at the bottom of the path.

The solid is then drawn.

The hierarchy is traversed recursively until all objects are drawn.

Let's use a

high explosive projectile as an example.  Figure 2 depicts the hierarchy involved.

If one were to display the combination "heround", six solids would

be drawn on the screen.  Table II lists each solid name, the

transformation applied to the solid's parameters, and the path used to

reach the solid.

.PP

One very useful type of combination is the INSTANCE.  An

instance is a referral to an object (solid or combination)  without

the object itself being duplicated.

To produce an instance of an object, a combination is created with the

object being instanced as a member object.  As with any other combination, this

member has an associated transformation, hence it can be edited as a unit.

The instance is most useful when

an object appears many times in a description.  The desired number of instances

are made and positioned in the description.  Any future changes in the

object require that only the original (prototype) be changed.

The modifications of the prototype object will

then be automatically reflected in all the instances of that object.

.sp 2

.ce

VII.  KEYBOARD COMMANDS

.PP

To execute GED, one simply types "ged file", where "file" is the data

file to be edited.

The GED keyboard commands are used to maintain overall control of the

system and to perform general housekeeping functions.

Each command is invoked by entering a single character followed by

other parameters (if necessary) separated by spaces.

In most cases, the response on the screen will be the word "done" once

the command is completed.

This response is not printed for the commands where the results of the

command are readily visible on the screen.

.PP

In the following pages, the GED keyboard commands will be presented

and discussed.  Examples will be given in appropriate cases.

Table III, at the end of this section, presents a summary of the GED

keyboard commands and the function of each.

.bp

.ce

------------------------------------------------------------------------

.sp 3

.ce

c oldsolid newsolid

.sp 2

This command is used to produce and display a copy of a solid.  In this case, the

solid "oldsolid" will be copied into a solid called "newsolid".  A new identical

solid record is added to the data file.

The solid parameters are copied as they appear

in the solid record.

.sp 2

Examples:     c arb8 hullbot.s

c arb8 turrettop.s

c tgc wheelrim.s

c tor tire1.s

.sp 3

.ce

------------------------------------------------------------------------

.sp 3

.ce

n old new

.sp 2

This command is used to rename objects in the data file.  In this

case, the object "old" will be renamed "new".

A note of caution:  the name is changed only in the object record itself, not

in any member records.  Thus if the object "old" appears as a member

of any other object, the name will not be changed there.

.sp 2

Examples:     n test hull

n g00 air

n g1 turret

.sp 3

.ce

------------------------------------------------------------------------

.sp 3

.ce

Z

.sp 2

This command clears (Zaps) the screen of all displayed objects.

.sp 3

.ce

------------------------------------------------------------------------

.bp

.ce

-----------------------------------------------------------------------

.sp 3

.ce

g group obj1 obj2 ..... objn

.sp 2

This command creates or appends to a combination record and

is used to group objects together either for editing or displaying

purposes.  In this case, "obj1" through "objn" are added as members

to the combination "group".  If "group" does not exist, it is

created and "obj1" through "objn" are added as members.

.sp 2

Examples:   g shell hull turret

g tank wheels engine crew shell

g tank track

.sp 3

.ce

------------------------------------------------------------------------

.sp 3

.ce

r region op1 sol1 op2 sol2 .... opn soln

.sp 2

This command is used to create regions or append to regions.

If "region" exists, then solids "sol1" through "soln" are

added as members with "op1" through "opn" as the defining operations.

If "region" does not exist, then it is created and solids "sol1" through

"soln" are added as members with "op1" through "opn" as the

defining operations.  A region is merely a combination

record with a flag set and is distinguished from other combinations (groups)

since it has meaning to the COMGEOM solid modeling system.

When a region is created, the item and air codes are set equal to zero.  To change

the item and air codes use the "I" command.

Note:  In the past all members of a region had to be solids, but

recently regions have been allowed as members of regions.  Hence,

the names "soli" can also be regions now.

.sp 2

Examples:    r hulltop.r + hulltop.s - hullleft.s - hullright.s

r gun + gun.s - gunin.s

r gunair + gunin.s

.sp 3

.ce

------------------------------------------------------------------------

.bp

.ce

------------------------------------------------------------------------

.sp 3

.ce

i object combname brname

.sp 2

This command is used to make an instance of an object.

An instance of an object is produced by creating a combination

and making the object a member.

In this case, an instance of "object" is made by

creating the combination record "combname" (if "combname" does not

already exist) and adding "object" as a member.  This member also

has a second name "brname" added to the member record, which can be

thought of as the name of this branch in the hierarchical data structure.

If "combname" already exists, then "object" is added as the next member

and "brname" is added as the branch name.

An instance is used to refer to an object, without making actual copies

of the object.  Instances are useful when one is adding a certain

component to a target description many times.

Furthermore, any modifications to an object which has been instanced need only be

done in the original (prototype) object.

These modifications will then be automatically reflected in all the

instances of the object.

.sp 2

Examples:       i heround ammo he1

i heround ammo he2

i heat ammo heat1

i heat ammo heat2

.sp 3

.ce

------------------------------------------------------------------------

.sp 3

.ce

f face distance

.sp 2

This command allows the user to project

a face of an arb (arbitrary polyhedron) being edited a normal distance to create a new arb.

The value of "face" is 4 digits such as 1256. If the face is projected

in the wrong direction use a negative "distance".  One use for this command is

for producing armor plates of a desired thickness.

.sp 2

.nf

Examples:     f 1234 20

f 2367 34.75

f 2367 -34.75

.sp 3

.fi

.ce

------------------------------------------------------------------------

.bp

.ce

------------------------------------------------------------------------

.sp 3

.ce

l object

.sp 2

This command is used to list information about objects in the data file.

The information listed depends on what type of record "object" is.

If "object" is a combination record, then the members are listed.

If "object" is a solid record, then the GED general solid type and

the parameters as presently in the data file are listed.

Note:  only the solid parameters as they exist in the solid record are

listed, no transformation matrix is applied.

Hence, if the solid was edited as a member of a combination, the "l"

command will not reflect the editing in the listed parameters.

.sp 2

Examples:      l hull

l turret

l turtop.s

l arb8

.sp 3

.ce

------------------------------------------------------------------------

.sp 3

.ce

m oldsolid newsolid axis

.sp 2

This command is used to create and display a new solid record which is

the mirror image of an existing solid.

The mirror image is about an axis and is created by changing the

signs on the solid's parameters depending on which axis the solid

is mirrored about.

In this case, a mirror image of the solid "oldsolid" will created

about the axis indicated by "axis" and the new solid record will

be called "newsolid".

The only acceptable values for the parameter "axis" are "x", "y", and "z".

.sp

Examples:     m tur.left.s tur.right.s y

m tur.top.s tur.bot.s z

m tur.front.s tur.back.s x

.sp 3

.ce

------------------------------------------------------------------------

.bp

.ce

------------------------------------------------------------------------

.sp 3

.ce

p dx [dy] [dz]

.sp 2

This command allows a user in the SOLID

EDITING mode to input exact parameter modifications by the

keyboard.  The meaning of the values typed after the p command

varies depending on what solid editing feature is being used.

Examples of this command will be given in the discussion on

solid editing.

.sp 3

.ce

------------------------------------------------------------------------

.sp 3

.ce

D comb mem1 mem2 .... memn

.sp 2

This command allows one to delete members from a combination record.

In this case, members "mem1" through "memn" will be deleted from

the combination "comb".

.sp

Examples:     D tank hull wheels

D region1 solid8 solid112

D turtop.r tursidel.s tursider.s turback.s

.sp 3

.ce

------------------------------------------------------------------------

.sp 3

.ce

I region item air

.sp 2

This command allows one to change the item or

air code numbers of a region.  If the air code ("air") is not included,

a zero is assumed.

To change the air code, a zero item code should be used (see second

example below).

.sp 2

Examples:

I region1 105

I region7 0 2

I region11 129 0

.sp 3

.ce

------------------------------------------------------------------------

.bp

.ce

-----------------------------------------------------------------------

.sp 3

.ce

e object

.sp 2

This command is used to display (draw) objects on the screen.

In this case, "object" will be displayed on the screen.

Note that "object" must be found in the table of contents.

.sp 3

.ce

------------------------------------------------------------------------

.sp 3

.ce

d object

.sp 2

This command is used to drop (delete) objects from the display.

In this case, "object" will be deleted from the screen display.

This command is opposite of the "e" command.

.sp 3

.ce

------------------------------------------------------------------------

.sp 3

.ce

k object

.sp 2

This command is used to remove (kill) objects from the data file.

In this case, "object" will be removed from the data file.

Note the distinction between the "d" command which deletes objects

from the display and the "k" command which removes objects from

the data file.

.sp 3

.ce

------------------------------------------------------------------------

.sp 3

.ce

t

.sp 2

This command produces a list of the table of contents of all objects in the data file.

The objects are listed a screenfull at a time.  A carriage return

will produce the next screenfull.

.sp 3

.ce

------------------------------------------------------------------------

.bp

.ce

------------------------------------------------------------------------

.sp 3

.ce

a arbname rot fb

.sp 2

This command is used to create and display a new arb8 solid record.

This new arb8 has two square (5 units x 5 units) parallel faces

which are 0.5 units apart.

These parallel faces are in planes whose orientations are defined

by the two input angles - the rotation (rot) angle and the fallback

(fb) angle.

The orientation of armor plates are frequently defined using

rotation and fallback angles.

The vertex of this new arb8 is located in the center of the screen.

.nf

Examples:     a hullsec3.s 45 30

a tursec2.s 90 20

a topglacis.s 0 60

.fi

.sp 3

.ce

------------------------------------------------------------------------

.sp 3

.ce

x

.sp 2

This command is the display debug command.  It will produce a list of

all objects that have been displayed , the

paths traversed to draw the objects, and whether or not the objects

are displayed in the present view.

.sp 3

.ce

------------------------------------------------------------------------

.sp 3

.ce

q

.sp 2

This command is used to quit the graphics editor code.  This is the

normal halt.

.bp

.sp 2

.ce

VIII.  PERIPHERAL DEVICES

.PP

Before we discuss the features of GED, we will introduce

the hardware devices used to implement them.

These devices are the "tools of the trade" for the GED user.

We will discuss only basic operational characteristics here.

Specific use of these devices will be covered in the later sections

on the viewing and editing features of GED.

.PP

The JOYSTICK is a mechanical device used to do the rotations in GED.

Any movement of the stick left or right rotates the display about the

x-axis.  Any movement of the stick up or down rotates the display

about the y-axis.  When the joystick is twisted in a clockwise or

counterclockwise direction, the display rotates about the z-axis.

Any combination motion of the stick will produce a "combined"

rotation about the appropriate axes.

All of these motions have a spring return to a null center position.

.PP

The FUNCTION SWITCH box contains thirty-two buttons.

When any of the buttons is pressed, an action occurs

or condition is set.  Figure 3 depicts the functions programmed

for each button.

The buttons in the shaded area are used for editing while the

rest are used for viewing the display.

The exact functions assigned to these buttons will be discussed

in the sections on viewing the display and on editing.

.PP

The CONTROL DIALS (knobs) are used to send digital information

to the computer.

As a knob is turned, a succession of numbers are available for

use by the computer.

Figure 4 depicts the functions assigned to each of the ten knobs.

The exact functions of each of these knobs will be discussed in

the angle distance cursor section and in the viewing features section.

.PP

The DATA TABLET is a graphics input device containing an X-Y coordinate

grid which corresponds to the grid on the screen.

Information is entered using a pen-like stylus.

The distance this pen is from the tablet is important.

If the pen tip is within one half inch of the tablet surface, the cursor

location on the screen corresponds to the X,Y location of the

pen on the tablet.  This condition is called the "near" position.

If the pen is more than one half inch from the tablet surface, the

cursor remains located in the center of the screen.

When the pen is pressed against the tablet surface, the pressure switch is activated and a bit

is set which is a signal used by GED.  The exact meaning of the pen

near and pen press depends on what is being done and will be covered

in the appropriate sections of this report.

.sp 2

.ce

IX.  ANGLE DISTANCE CURSOR (ADC)

.PP

The angle distance cursor is a construction aid used to measure

angles and distances. It should be noted that all measurements are

made in the projected space of the screen, so one should measure only

in a view normal to the surface where the measurement is to take place.

The ADC is placed on (or removed from) the display by pushing the "ADC"

function switch button (see Figure 3).

The ADC consists of three cursors which cover the entire screen.

Figure 5 depicts the ADC as it appears on the screen.

All the cursors are centered at the same point and can be moved to any

location on the screen.  Two of these cursors rotate for angle measuring

purposes. Angle cursor 1 is solid while angle cursor 2 is dashed.  Angle

cursor 1 has movable tic marks for measuring distances on the screen.

The two angle cursors move with the horizontal and vertical

lines of the main cursor.

The resulting effect is the moving of the center point

horizontally or vertically.

The ADC is controlled by the bottom row of the control dial knobs (see Figure 4) :

Knob 6   moves the center in the horizontal direction

Knob 7   moves the center in the vertical direction

Knob 8   rotates angle cursor 1  (alpha)

Knob 9   rotates angle cursor 2  (beta)

Knob 10  moves the tic marks

Whenever the ADC is on the screen, there is a readout at the bottom of

the screen listing pertinent information about the ADC.

This information includes the angles that angle cursors 1 and 2

have been rotated (alpha and beta), the distance the tic marks are

from the center of the ADC, and the location of the center of the ADC.

This information is continually updated on the screen.

.sp 2

.ce

X.  VIEWING FUNCTIONS

.PP

The GED viewing features are designed to allow one to examine a target description

in close detail.

Any of the viewing features can be invoked at any time.

It should be noted, that these functions do not change the actual

data, only the way these data are displayed.

.PP

Six standard views (front, rear, top, bottom, left, and right) and

one oblique view (azimuth 35, elevation 25) are each assigned to the function buttons (see Figure 3).

Hence, any of these views is immediately available at the press

of the appropriate function button.  The views available are not

limited to these standard views however, as the display can be

rotated to any view by using the joystick.  By pressing the function button

labeled "save view" (see Figure 3), the present viewing aspect angle of the display is saved.

At any time, the saved view can be immediately returned to the

screen by pressing the "restore view" function button (see Figure 3).  The "restore view" button

will be lit whenever a view has been saved.

The function button labeled "reset" (see Figure 3), restores the display

to the default view (front) when pressed.

.PP

The display can be panned or slewed on the screen in two ways - using the

data tablet and pen or by using the control knobs.

When one is editing, the tablet and pen are not available for

slewing, hence one must use the control knobs to slew the display.

If one is NOT editing, then whenever the pen is pressed, the display

moves in the direction of an imaginary vector drawn from the center

of the screen to the cursor location on the screen.

Recall that there is a one-to-one correspondence between the pen location

and the cursor location.

The longer this vector, the faster the display will move.

To slew the display using the control knobs, one uses the knobs labeled

"slew x" or "slew y" (see Figure 4).  The null positions on these knobs is in the

center or straight up.

If the "slew x" knob is turned clockwise of center, the display will move to

the right. If it is turned counterclockwise, the display

will move to the left.

For the "slew y" control knob, clockwise of the center moves the display up

and counterclockwise moves the display down.

The further these knobs are turned from center, the faster the display moves.

.PP

One can zoom the display by using the control knob labeled "zoom" (see Figure 4).

Again the null position of this knob is center or straight up.

Turning this knob clockwise of center causes the display to

increase in size producing a zoom-in effect.

Turning this knob counterclockwise of center causes the display

to decrease in size or zoom-out.

Again, the further the "zoom" knob is turned from center,

the faster the zooming will occur.

.PP

The viewing features of GED also include a slice mode.

To invoke the slice feature, one presses the "slice mode" function button (see Figure 3).

An imaginary slicing plane, parallel to the screen, can then be moved about

by turning the first control knob (see Figure 4).

Turning this knob clockwise moves the imaginary plane into the screen

and counterclockwise moves the plane out of the screen.

All portions of the display in front of the plane are eliminated

from the display.  The result of moving this plane into the

screen is a slicing effect with the parts of the display closest

disappearing first.

To leave the slice mode, just press the "slice mode" button again.

.sp 2

.ce

XI.  SELECTING OBJECTS FOR EDITING

.PP

Before we discuss the editing features of GED, we will discuss

how one selects objects for editing.

To be edited, an object must be displayed on the screen.

Since the object to be edited may be only a small portion of the display, a

procedure is needed to select an object from the many objects

being displayed.

This procedure relies on the hierarchical structure of the GED data base.

As stated before, when objects are displayed, every path of every object

is traversed and the bottom object, which is always a solid, is drawn.

Hence one can think of each solid drawn not as one solid, but as the

unique path used to reach that solid.

As an example, the six paths (solids) for the sample object "heround"

were presented in Table II.

.PP

To select an object for editing, one first must enter the illuminate

mode by pressing the function button labeled "illuminate" (see Figure 3).

The data tablet and pen are then used to select the object.

The surface of the data tablet is divided into imaginary horizontal strips.

Each of these strips corresponds to a path (solid) drawn on the screen.

For example, if the sample "heround" were displayed, the tablet surface

would be divided into six horizontal strips.

The pen is used in the "near" mode (within one half inch of tablet

surface) to illuminate the paths.

The path corresponding to the pen location is written in the upper

right hand corner of the screen.

The solid at the bottom of this path is illuminated (drawn brighter)

on the screen.

In this manner one can "scan" the complete display by simply

moving the pen up and down the tablet in the near mode.

Hence, the illuminate mode is also quite useful just to identify

objects in a description.

However, the primary purpose of the illumination mode is to select objects

for editing.

.PP

The actual selecting of an object for editing is a two-step process.

First, one selects the path desired, then one selects the particular

object of interest from that path.

One "selects" by pressing the pen on the tablet surface.

When a path is selected, the word PATH followed by the members of that

path is written in the upper right of the screen.

At that point, one is ready to select a specific member (object)

from that path for editing.

Again, the tablet is used for this selection.

The tablet surface is now divided into as many rows (zones) as there are

members in the selected path.

When the pen, in the near mode, is in a path member's assigned zone

on the tablet, that member's name will be illuminated in the path listing.

When the specific member to be edited is illuminated, it is selected

by pressing the pen.

The word EDIT will now appear in the upper right of the screen

ahead of the path listing.

One is now ready to edit this selected object.

.PP

The only way out of the EDIT mode is to accept or reject any

editing performed.

This is done by pressing the function buttons labeled "accept" or

"reject" (see Figure 3).

If at any time in the selection process an incorrect object is selected,

just press the "reject" button and begin the selection process again.

.sp 2

.ce

XII.  OBJECT EDITING

.PP

The heart of the GED system is its editing features.

The editing features are divided into two classes: object editing

and solid editing.

Object editing is designed to allow one to change the location,

size, and orientation of an object.

Recall that an object is defined as the basic data unit of the

GED system and includes both combinations and solids.

In the case of a solid, one needs to change not only its location,

size, and orientation, but also its "shape".

Changing the shape of a solid means changing any of its individual parameters.

Hence, solid editing is handled separately.

.PP

Any object in the data file may be selected for object editing.

The object is picked using the object selection procedure discussed earlier.

Recall that in the GED data structure, all members of combinations

have transformation matrices associated with them, allowing

assemblies to be edited as units.

Object editing is the vehicle for performing such tasks.

All object editing is done using homogeneous transformation matrices.

When the editing is accepted, new transformation matrices are

created in all appropriate member locations.

If the object happens to be a single-member path (solid) then the matrix

is applied to the solid's parameters.

Otherwise, all object editing is stored in transformation matrices

in the data base.

.PP

Once an object is selected for editing, the second row of function

buttons (see Figure 3) is used to select the editing to be done.

Then either the data tablet and pen or the joystick are used

to perform the actual editing.

.PP

To SCALE a selected object, one first presses the function button

labeled "scale object".

The data tablet and pen are then used to scale the object.

The object is scaled about the target origin, thus unless the object is

centered at the origin, some translation of the object will occur.

The location of the pen on the tablet when pressed determines the

scale factor used.

If the pen is above the center of the screen, the scale

factor will be greater than one.

The further the pen is located above the center, the larger the scale factor.

Likewise, if the pen is located below the vertical center, the scale

factor will be less than one and the further below the center, the

smaller the scale factor.

As long as the pen is pressed, the object will continue to increase

or decrease in size.

.PP

To ROTATE a selected object, one first presses the function button

labeled "rotate object".

The joystick is then used to rotate the object.

The object is rotated about the center of the view (screen).

If one wishes to rotate the object about another point of interest,

the entire display can be slewed until

the desired point is positioned at the center of the screen.

.PP

To TRANSLATE a selected object, one has three function buttons from

which to select (see Figure 3).

The data tablet and pen are then used to translate the object.

The center of the object "follows" the pen (cursor) location whenever

the pen is pressed.

The three function buttons define the direction allowed for movement

of the object.

The button labeled with a horizontal arrow (<--->), allows the

object to follow only the horizontal location of the pen.

The vertical location of the object does not change.

The function button labeled with a vertical arrow (  ), allows the

object to follow only the vertical location of the pen.

The horizontal location of the object does not change.

The function labeled with crossed arrows (<--->) allows the object

to follow the X,Y location of the pen.

.PP

The object editing features can be invoked in any order and at any time

once an object has been selected for editing.

During object editing, any of the viewing features, such as changing

views, zooming, and slewing, can be used and in fact are usually

quite useful.

Again, the only way to exit the object editing mode is to

accept or reject the editing.

If the "reject" button is pressed, the object will return to its pre-edit state.

If the "accept" button is pressed, the data base will be changed to

reflect the object editing performed.

.sp 2

.ce

XIII.  SOLID EDITING

.PP

The solid editing feature allows the user to interactively translate,

rotate, scale, and modify individual parameters of a solid.

Whenever one is in the solid edit mode, the parameters of the solid

being edited are listed and continually updated at the bottom of

the screen.

Certain parameters are also labeled on the solid being edited.

Solid editing is generally used to "build" objects by producing

solids of the desired shape and size in the correct orientation and position.

Once the object is built, object editing is used to scale, orient, and

position the object in the description.

The general philosophy of solid editing is to first create a copy of a prototype

solid with the desired name and then to edit this solid.

The prototype solids should not be edited.

As an example, suppose one were to build the sample object "heround"

mentioned earlier.

To produce the base of the shell, one would need a cylindrical shaped solid.

The prototype solid is the TGC (see Table I), so one would type:

.sp

.ce

c tgc shellcase.s

.sp

A new solid record called shellcase.s would be created and displayed

on the screen.

This shellcase.s solid would then be edited using solid editing to

produce the exact solid parameters desired.

The solid record tgc would be unchanged and available for copying

the next time a cylindrical solid is needed.

.PP

The procedure for solid editing is quite similar to that for object

editing.

First, the solid must be selected for editing.

A solid is selected for editing using the illuminate mode, just as in object

editing, except a solid must be selected.

Recall that the bottom object in every hierarchy path is a solid.

Second, one must push the function button labeled "solid edit" (see

Figure 3).

This button sets up the solid edit mode:

the solid parameters are listed at the bottom of the screen,

pertinent solid parameters are labeled on the display,

the solid editing function buttons are activated,

and a menu header depending on the solid type is written on the

right side of the screen.

The meaning of the menu will be discussed shortly.

Third, one selects (by pressing the appropriate function buttons) and performs the editing desired.

Finally, one exits the solid editing mode by accepting or rejecting

the editing performed, just as in object editing.

.PP

Recall, in the section on object editing it was mentioned that since

a solid is indeed an object, a solid can be edited (translated,

rotated, and scaled) as an object.

Hence, the solid editing mode is really only necessary to perform parameter modifications.

However, much of the object editing is done about the target or screen center, which can

be annoying when editing a solid.

Also, precise parameter modifications are possible (using the "p" keyboard

command) in solid editing.

Thus it was deemed desirable to include translation, rotation, and scaling

in solid editing also.

.PP

Solid TRANSLATION allows the user to place the solid being edited

anywhere in the description.

To invoke this option, one presses the function button labeled

"solid trans" (see Figure 3).

To move the solid, use the data tablet and pen.

Whenever the pen is pressed, the VERTEX of the solid moves

to that location on the screen.

One can read the actual coordinates of the vertex on the bottom

of the screen, along with the other parameters.

If the actual desired coordinates of the vertex are known, one

can place the solid exactly using the "p" keyboard command.

For example, to place a solid's vertex at the coordinates

(x, y, z) one would type

.sp

.ce

p x y z

.sp

The solid would then jump to this location.

.PP

The solid SCALE feature allows the user to scale the solid being

edited to any desirable size.

The scaling is done about the vertex of the solid, hence NO translation

of the solid occurs.

The scaling is performed using the data tablet and pen, just as in

object scaling.

One can input an exact scale factor using the "p" keyboard command.

For example, typing

.sp

.ce

p factor

.sp

will scale the solid by an amount equal to "factor".

The value of "factor" is absolute - the original solid is scaled.

The size of the original solid is defined when the "solid scale" function

button is pressed.

By setting "factor" equal to one, the original size solid will be

displayed on the screen.

.PP

Solid ROTATION allows the user to rotate the solid being edited

to any desired orientation.

The rotation is performed about the vertex of the solid.

To select this option, one presses the function button labeled

"solid rotate" (see Figure 3).

The actual rotation is done using the joystick.

One can input exact angles to rotate the solid by using the "p"

keyboard command.

For example, typing

.sp

.ce

p alpha beta gamma

.sp

will rotate the solid "alpha" degrees about the x-axis, "beta" degrees

about the y-axis, and "gamma" degrees about the z-axis.

Alpha, beta, and gamma are measured from the original "zero" orientation

of the solid, defined when the "solid edit" function button was pressed.

Hence, typing

.sp

.ce

p 0 0 0

.sp

will always return the solid to its original position (its position when the current solid

editing session began).

.PP

The PARAMETER editing feature allows the user to modify any of the

parameters which comprise the selected solid.

To invoke this feature, one presses the function button labeled

"menu" (see Figure 3).

A menu (listing) of parameters to edit will then appear on the

right side of the screen under the menu header.

The contents of this menu of course depend on the type of solid

being edited.

To select an item (parameter) from this menu, one uses the

data tablet and pen.

The immediate region (strip) surrounding each item of the menu "belongs"

to that item.

Whenever the pen is in one of these regions, that menu item will

get brighter in the menu listing on the screen.

A menu item is picked by pressing the pen when that item name is illuminated.

It should be noted that whenever the pen is in the "menu area", the code

assumes the user is attempting to pick a menu item.

Hence, the menu area of the screen (the far right edge) can NOT be used

by the pen for editing while the menu is on the screen.

.PP

For parameter editing, the solid type determines the menu items.

The GED code recognizes four general solid types for parameter editing.

All the specific solid types which fall into one of these general classes

are treated as the general type.

For example, all the cylindrical solid types are treated as the

general type - the TGC.

In the following paragraphs, we will discuss parameter editing

for each of these general types of solids.

.sp 2

.nf

A.  ARB PARAMETER EDITING

.fi

.PP

The GENERAL ARB class of solids represents all the convex polyhedrons

(RPP, BOX, RAW, and ARBs).

The ARBs comprise five classes of polyhedrons each with a characteristic

number of vertices.

These are the ARB8, ARB7, ARB6, ARB5, and ARB4, where the ARB8 has

eight vertices, etc.

GED handles all these ARBs as ARB8s, but with some vertices equal.

All the eight vertices of the GENERAL ARB are labeled on the screen, although

in certain cases (RAW,ARB7,ARB6,ARB5,ARB4) some vertices will be equal.

Figure 6 depicts typical ARBs as they would be displayed and labeled during solid editing.

.PP

To edit the ARB vertices, EDGES are moved.

Hence, the GENERAL ARB menu is a list of edges to move:

.sp

.ce 13

GENERAL ARB MENU

move edge 12

move edge 23

move edge 34

move edge 14

move edge 15

move edge 26

move edge 56

move edge 67

move edge 78

move edge 58

move edge 37

move edge 48

.sp

The edge to be moved is selected from this menu and is then moved

using the data tablet and pen.

When the pen is pressed, the line containing the selected edge is

moved so it goes through the pen location and is parallel to the original edge.

The endpoints of the new edge are then calculated to remain in

the "bounding" planes of that edge.

In the case of the ARB8, for example, the bounding planes for edge 58

are planes 4378 and 1265 (see Figure 6).

.PP

The information printed on the bottom of the screen is the x, y, z

coordinates of the eight vertices:

.ce 5

vertex         coordinates

1,2        x1 y1 z1  x2 y2 y3

3,4        x3 y3 z3  x4 y4 z4

5,6        x5 y5 z5  x6 y6 z6

7,8        x7 y7 z7  x8 y8 z8

These values are continually updated during solid editing.

.PP

The user can force the line containing an edge through any point using

the "p" keyboard command.

Hence typing

.sp

.ce

p x y z

.sp

will force the line containing the edge being edited to pass

through the point (x, y, z).

No check is made for concave ARBs or for ARBs with crossed edges.

Visual feedback from the display is considered adequate to detect these situations.

.sp 2

.nf

B.  TGC PARAMETER EDITING

.fi

.PP

The TGC general class of solids includes all the cylindrical COMGEOM solids.

The defining parameters of the TGC are two base vectors (A and B), a height

vector (H), two top vectors (C and D), and the vertex (V).

Reference 1 contains more information on these parameters.

The top vectors C and D are directed the same as the base vectors A and

B respectively, hence the top vectors are defined only by their lengths (c and d).

Figure 7 depicts these parameters on typical TGCs.

When a TGC is edited in the solid editing mode, only vectors A and B are

labeled on the display.

.PP

All the vectors (A,B,C,D,H) of the TGC can be scaled during parameter editing.

In addition, the height vector (H) and the base (AxB) can be rotated.

These parameter editing options are reflected in the TGC menu:

.ce 8

TGC MENU

scale H

scale A

scale B

scale c

scale d

rotate H

rotate AxB

The scaling of the lengths of the vectors is done using the data tablet

and pen in the same manner as object editing.

Exact vector lengths can be achieved easily using the "p" keyboard command.

If one is scaling a vector, then typing

.sp

.ce

p x

.sp

will make the vector being scaled have a length equal to "x".

.PP

To rotate vector H or surface AxB, one uses the joystick.

Since vectors C and D have the same direction as vectors A and B respectively,

the top and bottom surfaces of the TGC will remain parallel.

Hence rotating the base surface (AxB) will also rotate the top surface.

The "p" keyboard command is not used with the parameter rotation of the TGC.

.PP

During solid editing, the parameters of the TGC are constantly updated on the bottom

of the screen:

.ce 7

V   x y z

H   x y z |H| a b g rot fb

A   x y z |A|

B   x y z |B|

c   |c|

d   |d|

AxB a b g rot fb

where "a", "b", "g" are the direction cosines and "rot" and "fb" are

the rotation and fallback angles of the listed vector.

.sp 2

.nf

C.  ELLG PARAMETER EDITING

.fi

.PP

The ELLG general class represents all the ellipsoidal solids, including

spheres and ellipsoids of revolution.

The defining parameters of the ELLG are three mutually perpendicular

vectors (A, B, and C) and the vertex (V).

Reference 1 contains a more complete definition of these parameters.

Figure 8 depicts typical ELLGs with these parameters labeled.

When an ELLG is being edited, only vectors A and B are labeled on the display.

.PP

The parameter editing of the ELLG consists of scaling the lengths of the

vectors A, B, C.

These parameter editing options are listed in the menu:

.ce 4

ELLG MENU

scale A

scale B

scale C

The scaling of these vectors is done using the data tablet and pen in

exactly the same manner as in object scaling.

The "p" keyboard command again can be used to produce a vector of

desired length.

.PP

The parameters of the ELLG are continually updated on the bottom

of the screen during solid editing:

.ce 4

V   x y z

A   x y z |A| a b g rot fb

B   x y z |B| a b g rot fb

C   x y z |C| a b g rot fb

where "a", "b", "g" are the direction cosines and "rot" and "fb" are

the rotation and fallback angles of the listed vector.

.sp 2

.nf

D.  TOR PARAMETER EDITING

.fi

.PP

The TOR general class of solids contains only one type of torus, one with

circular cross-sections.

The defining parameters of the TOR are two radii (r1 and r2), a normal

vector (N), and the vertex (V).

The scalar r1 is the distance from the vertex to the midpoint of the

circular cross section.

The scalar r2 is the radius of the circular cross-section.

The vector N is used to orient the torus.

Reference 1 contains a more detailed definition of these parameters.

Figure 9 depicts these parameters.

During solid editing, none of these parameters are labeled on the screen.

.PP

The parameter editing of the TOR consists of scaling the radii, hence the

menu contains only two members:

.ce 3

TORUS MENU

scale radius 1

scale radius 2

As before, the scaling of these radii are done using the data tablet

and pen and the "p" keyboard command can be used to produce a radius

of any desired length.

.PP

The parameters of the TOR are continually updated on the bottom

of the screen during solid editing:

.ce 4

V   x y z

r1  |r1|

r2  |r2|

N   x y z a b g rot fb

where "a", "b", "g" are the direction cosines and "rot" and "fb"

are the rotation and fallback angles of the vector N.

.PP

Table IV presents a summary of parameter editing for each of the

general solid classes.

.sp 2

.ce

XIV.  CONCLUSION

.PP

BRL has developed the Army's first interactive solid modeling

system, called GED.  GED performs two basic functions:

viewing and editing.  The standard viewing capabilities of zooming, slewing,

slicing, and rotation are available.

Likewise, all the standard editing features are also available.

The user easily traverses the hierarchical data structure, applying

the editing functions of rotation, translation, and scaling to any

position in the hierarchy.

The hierarchical structure can be modified and regrouped and regions

created and modified.

Specific parameter editing can also be applied to the solids to produce

any shape solid desired.

GED has made great gains in reducing the bottleneck the target

description process has been creating in the vulnerability process at BRL.