DIAGS.EXE                                    DIAGS.EXE

Special Serial, Parallel, Video diagnostics for the PC             Special Serial, Parallel, Video diagnostics for the PC

Written by Joan Riff for:

Computerwise Consulting Services

P.O. Box 813, McLean, VA 22101

(703) 280-2809

Introduction      ____________

DIAGS is a tool for the advanced PC user, period. It is NOT for the

general public. This manual will not explain things in elementary terms.

If you know what a 6845 is, and what an 8250 is, and what parallel

handshaking lines are, then DIAGS will be extremely useful to you. If

not, then you may have better things to do with your time than try to

figure out this program.

DIAGS started out as a testbed for our new interrupt-driven RS232

libraries. The Serial Port Diagnostics were the first written, to test

the Microsoft version 3.0 'C' interface to these routines.

It has since grown. As we have added more assembler routines to our

library, we have added corresponding DIAGS functions to test them.

Additionally, DIAGS has been expanded as we experienced the need for new

diagnostics capabilities. So the Parallel Port diagnostics were added

when we had to debug yet another unknown printer's handshaking. And the

6845 diagnostics were added when we needed to test various video tricks

like displaying 30 lines of text, smooth scrolling, etc.

As it now stands, DIAGS is a very powerful facility for:

1) Testing all sorts of asynch communications lines

2) Testing printer handshaking and cable wiring

3) Examining an unknown system's DOS environment, including:

a) Interrupt vectors

b) Resident device handlers

4) Directly manipulating the 6845.

Main Menu     _________

The main DIAGS menu displays information about the machine and about

DOS, and presents a list of the various diagnostics that are available.

When you exit a particular diagnostic, you are returned to this main

menu.

(D)isk Info      ___________

Let's describe the weakest and least important diagnostic first.

The DISK INFO selection from the main menu displays just about every-

thing that can be determined about a disk. Make sure that you have a

diskette in the indicated drive before selecting this option.

Since there are other utilities which do just about everything that you

may want to do with a disk, we haven't done much with this diagnostic.

We just display disk information as reported by DOS as well as by the

disk's Boot Record.

Once in the Disk Info screen, the following keypresses are available to

you:

ESC - return to the main DIAGS menu           ESC

ALT-V - verify a diskette by reading every sector via DOS INT          ALT-V

025h. This is SLOW!!!! Think of this as a demo of just how

slow INT 025h can be.

ALT-R - reset drive. Doesn't do much, huh? That's what we          ALT-R

wanted to see.

A thru Z - select new drive for which statistics are to be          A      Z

displayed. Again, be sure that you have a disk in whatever

drive you select.

That's it. Not much to it. If you need to do more, then use:

The Norton Utilities

Quaid's Disk Explorer

Central Point Software's Option Board

(I)nterrupt Vector Display      __________________________

This selection from the main menu displays the system's Interrupt

Vectors. For each interrupt, the following information is displayed:

The interrupt number

The interrupt vector's address within segment zero

The Segment:Offset of its handler

The interrupt's name and/or use

If an interrupt points to an IRET, then that fact is reported by a

highlighted "[IRET]" displayed after the name.                   [IRET]

Once in the Interrupt screen, the following keypresses are available to

you:

ESC - exit back to main menu           ESC

Home - scroll display back to Interrupt 0           Home

End - scroll display to the end of the interrupts (F0 thru FF)          End

PgDn - scroll to the next page of vectors           PgDn

PgUp - scroll up to the previous page of vectors           PgUp

Up Arrow - scroll up 1 line           Up Arrow

Down Arrow - scroll down 1 line           Down Arrow

(P)arallel Diagnostics      ______________________

This is a more complicated set of diagnostics. It allows you to do all

sorts of useful things with the parallel ports. We use these routines

when we're trying to figure out whether or not some new printer is

working properly with the system.

At the top of the screen is displayed:

The number of parallel ports on the system (1 thru 4)

The number of the currently-selected printer (0 thru 3)

The base I/O address of the currently-selected printer

The trip character that is currently set (see below)

Whether interrupts are allowed or disallowed during sampling

The Parallel Port Diagnostics menu gives you the following choices:

(I)nit printer - does a ROM BIOS "init printer" call.           (I)nit printer

(E)nable Interrupts during sampling - toggles the indicator on          (E)nable Interrupts during sampling

the top line, which indicates whether INTs are to be dis-

allowed during sampling.

(N)ew Trip character - allows you to specify a new character          (N)ew Trip character

to be sent to the printer to 'trip' it. This is initially a

Linefeed.

(P)rinter switch - selects next printer (in sequence) as the          (P)rinter switch

current printer.

(T)rip printer - sends the current Trip character to the          (T)rip printer

printer, and then captures the printer Status port lines using

a special high-speed capture routine. These lines are dis-

played in oscilloscope format, allowing you to see at a glance

the operation of the BUSY line, ACK line, etc. Interrupts will

be inhibited during the sampling if the top line of the

display shows "No Ints". Press ESC to exit from the display of

the status lines.

(U)nique Printer setup - sends custom codes to set various          (U)nique Printer setup

specific printers into various modes. The printers currently

supported are:

Data Products 8070 (and perhaps IDS Color Prism)

IBM Graphics Printer (and certain Epsons)

Fujitsu DL-2600 (and perhaps Epson JX color series)

Okidata Microline 93

Diable 630 (and C.ITOH Starwriter, F-10, etc)

C.ITOH Prowriter

Follow the prompts that are displayed when you make this

selection.

(Q)uit parallel diagnostics - to return to main menu           (Q)uit parallel diagnostics

The following three selections are invaluable for checking

proper printer cabling and handshaking:

(D)ata lines - displays the parallel port Data I/O port lines          (D)ata lines

in oscilloscope-like format. Press ESC to exit the oscillo-

scope display.

(S)tatus lines - as above, but lines displayed are from          (S)tatus lines

the parallel Status I/O port.

(C)ontrol lines - as above, but lines displayed are from          (C)ontrol lines

parallel Control I/O port.

The following selections send various strings to the printer:

(F)ox message - sends a continuous FOX message to the printer          (F)ox message

via direct I/O (bypassing the ROM BIOS). Press ESC to stop it.

(A)scii character set - sends the complete ASCII character set          (A)scii character set

to the printer via direct I/O (bypassing the ROM BIOS). No

carriage returns are sent. Press ESC to stop it.

(H)oriz and vert alignment tests - send various Formfeed, Tab,          (H)oriz and vert alignment tests

line spacing and Backspacing sequences to printer. This is

useful for testing the results of (U)nique printer setups, and                                            (U)nique

to see if the printer supports true backspacing. This selec-

tion has its own submenu, which is exited via ESC.

(S)erial Port Diagnostics      _________________________

If you do PC communications as much as we do, then this part of DIAGS is

for you! It provides a very powerful look at the comm line. Its most

intriguing feature is the ability to serve as a line monitor between two

serial devices.

When you select "Serial Diagnostics" from the main menu, your screen

becomes a terminal screen. At the top of the screen is displayed various

port information as follows:

Whether the port is open ("OP") or closed ("CL")

The current serial port number

The current baud rate

The number of data bits being used

The type of parity being used

The number of stop bits being used

Whether the existing 8250 parameters are being re-used (this

feature is only used by us, for internal testing)

What port is being wrapped to (more on this later)

Whether keypresses are echoed to the screen

Whether DEBUG mode (HEX display) is set

To the right of this information are various stats, displayed only if

the port is open and stats have been enabled. These stats include:

The size of the RS232 output queue (in HEX)

The size of the RS232 input queue (in HEX)

The status of Carrier Detect and DSR

The number of times (in HEX) that the following have occurred:

RING

BREAK

Framing Error

Parity Error

Overrun Error

Once the port has been opened, any "normal" keys that you press are sent

over the comm line by being queued into the output queue. Incoming data

appears on the screen. Incoming and outgoing characters are displayed in

different colors, so that you can tell what's what.

The RS232 routines are entirely interrupt-driven. The output queue will

be serviced only if the 8250 is properly interrupting the 8088. This in

itself is a valuable test of proper system operation. If the output or

input queue sizes stay "stuck" at a non-zero value, then something is

wrong with your system.

Besides the "normal" keys, you may use the following keypresses:

ALT-S - switch display between port 0 and port 1 (COM1: and          ALT-S

COM2:).

ALT-M - bring up a menu of miscellaneous features:           ALT-M

HANGUP the phone by dropping DTR

PICKUP the phone by raising DTR

RESET the Hayes 1200b by OUT1 toggle

FLUSH input and output queues

CRLF control (how to display incoming CR and LF)

STATS control - turning the stats off can speed up the

terminal display considerable

TRAP - control what happens when you press ALT-T to trap

some 8250 register

ALT-X - initiate an XMODEM transfer. This we use to test the          ALT-X

accuracy of XMODEM as implemented in other software packages.

ALT-P - control port parameters. This includes:           ALT-P

port OPEN and CLOSE

BAUDRATE

DATA bit selection

PARITY setting

STOP bit selection

REUSE of the 8250 params (for our internal use)

ECHO (of local keypresses) on/off

WRAP of incoming data to another comm port

DEBUG display toggle

ALT-T - to trap the data lines from a previously-defined 8250          ALT-T

register

ALT-R - display various 8250 registers           ALT-R

ALT-B - to send a BREAK           ALT-B

ALT-W - wait (do nothing) till next keypress. This is useful          ALT-W

for pausing a fast-miving display.

ALT-Q - quit back to main menu           ALT-Q

ALT-F - send a single FOX message over comm line           ALT-F

Most of these functions contain their own prompts, and require no

further explanation. Some, however, need to be explained in detail.

The continuous updating of the stats on the top line can slow down the

terminal display. Turning stats off (ALT-M S) can speed things up

considerably.

The baudrate is continuously variable, from 2 to 38400. You are not

limited to "standard" PC baud rates.

If you want to clear the screen and reset the stats, try switching ports

twice (ALT-S ALT-S).

To trap and display an 8250 register in the middle of communication, you

must first define the register to be trapped (ALT-M T) and whether INTs

are to be allowed during the sampling that follows.

Having done that, you cause the trap by pressing ALT-T. At that instant,

a special high-speed assembler routine is executed which samples the

designated register once every 50 machine cycles or so until 420 samples

have been taken. These sample bytes are then displayed as a screenful of

HEX bytes. You may highlight a particular byte via the arrow keys. The

highlighted byte's interpretation is displayed on the message line

toward the top of the screen. Press ESC to exit this trap display.

The "wrap" feature is very useful. It may be used to allow DIAGS to

serve as a line monitor located between two serial devices, among other

things. Every incoming character is "wrapped" as an output to whatever

port you have specified as the "wrap" port. If you select as the "wrap"

port the same port that you're using, then you have in effect become a

full-duplex system. The serial device connected to that port will see

its "keypresses" echoed back to itself.

Assuming that you have two serial ports, you may use DIAGS as a line

monitor by wrapping each port's input to the other port. You can then

switch between ports (ALT-S) and generally watch everything that goes in

either direction. You can turn on the DEBUG display (ALT-P G) to see

things in more detail.

(R)esident device handler display      _________________________________

This main menu selection allows you to see a list of all resident device

handlers. As you highlight each handler's name (via the left and right

arrow keys), the device's attributes are expanded on the screen. Press

ESC to exit this display.

Block devices are indicated by device names of the form "BLOCKnnn",

where nnn is the number of devices supported by this block device

handler.

(V)ideo Manipulation      ____________________

If you want to do some fancy 6845 programming, then the Video diagnostic

will be of great interest to you. It allows you to examine and alter the

6845 registers in a most graphic fashion.

Toward the top of the screen is a two-column list of all 6845 registers,

and one or two of the special IBM PC video registers (the MODE and/or

COLOR registers). There is a ">" pointer, showing you which register is                                   >

being worked with.

For each 6845 register, two values are shown:

1) The original value, as set by the ROM BIOS

2) The current value as set by you

You move the ">" pointer to various 6845 registers via the up and down                   >

arrow keys. As you select each 6845 register, that register's bit

pattern is expanded on the bottom half of the screen.

You can change the currently-selected 6845 register's value by using the

"+" or "-" keypad keys to increment or decrement its value.       +      -

You can also use the left and right arrow keys to move the "bit pointer"

across the various bits of the expanded byte on the bottom half of the

screen. Pressing "1" or "0" sets or clears the bit currently pointed                       1      0

to. This is an alternate way to change the current value of a 6845

register.

The updated 6845 registers are loaded into the 6845 ONLY WHEN YOU PRESS

RETURN. This allows you to make extensive changes to registers without

having intermediate states foul up the display.

If you hose things up and get an unreadable screen, just press ESC to

exit the Video Diagnostics, then enter it again. The 6845 is reset on

exit from the Video Diagnostics.

If you are using a Color/Graphics Adapter, then you may press "S" to                                                                    S

toggle Snow on or off. The upper-right corner of the display shows the

current status of snow as follows:

"Snow: YES" means that no effort is being made to eliminate           Snow: YES

snow, and that snow is present (although you won't see it if

you have a good Color/Graphics board)

"Snow: NO" means that snow is being eliminated by our video           Snow: NO

software.