DOCUMENTATION FOR LISS PROGRAMS

These programs plot Lissajous patterns on the graphics screen of your MS-DOS

micro-computer.

Two programs are included, LISS.COM which runs on any MS-DOS compatible machine

and LISS-87.COM, which runs much faster but requires that a math co-processor

chip be installed.  Both were written in Turbo Pascal and compiled therewith.

Lissajous patterns result when one function of an independent variable is

plotted on the X-axis of an orthogonal curve, and another function of the same

independent variable is plotted on the Y-axis.

The X function of the Lissajous pattern plotted by these programs is:

f(x) = c1 cos(h1*a + p1) + c2 cos(h2*a + p2)

+ c3 cos(h3*a + p3) + c4 cos(h4*a + p4)

The coefficients c1..c4, the harmonics h1..h4, and the phase angles p1..p4

for the 4 terms come from a random number generator.

Similarly, the Y function is:

f(y) = c1 sin(h1*a + p1) + c2 sin(h2*a + p2)

+ c3 sin(h3*a + p3) + c4 sin(h4*a + p4)

where a = the instantaneous angle.

The coefficients, harmonics, and phase angles for the Y-function are also

randomly generated.  They are not the same as those for the X-function.

The functions are plotted for values of a from 0 to 2 pi radians in increments

of 2*pi/100 radians.

The programs calculate the functions for a given set of random coefficients,

harmonics and phase angles, normalize them to produce a full screen image,

display them, and repeat the cycle until you hit "Esc" to quit.

It can take anywhere from 1/2 second to 18 seconds to go through a cycle. The

speed depends on the type of CPU chip, memory chips, clock frequency, and the

presence of a math co-processor.

When the programs are first invoked, you will see a screen summarizing the

above introduction.

You will then be asked if you want to see tables of coefficients, harmonics and

phase angles for each of the functions, and if you want to see the X-Y axes of

the curve, and if you want the display to show how many plots have been drawn.

If you press "R" for Review while the program is running, you will get a

a chance to review these screens re-answer the questions.

If you press any key after a plot is drawn, it will stay on the screen until

you again press any key.

You can hit "Esc" to quit the program.

There really isn't much to do with the programs besides look at the pictures.

Remember, you must have a color-graphics adapter, but not necessarily a color

monitor.  If you run LISS-87.COM, then you must have an 8087, 80187 or 80287

math co-processor chip installed.