xx

MEMORY CLEAN FOR THE IBM PERSONAL COMPUTER

Version 1.00

May 3, 1983

Robert J. Beilstein

413 Wells Avenue, West

North Syracuse, New York 13212

One of the unfortunate "features" of the IBM Personal Computer is

that, while the 8088 processor will address 1024K of memory, the BIOS

(and, thus, DOS) will only recognize the first 640K (on "regular"

PC's, without the expansion box and ROM, 544K) of memory.

While there are a number of programs around which are able to ignore

the memory size passed from DOS, and make use of the additional

memory, there is still a fairly serious problem that needs to be

addressed.

When the machine is powered up, the contents of memory are

unpredictable.  In particular, there is probably only a 50%

probability that any given memory location has the proper parity.

The ROM BIOS "initial reliability tests" write to both planar and I/O

RAM during initialization.  This not only sets proper parity in the

memory locations (this happens automatically when the locations are

written into), it also tests the memory for proper operation (though,

curiously, it never attempts to write  an odd-parity value).

Unfortunately, this setting and testing of memory only occurs in the

first 640K (544K) of memory.  Thus, any additional memory in the

system is left in an uninitialized state.

If a program attempts to reference a location in this uninitialized

memory before first writing to it (extremely poor programming

practice, it is true), then there is a 50% chance that the result will

be a "PARITY CHECK 2".

This seems like an awfully stiff penalty to pay for a program bug, as

it requires one to power down the machine in order to clear it.

MEMCLEAN is a program which will write an initial value (with good

parity) into all of the memory locations which IBM misses.

When called without additional parameters, the program will clear the

high memory from X'C0000' through X'EFFFF' (768K to 960K).

If a parameter of "544K" is specified, locations X'88000' through

X'9FFFF' (544K to 640K) will be cleared.  This is for compatibility

with older (non XT) PC's.

If "SEGA" is specified, then locations X'A0000' through X'AFFFF' are

included.  This area is defined to be "reserved" by IBM, but is (for

now) available for use.

Specifying "NOHIGH" will inhibit writing into the high memory area

(X'C0000' through X'EFFFF').

Note that writing into nonexistent memory locations causes no problems

(other than using up some time).  So there is no problem if there is

memory only up to (say) 576K.

The following examples illustrate the use of MEMCLEAN:

1.  A PC/XT with memory in the high area, but not in the "reserved"

area:

A>memclean

2.  A "regular" PC with 704K of contiguous memory (i.e. up through

X'AFFFF'):

A>memclean sega 544k nohigh

3.  A "regular" PC with every available memory location populated:

A>memclean 544k sega

4.  A PC/XT with 704K of contiguous memory:

A>memclean sega nohigh

Permission is given to use and copy this program and documentation

freely, as long as no charge is made for its distribution or use, and

as long as the notices and copyright statements in the program and

documentation are left intact.  For complete details of the terms and

conditions, please see the source code.

All other rights are reserved.  Program and documentation Copyright

(C) Robert J. Beilstein, 1983.

65399 '** DONE - PRESS ENTER TO RETURN TO MENU **

ed.  Program and documentation Copyright

(C) Robert J. Beilstein