VCOPY

Ver.  1.30

by

W.G. Madison

W.G. Madison and Associates, Ltd.

Consultants / Information Systems

P. O. Box 898

Greenbelt, MD 20770

(301)552-7234

(CompuServe  73240,342)

The Author is a member of the Association of Shareware

Professionals.

========================================================

|                       Shareware                      |

|                                                      |

| We encourage you to freely copy and share unmodified |

| copies  of this program  with other  people.  If you |

| decide to use the  program,  support  the  Shareware |

| marketing  concept by  becoming  a registered  user. |

========================================================

PURPOSE:

When the DOS COPY command is used, no true read-after-write

verification is performed. The only way to assure that the re-

cording was correct is to read the file after writing, and com-

pare the output file, byte for byte, with the input. For very

long files this can result in unacceptable delays.

A reasonable compromise between speed and reliability is to

calculate the cyclic redundancy check (CRC) word of the input

file as it is being copied, and then on completion of the copying

operation, read back the output file and calculate its CRC. If

the two compare, the probability that the two files differ is

extremely small.

Further, when using the DOS COPY command, files are copied

unconditionally -- possibly accidentally overwriting existing

files.

Finally, when using the usual DOS facilities, files can only

be moved from one subdirectory to another through a two step

process. First, the file must be COPY'd to the target subdirect-

ory; and second, it must be DELeted from the source subdirectory.

VCOPY answers these three problems in a single efficient

program.

Whenever VCOPY copies a file, the source file CRC is calcu-

lated during the copying process. On completion of the copying,

the destination file is read and the CRC re-calculated. If the

1

two do not compare, a warning is issued and the destination file

is deleted. VCOPY preserves the date/time stamp of the source

file.

Whenever VCOPY copies a file, it first checks to see if a

file of the same name already exists in the target subdirectory.

If it does, the user is presented with the option of allowing the

copying to proceed, or skipping that copy and proceeding to the

next. VCOPY can, however, be directed to perform the copies

unconditionally, by use of a command line switch.

VCOPY can also be instructed (again, using a command line

switch) to move a file from one subdirectory to another.

If no file of the same name exists in the target subdirect-

ory, the file will be moved over directly, without copying.

If a file of the same name does exist in the target subdi-

rectory, then VCOPY will revert to the copying mode described

above. In this case, however, the file will be deleted from the

source subdirectory after a verified copy has been completed.

USAGE:

The syntax is similar to the DOS COPY command, except that

files cannot be renamed during the copying process.

For a complete description of the VCOPY syntax, type VCOPY

at the DOS prompt with no arguments.

HISTORY AND BACKGROUND:

For those unfamiliar with CRCs, suffice to say that they are

very powerful generalizations of the more familiar 'parity check'

schemes, such as those which have been used at least since the

early days of digital magnetic tape recording.

As such, calculating the CRC of a block of data prior to its

transmission and again following transmission, and then comparing

the results of the two calculations, provides an excellent indi-

cation of whether the transmission process introduced any errors

into the data.

The CCITT (for those who enjoy living life the hard way,

CCITT is the designation for the International Consultative

Committee for Telephone and Telegraph, a branch of the United

Nations) has recommended the use of a specific generator poly-

nomial for use in calculating the CRC for data blocks in asyn-

chronous communications. This polynomial is

(X^16 + X^12 + X^5 + 1).

While its recommended use primarily deals with block trans-

mission of data (as in the X/Y/ZMODEM protocols), it has come

into widespread use also as a generator polynomial for file-level

2

CRC. This is therefore the generator polynomial which has been

used for VCOPY.

Those sufficiently interested and/or masochistic as to want

to find additional information regarding the theory of CRC are

referred to:

1. SOBOLEWSKI, J.S., "Cyclic Redundancy Check", in Ralston,

A. (ed.), Encyclopedia of Computer Science and Engineering (2nd

Ed.), Van Nostrand Reinhold Co., 1983, p.434ff

2. PLESS, V., "Introduction to the Theory of Error-

Correcting Codes", John Wiley and Sons, 1982, p. 59ff

Both of these references include bibliography for additional

study.

SYSTEM REQUIREMENTS:

Hardware: PC/XT/AT or close compatible.

Software: DOS 2.0 and above.

INSTALLATION:

If you are using DOS 2.x, VCOPY ***MUST*** be installed in

one of the directories specified in your PATH, and ***MUST NOT***

be renamed to anything else. If you are unfamiliar with the PATH

concept, please refer to your DOS reference manual.

If you are using DOS 3.x or later, these restrictions do not

apply. VCOPY may be installed anywhere, and renamed to anything.

AND FINALLY:

-  The software described herein and the accompanying documen-

tation is copyrighted and the sole the property of W. G.

Madison and Associates, Ltd.

-  W. G. Madison and Associates, Ltd. reserves the right to

make improvements in the product described in this documen-

tation and/or to the documentation at any time and without

notice.

Registration

If you find the program useful, and continue to use it past

a reasonable trial period, you are required to send a check or

money order in the amount of  $10.00 (if foreign funds, at current

exchange rate) to the copyright holder at the address shown at

the top of this file.

For those wishing to use VISA or Master Charge, on-line

registration can be made by calling the Shareware Products BBS at

619/455-5226 between the hours of 7:00AM and 11:00PM (PST).

3

Registration provides the following benefits:

0. Elimination of the annoyance and delay required for the

display of the reminder line.

1. Full Telephone support. We will provide telephone support to

registered users ONLY.

2. Free major updates for a small handling fee

3. Inclusion on our mailing list so that you may receive new

product announcements.

You can register your copy of this program by completing the

accompanying registration form and sending it, with the appro-

priate fee to the address shown.

Support

Registered users are provided with automatic notification of

major updates, and full telephone support. A recorder is provided

on the telephone number listed above, in case no one is in the

office, and a call-back will normally occur no later than the

next business day. Bug fixes or work-arounds will be mailed to

the complainant, if a registered user, and will also be uploaded

to CIS (IBMSW) and to major BBS's in the metropolitan DC area.

Contact

Telephone contact may normally be made between the hours of

7:00AM and 9:00PM (Eastern time), but see above. Non-registered

users are welcome to call, and will be accommodated on a "time

available" basis. Contact may, of course, always be made through

CIS EMAIL on the PPN given above.

License

A limited license is granted to any user of the program to

freely copy and distribute the product subject to the following

restrictions:

You may not:

-  Sell and/or distribute the product for a fee, either alone

or along with other products, without express written per-

mission.

-  Grant sublicense, leases, or other rights in the software to

others.

-  Modify or remove the Copyright notice from the program,

diskette or the documentation.

-  Modify the program, diskette or documentation and/or merge

it into another program for any machine.

Distribution and Warranty

VCOPY is specifically authorized for distribution, under

the SHAREWARE principle, via public electronic bulletin boards or

by individual copying. Any distribution thus made must be in the

form of this entire .ARC file, UNALTERED, including both the

program executable file and this documentation file.

NO OTHER FORM OF DISTRIBUTION, INCLUDING DISTRIBUTION AS

PART OF A LARGER PACKAGE, NOR DISTRIBUTION FOR A FEE (other than

4

a nominal copying charge not to exceed $6.00), IS AUTHORIZED

WITHOUT SPECIFIC WRITTEN PERMISSION OF THE COPYRIGHT HOLDER.

Limited Warranty

THE PROGRAM IS PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND,

EITHER EXPRESSED OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE

IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTIC-

ULAR PURPOSE. THE ENTIRE RISK AS TO QUALITY AND PERFORMANCE OF

THE PROGRAM IS WITH YOU. SHOULD THE PROGRAM PROVE DEFECTIVE, YOU

ASSUME THE ENTIRE COST OF ALL NECESSARY SERVICING, REPAIR OR

CORRECTION. SOME STATES DO NOT ALLOW THE EXCLUSION OF IMPLIED

WARRANTIES SO THE ABOVE EXCLUSION MAY NOT APPLY TO YOU. THIS

WARRANTY GIVES YOU SPECIFIC LEGAL RIGHTS AND YOU MAY ALSO HAVE

OTHER RIGHTS WHICH VARY FROM STATE TO STATE.

Limitation of Remedies

IN NO EVENT WILL W. G. MADISON AND ASSOCIATES, LTD. BE LIABLE FOR

ANY DAMAGES INCLUDING ANY LOST PROFITS, LOST SAVINGS OR OTHER

INCIDENTAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OR

INABILITY TO USE SUCH PROGRAM EVEN IF W. G. MADISON AND

ASSOCIATES, LTD. HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH

DAMAGES OR ANY CLAIM BY ANY OTHER PARTY.

5

Other Shareware Products

You may wish to try these other products of Madison and

Associates:

STORM - A comprehensive hard disk management aid. Provides:

1. Directory listing of all files on the disk,

sorted by NAME.EXT, Directory, or Size

(Decreasing).

2. Directory listing of all potentially duplicate

files on the disk.

3. Global deletion of all files matching a user-

supplied mask.

4. WHEREIS-type function to locate all files match-

ing a user-supplied mask.

5. Summary of disk usage by subdirectory.

6. Visual presentation of the directory structure of

the disk.

BPRINT - A utility to print files with banner pages. Prints

the name and extension in large block letters, fol-

lowed by a date/time stamp of when printed and the

home directory of the file.

QCRC -   A utility to calculate the cyclic redundancy check

value of files. These CRC's may be directed to a

file, if desired. The file may later be used for

verification purposes, and may be freely edited to

include additional text. This combination of proper-

ties makes it extremely useful as a 'Table Of Con-

tents' file for diskettes.

VCOPY - An efficient utility for copying or moving files between

directories or devices, with true read-after-write

verification.

POTPOURI - A collection of (trivial and sophisticated) subpro-

grams for users of TURBO/PASCAL. Includes:

1. Procedures for displaying and reading data from

menus;

2. Transcendental functions for TURBO/BCD;

3. Procedures for manipulating dates, without

restriction (provided only that the dates being

manipulated are *after* October 15, 1582 AD, and

are *before* January 1, 7496237 AD);

4. Procedures for reading the DOS environment space;

5. Procedures for examining and manipulating direct-

ory trees and files; and

6. Some out-of-the-ordinary string manipulation

procedures.

6

WGM&AL Registration & Comment Form

Name:___________________________________________________________

Company:________________________________________________________

Address:________________________________________________________

City:__________________________ State:_____  ZIP _______________

Telephone:(_____) _____-_______

VCOPY Version:_____._____

Comments:

Payment:

Please register my copy of VCOPY

$10.00

(MD residents only) MD 5% Sales Tax ____.__

TOTAL ____.__

Mail To:

W. G. Madison and Associates, Ltd.

P.O. Box 898

Greenbelt MD 20770

(301) 552-7234

7

########################