Software diskette #1

-----------------------------

1.Programmer Files :

EPP512.EXE   : EPROM programmer software.

2716 to 27512,

2516 to 2564,

8764 to 87256,

2816 to 2864A,

CYPRESS CY7C251,7C261,7C271...

NS NMC 27C16 to 27C512...

EPP1024B.EXE : 1 M bit EPROM include

BYTE wide(27010,27101,271000,271001)

PAGE mode(27513,27011..),

MASK ROM pinout EPROM(231024,27301,271000,271001),

BYTE WIDE 2M,4M bits EPROM(TC572000,574000).

EPP1024W.EXE : 1 M bit EPROM include

WORD wide(27210)

BPPGM.EXE    : BIPOLAR PROM NON-register type

32x8 to 4096x8,

CYPRESS CY7C291/292,27C291/292...

SEEP1.EXE    : NS 9306/07/46,93CS06/26/46/56/66.

XC1736/A,MB8541P.(SEE 2. REVISION)

MPU1.EXE     : MPU HITACHI 63701V0, TI 7742,77C82

EEP1.EXE     : GENERAL EEPROM programmer software.

2816,2816A,2817,2864A,28256A,28F256,28F512

PGM48.EXE    : MPU 8748 series.

PGM51.EXE    : MPU 8751 series.

PGMZ8.EXE    : MPU Z8 SERIES,86E11,86E21

EPLD1.EXE    : ALTERA EPLD 310,320,600,900

TI 610,910

FPLP1.EXE    : FPLA series of SIGNETICS

PLS153/A,PLUS153,PLS173,PLUS173...

PLHS18P8A,PLHS16L8A/B.

FPLP2.EXE    : FPLA series of SIGNETICS.

SIGNETICS PLC42VA12.

GAL1.EXE     : Electrial erasable GAL(16V8,20V8,16V8A,20V8A,22V10),

GAL2.EXE     : Electrial erasable PALCE(16V8H/Q,20V8H/Q,22V10H/Q),

GAL3.EXE     : Electrial erasable GAL(6001),

PALP1.EXE    : 1st generation PAL programmer software.

16R8 series,20R8 series...

PALP2.EXE    : 1st generation PAL with polarity fuse.

16RA8 series,20RA10 series...

PALP3.EXE    : new generation EPLD(macro cell output) include

UV erasable PAL(20G10,22V10),

Electrial erasable GAL(16V8,20V8,16V8A,20V8A),

or PEEL(18CV8,22CV10)....(Use GAL1.EXE)

NOTE:

During programming ICT PEEL 18CV8,22CV10, please

add 0.01 UF cap between pin 30 and pin 20.(Use PEEL1.EXE)

PALP4.EXE    : new generation EPLD(macro cell output) include

UV erasable TIBPAL22V10/A/-15,22VP10/-20/-25,

TICPAL18V8,

SIGNETICS PLC16V8,PLC20V8,PLC18V8Z,PLC22V8Z.

PALP5.EXE    : AMD22V10-15/20,ATMEL22V10/10L are added,

user has to add a capacitor 0.01 uF(or greater) between

pin 32 and pin 20.

PEEL1.EXE    : PEEL(18CV8,22CV10,153,173,253,273....)

PEEL2.EXE    : PEEL(20CG10)

EPC.EXE      : ALTERA EPC1064, EPC1213.

2.REVISION of programmer software :

Hardware Revision on  :

VERSION 1 : 1st version hardware.4/15/1990.

current limit resistor from 22 ohm

to 10 ohm.

VERSION 2 : 2nd version hardware.12/1/1990.

Add GND control for PIN 30 to solve

40 PIN word-wide EPROM problom.

EPP512.EXE

V1.00 : 1st released version.

V1.01 : Software bug on 2564 programming is solved

V1.02 : Add a powerful screen editor to replace DEBUG.COM,

please ref. section 5.

V1.03 : NMC 27C256A problem solved.

V1.04 : NMC9817J-35 problem solved.

V3.00 : new version.

V3.10 : 1/1/91, Split EEPROM to EEP1.EXE.

V3.11 : 2/8/91, Add INTEL 87C257.

V3.12 : 3/1/91, Revision of programming algorithm and delay

time for NS Mfr. as well as program voltage

and delay time for CYPRESS.

V3.13 : 4/1/91, Update load file.

Add ATMEL 27C513,27C256R,27C512R,27C513R.

V3.14 : 6/20/91,Add SEEQ 2764,27128,27C256.

V3.15 : 7/1/91, Add MXIC 27C256,27C512,27C1000.

V3.16 : 10/1/91,Add MITSUBISHI M5M27C256AK-I,M5M27C512AK-I.

HITACHI HN27C256AG/HG,HN27C512G.

INTEL D27C256,TI 87C257,NEC D27256.

V3.18 : 12/20/91,Revision of programming algorithm for CYPRESS.

V3.19 : 1/15/92, Add GENERAL 27C513, MICROCHIP 27C513,

INTEL 27C513.

V3.22 : 2/19/92, Modify the TI 27C512 programming voltage from

6.5V to 6.0V.

V3.23 : 3/20/92, Add TOSHIBA TC57256A, MITSUBISHI 27C128K.

Extend the haddisk driver to F for load file.

V3.24 : 4/1/92,  From 4/1/92 this program is also good for

ALL-03A modules.

V3.25 : 4/3/92,  Revision of programming algorithm for

AMD/MMI 2716B.

Revision of programming algorithm for WSI Mfr..

V3.26 : 5/26/92, Add the display of color screen.

Enhance the HEX load file function.

V3.27 : 7/1/92,  Extend the harddisk driver to Z for load file.

V3.31 : 10/1/92, Enhance the HEX load file function.

Add SONY CXK27C256,CXK27C512. NEC 27C256A.

NS NM27LC64,NM27C256/LC256,NMC87C257,

NM27LC512/LV512,NM27P512.

V3.32 : 10/1/92, Modify the programming algorithm for

CYPRESS  7C271.

Add CYPRESS  7C274.

V3.33 : 10/27/92,Add MICROCHIP  27LV256, 27LV512, AMD  27H256.

Modify the programming voltage.

V3.35 : 12/29/92,Modify screen display.

Modify load menu function.

Add AMD  27H256, 27ZLV512,

MICROCHIP  27LV256, 27LV512,

SGS-THOMSON  87C257,

ICT  27CX256

V3.39 : 4/2/93,  Increase the file number of load file functio

to 1200.

Add ATMEL - AT27LV256R, AT27LV512R.

HITACHI - 482764,  INTEL - 27C128.

Modify programming algo of NS - 27CP128.

Add Modify buffer structure function.

Modify INTEL HEX and MOTOROLA HEX modes.

V3.40 : 6/29/93, modify programming algo and screen display.

V3.42 : 7/26/93, modify programming algo.

V3.43 : 9/15/93, modify programmig algo and add

SEEQ - 82005, 86063, 82025,

MICROCHIP - 27LV64,

SGS - M87C512,

INTEL - D68C257.

V3.44 : 12/27/93, modify programming algo.

add OKI  27C256ZB, 27C256ZC, 27C256H,

27C256HZB;

CYPRESS  CY7C277, CY7C279.

V3.45 : 3/11/94, add SGS  27C128A

V3.47 : 6/24/94, modify current test functon.

V3.48 : 8/10/94, modiy blank check function on CYPRESS 27C256.

V3.49 : 8/17/94, modify blank check function on CYPRESS series.

V3.50 : 9/16/94, version update.

V3.51 : 12/7/94, modify programming algo on NS  NM series.

V3.52 : 3/15/95, version update.

EPP1024B.EXE

V1.00 : 1st released version.

V1.01 : Add a powerful screen editor to replace DEBUG.COM,

please ref. section 5.

V1.02 : Add BYTE WIDE 2M,4M bits EPROM(TC572000,574000).

If memory size is not enough for extra 512K BYTES

allocation,this software can not program 4M bits

EPROM.

If memory size is not enough for extra 256K BYTES

allocation,this software can not program 2M bits

EPROM.

If memory size is not enough for extra 128K BYTES

allocation,this software can not program 1M bits

EPROM.

V1.03 : Bug on 27513 content reading is solved.

V3.00 : new version.

V3.08 : 1/1/91, Add new function "COPY file to EPROM",

"READ EPROM to file".

V3.10 : 2/8/91, Enhance S/W to calculate the check sum

while in "EPROM to File" function.

Enhance S/W to calculate the sheck sum

while in "File to EPROM" function.

V3.11 : 3/1/91, Retrieve programmer function.

V3.12 : 4/1/91, Update load file.

Add ATMEL 27C010/L, 27C1024/L, 27C011.

V3.13 : 5/1/91, Transfer file to EPROM by way of drive A or B

can be selected.

V3.14 : 6/1/91, Add ATMEL  AT32C16, WSI  57C65, 57C257, 57C201F

V3.15 : 7/1/91, Add MXIC  27C1000.

V3.18 : 9/5/91, Add FUJISU 27C4001.

V3.20 : 10/1/91,Increase the range of "read EPROM to file".

Increase the range of "copy file to EPROM"

V3.23 : 11/13/91,Enhance S/W to calculate the check sum while

in "EPROM to file" function.

V3.25 : 1/15/92, Add "VERIFY file with EPROM" function.

Seperate special Byte wide portion from

EPP1024.EXE

V3.28 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.30 : 5/26/92,Add the display of color screen.

Enhance the HEX load file function.

V3.31 : 7/1/92, Enhance the modify buffer structure funtion.

V3.32 : 7/13/92,Modify the change function of file buffer.

V3.33 : 10/1/92,Enhance the HEX load file function.

V3.36 : 12/28/92,Modify debug function.

V3.37 : 1/28/93, Add PGMspeed to status column.

V3.39 : 4/2/93, Modify batch file.

Modify save file function.

V3.40 : 7/21/93, modify programming algo.

V3.42 : 7/26/93, modify programming algo.

V3.43 : 9/15/93, add MXIC  27C2000.

V3.44 : 12/27/93, add TOSHIBA  57H1000, 57H1001,

54H1000, 54H1001.

OKI  27C101/H, 27C2000, 27C401.

modify programming algo on ATMEL  27C040.

V3.45 : 3/11/94, add SGS  27V101, 27V201, 27V401.

add M27C516.

V3.47 : 9/16/94, version update.

V3.48 : 12/23/94, modify screen display.

V3.49 : 3/15/95, version update.

EPP1024W.EXE

V3.25 : 1/15/92, Add "VERIFY file with EPROM" function.

Seperate special Word wide portion from

EPP1024.EXE

V3.28 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.30 : 5/26/92,Add the display of color screen.

Enhance the HEX load file function.

V3.31 : 7/1/92, Enhance the modify buffer structure funtion.

V3.28 : 7/13/92,Modify the change function of file buffer.

V3.35 : 10/1/92,Enhance the HEX load file function.

V3.41 : 12/28/92,Modify check sum, buffer structure, and debug

function.

V3.42 : 1/28/93, Add PGMspeed to status column.

V3.45 : 4/2/93, Modify batch file.

Modify save file function.

V3.47 : 5/24/93, add OMNI  27C1024, FUJITSU  27C2048,

PHILIPS  27C240, SGS  27C4002.

V3.48 : 7/21/93, add MXIC  1024, 2048, 4096,

OMNI  27C1024,

FUJITSU  27C2048, 27C4096,

PHILIPS  27C240,

SGS  27C4002.

V3.50 : 9/15/93, support follwoing additional items:

OKI - MSM27C1024AS,

TOSHIBA - 57C096D,

NATIONAL - 27C210, 27LV210.

V3.51 : 12/27/93, add TOSHIBA  54H1024A, 544096, 544200.

modify programming algo.

V3.52 : 3/11/94, add M27C516.

V3.56 : 9/16/94, version update.

V3.57 : 12/23/94, modify screen display.

V3.58 : 3/15/95, version update.

BPPGM.EXE

V3.24 : 4/1/93,  first released version.

V3.25 : 4/30/93, modify programming algo on PHILIPS 82S115, and

AMD 27s series.

V3.26 : 5/24/93, modify programming algo on CYPRESS CY7C265.

V3.27 : 6/29/93, modify programming algo and screen display.

V3.28 : 8/6/93, modify programming algo on ICs of manufacturers:

ATMEL, AMD, WSI, TI, ICT, ASAHI KASEI, PHILIPS,

MICROCHIP, SGS-THOMSON.

V3.30 : 11/4/93, add MITSUBISHI  M54700, M54701,

SHARP  LH5749/J.

V3.31 : 3/11/94, version update.

V3.32 : 6/21/94, add CYPRESS  CY7C225A, CY7C235A, CY7C281A,

CY7C282A, CY7C285.

NEC  uPD27HC65

V3.33 : 9/16/94, modify F1, Home, End keys to operate with load

function.

V3.34 : 11/2/94, modify load HEX file function.

V3.35 : 12/23/94, modisy screen display.

add PHILIPS  82S147B.

V3.36 : 3/15/95, add AMD  27S37, 27S47.

SEEP1.EXE

V3.00 : 1st released version.

V3.03 : for EXPRO-40 hardware modification as follows :

1.During NS93 & XC1736/A series PROM programming,

user has to connect pin 11 to pin 21,

and connect pin 28 to pin 24.

2.During 8541 series PROM programming,

user has to connect and connect pin 28 to pin 24.

V3.08 : 1/1/91.

V3.10 : 2/8/91, Add OKI MSM16911RS

Add MATSUSHITA MN12C201D(must short TEXTOOLS

PIN14 and PIN21).

V3.11 : 3/27/91,Add XICOR X24C04,X24C16.

V3.12 : 4/1/91, Add ICT 93C46/A,93CX46,93C56/CX56,93C66/CX66.

V3.14 : 7/1/91, Add SAMSUNG 93C06,93C07,93C46. XILINX XC1765.

XICOR X2402/I,X2404/I,X24C01/I,X24C02/I.

V3.15 : 8/17/91,Revision of programming algorithm for

XICOR X24C01/I.

V3.16 : 10/1/91,Add XICOR X2444, X2444I, X24C44, X24C44I,

X24C45, X24C45I.

SIEMENS SDA2506,SDA2516.

SGS-THOMSON ST24C02A,ST24C04,ST25C02A,ST25C04

ST93C06,ST93C46A.

V3.18 : 11/15/91,Revision of text on the screen.

V3.19 : 1/15/92, Add NS:95C12, AKM:AK93C46,  GI:PCD8572,

PHILIPS:PCF8582E,PCF8582A, ISSI:IS93C56,

SGS-THOMSON:ST93CS46,ST93CS47,ST93CS56,ST93CS57.

V3.21 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.22 : 5/22/92,Add the display of color screen.

Enhance the HEX load file function.

Add EXEL XL93C46, HYUNDAI HY93C46,

MOTOROLA MCM2801, NS NMC9313B, NMC9314B ,

SONY CXK1012P, CXK1011P,

ATMEL AT24C01/C02/C04/C08/C16,

AT93C46(128*8,64*16), AT93C56(256*8,128*16),

AT93C66(512*8,256*16), AT59C11(128*8,64*16),

AT59C12(256*8,128*16), AT59C13(512*8,256*16).

V3.23 : 7/1/92, Extend the harddisk driver to Z for load file.

Correct the errors in date and readdir.

Add CATALYST CAT35C102(256*8), CAT35C102(128*16).

V3.25 : 10/1/92,Add XICOR - X2444P, X24C44P.

SONY - CXK1013P, CXK1023P,

CXK1024P(256*8,128*16).

Enhance the inspection capability of Display

Card.

V3.26 : 10/6/92,modify memory buffer size.

V3.27 : 12/29/92, Modify screen display.

Add NS: NM93C56/56L, NM93C66/66L, NM93C46A(64x16),

NM93C46A(128x8), NM59C11(128x8), NM59C11(64x16),

NM24C02/02L, NM24C03/03L, NM24C04/04L, NM24C05/05L,

NM24C08/08L, NM24C09/09L, NM24C16/16L, NM24C17/17L;

MICROCHIP:

24C16/LC16, 93LC46(128x8), 93LC46(64x16),

93C/LC56(256x8), 93C/LC56(128x16),

93C/LC66(512x8), 93C/LC66(256x16):

XICOR: X24C00

AT&T: ATT1736, ATT1765, ATT17128

CATALYST:

CAT24C44,

CAT33C104(512x8), CAT33C104(256x16),

CAT33C204(512x8), CAT33C204(256x16),

CAT35C104/H/I(512x8), CAT35C104/H/I(256x16),

CAT35C202/H/I(128x16), CAT35C202/H/I(256x8),

CAT35C204/H/I(256x16), CAT35C204/H/I(512x8),

CAT59C11/A/H/I(128x8), CAT59C11/A/H/I(64x16),

CAT93C46/H/I(128x8), CAT93C46/H/I/A/AH/AI(64x16),

SEIKO:

S-24H/S30R/I, S-24H/S45R/I,

S-2913AR/I, S-2914AR/I, S-2919AR/I, S-2922AR/I,

S-2924AR/I, S-2929AR/I, S-2934AR/I, S-2939AR/I,

S-2917I(64x16), S-2917I(128x8) S-2918I, S-2940I,

S-2961I, S-2980I,

S-2911R(64x16), S-2911R/I(128x8), S-2914R(64x16),

S-2914R(128x8), S-2924R(128x16), S-2924R(256x8),

S-2913CR/I, S-2919CR/I, S-2923CR/I, S-2929CR/I,

S-2933CR/I, S-2939CR/I,

S-2919GR/I, S-2929GR/I, S-2939GR/I,

V3.28 : 2/12/93, SGS-THOMSON - 24C04, modify read function.

V3.29 : 4/2/93, Add AT & T - ATT1736/F, ATT1765/F, ATT17128/F.

Increase the file number of the load file

function to 1200.

V3.30 : 6/2/93, add MICROCHIP  ER5911 (64*16, 128*8),

CATALYST  CAT33C101/I (64*16, 128*8)

XILINX   X1718D, X1736D, X1765D.

modify programming algo on XICOR  X24C45.

V3.31 : 6/29/93, modify programming algo.

V3.32 : 8/3/93, modify programming algo. on

XILINX  XC1718D, XC1736D, XC1765D.

V3.33 : 8/13/93, modify programming algo.

V3.34 : 8/17/93, modify programming algo on ZILINX..

V3.35 : 9/14/93, version update.

V3.36 : 12/10/93, moidify programming algo on ATP765/F.

V3.37 : 3/11/94, add XICOR  X24C16 & version update.

V3.38 : 6/22/94, add XILINX  XC1718L,  XC1736L, XC1765L.

MICROCHIP  24C65.

V3.39 : 7/19/94, add EXEL  XL24C01A, XL24C02, XL24C04,

XL24C04-3, XL24C16.

V3.40 : 8/12/94, add MICROCHIP:

24C08B/LC08B, 24AA01, 24AA02, 24AA04, 24AA08,

24AA16, 24C32, 24LC32, 24AA32, 24LC65, 24AA65.

SGS-THOMSON:

ST24C01, ST24C08, ST25C01, ST25C08.

V3.42 : 9/16/94, modify F1 key to operate with select function.

add SGS-Thomson  ST24C16C, ST93CS66,

modify read function on ZILINX XC17128.

V3.43 : 10/6/94, add  ATMEL  24C164.

modify NS  93CS series.

V3.47 : 12/23/94, modify screen display.

modify write protect function on 93CS series.

add MICROCHIP  24LC21

XICOR  X25C02, X25010, X25020, X25040,

X25160, X25640.

XILINX  XC171280, XC172560.

modify programming algo on MICROCHIP  24C04,

SGS-THOMSON  24C04,

SEIKO  S2980I.

V3.49 : 3/15/95, add XICOR  X25642, X25128.

SEEP2.EXE

V3.00 : 4/2/93, Increase the file number of load file function

to 1200.

V3.01 : 6/29/93, modify programming algo.

V3.02 : 9/14/93, version update.

V3.03 : 3/11/94, version update.

V3.04 : 9/16/94, modify F1 key to operate with select function.

V3.05 : 12/23/94, modify screen display.

V3.07 : 3/15/95, add PHILIPS  PCF8522E,

SGS-THOMSON  ST24E32, ST25E32.

MPU1.EXE

V3.00 : 1st version.

V3.01 : 1/1/91.

V3.02 : 4/1/91, Update load file.

V3.03 : 7/1/91, NEC UPD77P25.

V3.04 : 8/29/91,Revision of programming voltage for TMS77C82.

V3.05 : 10/1/91,TMS77C82 additional program protect bit

function.

Revision of text on the screen.

V3.07 : 1/15/92.

V3.09 : 2/11/92,Prohibit the programming on dummy address

(4n+3) of uPD77P25.

V3.10 : 3/20/92,Extend the harddisk driver to F for load file.

V3.11 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

V3.12 : 4/28/92,Revision of TMS77C82 verify funtion.

V3.15 : 5/26/92,Add the display of color screen.

Enhance the HEX load file function.

Modify the I/O change function.

V3.16 : 7/1/92, Extend the harddisk driver to Z for load file.

V3.17 : 10/1/92,Enhance the inspection capability of Display

Card.

V3.18 : 11/18/92,modify programming algorithm of

HITACHI 63701V0.

V3.19 : 12/29/92,Modify screen display.

Modify batch file function.

V3.20 : 4/2/93, Increase the file number of load file function

to 1200.

V3.21 : 6/29/93, modify programming algo.

V3.22 : 9/14/93, version update.

add HITACHI  HD637A01V0, HD637B01V0.

V3.23 : 3/11/94, version update.

V3.24 : 9/16/94, modify F1 key to operate with select function.

V3.25 : 12/23/94, modify screen display.

V3.27 : 3/15/95, modify programming algo.

EEP1.EXE

V3.00 : 1st version.1/1/91.

V3.01 : 2/8/91, Add HITACHI HN58C256

Add INTEL   28F010, 28F202

Add SEEQ    48F512, 48F010

Add HUGHES  H3300

V3.02 : 3/1/91, Revision of text on the screen.

Revision delay time for SEEQ 2816, 2817, 2864.

V3.03 : 4/1/91, Update load file.

Add ATMEL 28C04, 28MC010, 28C010, 28C1024,

28C191/L, 28C292/L.

EXEL XL46C15.

V3.05 : 7/1/91, Update load file.

Add AMD 28F256, 28F010, 28F020. SAMSUNG  28C256.

SEEQ 52B13/H, 52B33/H, 2804A, 36C16, 36C32,

38C16, 38C32, 47F512, 47F010.

XICOR 2804A/I, 2816C/I, 28C64/I, 28C512/I,

28C010/I.

V3.06 : 8/12/91,Add Data Protection function.

XICOR 28C64/C64I, 28C512/C512I,

28C256/C256I, 28C010/C010I.

V3.07 : 10/1/91,Add ATMEL AT29C256, AT29C257, AT29C010.

INTEL 28F001BX-B, 28F001BX-T.

XICOR X2201A, X2001/I, X2004/I/M.

V3.09 : 1/15/92,Add XICOR:2816A/16AI, X88C64/C64I.

DALLAS:DS1220AB/AD, DS1225D/E, DS1230Y/AB.

MITSUBISHI:M5M28F010.

V3.10 : 2/19/92,Modify the ATMEL AT29C010 page from byte to 128

byte.

V3.11 : 3/20/92,Add TI 29F256, 29F258, 29F259, 29F512, 29F010.

Extend the harddisk driver to F for load file.

V3.12 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

V3.13 : 4/3/92, Add data protection function for

ATMEL AT28C256/HC256/L AT28MC010/C010.

V3.14 : 4/21/92,Modify the erase order of Flash EEPROM of AMD and

INTEL.

V3.15 : 5/26/92,Add the display of color screen.

Enhance the HEX load file function.

Modify the I/O change function.

V3.16 : 7/1/92, Extend the harddisk driver to Z for load file.

Allow file to be assigned as buffer.

Modify the protection of page mode.

Add AMD  29F010 ,HITACHI HN58C66 ,

BOWMAR WHITE  8023.

V3.17 : 10/1/92,Add ASAHI-KASEI AK28C64.

Enhance the HEX load file function.

Revision of programming algorithm.

Enhance the inspection capability of Display

Card.

V3.18 : 10/14/92,add SGS-THOMSON MK48T02B/12B, MK48T08B/18B.

V3.19 : 10/19/92, add the data protection to AMD 29F010.

V3.20 : 10/21/92,add data unprotection function to AMD 29F010.

V3.21 :  12/29/92,Modify screen display.

Modify check sum unit.

Modify batch file function.

Modify load HEX file function.

Add ATMEL - AT29C512, AT29LV512, AT29C040

CATALYST - CAF28065A, CAT28C256,

SEIKO - S2840R/A, S28F512A,

HITACHI - HN58C1000, HN29C101, HN29C101B,

HN28F4001

V3.22 : 1/19/93, AMD - 29F010, modify data protection function.

Add ATMEL - AT28MC020, ATMC040

CATALYST - CAT28F020

AMD - 2864BE, 9864, 28C256

V3.23 : 2/3/93, Add SST 28EE011.

V3.24 : 4/2/93, Add HITACHI - 28F101, DALLAS - DS1245Y/AB

AMD - 29F010, add sector erase function.

SAMSUNG - KM28C64/C65, add page mode programming.

Modify erase function for HITACHI - 28F4001

and AMD - 28F010, 28F020.

Increase file number of the load file function

to 1200.

V3.29 : 8/13/93, modify programming algo.

V3.30 : 9/12/93, modify programming algo on HITACHI, DALLAS.

add DALLAS - DS1630Y/AB, DS1645Y/AB, DS1650Y/AB.

DS1243Y, DS1244Y, DS1248Y, DS1235Y.

HITACHI - 58C1001, 58V1001.

SEIKO - 52812A, 52817A, 52860A,52864A.

WINBOND - W27E256.

V3.31 : 1/4/94, modify programming algo and erase function.

add SGS-THPMSOM  28F256A, 28F102, 28F201,

28C64C, MK48C02A/12A, MKI48Z02/12,

MK48Z08/18, MK48Z/09/19, MKI48Z18,

MK48Z30/Y, MK48Z32/Y, MK48Z128/Y,

MK48Z256/Y, MK48Z512Y, MK46Z128Y, MK46Z256/Y.

WINBOND   29EE011;

EXEL  XL28C16B, 28C64, 28C64B, 28C256;

XICOR  28HC16, 28HC256/VC256, XM28C040,

XM28C4096, X20C04, X20C05, X20C16,

X68C64, X86C64, XM20C64, XM20C64FR;

AMD  29F040.

V3.32 : 3/11/94, add WINBOND  27F010.

V3.33 : 5/13/94, add ISSI  28F010,   ATMEL  29C020,

modify programming command on MXIC 28F1000,

28F4000, modify block protect function on 8864,

modify disable protect function on 28HC64.

V3.34 : 9/16/94, add WINBOND  W27F512

ATMEL  AT29LV256, 29LV010, 29LV040

TI  TMS28F210

AMD  AM28F256A, AM28F512A;

modify F1, Home, End keys to operate with load

function.

V3.35 : 10/17/94, add ATMEL  AT28LV256,

SST  29LE010, 29EE512.

modify erase function.

V3.36 : 12/23/94, modify screen display.

add WINBOND  78T064,

DALLAS  12877,

ATMEL  AT29LV020.

modify programming algo on

INTEL  2816,

XICOR  20C04, 20C05, 20C16,

ATMEL  28C1024

V3.37 : 2/28/95, modify load file function.

add SST  28SF040,

WINBOND  27E256,

ATMEL  29C1024.

V3.38 : 3/15/95, modify load file function.

PGM48.EXE

V1.00 : 1st released version.

V1.02 : Add a powerful screen editor to replace DEBUG.COM,

please ref. section 5.

V3.00 : new version.

V3.02 : 1/1/91.

V3.03 : 4/1/91, Update load file. Add 8742.

V3.04 : 7/1/91, Revision of text on the screen.

V3.05 : 10/1/91,Revision of text on the screen.

V3.07 : 1/15/92.

V3.10 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the harddisk driver to F for load file.

V3.11 : 5/26/92,Add the display of color screen.

Enhance the HEX load file function.

V3.12 : 7/1/92, Extend the harddisk driver to Z for load file.

V3.13 : 7/14/92,Enhance the inspection capability of Display

Card.

V3.14 : 10/1/92.

V3.16 : 12/29/92,Modify programming voltage.

Modify screen display.

V3.18 : 4/2/93, Increase the file number of load file function

to 1200.

Modify save file function.

V3.20 : 7/14/93, add '/BW' function.

add type 87C42.

V3.21 : 9/14/93, version update and modify HEX code.

V3.22 : 3/11/94, version update.

V3.23 : 9/16/94, modify F1 key to operate with select function.

V3.24 : 12/23/94, modify screen display and programming algo.

V3.25 : 3/15/95, version update.

PGM51.EXE

V1.00 : 1st released version.

V1.02 : Add a powerful screen editor to replace DEBUG.COM,

please ref. section 5.

V1.03 : Bug in 8752BH reading is solved.

V3.00 : new version.

V3.06 : 1/1/91.

V3.07 : 4/1/91, Add SIGNETICS 87C652,87C654.

V3.08 : 7/1/91, Add AMD 87C52T2, SIGNETICS 87C528.

V3.09 : 8/27/91,Increase the programming capacity for 87C528 from

16K to 32K.

V3.11 : 10/1/91,Add INTEL 87C75PF, 87C54, 87C58.

Revision of text on the screen.

Revise the programming method of 87C51FA and

87C51FB to the same as that of 87C51C.

Revise the Lock Bit display method for

87C51FA/B/C, 87C54 and 87C58.

V3.13 : 1/15/92.Add INTEL 87C51(FX-CODE)

NEC 8755A

TOSHIBA 8755A

V3.16 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the harddisk driver to F for load file.

V3.17 : 4/16/92,Change the encryption code of INTEL 87C51 to

64 byte.

V3.18 : 5/22/92,Add the display of color screen.

Enhance the HEX load file function.

Change the encryption code of SIGNETICS 87C528

to 64 byte.

V3.19 : 7/1/92, Extend the harddisk driver to Z for load file.

Correct the errors in date and readdir.

V3.20 : 10/1/92,Revision of programming algorithm for

INTEL 87C75PF.

Enhance the inspection capability of Display

Card.

V3.21 : 10/12/92,enhance the HEX file load function.

V3.22 : 12/29/92,Modify screen display.

Modify batch file function.

Modify load file function.

V3.23 : 1/29/93, Modify I/O address fetch of FILE.DAT

V3.24 : 4/2/93, Increase the file number of load file function

to 1200.

Modify manufacturer YDC as ADRITE.

Modify manfuacturer SIGNETICS as PHILIPS.

Modify PHILIPS - 87C51 voltage.

Add PHILIPS - 87C524, 87C51FB.

V3.25 : 6/29/93, modify screen display,

add PHILIPS 87C575.

V3.26 : 7/23/93, add ATMEL  AT89C51.

V3.27 : 9/14/93, version update.

V3.28 : 10/28/93, add INTEL  87C52.

V3.29 : 11/16/93, add PHILIPS  87C51FA.

V3.30 : 3/11/94, version update.

V3.31 : 9/16/94, modify F1, Home, End keys to operate with load

function.

add ATMEL  AT89C52,

PHILIPS  87C504, 87C51FC.

V3.32 : 10/3/94, add DALLAS  DS87C520.

V3.33 : 12/7/94, add ATMEL  AT89C2051.

V3.34 : 12/23/94, modisy screen display.

add INTEL  87L52, 87L54, 87L58,

87L51FA, 87L51FB, 87L51FC.

V3.35 : 3/15/95, add ATMEL  89LV51, 89C1051.

PGMZ8.EXE

V1.00 : 1st released version.

V3.00 : new version.

V3.03 : 1/1/91.

V3.04 : 4/1/91, Update load file.

V3.05 : 7/1/91, Revision of text on the screen.

V3.06 : 8/1/91, Revision programing function.

V3.07 : 10/1/91,Revision of text on the screen.

V3.09 : 1/15/92.

V3.12 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the harddisk driver to F for load file.

V3.13 : 4/10/92,Revision programming function of "Load File".

V3.14 : 5/26/92,Add the display of color screen.

Enhance the HEX load file function.

V3.15 : 7/1/92, Extend the harddisk driver to Z for load file.

V3.17 : 10/1/92.

V3.18 : 11/9/92,Add ZILOG   Z86E30, Z86E23.

V3.19 : 12/29/92,Modify screen display.

V3.21 : 4/2/93,  Increase the file number of load file functio

to 1200.

Modify save file function.

V3.22 : 7/14/93, add '/BW' function.

V3.23 : 9/14/93, version update and modify HEX code.

V3.24 : 3/11/94, version update.

V3.26 : 9/16/94, modify F1 key to operate with select function.

modify RAM protect and RC oscillator select

function.

delet ZILOG  Z86E11, Z86E21, add Z86E31.

V3.28 : 12/23/94, modify screen display and address of Z86E30.

V3.30 : 3/15/95, modify programming algo.

EPLD1.EXE

V3.00 : 1st released.

V3.14 : 1/1/91.

V3.16 : 4/1/91.

V3.17 : 5/1/91, Revision of Blank check.

V3.18 : 7/1/91, Revision of text on the screen.

Revision of NON-TURBO ADDER.

Add INTEL 85C224,5AC312.

V3.19 : 7/3/91, Revision of text on the screen.

V3.20 : 7/11/91,Revision of device type for ALTERA Mfr.

Improve INTEL 85C312 programing ALGO.

Improve TI,ALTERA EP330/630 programing pulse.

V3.21 : 8/6/91, Revision of text on the screen.

Revision of programming voltage for TI EP330/630.

V3.23 : 12/10/91,Add ID code for EP330.

V3.24 : 12/19/91,Revision of programming voltage for TI EP330/630.

V3.25 : 1/15/92, Add ALTERA EP910-30T/-35T/-40T.

Revision of programming voltage for TI EP910.

V3.28 : 2/24/92, Improve TI EP330/630 programming ALGO.

V3.29 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.30 : 5/22/92,Add the display of color screen.

Revision of programming algorithm for TI EP610,

EP330.

V3.31 : 7/1/92, Extend the harddisk driver to Z for load file.

Make JEDEC file comply with DATA I/O specifications.

V3.32 : 7/29/92,Revision of programming voltage for TI EP330/630.

V3.33 : 10/1/92,Revision of programming algorithm for TI EP6XX,

Enhance the inspection capability of Display

Card.

V3.34 : 12/30/92,Modify screen display.

Modify programming algorithm of

TI  EP630,  INTEL  85C060, 85C090, and

ALTERA  EP610-25/-20/-15.

V3.35 : 2/10/93, Modify AUTO function.

INTEL - 5AC312, modify programming algo.

INTEL - 5C031, modify verify function.

V3.36 : 4/2/93, Modify batch file function.

Modify manufacturer YDC as ADRITE.

Increase the file number of load file function

to 1200.

Modify quit screen.

V3.38 : 7/23/93, add INTEL  iPLD610, iPLD910,

ALTERA  EP910A.

modify programming algo.

V3.40 : 9/15/95, modify programming algo.

add CYPRESS   CY7B326(PLD610).

V4.42 : 1/4/94, modify programming algo.

V3.45 : 3/11/94, modify programming algo.

V3.46 : 9/16/94, modify load file function with F1, Home, End keys.

modify rerify voltage on ALTERA EP610, EP910A.

V3.47 : 12/27/94, modify screen display and programming algo.

V3.48 : 3/16/95,  modify programming algo.

FPLP1.EXE

V1.00 : 1st released version of FPLA SERIES.

FOR SIGNETICS PLS153/A,PLS173,PLUS153,PLUS173.

V1.01 : Add signetics PLHS18P8A,PLHS16L8A/B.

NOTE : PLHS18P8A,16L8A/B Blank fuses are opened.

In JEDEC fuse map, We define opened fuse to be

'1'.

But in PLHS18P8A, XOR fuse configuration are

inversed from JEDEC AND AMD18P8 configuration,

so we define Blank xor fuse to be '0' and at

low(or invert) output polarity.

V1.02 : Add AmdPAL18P8.

V3.00 : new version.

V3.03 : 1/1/91.

V3.06 : 7/1/91, Revison of text on the screen.

V3.08 : 10/1/91,Revision of text on the screen.

V3.11 : 12/6/91,Revision of LOAD file function.

V3.13 : 1/15/92,Revision of text on the screen.

Revision of PLS153A, PLHS18P8 programming

voltage.

V3.17 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.19 : 7/1/92, Extend the harddisk driver to Z for load file.

Make JEDEC file comply with DATA I/O

specifications.

V3.20 : 10/1/92,Enhance the inspection capability of Display

Card.

V3.21 : 12/30/92,Modify screen display.

V3.22 : 4/2/93, Modify batch file function.

Modify manufacturer YDC as ADRITE.

Modify manufacturer SIGNETICS as PHILIPS.

Increase the file number of load file function

to 1200.

Modify quit screen.

V3.23 : 7/23/93, add '/BW' function.

V3.24 : 9/15/93, modify save function.

V3.25 : 1/4/94, modify programming algo.

V3.26 : 9/16/94, modify load file function with F1, Home, End keys.

V3.27 : 12/27/94, modify screen display and programming algo.

V3.28 : 3/16/95, modify programming algo.

FPLP2.EXE

V3.00 : 1st released version of FPLA SERIES.

6/3/91, SIGNETICS PLC42VA12.

V3.02 : 10/1/91,Revision of text on the screen.

V3.04 : 12/6/91,Revision of LOAD file function.

V3.05 : 1/15/92, Revision delay time for PLC42VA12.

V3.06 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.10 : 5/22/92,Add the display of color screen.

V3.11 : 7/1/92, Extend the harddisk driver to Z for load file.

Make JEDEC file comply with DATA I/O

specifications.

V3.12 : 10/1/92,Enhance the inspection capability of Display

Card.

V3.13 : 12/30/92,Modify screen display.

V3.14 : 4/2/93, Modify batch file function.

Modify manufacturer YDC as ADRITE.

Modify manufacturer SIGNETICS as PHILIPS.

Increase the file number of load file function

to 1200.

Modify quit screen.

V3.15 : 7/23/93, add '/BW' function.

add PHILIPS  PLC153.

V3.16 : 9/15/93, modify save function.

V3.17 : 3/11/94, version update.

V3.18 : 9/16/94, modify load file function with F1, Home, End keys.

V3.19 : 12/27/94, modify screen display and programming algo.

V3.20 : 3/16/95, modify programming algo.

GAL1.EXE

V3.00 : 1st released.

V3.15 : 1/1/91. New version GAL.

V3.16 : 2/8/91, Correct the problem that the characters(P and H)

can't be typed while executes "LOAD File"

function.

V3.17 : 4/1/91, Update load file.

Add LATTICE 16V8/B,20V8/B,22V10USE,22V10(NO USE),

20RA10USE,20RA10(NO USE),18V10/B.

V3.18 : 7/1/91, Add NS GAL16V8A/QS/-7,GAL20V8A/QS/-7,GAL20RA10.

V3.19 : 8/6/91, Revision of text on the screen.

Revision of programming voltage NS Mfr.

V3.20 : 10/1/91,Revision of text on the screen.

Revision of programming algorithm and delay

time for GAL16V8A,GAL20V8A.

Add SGS-THOMSON GAL16V8S,GAL20V8S.

V3.21 : 10/30/91,Revision of ID code for NS GAL20RA10.

V3.22 : 11/15/91,Revision of NS 20RA10 programming voltage.

V3.26 : 1/15/92, Revision of DEVICE code for NS 20V8.

V3.29 : 3/4/92, Modify the "ERASE" in the Auto function.

V3.30 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.31 : 4/29/92,Add LATTICE GAL20XV10/B.

V3.32 : 5/12/92,Modify the compatible program for NS 16V8/20V8 and

16V8A/20V8A, that have the same device code.

V3.33 : 5/22/92,Add the display of color screen.

V3.34 : 6/19/92,Extend the harddisk driver to Z for load file.

V3.35 : 7/9/92, Revision of text on the screen.

V3.36 : 8/27/92,Revision of NS and LATTICE programming voltage.

V3.37 : 10/1/92.

V3.38 : 1/12/93.

V3.39 : 3/4/93, Modify delay time operation.

Modify buffer range of the load file function.

Enhance the key batch file function.

Modify the screen of QUIT function.

Modify ERASE function of GAL22V10.

V3.40 : 4/2/93, Increase file number of the load file function

to 1200.

Modify manufacturer YDC as ADRITE.

Modify manufacturer SIGNETICS as PHILIPS.

V3.41 : 7/20/93, modify programming algo.

add LATTICE  GAL16V8C,

SGS  GAL16AS, GAL20AS.

V3.42 : 9/14/93, modify JEDEC format.

add LATTICE  GAL16V8Z, GAL20V8Z, GAL20V8C,

GAL22V10B-QP.

V3.43 : 10/2/93, add LATTICE  GAL22V10C,

modify programming algo for

NS GAL20RA10, GAL16V8B.

V3.44 : 12/30/93, modify programming algo.

V3.45 : 3/10/94, modify programming & version update.

V3.46 : 4/1/94, modify specification on LATTICE  GAL18V10.

V3.47 : 6/29/94, add LATTICE  GAL16VP8B, GAL20VP8B,

modify LATTICE  GAL16V8B, GAL20V8B.

V3.48 : 9/16/94, version update.

V3.49 : 12/29/94, modify screen display and programming algo.

V3.50 : 3/15/95, modify programming algo.

GAL2.EXE

V3.12 : 1st released.1/1/91.

V3.13 : 2/8/91, Correct the problem that the characters(P and H)

can't be typed while executes "LOAD File"

function.

V3.14 : 4/1/91, Update load file.

Add AMD PALCE22V10Z.

V3.15 : 7/1/91, Add PALCE16V8Z.

V3.17 : 10/1/91,Add AMD PALCE610H.

V3.18 : 10/30/91,Revision of ID code for 22V10. Add PALCE22V10/4.

V3.21 : 12/19/91,Revision of PALCE22V10 programming voltage.

V3.22 : 1/15/92.

V3.24 : 3/4/92, Modify the "ERASE" in the Auto function.

Add NS 16V8-10, 20V8-10.

V3.26 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.27 : 4/20/92,Add AMD PALCE29M16/4 PALCE29MA16/4.

V3.28 : 5/22/92,Add the display of color screen.

V3.29 : 5/29/92,Revision of programming voltage for AMD

PALCE29M16/4 ,PALCE29MA16/4.

V3.30 : 6/19/92,Extend the harddisk driver to Z for load file.

V3.31 : 7/1/92, Revision of AMD PALCE22V10H-25/4 programming

voltage.

V3.32 : 7/9/92, Revision of LATTICE GAL22V10/B programming voltage.

Revision of text on the screen.

V3.34 : 8/12/92,Add AMD PALCE16V8H/5.

V3.36 : 8/12/92,Revision of text on the screen.

V3.38 : 10/1/92.

V3.40 : 11/9/92,modify programming algorithm of

AMD PALCE22V10H/Q/4/5.

V3.41 : 12/29/92,Add AMD  PALCE20V8/5.

V3.42 : 3/4/93, Modify delay time operation.

Modify buffer range of the load file function.

Enhance the key batch file function.

Modify the screen of QUIT function.

Modify ERASE function of GAL22V10.

V3.43 : 4/2/93, Increase file number of the load file function

to 1200.

Modify manufacturer YDC as ADRITE.

Modify manufacturer SIGNETICS as PHILIPS.

V3.44 : 4/21/93, add AMD PALCE20RA10H.

modify programming algo on 16V8HD.

change names of PALCE29M16/4 to PALCE29M16H/4.

PALCE29MA16/4 to PALCE29MA16H/4.

V3.45 : 7/20/93, add '/BW' function.

V3.47 : 9/14/93, modify JEDEC format.

add AMD  PALCE20RA10H/Q/4.

V3.48 : 12/30/93, modify programming algo on AMD PALCE22V10.

V3.49 : 3/10/94, modify programming & version update.

V3.50 : 6/29/94, add AMD  PALLV16V8, PALLV16V8Z, PALLV22V10Z.

V3.51 : 7/22/94, add AMD  PALLV22V10,

modify programming algo on PALLV22V10Z.

V3.52 : 9/16/94, version update.

V3.53 : 12/29/94, modify screen display and programming algo.

V3.54 : 3/15/95, modify programming algo.

add PALCE20V8H(Q)/4(5).

GAL3.EXE

V3.10 : 1st released.1/1/91.

V3.11 : 2/8/91, Correct the problem that the characters(P and H)

can't be typed while executes "LOAD File"

function.

V3.12 : 4/1/91, Update load file.

V3.13 : 8/6/91, Revision of text on the screen.

Revision of programming voltage for NS Mfr.

V3.14 : 10/1/91,Revision of programming algorithm and delay time.

V3.18 : 12/19/91,Revision of text on the screen.

V3.19 : 1/15/92.

V3.22 : 3/4/92, Modify the "ERASE" in the Auto function.

V3.23 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.24 : 5/22/92,Add the display of color screen.

V3.25 : 6/19/92,Extend the harddisk driver to Z for load file.

V3.26 : 10/1/92.

V3.27 : 1/1/93.

V3.28 : 3/5/93, Modify delay time operation.

Modify buffer range of the load file function.

Enhance the key batch file function.

Modify the screen of QUIT function.

Modify AMD - AmPAL20L10 security algorithm.

V3.29 : 4/2/93, Increase file number of the load file function

to 1200.

Modify manufacturer YDC as ADRITE.

Modify manufacturer SIGNETICS as PHILIPS.

Add LATTICE - GAL6001B#, GAL6002B#.

V3.30 : 7/20/93, add '/BW' function.

V3.31 : 9/14/93, modify JEDEC format.

V3.32 : 12/30/93, modify programming algo.

V3.33 : 3/10/94, modify programming & version update.

V3.34 : 9/16/94, version update.

V3.35 : 12/29/94, modify screen display and programming algo.

V3.36 : 3/15/95, version update.

PALP1.EXE

V1.00 : 1st released version.

V1.01 : clarify type of TIBPAL.

V1.02 : increase 20L8B-2/D,20R8B-2/D,20R6B-2/D,20R4B-2/D.

JEDEC file saving error on 10H8,10L8 are solved.

V1.03 : add old 16L8-12/-15/-25 series security fuse programming

(21 volt).

new 16L8-12/-15/-25 series security fuse programming

voltage is 14 volts.

V1.04 : Add SIGNETICS PLUS16L8/R4/R6/R8,PLUS20L8/R4/R6/R8

V2.00 : TIBPAL 16L8-7/H-15,R4-7/H-15,R6-7/H-15,R8-7/H-15 included

V3.00 : new version.

V3.14 : 1/1/91.

V3.16 : 2/8/91, Add TI TIBPAD16N8-7,TIBPAD18N8-6

TIBPAL16xx-5. xx=L8,R4,R6,R8.

TIBPAL20xx-5. xx=L8,R4,R6,R8.

Improve AMD PAL16L8-5/-7/H-10 programming ALGO.

Correct the problem that the characters(P and H

) can't be typed while executes "LOAD File"

function.

V3.17 : 4/1/91, Update load file.

V3.18 : 6/28/91,Revision of programming voltage for TI Mfr.

V3.19 : 7/1/91, Revision of programming algorithm for

TI 20l8-15/25, MMI 16l8D/B.

V3.20 : 8/27/91,Revision of text on the screen.

Revision of programming algorithm for NS PAL.

V3.24 : 1/15/92,Revision of TIBPAL20R4/R6/R8/L8-5,TIBPAL16N8-7

programming voltage.

V3.27 : 3/4/92, Modify the TIBPAL16XX-5/20XX-5 programming

voltage.

V3.28 : 3/20/92,Modify the SIGNETICS PLUS16XXd/-7 ,20XXD/-7

secunty programmer.

V3.29 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.30 : 4/29/92,Add CYPRESS PAL16L8/B ,PAL16R8/B ,PAL16R6/B ,

PAL16R4/B.

V3.31 : 5/12/92,Revision CYPRESS PAL16L8-5/-7/B ,PAL16R8-5/-7/B ,

PAL16R6-5/-7/B ,PAL16R4-5/-7/B .

V3.32 : 5/22/92,Add the display of color screen.

V3.33 : 5/29/92,Revision of programming voltage for AMD PAL16L8-5

V3.34 : 6/19/92,Add SAMSUNG CPL16L8/R8/R6/R4 ,CPL20L8/R8/R6/R4

CPL20L10.

Extend the harddisk driver to Z for load file.

V3.35 : 7/1/92, Revision of programming voltage for AMD PAL16L8-5

series.

V3.37 : 7/29/92,Revision of programming voltage for TI

TIBPAL16XX-7/-10/-12/-15/-25,TIBPAL20XX-7/-10,

TIBPAL20XX-15/-25.

V3.38 : 9/3/92, Revision of programming voltage for AMD

PALCE20L8-10PC, TI TIBPAL20L8-15, MMI PAL16RBCN.

V3.39 : 10/1/92.

V3.40 : 11/9/92,modify programming algorithm of

AMD PAL16L8-5 series and SAMSUNG CPL24 series.

V3.41 : 1/1/93.

V3.42 : 1/14/93, AMD - AmPAL16xx - modify programming algorithm.

V3.43 : 2/10/93, TI - TIBPAL16R8-15NC, modify programming algo.

V3.44 : 3/5/93, Modify delay time operation.

Modify buffer range of the load file function.

Enhance the key batch file function.

Modify the screen of QUIT function.

Modify AMD - AmPAL20L10 security algorithm.

V3.45 : 4/2/93, Increase file number of the load file function

to 1200.

Modify manufacturer YDC as ADRITE.

Modify manufacturer SIGNETICS as PHILIPS.

V3.46 : 4/21/93, modify programming algo on CYPRESS ICs.

V3.47 : 6/10/93, modify programming algo on

AMD  PAL20xx-7,  TI  PALxx-15.

modify mfr. and type names of AMD.

V3.49 : 7/20/93, add '/BW' function.

V3.50 : 9/14/93, modify JEDEC format.

V3.51 : 10/2/93, modify programming algo on NS  PAL16L8A series.

V3.52 : 12/30/93, modify programming on TI and MMI items.

V3.53 : 1/27/94, modify programming algo.

V3.54 : 3/10/94, modify programming & version update.

V3.55 : 6/29/94, modify programming algo on MMI PALC20XX.

V3.56 : 7/22/94, modify programming algo on TI  TIBPAL16L8-10.

V3.57 : 9/16/94, version update.

V3.58 : 12/29/94, modify screen display and programming algo.

V3.59 : 3/15/95, modify programming algo.

PALP2.EXE

V3.00 : new version.

V3.11 : 1/1/91.

V3.12 : 2/8/91, Correct the problem that the characters(P and

H) can't be typed while executes "LOAD File"

function.

V3.13 : 4/1/91, Update load file.

V3.15 : 7/1/91, Add SEEQ PQ20RA10Z-45.

V3.16 : 8/27/91,Revision of programming algorithm for NS 20RA10.

V3.18 : 12/19/91,Add RICOH EPL16P8B.

Add capacity for "Load file".

V3.19 : 1/15/92,Add RICOH EPL16P8B.

V3.22 : 3/20/92,Add TI TIB82S167.

V3.23 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.24 : 5/22/92,Add the display of color screen.

V3.25 : 6/19/92,Extend the harddisk driver to Z for load file.

V3.26 : 7/29/92,Revision of programming voltage for TI TIB82S167.

V3.27 : 10/1/92.

V3.28 : 1/1/93.

V3.29 : 3/5/93, Modify delay time operation.

Modify buffer range of the load file function.

Enhance the key batch file function.

Modify the screen of QUIT function.

Modify AMD - AmPAL20L10 security algorithm.

V3.30 : 4/2/93, Increase file number of the load file function

to 1200.

Modify manufacturer YDC as ADRITE.

Modify manufacturer SIGNETICS as PHILIPS.

V3.31 : 4/21/93, modify programming algo on TIB82S167.

V3.32 : 7/20/93, add '/BW' function.

V3.33 : 9/14/93, modify JEDEC format.

V3.34 : 10/2/93, modify programmig algo on

NS PAL16RA8 and 20RA8 series.

V3.35 : 3/10/94, modify programming & version update.

V3.36 : 8/4/94, modify programming algo on RICOH EPL16RP8B series.

V3.37 : 9/16/94, version update.

V3.38 : 12/29/94, modify screen display and programming algo.

V3.39 : 3/15/95, version update.

PALP3.EXE

V1.00 : 1st released version.

V1.01 : solve AUTO function bug.

V1.02 : solve GAL 16V8,20V8 JEDEC file loading error at UES,PTD

V1.03 : solve GAL 16V8,20V8 programming at 1's and 0's.

We made a mistake on GAL JEDEC file in 1's and 0's

definition,the fuses were programmed to it's inverted

status at V1.02 or less version.

We are very appreciated that MICROWARE,EAGLE,BLISS,BALTEC

payed their attention and help us to solve these

problems.

V1.04 : ICT and HYUNDAI PEEL 18CV8 are available now.

V1.05 : ICT PEEL 22CV10 are available now.

V1.06 : SECURITY VOLTAGE OF ICT 18CV8 down to 15.0 VOLTS.

ADD TICPAL22V10.

ADD SAMSUNG 22V10.

ADD VLSI,SGS GAL16V8 and GAL20V8.

ADD AMI,GOULD PEEL18CV8.

V1.07 : A VERSION GAL(16V8A,20V8A) ARE AVAILABLE.

V3.00 : new version.

V3.13 : 1/1/91.

V3.14 : 2/8/91, Correct the problem that the characters(P and H

) can't be typed while executes "LOAD File"

function.

V3.15 : 4/1/91, Update load file.

V3.16 : 7/1/91, Add CYPRESS PLDC20G10/A,PALC22V10/A,PLDC20G10B,

PALC22V10B.

V3.17 : 8/16/91,Revision of programming voltage for

CYPRESS PLDC20G10B,PALC22V10B.

V3.18 : 10/1/91,Add CYPRESS PLD20RA10.

V3.20 : 1/15/92.

V3.23 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

Add CYPRESS 22V10C,22VP10C.

V3.24 : 5/22/92,Add the display of color screen.

Enhance the HEX load file function.

V3.25 : 7/1/92, Extend the harddisk driver to Z for load file.

Correct the errors in date and readdir.

V3.26 : 10/1/92,Enhance the inspection capability of Display

Card.

V3.27 : 12/29/92,Modify screen display.

Modify batch file function.

V3.28 : 1/29/93, Modify I/O address fetch of FILE.DAT

V3.29 : 4/2/93, Increase the file number of load file function

to 1200.

Modify manufacturer YDC as ADRITE.

Modify manfuacturer SIGNETICS as PHILIPS.

Add CYPRESS - PALC22V10D.

AMD - AmPAL22V10-7, modify programming algo.

V3.30 : 4/28/93, modify programming algo on CYPRESS 22V10D.

V3.31 : 6/29/93, modify programming algo and screen display.

V3.33 : 9/14/93, add INTEL  iPLD22V10.

CYPRESS  22V10G, 22VP10G.

V3.34 : 3/11/94, version update.

V3.35 : 9/16/94, modify F1, Home, End keys to operate with load

function.

add CYPRESS  PALCE16V8, PLDC20G10C.

V3.36 : 12/23/94, modify screen display.

add ALTERA  EP22V10ELC/EPC, EP22V10LC/PC.

V3.37 : 3/15/95, add CYPRESS  16V8(16R4), 16V8(16R6),

16V8(16R8), 16V8(16L8).

PALP4.EXE

V1.00 : 1st released version.

V3.00 : new version.

V3.02 : 1/1/91.

V3.03 : 5/1/91.

V3.04 : 7/1/91, Revision of text on the screen.

V3.08 : 1/15/92.

V3.11 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.12 : 5/26/92,Add the display of color screen.

Enhance the HEX load file function.

Modify the I/O change function.

V3.13 : 7/1/92, Extend the harddisk driver to Z for load file.

V3.14 : 7/29/92,Revision of programming voltage for TI

TIBPAL22V10/VP10.

V3.15 : 9/2/92, Adjust the VCC voltage of FULL LOAD and NO LOAD

in the range between MIN and MAX.

V3.16 : 10/1/92,Enhance the inspection capability of Display

Card.

V3.17 : 12/29/92,Modify screen display.

Modify batch file function.

Add TI  TIBPAL22V10-7.

V3.18 : 4/2/93, Increase the file number of load file function

to 1200.

V3.19 : 6/29/93, modify programming algo.

V3.20 : 9/14/93, version update.

V3.21 : 3/11/94, version update.

V3.22 : 9/16/94, version update.

V3.23 : 12/29/94, modify screen display and programming algo.

V3.25 : 3/15/95, modify programming algo.

PALP5.EXE

V1.00 : 1st released version

AMD22V10-15/20,ATMEL22V10/10L are added, but user has to

add a capacitor 0.01 uF(or greater) between pin 32

and pin 20.

V3.00 : new version.

V3.03 : 1/1/91.

V3.04 : 2/8/91, Add TI TIBFPLS506,TIBPSG507

Correct the problem that the characters(P and H

) can't be typed while executes "LOAD File"

function.

V3.05 : 4/1/91, Update load file.

V3.06 : 7/1/91, Revision of programming voltage for TI Mfr.

V3.07 : 8/16/91,Revision of programming voltage for AMD 22V10.

V3.08 : 9/27/91,Add SIGNETICS PLV750.

V3.09 : 10/1/91,Eliminate errors occured in reading for

ATMEL AT22V10L.

V3.11 : 10/28/91,Revision delay time for AMD AmPAL22V10.

V3.13 : 1/15/92.

V3.16 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.17 : 5/22/92,Add the display of color screen.

Enhance the HEX load file function.

V3.18 : 7/1/92, Extend the harddisk driver to Z for load file.

Correct the errors in date and readdir.

V3.19 : 7/20/92,Revision of programming voltage for TI TIBFPLS506

V3.20 : 10/1/92,Enhance the inspection capability of Display

Card.

V3.22 : 12/29/92,Modify screen display.

Modify batch file function.

Modify AT22V10 programming voltage.

V3.23 : 1/29/93, Modify I/O address fetch of FILE.DAT

V3.24 : 4/2/93, Increase the file number of load file function

to 1200.

Modify manufacturer YDC as ADRITE.

Modify manfuacturer SIGNETICS as PHILIPS.

V3.25 : 6/29/93, modify screen display.

V3.26 : 9/14/93, add  ATMEL  AT22V10B,

PHILIPS  PLQ22V10

V3.27 : 3/11/94, add ATMEL  ATV750B/L, ATF22V10B/L.

V3.28 : 4/26/94, ATMEL  ATF22V10B/L

modify user edit bit and architecture fuse.

ATMEL  ATV750B/L

modify verify function of security fuse,

add home and end functions of load file function.

V3.30 : 7/15/94, add AMD  AmPAL22P10, AmPAL22RP4, AmPAL22RP6,

AmPAL22RP8, AmPAL22RP10.

ATMEL  ATV750BQ/BQL, ATF22V0BQ/BQL,

ATF20V8B/BL/BQ/BQL, ATF16V8B/BL/BQ/BQL.

PHILIPS  LVT(P3Q)22V10

V3.31 : 9/16/94, modify F1, Home, End keys to operate with load

function.

V3.32 : 12/23/94, modify screen display.

V3.33 : 3/15/95, add ATMEL  16V8(16R4), 16V8(16R6),

16V8(16R8), 16V8(16L8).

PEEL1.EXE

V3.00 : 1st released.

V3.07 : 1/1/91. High yield rate.

V3.08 : 2/8/91, Correct the problem that the characters(P and H)

can't be typed while executes "LOAD File"

function.

V3.09 : 4/1/91, Update load file.

V3.10 : 7/1/91, Add ICT,AMI,GOULD 22CV10Z.

V3.11 : 8/16/91,Eliminate errors occured in programming Signature

for PEEL253 and PEEL273.

V3.12 : 10/1/91,Revision of text on the screen.

Add SIGNETICS PL22V10.

V3.14 : 1/15/92,Revision of MACRO CELL for 22CV10Z.

V3.15 : 1/21/92,Modify short-test subprogram.

V3.17 : 3/4/92, Modify the fetch address of PEEL22CV10 MACROCELL

V3.18 : 3/17/92,Delete short-test subprogram.

V3.19 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.20 : 5/25/92,Add the display of color screen.

V3.2l : 7/1/92, Extend the harddisk driver to Z for load file.

V3.22 : 9/9/92, Add PRN-MODEL function and modify I/O command.

V3.23 : 10/1/92,Revision of text on the screen.

Add ICT PEEL22CV10A.

Enhance the inspection capability of Display

Card.

V3.24 : 11/20/92,modify programming algorithm.

V3.26 : 12/29/92,Modify screen display.

Modify save buffer to file function.

Modify programming algorithm of 22CV10E, 22CV10A+.

Modify ID check function.

Add ICT PEEL22CV10A+.

V3.27 : 2/26/93, Modify buffer range of the load file function.

Modify operation of batch file.

Modify PEEL273 programming algorithm.

V3.28 : 4/8/93, Modify manufacturer YDC as ADRITE.

Modify print port function.

Modify manufacturer SIGNETICS as PHILIPS.

V3.29 : 6/2/93, add NS  GAL22CV10.

modify programming algo on

ICT  22CV10A, 22CV10Z, 22CV10A+.

modify programming for 18CV8.

modify erase function.

V3.30 : 7/22/93, add '/BW' function.

modify address location.

V3.31 : 9/14/93, modify read and save functions.

V3.34 : 12/9/93, modify manufacturer category.

modify programming algo on 22CV10 and 18CV8.

V3.35 : 3/11/94, version update.

V3.36 : 9/15/94, modify F1 key to operate with load file function.

V3.37 : 12/23/94, modify screen display and programming algo.

add ICT  PEEL22CV8.

V3.38 : 3/15/95, version update.

PEEL2.EXE

V3.00 : 1st released. 1/1/91.

V3.01 : 4/1/91, Update load file.

V3.02 : 7/1/91, Revision of text on the screen.

V3.03 : 10/1/91,Revision of text on the screen.

V3.05 : 1/15/92.

V3.06 : 3/17/92,Delete short-test subprogram.

V3.07 : 4/1/92, From 4/1/92 this program is also good for

ALL-03A modules.

Extend the haddisk driver to F for load file.

V3.08 : 5/25/92,Add the display of color screen.

V3.09 : 7/1/92, Extend the harddisk driver to Z for load file.

V3.10 : 10/1/92,Revision of text on the screen.

Enhance the inspection capability of Display

Card.

Add PRN-MODEL function and modify I/O command.

V3.11 : 12/29/92,Modify screen display.

Modify save buffer to file function.

Modify ID check function.

Add ICT PEEL20CG10A+.

V3.12 : 2/26/93, Modify buffer range of the load file function.

Modify operation of batch file.

V3.13 : 4/8/93, Modify manufacturer YDC as ADRITE.

Modify print port function.

V3.14 : 7/22/93, add '/BW' function.

modify address location.

V3.15 : 9/13/93, modify save function.

V3.16 : 12/9/93, modify manufacturer category.

modify programming algo.

V3.17 : 3/11/94, version update.

V3.18 : 9/15/94, modify F1 key to operate with load file function.

V3.19 : 12/23/94, modify screen display and programming algo.

V3.20 : 3/15/95, add ICT  PEEL16V8, PEEL20V8,

PEEL16V8(16R4), PEEL16V8(16R6),

PEEL16V8(16R8), PEEL16V8(16L8).