Table of Contents:

Section 1.  Introduction

1.1 UltraStor ULTRAOPT

1.2 License

1.3 Limited Warranty

1.4 Copyright

1.5 Special Notes

Section 2.  Preparations and Precautions

2.1 Protecting Your Data

2.2 Ensuring the Proper Attachment of Peripherals

2.3 Power Requirements

2.4 DOS Operating Environment

Section 3.  Technical Assistance

3.1 Resellers

3.2 Compuserve

3.3 UltraStor Technical Support

Section 4.  Description of files for UltraStor ULTRAOPT

Section 5.  Installation

5.1 Warning

5.2 Executing the ULTRAOPT Install Program

Section 6. Tested SCSI Peripherals

Section 7. Software Operation

Section 8. Sample CONFIG.SYS and AUTOEXEC.BAT Files

PLEASE READ THIS README FILE BEFORE PROCEEDING TO INSTALL.  THERE ARE

MANY PRECAUTIONS TO CONSIDER AND YOUR TIME WILL BE WELL SERVED

BY STUDYING THIS README FILE BEFORE ATTEMPTING TO INSTALL.

Section 1.  Introduction

1.1 UltraStor ULTRAOPT

UltraStor ULTRAOPT contains :

* This Readme file.

* Installation program.

* ASPI Managers for UltraStor ULTRA 14N, ULTRA 14F and ULTRA 24F

Host Adapters.

* UltraStor device drivers and utilities to support:

- Removable hard drives

- Magneto Optical drives

- CD-ROMs

- Floptical drives

- Third floppy drive

1.2 License

You may use the programs on a single machine which contains an

UltraStor Host Adapter.  You may make backup copies for the same

machine.  This license is only transferable to another party if

the other party agrees to accept all of the conditions of this

license.  If you transfer the programs, all programs, disks,

materials and copies of the programs must be transferred at the

same time.  If you transfer or yield possession of the programs

or any portion thereof, your license is automatically

terminated.  This license is effective until terminated.  You may

terminate the license at any time by sending all programs,

disks, materials, and copies to UltraStor.  This license will

also terminate if you fail to comply with the terms or

conditions of this license or if you infringe upon UltraStor's

copyrights.  You agree upon termination to destroy the programs,

disks, materials, and copies in any form.

1.3 Limited Warranty

The programs are provided "as is" without any warranty of any

kind, either expressed or implied.  UltraStor assumes no risk for

the use of the programs.  If your use of the programs, disks,

materials and copies proves defective, you assume the total cost

for repair or correction.  UltraStor does not warrant that the

programs, disks, materials, and copies will meet your

requirements or that the operations of said programs, disks,

material, and copies will be error-free.  UltraStor warrants that

the diskette(s) containing the programs to be free from defects

in material and workmanship under normal and intended use for a

period of 90 days from the date of delivery to you.

1.4 Copyright

The programs, disks, materials, and copies are proprietary

products of UltraStor Corporation and are protected by the

Copyright Laws of the United States.  Except as allowed in the

license, you may not copy any portion of the programs, disks,

and materials.

1.5 Special Notes

The installation program will lead you through the procedure to

install appropriate ASPI managers and device drivers, and make

necessary modifications to CONFIG.SYS and AUTOEXEC.BAT files.

Please see the attached list of tested SCSI peripherals to

ensure your peripheral has been tested.

Removable Hard Drive :

----------------------

If you have removable hard drives connected to your system  you

should install ULTRAREM.SYS, device driver for removable hard drives.

ULTRAREM.SYS supports up to 4 removable hard drives and you can

reserve up to 4 logical drives for each device.  When you install

ULTRAREM.SYS, UFDISK.EXE file will also be copied to the selected

target directory.  This file is an utility program to manipulate

partitions in the removable hard disks when they are supported by

ULTRAREM.SYS. Please consult the UltraStor SCSI controller

manual for specific support of removable drives; some special

settings may be required.

There are 2 methods to install the ULTRAREM.SYS driver:

* Default installation:

- If there is no disk in the drive, the number of logical

drives reserved for the drive is 1.

- If a disk is already inserted to the drive, the number of

logical drives reserved for the drive will be the same as

the number of partitions exist in the disk.

* Custom installation:

- You can specify the number of logical drives you want to

reserve for the drive.

"DEVICE=TargetDirectory\ULTRAREM.SYS" statement will be added to

CONFIG.SYS and the old CONFIG.SYS will be copied to CONFIG.KIT.

The default setting of ULTRAREM driver supports both hard disks,

which are not supported by BIOS INT13, and removable drives.

Options:

/H : to support hard disks only.

/R : to support removable drives only.

These options are to be added following the driver statement in the

CONFIG.SYS file.

e.g.

DEVICE=c:\TargetDirectory\ULTRAREM.SYS /H

CD-ROM :

--------

The device driver for CD-ROMs is ULTRACD.SYS.  The default name

used to access the CD-ROM driver is MSCD0001.  If you wish to use

a different name you have to change it manually in the CONFIG.SYS.

This CD-ROM driver also supports KODAK Multisession Photo CD for

CHINON, NEC, SONY, TEAC, TEXEL and TOSHIBA drives.

The UAUDIO.EXE file will also be copied into the target directory

when you choose to install ULTRACD.SYS. UAUDIO is a DOS-based

utility program which allows the user to control the playing of

Compact Audio Discs (CD-DA) in a CD-ROM drive with sound output

capabilities.

"DEVICE=TargetDirectory\ULTRACD.SYS  /D:MSCD0001" statement will

be added to CONFIG.SYS.

"DEVICE=MSCDEXDirectory\MSCDEX.EXE /D:MSCD0001" statement will be

added to AUTOEXEC.BAT.

FLOPTICAL DRIVE :

-----------------

FLPTICAL.sys is the device driver for floptical drives. This

driver enables the floptical drive to access 21M bytes floptical

diskettes as well as 1.44M and 720K floppy diskettes. Please

consult the UltraStor SCSI controller manual for specific

support of removable drives; some special settings may be required.

In the CONFIG.SYS, the following statement will be added :

"DEVICE=TargetDirectory\flptical.sys"

Third Floppy Drive :

--------------------

EXFDRV.SYS is UltraStor device driver to support a third floppy

drive under MS DOS. You will need a copy of DRIVER.SYS file which

comes with DOS v3.2 and above.  In the CONFIG.SYS file, the

following statements will be inserted:

"DEVICE=TargetDirectory\EXFDRV.SYS  /1:xx"

"DEVICE=SourceDirectory\DRIVER.SYS  /D:2/F:aa/H:2/S:bb/T:cc"

Please refer to the UltraStor SCSI Host Adapter manual for further

definitions of the above command line options.

Software Caching Programs and MS Smartdrv.exe:

----------------------------------------------

Software caching programs may be used with UltraStor SCSI

controllers as long as the caching program supports the Virtual

DMA Services (VDS) specification. When the USPI (UltraStor's

ASPI manager) is loaded, double buffering is not required,

unless the controller is the ULTRA 14F or 14FB and the system

is populated with more than 16MB of system memory; in the

latter case, some drivers may not function properly, due to the

double buffering.

If Microsoft Windows 3.1 or MS DOS 6 adds the "device=Smartdrv.exe

/double_buffer " to your config.sys file, delete it if using

the UltraStor USPI manager.

Section 2.  Preparations and Precautions

2.1 Protecting Your Data

In order to protect your data, we strongly recommend that you

backup all data on your hard drive(s).  Loss of data is never

expected and the best protection is a good set of backups.

2.2 Ensuring the Proper Attachment of Peripherals

During the installation of various SCSI peripherals, it

will be necessary for you to locate and determine whether the

peripheral has terminating resistors installed, which SCSI ID

has been set on the SCSI peripheral(s), and where "Pin 1" exists

on the peripheral's SCSI interface connector.

A proper placement of terminating resistors will ensure

reliable and unimpeded data flow on the SCSI bus.  Only

the two peripherals, including the UltraStor SCSI Host Adapter,

at each end of the SCSI bus will have terminating resistors installed.

All other SCSI peripherals will have their terminating resistors

removed or disabled.

The SCSI bus provides for eight (8) physical unit IDs, starting

with zero (0) and running upto seven (7).  The last ID (7) is

usually reserved for the SCSI Host Adapter.  The first two IDs,

zero (0) and one (1) are usually reserved for the booting hard

drive and second physical hard drive; more than two (2) hard

drives can be supported with the UltraStor SCSI Host Adapters with

either a special jumper setting or an EISA configuration board

option selection.  Keep in mind that you have only seven (7) IDs

to choose since the UltraStor SCSI Host Adapter requires one.  It

is usually a good idea to write out the intended assignments for

IDs before installation.

The SCSI cable can only be properly attached to peripherals in

one way; attaching the cable incorrectly will cause problems

with the installation.  Use the peripheral's installation guide

or manual to locate "Pin 1" for its SCSI connector.  This is

important as some damage may occur to the peripheral if the SCSI

cable is reversed and power is turned onto the system.  Many SCSI

peripherals set "Pin 1" on its SCSI connector to be closest to

the power supply; in all cases, verify its location.  The cable

provided by UltraStor has one side of the cable marked by

writing on the entire length.  This marking indicates where "Pin

1" is located; line up "Pin 1" on all peripherals and your SCSI

bus will operate properly.

2.3 Power Requirements

Ensuring proper power supply to the entire system, including the

SCSI bus peripherals is extremely important.  Make sure your

system's power supply is adequate; should your power supply not

be able to supply sufficient power to all peripherals, the

system will not operate properly and may cause damage to some

peripherals and perhaps to the system.  Some SCSI peripherals

require a peak power supply at power on time; this is a key

point for many older SCSI drives and tape drives as the peak

power necessary to start spinning a 15 platter drive or start

spinning a tape cartridge is rather high.  If the system

successfully recognizes all peripherals after the power up

sequence is completed, then your system will most likely contain

sufficient power to continue operations.

2.4 DOS Operating Environment

The UltraStor ULTRAOPT  has been tested with MS DOS 3.3 and

above.  Certain versions of MS DOS operating system utilities may

operate differently; please refer to the actual documentation.

Use of memory managers and caching software may alter the

behavior of some UltraStor utilities; every effort has been made

to ensure compatibility with standard operating environments.

However, it is possible to configure memory managers and caching

software in a way that impedes operation or slows performance of

the SCSI peripherals.  Should you experience problems, we

strongly recommend that you limit or omit your usage of these

types of software until such time you have successfully	installed the

SCSI subsystem.

Section 3. Technical Assistance

3.1 Technical Support

If you purchased this UltraStor product from a reseller, please

contact that reseller for further assistance.  Since the reseller

may have provided additional equipment and software, it is

always best to contact the reseller first.

3.2 Compuserve

Additionally, technical assistance may be obtained by accessing

the Compuserve Information Network.  Once online, issue the "GO

IBMHW" command to enter the IBM Compatible Hardware forum.

Select the section on Host Adapters and drives (as of this

writing, that section is "4") and leave a message to all or

UltraStor.  An UltraStor technical expert will generally read

those messages and offer assistance. The UltraStor

Compuserve ID is 71005,1655, should you need to send queries

directly.

3.3 UltraStor Technical Support

UltraStor maintains a technical support hotline for our

resellers and those customers who are unable to obtain

support from the other methods listed above.  That telephone

number is 1-510-623-8955.  As the staff of this organization is

continuously called upon for help, please be patient when

requesting assistance.  It is not uncommon for your name and

telephone number to be requested so that a call back can be

scheduled.  Since call backs are scheduled on a first come first

served, please expect delays of one or more days.  Again, we

strongly recommend that you contact the reseller or log on to

Compuserve for assistance.

In addition to the voice hotline, UltraStor maintains a Driver

Download Facility.  Licensed UltraStor users may download the

latest operating system level drivers from this system.  Note:

UltraCD, UltraREM and MSCDEX are licensed products

which may not be downloaded from this facility; the latest

versions of the USPI driver and the third floppy driver may be

downloaded.  Access the UltraStor Driver Facility by having your

modem set for 8bits, No parity, 1 start/stop bit, 2400 to

14400bps speed, and then dialing 1-510-623-9091.  Please do not

leave requests for technical support on this system as they

cannot be answered by the technical support organization.

Section 4. Description of files for UltraStor ULTRAOPT

README  .DOC - This readme file.

INSTALL .EXE - UltraStor ULTRAOPT installation program.

USPI14N .SYS - DOS device driver for supporting ASPI compatible device

module interface used with ULTRA 14N AT SCSI

Host Adapter.

USPI14  .SYS - DOS device driver for supporting ASPI compatible device

module interface used with ULTRA 14F AT Bus Master

SCSI Host Adapter.

USPI24  .SYS - DOS device driver for supporting ASPI compatible device

module interface used with ULTRA 24F EISA Bus Master

SCSI Host Adapter.

ULTRAREM.SYS - Provides the low level interface to your removable and

optical drives.

UFDISK  .EXE - Allows the partitioning of removable drives.

ULTRACD .SYS - Provides the low level interface to your CDROM drive.

UAUDIO  .EXE - UltraAudio is a DOS based program which controls the

playing of audio CDs.

FLPTICAL.SYS - Provides the low level interface to your floptical

drive.

EXFDRV  .SYS - Allows three floppy drives to operate under DOS.

Section 5. Installation

5.1 Warning

Warning: this install program will modify the AUTOEXEC.BAT and

CONFIG.SYS files on your booting hard drive.  Please make copies

so that you can recover should problems occur.  Please make

backups of all data and programs on your hard drive.

5.2 Executing the ULTRAOPT Install Program

Insert the ULTRAOPT floppy into drive A: (or B:) and at the

DOS command line, change the drive letter and then enter the

"install" command:

e..g	C:>a:		change to A: drive

A:>install	invoke install command from A: drive

The screen will display the License Agreement.  If you agree

to accept the License Agreement the install program will continue

and display installation menu screen; otherwise it will terminate.

The installation menu consists of selections for drive path, ASPI

manager for UltraStor controller cards, ASPI device modules for

different SCSI devices and HELP menu.

Once in the menu, use <TAB> to move to next field, use <ARROW KEYS>

to move within a field, use <ENTER> to select, use <ALT-Underscored

char> to select, use <ALT-F4> to close a window.

The drive path lets the user specify the source drive, destination

drive\path and the boot drive.

The program automatically scans for UltraStor Host Adapters and SCSI

devices installed in the system. Then, it chooses the appropriate

ASPI manager(s) and device drivers to be installed.

Check the appropriate boxes if you want to install additional ASPI

managers or device drivers.

The ASPI manager should be selected for each controller card in

the system. Multiple ASPI managers are allowed if different

controller cards are installed in the system. If none of

the controller cards are selected, whatever ASPI manager

exists in the current config.sys will remain intact.

The ASPI device module is the device driver that

communicates with the ASPI manager for the different SCSI devices.

If you have CD-ROM, Removable disk(Including Magneto Optical drive)

or Floptical drive in the system, you must select the

corresponding device modules.

If you choose to "Install", the install program will copy the

selected files and modify the CONFIG.SYS and AUTOEXEC.BAT files

accordingly. If you choose to install CD-ROM driver, you will be

prompted to specify the directory where you have the MSCDEX.EXE

file.

After the modules have been installed and after the CONFIG.SYS

and AUTOEXEC.BAT files have been modified, the install program

will warn user to reboot the system so that the new modules and

the modified files can be loaded.

After the system reboots, you should now have the selected

support as requested during the install procedure.

Section 6. Tested SCSI Peripherals

The following hard drive manufacturers have supplied a representative

sampling of all their SCSI drive families for compatibility testing

with UltraStor SCSI Host Adapters:

Connor

Fujitsu

Hitachi

HP

Magtron

Maxtor

Micropolis

Microscience

NEC

Quantum

Seagate

Toshiba

Western Digital

The following peripherals were specifically certified with the ULTRAOPT

software.

Removable Drives

Bernoulli IOMega 90MB

Syquest 555

Syquest 5110

Insite Floptical  I325

I-OMega Floptical IO20S

MO Disks

Sony SMO-E501

HP C1711A

Hitachi OD112-1

Maxoptics Tahiti II

Ricoh RO-3010E

Ricoh RO-5031E

Pioneer DE-U7001

CD-ROM Drives

Sony CDU-541-01 **

Sony CDU-7211UU **

Sony CDU-561 ** ++

Toshiba XME-3201B

Toshiba XME-3100A

Toshiba XME-3201A1 **

Toshiba XME-3301E1 **

Toshiba XME-3401B ** ++

Hitachi CDR-1750S **

Hitachi CDR-3650U **

Chinon CDX-431 **

Chinon CDS-431 **

Chinon CDC-431 **

Chinon CDS-435 **

Chinon CDX-435 **

Chinon CDS-535 ** ++

NEC CDR-37 **

NEC CDR-74 **

NEC CDR-84 **

NEC CDR-84-1 ** ++

Panasonic CR-501 **

Panasonic LK-MC501

Pioneer DRM-600/604 **

ShinaKen DM-3x1S

TEAC CD-50 ** ++

Texel DM-5021 **

Texel DM-3021 **

Texel DM-5024 ** ++

Texel DM-3024 ** ++

Texel DM-5028 ** ++

Texel DM-3028 ** ++

** Indicates certification with Audio features of the drives

++ Indicates certification with KODAK Multisession Photo CD

features of the drives

Section 7. Software Operation

The following sections describe basic operation of specific

software modules.

7.1 UAUDIO: Playing Music on your CD-ROM Drive

UltraAudio is a MS DOS based program which allows you to play

standard music CDs oa your CD-ROM drive. Your CD-ROM

drive must support this function and that CD-ROM must be one of

the drives that has been certified with this UltraOPT; check

Section 6.0.

At the DOS prompt, enter "Uaudio". The program will check for a

valid music CD and attempt to read the table of contents. If it

does not find a valid Music CD and/or cannot read the table of contents, the

program will display an error message indicating so, and end it's

execution gracefully.

If it is successfull in reading the table of contents, the program will

display a screen with various status boxes and control functions.

The available control functions are:

Screen Notation                    * Description

Backspace                        track play by 10 second increments

Advance                          track play by 10 second increments

Track                            Backspace CD play by 1 track increments

Track                            Advance CD play by 1 track increments

STOP                             Stop CD play

Play                             Start CD play

Pause                            Pause CD play

Eject                            Eject the CD from the drive

* Some drives may not support all functions.

The screen also shows various status boxes during music play. All

functions are similar to those found on standard music CD players.

Screen Notation                    Description

Current                          Shows which sequential track is in play

Total                            The total number of tracks on this CD

Elasped                          The elasped time of disk in play

[DISC]

Elasped                          The elasped time of track in play

[Song]

Total                            The total amount of playing time available

[DISC]

Total                            The total amount of playing time available

[Song]

In addition, the "About" menu item at the top of the screen will

display the current version number of UltraAudio. It may be

selected by entering "A" and then pressing the Enter key or by

double clicking with the mouse.

UltraAudio may be started and terminated, without loss of music

play. Just position the cursor over the upper left corner of the

screen, with the little box with a dash, and double click or press the

"Enter" key twice. Alternately, you may press the Escape key twice

to end the program; the music CD will continue to play.

7.2 UFDISK: Setting up Partitions on Removable Drives

UltraFDISK is a MS DOS based program which will allow the

partioning and high level formatting of hard disk drives,removable

drives, and magnetec optical (MO) drives.UltraFDISK requires the

USPI system level driver and the UltraREM driver to be loaded

beforehand in order to function with removable and /or MO drives;

operate the same as MS DOS FDISK UltraREM is not required for

hard drives.

UltraFDISK combines the functions of the DOS FDISK and Format

commands into one simple, full status screen. All functions

operate the same as MS DOS FDISK with the exception that when

you perform a partioning function, a high level format command

is also performed automatically.

The available functions are:

Creat Primary            Creates a standard bootable partition

DOS Partition

Create Extended          Creates a standard non-bootable partition

DOS Partition

Create DOS               Creates a standard logical drive on the

Logical Drive            selected extended partition

Set Partition Active     Set the primary partition to be active and

bootable

Delete Partition         Deletes a partition

Change Current Drive     Changes from one drive to another

Exit                     Gracefully ends program operation

Section 8.  Sample CONFIG.SYS and AUTOEXEC.BAT Files

The following sample system configuration file listings are

provided to assist in trouble shooting. Other statements may be

required in your files and you should consult the installation

guides for the other hardware and software which is or will be

installed in the systen. These files are based on the ULTRA 24F

EISA SCSI controller.

rem beginning of Config.sys file

buffers=30

Files=30

Device=C:\ULTRA\USPI24.SYS

rem Necessary for ASPI Manager

Device=C:\ULTRA\ULTRAREM.SYS

rem UltraStor's removable MO peripheral driver

Device=C:\ULTRA\FLPTICAL.SYS

rem UltraStor's floptical device driver

Device=C:\ULTRA\ULTRACD.SYS /d:MSCD000l

LASTDRIVE=M

rem Necessary for adding CD-ROM drive

Stacks=9,256

rem end of config.sys file

rem beginning of Autoexec.bat file

echo off

PROMPT $P$G

PATH=C:\;C:\DOS;C:\TEMP;C:\BAT;C:\ULTRA;

set comspec= C:\DOS\command.com

SET TEMP=C:\TEMP

echo on

C:\DOS\MSCDEX /D:MSCD0001

rem Necessary for CD-ROM operation

C:\DOS\Smartdrv.exe 2048 2048 a-b-

rem end of autoexec.bat file

#