How To Use NetWare DOSODI Driver PCNTNW.COM

TABLE OF CONTENTS

1.0 NETWARE DOS ODI INSTALLATION NOTES FOR 2.2 & 3.1x SERVERS:

2.0 NETWARE 4.0 & 3.12 DOS WORKSTATION INSTALLATION NOTES:

3.0 NETWARE LITE INSTALLATION NOTES:

1.0 NETWARE DOS ODI INSTALLATION NOTES FOR 2.2 & 3.1x SERVERS:

Your server must be running NetWare 2.2 or 3.x or greater to support

ODI workstations.  (For information on configuring an ODI workstation

with a NetWare 4.0 & 3.12 server, see NETWARE 4.0 & 3.12 DOS WORKSTATION

INSTALLATION NOTES ). For more information on ODI workstations see Novell's

documentation "ODI Shell for DOS".

PROCEDURE:

----------

1.Run the LSL program of NOVELL Operating System.

2.Execute PCI ODI driver.(PCNTNW.COM)

The PCNTNW.COM driver defaults to Ethernet_802.2 frame type. If your

network uses another frame type, you will need a NET.CFG file to

communicate to the different settings. For an 802.3 frametype, create

a NET.CFG file like the one below. It is an ascii file, which is sensitive

to the placement of certain parameters. Be sure that the link driver

line is left justified and the indented lines are preceded by either

ONE SPACE, or ONE TAB.

Link Driver PCNTNW

Frame Ethernet_802.3

PROTOCOL IPX 0 ETHERNET_802.3

PB Buffers=2

Note: 1.FrameType - for 2.2 or 3.11 networks, 802.3 is generally the default

frame type.  NetWare 4.0 or 3.12 will use the 802.2 frame type.

2.You can unload the ODI driver using -U parameter.

3.Run the IPX Protocol Stack in NOVELL named IPXODI.COM

4.Run the NOVELL DOS SHELL NETX.EXE or BNETX.COM, if the server supports

IPXBURST mode.

5.Ensure your CONFIG.SYS file includes FILES and BUFFERS statements.

(Novell recommends a minimum of 20 for each.)  If a LASTDRIVE statement

appears, it should indicate the drive letter of your last local drive.

The following letter will be your first NetWare drive.  For example,

a LASTDRIVE=E statement, would give you Drive F as the first NetWare

drive.

2.0 NETWARE 4.0 & 3.12 DOS WORKSTATION INSTALLATION NOTES:

There are two different methods to connect to a NetWare 4.0 or 3.12

server. The first one describes using NETX.EXE or BNETX.COM, and this

installation is the same as NetWare 2.2 or 3.1x. The second installation is

for using Novell's VLMs and performing the installation through NetWare 4.0

( or 3.12 )'s INSTALL.EXE utility.

Novell's INSTALL program will copy over the necessary drivers and

software to connect to the NetWare 4.0 ( or 3.12 ) server. If you have

installed the System Software's socket service & card service already,

temporarily unload it in order for the INSTALL program to function correctly.

PROCEDURE:

----------

1. Run INSTALL.EXE from Novell's Workstation for DOS Diskette.

2. The NetWare Client Install program screen will appear.  You must

complete each of the questions.  These options are specific to your

needs.

A. Step #1 specifies the directory on the hard drive that you

want to install the client software.  The default directory

is C:\NWCLIENT. You can specify another directory if you desire.

B. Step #2 gives you the option to automatically update the

CONFIG.SYS and the AUTOEXEC.BAT files or modify them at a later

time manually.

C. Step #3 will install Windows software for your system if your

workstation supports Windows, and will ask you to specify the

directory which Windows is located.

D. Step #4 asks you to select a network driver and provide

configuration options.  It uses the specifications you provided

to create the files NET.CFG and STARTNET.BAT.  When it prompts

you for the driver disk, insert the Distributed Disk and select

the PCnet-PCI adapter.

E. Step #5 is the final step and the INSTALL program will copy

the appropriate files to your harddrive.

3. Run the STARTNET.BAT and connect to the NetWare 4.0 ( or 3.12 ) server.

4. Ensure your CONFIG.SYS file includes LASTDRIVE=Z statement.

If your last local drive is D, would give you Drive E as the first NetWare

drive.

4.0 NETWARE LITE INSTALLATION NOTES:

This section is based on NetWare Lite versions 1.0 and 1.1.

For more details on NetWare Lite, see Novell's documentation.

PROCEDURE

---------

1.  Start Novell's INSTALL program and follow Novell's instructions

for "other" drivers.  When prompted to insert the "other" driver

diskette, using the 3.5" distributed diskette, Select the

PCnet-PCI driver from the list of available drivers.

INSTALL will create a file called STARTNET.BAT. STARTNET.BAT for

the PCI Ethernet card should look like this:

LSL

PCNTNW

IPXODI A

SHARE

SERVER

CLIENT

NOTE: If this machine is a client only, the SHARE and SERVER command

will not be included.

2.  The PCNTNW.COM driver defaults to Ethernet_802.2 frame type. If your

network uses another frame type, you will need a NET.CFG file to

communicate to the different settings. For an 802.3 frametype, create

a NET.CFG file like the one below. It is an ascii file, which is sensitive

to the placement of certain parameters. Be sure that the link driver

line is left justified and the indented lines are preceded by either

ONE SPACE, or ONE TAB.

Link Driver PCNTNW

Frame Ethernet_802.3

PROTOCOL IPX 0 ETHERNET_802.3

3.  Reboot the computer, run STARTNET and watch for the driver's logon

message.