Cirrus Logic DMI Implementation

The Desktop Management Interface (DMI) provides a standard interface that handles communication between any management application and all the manageable products on or attached to a desktop PC or server.  The primary elements of the DMI are the Service Layer (SL),  which is provided by our OEM’s, and the Management Information Format (MIF), which is provided by Cirrus Logic.

The Service Layer is a program, running on the local desktop machine or server, that collects information from products, manages that information in the MIF database, and passes the information to management applications as requested.  It controls communication between itself and management applications by means of Management Interface (MI), and between itself and manageable products by means of the Component Interface (CI).

The MIF database contains the information about the products on the system.  The Service Layer manages the information in the database.  The information comes from MIF files provided with each manageable product, i.e., a CL-GD5436.  Individual MIF files are compiled into the database when a manageable product is installed into a DMI-enabled system.

Each product provides information to the MIF database by means of a MIF file, which contains the pertinent management information for that product.  OEM’s will provide a MIF file describing their system’s features.  This file is called SL.MIF.  Cirrus Logic provides a MIF file called Video_CL.mif which describes the features and attributes of the video adapter. The MIF file can be easily customized by modifying some of the static data to reflect customer-specific names or configurations.  A sample of the MIF file code is attached.  The information in the MIF file is compiled into the MIF database when the product is installed.  Separate MIF files can be added and deleted as necessary.

Installation Instructions

There are two ways to implement use of the Cirrus Logic MIF file.

- Load the Cirrus Logic MIF file with other system MIF files on boot up.  Place the MIF file (Video_CL.MIF) in the same location as the system and/or other product’s MIF files in order to create the MIF database, OR

- Insert the information found in the Video_CL.MIF file, via cut and paste, into the system MIF file (SL.MIF).

After you have selected which method of installation you will use, you must do the following.  These two steps are required to insure correct operation.

- Copy the file Video_CL.INI, which contains the static information, to the system’s WINDOWS directory.

- Modify The PATH section of the MIF file to point to the file, Video_CL.EXE, that contains both static and dynamic data about the video adapter.  This data contains information about the video group, the video BIOS group, and the video BIOS characteristics that is extracted into the MIF database.

For example:

Start Path

Name = “CIRRUS”

WIN16 = “D:\\CIRRUS\\VIDEO\\VIDEO_CL.EXE”

End Path

***

Sample Video_CL.MIF File:

//-----------------------------------------------------------------------------

// CIRRUS LOGIC CL-GD543x          VERSION 0.90

//

// Description of CL-GD543x Video characteristics according to

//   the DMTF specifications

//

//-------------------------------------------------------------------------------

Start Component

Name = "CIRRUS Logic CL-GD543x MIF"

Description = " This MIF file describes all the video features of CL-GD543x family"

Start Path

Name = "CIRRUS"

WIN16 = "D:\\LV13\\CIRRUS\\VIDEO\\VIDEO_CL.EXE"

End Path

Start Enum

Name = "BOOL"

Type = Integer

0 = "FALSE"

1 = "TRUE"

End Enum

Start Enum

Name = "System Enclosure Status"

Type = Integer

0x01 = "Other"

0x02 = "Unknown"

0x03 = "Safe"

0x04 = "Warning"

0x05 = "Critical"

End Enum

//------------------------------------------------------------------------

// Video Groups

//------------------------------------------------------------------------

Start Group

Name = "Video"

Class = "DMTF|Video|001"

ID = 20

Key = 1

Description = "This group defines the attributes of CL-GD543x."

Start Attribute

Name = "Video Index"

ID = 1

Description = "An index into the video table"

Access = READ-ONLY

Type = Integer

Value = *"CIRRUS"

Storage = COMMON

End Attribute

Start Attribute

Name = "Video Type"

ID = 2

Description = "The architecture of the video subsystem "

Access = READ-ONLY

Type =   Start Enum

Name = "Video Type"

Type = Integer

0x01 = "Other"

0x02 = "Unknown"

0x03 = "CGA"

0x04 = "EGA"

0x05 = "VGA"

0x06 = "SVGA"

0x07 = "MDA"

0x08 = "HGC"

0x09 = "MCGA"

0x0A = "8514A"

0x0B = "XGA"

0x0C = "Linear Frame Buffer"

End Enum

Value = *"CIRRUS"

Storage = COMMON

End Attribute

Start Attribute

Name = "Current Video Mode"

ID = 3

Description = "The video mode in this system as returned by Video BIOS"

Access = READ-ONLY

Type = Integer

Value = *"CIRRUS"

Storage = SPECIFIC

End Attribute

Start Attribute

Name = "Minimum Refresh Rate"

ID = 4

Description = "The minimum refresh rate for CL-GD543x in Hz."

Access = READ-ONLY

Type = Integer

Value = *"CIRRUS"

Storage = COMMON

End Attribute

Start Attribute

Name = "Maximum Refresh Rate"

ID = 5

Description = "The maximum refresh rate for CL-GD543x in Hz"

Access = READ-ONLY

Type = Integer

Value = *"CIRRUS"

Storage = COMMON

End Attribute

Start Attribute

Name = "Video Memory Type"

ID = 6

Description = "The type of video memory for CL-GD543x"

Access = READ-ONLY

Type =   Start Enum

Name = "Video Memory"

Type = Integer

0x01 = "Other"

0x02 = "Unknown"

0x03 = "VRAM"

0x04 = "DRAM"

0x05 = "SRAM"

End Enum

Value = *"CIRRUS"

Storage = COMMON

End Attribute

Start Attribute

Name = "Video RAM Memory Size"

ID = 7

Description = "Video adapter memory size in kilobytes"

Access = READ-ONLY

Type = Integer

Value = *"CIRRUS"

Storage = COMMON

End Attribute

Start Attribute

Name = "Scan Mode"

ID = 8

Description = "The scan mode for CL-GD543x"

Access = READ-ONLY

Type =Start Enum

Name = "Scan Mode"

Type = Integer

0x01 = "Other"

0x02 = "Unknown"

0x03 = "Interlaced"

0x04 = "Non Interlaced"

End Enum

Value = *"CIRRUS"

Storage = COMMON

End Attribute

Start Attribute

Name = "Video Physical Location"

ID = 9

Description = "The location of the video controller circuitry."

Access = READ-ONLY

Type =Start Enum

Name = "Video Location"

Type = Integer

0x01 = "Other"

0x02 = "Unknown"

0x03 = "Integrated"

0x04 = "Add-on Card"

End Enum

Value = *"CIRRUS"

Storage = SPECIFIC

End Attribute

Start Attribute

Name = "Current Vertical Resolution"

ID = 10

Description = "The current number of vertical pixels."

Access = READ-ONLY

Type = Integer

Value = *"CIRRUS"

Storage = COMMON

End Attribute

Start Attribute

Name = "Current Horizontal Resolution"

ID = 11

Description = "The current number of horizontal pixels"

Access = READ-ONLY

Type = Integer

Value = *"CIRRUS"

Storage = COMMON

End Attribute

Start Attribute

Name = "Current Number of bits per pixel"

ID = 12

Description = "The number of bits used to display each pixel for CL-GD543x"

Access = READ-ONLY

Type = Integer

Value = *"CIRRUS"

Storage = COMMON

End Attribute

Start Attribute

Name = "Current Number of rows"

ID = 13

Description = "The number of rows in character mode for Cl-GD543x"

Access = READ-ONLY

Type = Integer

Value = *"CIRRUS"

Storage = COMMON

End Attribute

Start Attribute

Name = "Current Number of columns"

ID = 14

Description = "The number of columns in character mode for CL-GD543x"

Access = READ-ONLY

Type = Integer

Value = *"CIRRUS"

Storage = COMMON

End Attribute

Start Attribute

Name = "Current Refresh Rate"

ID = 15

Description = "The current refresh rate in Hz for CL-GD543x"

Access = READ-ONLY

Type = Integer

Value = *"CIRRUS"

Storage = COMMON

End Attribute

Start Attribute

Name = "FRU Group Index"

ID = 16

Description = "If this is a Field Replaceable Unit or if it is part of another FRU,"

"this provides an index into the FRU table."

" 'Value = -1' indicates that the group is not a FRU"

Access = READ-ONLY

Storage = COMMON

Type = Integer

Value = unsupported

End Attribute

Start Attribute

Name = "Operational Group Index"

ID = 17

Description = "This attribute gives the instance index in the Operation State"

"Group"

Access = READ-ONLY

Storage = COMMON

Type = Integer

Value = *"CIRRUS"

End Attribute

End Group

//------------------------------------------

Start Group

Name = "Video BIOS"

Class = "DMTF|Video BIOS|001"

ID = 21

Description = "This group defines the attributes for the Video BIOS."

Key = 1

Start Attribute

Name = "Video BIOS Index"

ID = 1

Description = "The index into the Video BIOS table."

Access = Read-Only

Storage = Common

Type = Integer

Value = *"CIRRUS"

End Attribute

Start Attribute

Name = "Video BIOS Manufacturer"

ID = 2

Description = "The name of the company that wrote this Video BIOS."

Access = Read-Only

Storage = Specific

Type = String(80)

Value = *"CIRRUS"

End Attribute

Start Attribute

Name = "Version"

ID =  3

Description = "The version number or version string of this Video BIOS."

Access = Read-Only

Storage = Specific

Type = String(64)

Value = *"CIRRUS"

End Attribute

Start Attribute

Name = "Video BIOS Release Date"

ID = 4

Description = "The Video BIOS release date."

Access = Read-Only

Storage = Common

Type = Date

Value = unsupported

End Attribute

Start Attribute

Name = "Video BIOS Shadowing State"

ID = 5

Description = "If TRUE, the Video BIOS is currently being shadowed"

Access = Read-Only

Storage = Common

Type = "BOOL"

Value = *"CIRRUS"

End Attribute

End Group

//-------------------------------------------

Start Group

Name = "Video BIOS Characteristic"

Class = "DMTF|Video BIOS Characteristic|001"

Description = "This group defines the characteristics for the Video BIOS."

Key = 1,2

Start Attribute

Name = "Video BIOS Characteristics Index"

ID = 1

Description = "This is an index into the Video BIOS Characteristics table. "

Access = Read-Only

Storage = Common

Type = Integer

Value = 0

End Attribute

Start Attribute

Name = "Video BIOS Number "

ID = 2

Description = "This is the Video BIOS number which correlates to the Video BIOS Index."

Access = Read-Only

Storage = Common

Type = Integer

Value = 1

End Attribute

Start Attribute

Name = "Video BIOS Characteristics"

ID = 3

Description = "This field contains different attributes supported by this "

"version of the Video BIOS"

Access = Read-Only

Storage = Common

Type =Start Enum

Name = "Video BIOS Char"

Type = Integer

0x01 = "Other"

0x02 = "Unknown"

0x03 = "Unsupported"

0x04 = "Standard Video BIOS"

0x05 = "VESA BIOS Extension Supported "

0x06 = "VESA Power Management Supported"

0x07 = "VESA Display Data Channel Supported"

0x08 = "Video BIOS Shadowing allowed"

0x09 = "Video BIOS Upgradeable"

End Enum

Value = 0

End Attribute

Start Attribute

Name = "Video BIOS Characteristics Description"

ID = 4

Description = "Expanded description on this Video BIOS characteristic"

"Characteristic."

Access = Read-Only

Storage = Specific

Type = String(80)

Value = ""

End Attribute

End Group

Start Table

Name = "Video BIOS Table"

ID = 22

Class = "DMTF|Video BIOS Characteristic|001"

{1, 1, 4,""}

{2, 1, 5,""}

{3, 1, 6,""}

End Table

End Component