#

| LIESMICH Datei fuer

|

| ELSA QuickStep 1000

| Version 2.10

|

| Copyright (c) 1993-97 ELSA GmbH, Aachen (Germany)

|

| Sonnenweg 11

| D-52070 Aachen

|

| Telefon    : +49/0-241-9177-0

| FaxBox     : +49/0-241-9177-4

| Support-Fax: +49/0-241-9177-113

| BBS (Modem): +49/0-241-9177-981

| BBS (ISDN) : +49/0-241-9177-7800

|              +49/0-241-9177-7810

| CompuServe : GO ELSA

| WWW        : http://www.elsa.de

|

| 01.02.97

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Inhaltsuebersicht

1.      Uebersicht

2.      Inhalt

3.      Installation

4.      Bemerkungen

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

1.      Uebersicht:

Diese Datei enthaelt Bemerkungen und Aenderungen zu

den ISDN-Treibern.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

2.      Inhalt:

Dieses Verzeichnis enthaelt folgende Dateien:

LIESMICH.TXT  Diese Datei

README.TXT    Englische Version dieser Datei

SETUP.EXE     Installationsprogramm fuer Windows 3.1x

2.1     [DRIVER]

DLL (dynamic link library) fuer 1TR6 (nationales ISDN):

CAPI1TR.DLL  CAPI-Version 1.1 Treiber fuer ELSA QuickStep 1000

D-Kanalprotokoll : 1TR6 (nationales ISDN)

B-Kanalprotokolle: HDLC-transparent, bittransparent,

X.75, T.70NL, V.120

ISO 8208 (X.25), T.90

CA201TR.DLL  CAPI-Version 2.0 Treiber fuer ELSA QuickStep 1000

D-Kanalprotokoll : 1TR6 (nationales ISDN)

B-Kanalprotokolle: HDLC-transparent, bittransparent,

X.75, T.70NL, V.120

ISO 8208 (X.25), T.90

DLL (dynamic link library) fuer DSS1 (EURO-ISDN):

CAPIDSS.DLL  CAPI-Version 1.1 Treiber fuer ELSA QuickStep 1000

D-Kanalprotokoll : DSS1 (EURO-ISDN)

B-Kanalprotokolle: HDLC-transparent, bittransparent,

X.75, T.70NL, V.120

ISO 8208 (X.25), T.90

CA20DSS.DLL  CAPI-Version 2.0 Treiber fuer ELSA QuickStep 1000

D-Kanalprotokoll : DSS1 (EURO-ISDN)

B-Kanalprotokolle: HDLC-transparent, bittransparent,

X.75, T.70NL, V.120

ISO 8208 (X.25), T.90

CAPI.DLL     fuer CAPI 1.1, erfordert DOS-TSR

CAPI20.DLL   fuer CAPI 2.0, erfordert DOS-TSR

2.2     [UTILS]

clrauto.exe   loescht ggf's Zeile fuer das Laden des DOS-TSR aus

der autoexec.bat

isdnmon.exe   ELSA ISDNmonitor fuer Windows 3.1x

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

3.      Installation:

Die folgende Abbildung zeigt das Montageblech der

ISDN-Plug and Play-Adapterkarte :

_____

\    |

|   |

|   |

|   |

|   |

|___|

|___|<- ISDN S0-Leitungsanschluss

|   |

| o |<- ISDN-Verbindungsanzeige (gelbe LED)

| o |<- ISDN-Zustandsanzeige (gruene LED)

|___|

_

|

|

|

|

________________________________|

|                               _|

|                              |_ <- ISDN S0-Leitungs-

|                                |           anschluss

|                                o<- gelbe LED

|    _                      __   o<- gruene LED

|___| |____________________|  |__|

Die Plug and Play-Karte ist vollkommen Software konfigurierbar

und hat KEINEN Jumper.

- - - - - - - - - - - - - - - - - - - - - - - -

a.) Installation mit Plug and Play:

Wenn Sie einen Plug and Play-faehigen Rechner besitzen,

so muessen Sie nur die DLL installieren. Verwenden Sie

das Setup-Programm im Verzeichnis WIN31.

- - - - - - - - - - - - - - - - - - - - - - - -

b.) Installation ohne Plug and Play:

Sollte Ihr Rechner nicht Plug and Play-faehig sein, so

installieren Sie die DLL's ebenfalls mit dem Setup-Programm

aus dem WIN31-Verzeichnis. Folgende Standardwerte werden

festgelegt:

I/O-Addresse : 0x160h

IRQ          : 15

Sind diese Parameter bereits von einer anderen Komponenten

in Ihrem System belegt, so koennen Sie andere Werte waehlen.

Hierzu gehen Sie wie folgt vor:

- Wechseln Sie auf DOS-Ebene (keine DOS-Box) in das

Verzeichnis \WINDOWS.

- Oeffnen Sie die Datei CAPI.INI mit einem ASCII-Editor

(z.B.: "EDIT CAPI.INI")

- Suchen Sie nach der Sektion [CAPI PARAM]

- Fuegen Sie die Zeile "ADDPARAM [/a<I/O-Adresse>] [/r<IRQ>]"

ein.

Beispiel:  ADDPARAM = /a160 /r12

- Starten Sie nun Windows neu.

- Um die Aenderungen zu uebernehmen rufen Sie nun das Setup-

Programm aus der Programmgruppe ELSA ISDN Utilities erneut

auf.

- Ist waehrend des Setup eine Applikation aktiv, die auf die

DLL zugreift, z.B. cFos oder ISDNmon, so muessen Sie

Windows erneut starten, damit die CAPI.INI neu gelesen wird.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

4.      Bemerkungen:

V.120 unter CAPI 1.1 und CFOS (Version juenger als 15.7.96):

Geben Sie folgenden AT-Befehl:

AT B12 S17=1 S76=1

Nun wird bei allen Anwahlversuchen mit "ATDn" (n = Rufnummer)

versucht, eine V.120 Verbindung aufzubauen.

Wenn Sie nun wieder X.75 konfigurieren wollen, geben Sie:

AT&F

ein.

- - - - - - - - - - - - - - - - - - - - - - - -

V.120 unter CAPI 2.0 und CFOS (Version juenger als 15.7.96):

Geben Sie folgenden AT-Befehl:

AT B12 S17=1 S76=1

Nun wird bei allen Anwahlversuchen mit "ATDn" (n = Rufnummer)

versucht, eine V.120 Verbindung aufzubauen.

Wenn Sie nun wieder X.75 konfigurieren wollen, geben Sie:

AT&F

ein.

- - - - - - - - - - - - - - - - - - - - - - - -

Die neuesten Informationen sind in der Support-Mailbox ELSA ONLINE

oder auf unserer ELSA Homepage (http:\\www.elsa.de) verfuegbar.

Wir wuenschen Ihnen viel Freude beim Arbeiten mit Ihrer

ISDN-Karte.

#

| README file for

|

| ELSA QuickStep 1000

| Version 2.10

|

| Copyright (c) 1993-97 ELSA GmbH, Aachen (Germany)

|

|                                     Subsidiary:

| ELSA GmbH                           ELSA Inc.

| Sonnenweg 11                        2150 Trade Zone Blvd.

| D-52070 Aachen                      Suite 101

| Germany                             San Jose, CA 95131

|                                     USA

|

| Phone      : +49-241-9177-0         Phone: +1-408-935-0350

| Support Fax: +49-241-9177-113       Phone:  1-800-272-ELSA

| BBS (modem): +49-241-9177-981       Fax  : +1-408-935-0370

| BBS (ISDN) : +49-241-9177-7800      BBS  : +1-408-935-0380

| CompuServe : GO ELSA                CIS  : GO ELSA

| WWW        : http://www.elsa.de     WWW  : http://www.elsa.com

|

| 02/01/97

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Table of Contents

1.      Overview

2.      Contents

3.      Installation

4.      Notes

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

1.      Overview:

This file contains notes and changes to the

ISDN drivers.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

2.      Contents:

This directory contains the following files:

README.TXT    this file

LIESMICH.TXT  german version of this README.TXT

SETUP.EXE     installation tool for Windows 3.1x

2.1     [DRIVER]

DLL (dynamic link library) for 1TR6 (national ISDN):

CAPI1TR.DLL  CAPI version 1.1 driver for ELSA QuickStep 1000

D channel protocol : 1TR6 (national ISDN)

B channel protocols: HDLC transparent, bit transparent,

X.75, T.70NL, V.120

ISO 8208 (X.25), T.90

CA201TR.DLL  CAPI version 2.0 driver for ELSA QuickStep 1000

D channel protocol : 1TR6 (national ISDN)

B channel protocols: HDLC transparent, bit transparent,

X.75, T.70NL, V.120

ISO 8208 (X.25), T.90

DLL (dynamic link library) for DSS1 (EURO-ISDN):

CAPIDSS.DLL  CAPI version 1.1 driver for ELSA QuickStep 1000

D channel protocol : DSS1 (EURO-ISDN)

B channel protocols: HDLC transparent, bit transparent,

X.75, T.70NL, V.120

ISO 8208 (X.25), T.90

CA20DSS.DLL  CAPI version 2.0 driver for ELSA QuickStep 1000

D channel protocol : DSS1 (EURO-ISDN)

B channel protocols: HDLC transparent, bit transparent,

X.75, T.70NL, V.120

ISO 8208 (X.25), T.90

CAPI.DLL     for CAPI 1.1, DOS-TSR expected

CAPI20.DLL   for CAPI 2.0, DOS-TSR expected

2.2     [UTILS]

clrauto.exe   releases autoexec.bat to not load DOS CAPI

automaticly,

isdnmon.exe   ELSA ISDNmonitor forWindows 3.1x

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

3.      Installation:

The following illustration shows the mounting bracket of

your ISDN plug and play adapter board :

_____

\    |

|   |

|   |

|   |

|   |

|___|

|___|<- ISDN line connector

|   |

| o |<- ISDN connection status display (yellow LED)

| o |<- ISDN interface status display (green LED)

|___|

_

|

|

|

|

________________________________|

|                               _|

|                              |_ <- ISDN line connector

|                                |

|                                o<- yellow LED

|    _                      __   o<- green LED

|___| |____________________|  |__|

The ISDN plug and play adapter board is full software-

configurable and has NO jumper.

- - - - - - - - - - - - - - - - - - - - - - - -

a.) Installation with plug and play:

If you system supports plug and play you just have to

install the driver without any parameters. Use the setup-

program from the directory WIN31.

- - - - - - - - - - - - - - - - - - - - - - - -

b.) Installation without plug and play:

If your system doesn't support plug and play, you also have

to install the DLL's with the setup tool from the windows

directory. The driver uses the following defaults:

I/O-address : 0x160h

IRQ         : 15

If these parameters are used by an other component in your

system, you have to change the defaults as shown below:

- exit Windows and move to the directory \WINDOWS

- open the file CAPI.INI with an ASCII editor

(e.g. EDIT CAPI.INI)

- search for the section [CAPI PARAM]

- insert the line "ADDPARAM [/a<I/O-addresse>] [/r<IRQ>]"

e.g.: ADDPARAM = /a160 /r12

- start windows

- to takeover the changes start the setup tool from the

program group "ELSA ISDN Utilities" and go though it

without changes.

- if there is an application active, whitch uses the DLL,

for example cFos or the ISDNmon, you have to restart

Windows for the changed CAPI.INI is read again

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

4.      Notes:

V.120 under CAPI 1.1 and CFOS (version newer than 07/15/96):

Enter the following AT-command:

AT B12 S17=1 S76=1

Now, the ISDN adapter board tries to get a V.120 connection

erverytime you dial "ATDn" (n = number)..

If you want to get back to X.75 mode, you have to enter:

AT&F

- - - - - - - - - - - - - - - - - - - - - - - -

V.120 under CAPI 2.0 and CFOS (version newer than 07/15/96):

Enter the following AT-command:

AT B12 S17=1 S76=1

Now, the ISDN adapter board tries to get a V.120 connection

erverytime you dial "ATDn" (n = number)..

If you want to get back to X.75 mode, you have to enter:

AT&F

- - - - - - - - - - - - - - - - - - - - - - - -

The latest information you will always find in the

Support BBS ELSA ONLINE or on our homepage http:\\www.elsa.de.

Have fun with your ISDN board.