Win/TV

Software Development Guide

for the

Win/TV-Celebrity

Win/TV-HighQ

Win/TV-NG

Win/TV-Cinema

Release 3.82

June 10, 1996

Part No. 4704-05

Copyright Notice

CopyrightÓ 1995 Hauppauge Computer Works, Inc.  ALL RIGHTS RESERVED.

This manual is copyrighted by Hauppauge Computer Works, Inc.  You may not reproduce, transmit, transcribe, store in a retrieval system, or translate into any language or computer language, in any form or by any means, electronic, mechanical, magnetic, optical, chemical , manual, or otherwise, any part of this publication without the express written permission of Hauppauge Computer Works, Inc.

Restricted Rights Legend

Use, duplication, or disclosure by the Government is subject to restrictions set forth in subparagraph (c)(1)(ii) of the Rights in Technical Data and Computer Software clause at 252.277-7013.

Trademark Acknowledgment

is a registered trademark of Hauppauge Computer Works, Inc.

Win/TV is a  trademark of Hauppauge Computer Works, Inc.

IBM AT, XT, PS/2, Micro Channel, Personal System/2, Enhanced Graphics Adapter, Color Graphics Adapter, Video Graphics Adapter, IBM Color Display, and IBM Monochrome Display are trademarks of International Business Machines.

MS-DOS and Windows are trademarks of MicrosoftÒ

MultiSync is a trademark of Nippon Electric Company (NEC).

Disclaimer

This document is provided for the general information of the customer.  Hauppauge Computer Works Inc., reserves the right to modify the information contained herein as necessary and the customer should ensure that it has the most recent revision of the document.  Hauppauge Computer Works, Inc. makes no warranty for the use of its products and bears no responsibility for any errors which may appear in this document.  The customer should be on notice that the field of personal computers is the subject of many patents held by different parties.  Customers should ensure that they take appropriate action so that their use of the products does not infringe upon any patents.  It is the policy of Hauppauge Computer Works, Inc. to respect the valid patent rights of third parties and not to infringe upon or assist others to infringe upon such rights.

Table of Contents

Introduction

Theory of Operation

Win/TV Software

Sample Windows application

Sample DOS application

Sample Visual Basic application

Win/TV Windows/DOS Library

Win/TV Basic Functions

PCV_ClearVideoRect

PCV_CreateWindow

PCV_DisableColorKey

PCV_DisableInterlace

PCV_DisableVideo

PCV_EnableColorKey

PCV_EnableInterlace

PCV_EnableVideo

PCV_Exit

PCV_FreezeVideo

PCV_GetColor

PCV_GetInputFormat

PCV_GetIRQUsed

PCV_GetPortAddress

PCV_GetProfileInt

PCV_GetProfileString

PCV_GetRegister

PCV_GetSkewFactor

PCV_GetSystemMetrics

PCV_GetVideoAddress

PCV_GetVideoSource

PCV_HorizontalZoom

PCV_Initialize

PCV_LoadConfiguration

PCV_LoadClipboardFormat

PCV_LoadImageInRect

PCV_LoadImageRect

PCV_PanWindow

PCV_ResetColors

PCV_ResetSkewFactors

PCV_SaveConfiguration

PCV_SaveClipboardFormat

PCV_SaveImageRect

PCV_SetAcquisitionWindow

PCV_SetCaptureAddress

PCV_SetColor

PCV_SetColorKey

PCV_SetDisplayWindow

PCV_SetInputFormat

PCV_SetPortAddress

PCV_SetRegister

PCV_SetSkewFactor

PCV_SetVideoAddress

PCV_SetVideoMode

PCV_SetVideoScaling

PCV_SetVideoSource

PCV_SetWindowPosition

PCV_SetWindowSize

PCV_UnFreezeVideo

PCV_VerticalZoom

PCV_WaitVGARetrace

Win/TV Extended functions

WTV_CheckQFactor

WTV_CreateDIB

WTV_Exit

WTV_GetAudio

WTV_GetBoardType

WTV_GetCaptureColor

WTV_GetErrorCode

WTV_GetKeyColor

WTV_GetStatus

WTV_GetSVideo

WTV_GetTunerChannel

WTV_GetTunerType

WTV_GetVersion

WTV_GetVerticalInterpolation

WTV_HWOptions

WTV_LoadConfiguration

WTV_ReadI2C

WTV_ResetAudio

WTV_ResetCaptureColor

WTV_SetAudio

WTV_SetCaptureColor

WTV_SetFreezeResolution

WTV_SetSVideo

WTV_SetTunerChannel

WTV_SetTunerType

WTV_SetVerticalInterpolation

WTV_WriteI2C

WTV_WriteProfileInt

WTV_WriteProfileString

Win/TV Extended Accessory functions

WTVacc_AdjustColors

WTVacc_AlignVideo

WTVacc_PrinterSetup

WTVacc_PrintImage

WTVacc_ReadCmos

WTVacc_ConfigureHardware

Windows Dynamic Link Libraries

Calling functions in PCVIDEO.DLL

Linking with PCVIDEO.DLL

How the loader resolves references to the DLL

Using the PCVideo dynamic link library with Asymetrix ToolBook

DOS Demonstration Program

APPENDIX A

USA Tuner Frequency List

APPENDIX B

Media Control Interface (MCI)

Introduction

About MCI Device Commands

Driver Installation

Video Overlay Device Command Overview

Creating Command Strings

About MCI Device Types

Opening and Closing MCI Devices

Obtaining Information from MCI Devices

Using Flags

Supported Video Overlay Commands

# Introduction

Win/TV is the link between the worlds of video and computers. Full, crisp, live-action

video can be viewed in an infinitely re-sizable window. Images can be captured as individual

frames using the Win/TV application, and stored in a number of file formats. Live-action video clips can be captured using the Hauppauge Computer Works video capture driver and Microsoft's Video for Windows.

Size, position, image save/restore, and a full range of color adjustments are all under software control. The Software Development Guide that follows will allow users to create their own video application using a variety of development environments. Familiarity of the software environments presented is assumed, and the developer is referred to their documentation for specific details.

The software development guide includes...

3	A Windows 3.1 dynamic link library (DLL). The Windows DLL can be used with 			most Windows development tools including Asymetrix ToolBookÒ and 				Microsoft's Visual Basic applications.

3	DOS and Windows applications that demonstrate Win/TV  functionality 			3	Source to the Windows and DOS sample applications,

3	A stand-alone DOS demonstration program for Win/TV,

For software development you will need...

3	MicrosoftÒ  7.0 Optimizing Compiler, Visual C++ 1.5

3	MicrosoftÒ MASM 5.1 Assembler,

- OR -

3	MicrosoftÒ Visual Basic 1.0, 2.0, 3.0

- OR -

3	Any Windows development tool capable of DLL calls

If you are developing an application for Microsoft Windows you will need...

3	MicrosoftÒ Windows  3.1 Software Development Kit (SDK).

# Theory of Operation

The following discussion refers to the block diagram in figure 1. The Win/TV can digitize video sources in a variety of encoding standards. The video data, once digitized, can be stored in the on-board frame buffer and manipulated by the host CPU.

The frame buffer consists of 3 dual-ported Video RAMs. The parallel input port receives the data from the input video stream, while the serial output port will eventually become part of the VGA output stream. The video RAM array is controlled by a VXP500 Video Windowing Controller. This device can manipulate the video addresses of both the input parallel port, and the output serial port.  By manipulating these addresses, the image can be "decimated", I.E.: the device can drop every Nth pixel, or every Mth scan line where M and N are variables under software control.

The decimated image coming out of the RAM serial port is then converted back to the

analog domain via a DAC. The analog output from the DAC is fed into a video processor. The

video processor converts the DAC output to RGB, and also contains controls to adjust the

intensity and offsets of the RGB signal.

The RGB signal is fed into one input of a dual input analog multiplexer; the VGA-RGB video stream is fed into the other input. The MUX output is selected via a key signal generated by the VXP500. This key signal determines which video source is to be displayed on the VGA monitor during the video scan.

Figure 1 - Win/TV Block Diagram

# Win/TV Software

The software with the development guide can be used to evaluate the Win/TV product, and for development of demonstration programs, applications software, and drivers in the DOS and Windows environments. The Windows dynamic link library (DLL) or DOS object library (Large model - SDK Disk Version 3.7-3.79 does not suport DOS) can be used to incorporate live-video into your application software.

The DLL and Library routines control the overlay controller, Digital Video ICs, and the various audio and video DACs on the Win/TV board. Since the DOS and Windows libraries are built from the same source code, they have the same functions and interface. There are, however, some calls that rely on a Windows operating environment; these calls are not supported under DOS and are indicated in the text. Sample DOS and Windows applications (source and executable code) are provided as a starting point for your own applications.

PCVIDEO.LIB		Library file to be used in the link statement when linking applications 				that will call routines in PCVIDEO.DLL. This can be created from 					PCVIDEO.DEF using the IMPLIB utility. There is another PCVIDEO.LIB 				file in the DOS directory.

WINTVACC.LIB	Accessory functions library.

WINTV.H		Include file for primary WinTV functions.

WINTVACC.H		Include file for WTVACC functions.

WINTVCON.BAS	Declaration file for WinTV variables.

WINTVDEC.BAS	Declaration file for WinTV calls.

# Sample Windows application

WINWTV.EXE		Sample Windows application that  uses PCVIDEO.DLL. The 					program demonstrates all the basic functions of the Win/TV product.

WINWTV.C		C source code for the Windows application

WINWTV.H		Include file for the Windows demo application

WINWTV.DEF		Definition file for the Windows demo application

MAKEFILE.		Make for the Windows demo application

WINWTV.RC		Include file for the Windows demo application

# Sample DOS application

CHKWIN.EXE		Example DOS application which demonstrates all the basic functions

of the Win/TV product.

CHKWIN.C		Source code for the sample DOS application

MAKEFILE		Make file for the Example program.

CHKWIN.H		Include file for DOS application

REFPIC.TGA		High-resolution TGA format file used in the demonstration

# Sample Visual Basic application

EXAMPLE.EXE	Compiled Visual Basic application

EXAMPLE.MAK	MAKEFILE for Visual Basic compiler

GLOBAL.BAS		Global declarations for Visual Basic application

WINTV.FRM		Visual Basic Application.

# Win/TV Windows/DOS Library

# Win/TV Basic Functions

## PCV_ClearVideoRect

Syntax	int PCV_ClearVideoRect(xLeft, yTop, xWidth, yHeight)

This function clears a rectangular area of the capture buffer to zero.

Parameter	Type	Description

xLeft		Word	Pixel coordinate of left side of rectangle

yTop		Word	Pixel coordinate of top side of rectangle

xWidth		Word	Width of rectangle in pixels

yHeight		Word	Height of rectangle in pixels

Return Value	1	Success

0	Error writing the video buffer

In Visual Basic:

Declare Function PCV_ClearVideoRect Lib "pcvideo.dll" (ByVal xpos As Integer,

ByVal ypos As Integer,

ByVal xWidth As Integer,

ByVal yHeight As Integer ) As Integer

ClearRect = PCV_ClearVideoRect(0, 0, 1024, 512)

Comments	Make sure that the Win/TV frame buffer is not located where it will conflict with 			system memory.

See also	PCV_SetVideoAddress

## PCV_CreateWindow

Syntax	void PCV_CreateWindow(xLeft, yTop, xWidth, yHeight, fFitToWindow)

This function creates a video display window on the VGA at the specified position.

Parameter		Type	Description

xLeft			Word	Position in pixels of the left side of the video window on the VGA 					display

yTop			Word	Position in pixels of the top of the video window on the VGA 					display

xWidth			Word	Width of window in pixels

yHeight			Word	Height of window in pixels

fFitToWindow    	Bool	Flag indicating whether to fit video to the window.

1 = fit to window

0 = keep video full size and crop

Return Value	None

In Visual Basic:

Declare Sub PCV_CreateWindow Lib "pcvideo.dll" (ByVal X As Integer,

ByVal Y As Integer, 							  			    ByVal xWidth As Integer,

ByVal yHeight As Integer,

ByVal i As Integer)

Call PCV_CreateWindow(WinPos.x, WinPos.y, 125, 125, 1)

See also	PCV_SetWindowSize, PCV_SetWindowPosition, PCV_PanWindow

## PCV_DisableColorKey

Syntax	void PCV_DisableColorKey()

This function disables the color key comparator function.

Parameters	None

Return Value	None

Comments	Video will be displayed within the rectangle drawn by PCV_CreateWindow.

It will be the highest-priority displayed entity, IE: it will overlay anything

at the location specified by the PCV_CreateWindow call, including the

mouse cursor. See PCV_EnableColorkey if VGA graphics should be

allowed to overlay the live video.

See also	PCV_EnableColorKey, PCV_EnableVideo

## PCV_DisableInterlace

Syntax	void PCV_DisableInterlace()

This function disables interlaced output of the live video window.

Parameters	None

Return Value	None

Comments	This parameter must follow the display mode of the VGA card. The interlace

state is set during PCV_Initialize to match that of the VGA.

See also	PCV_EnableInterlace, PCV_GetSystemMetrics

## PCV_DisableVideo

Syntax	void PCV_DisableVideo()

This function disables the display of video.

Parameters	None

Return Value	None

See also	PCV_EnableVideo

## PCV_EnableColorKey

Syntax	void PCV_EnableColorKey()

This function enables use of a color key to display  video.  Video will be displayed at 	every pixel on the VGA display matching the color specified in the PCV_SetColorKey 	function, and inside the video display window specified with the PCV_SetWindowSize 	and PCV_SetWindowPosition or PCV_CreateWindow functions. For a discussion of

color keying, see PCV_SetColorKey.

Parameters	None

Return Value	None

Comments	The video display window and color key must also be set up before video will be 			displayed. Color keying is on by default. The default value for color key is set-			up by PCV_Initialize.

See also	PCV_DisableColorKey, PCV_DisableVideo, PCV_SetColorKey,

WTV_GetKeyColor, WTV_GetKeyColorIndex

## PCV_EnableInterlace

Syntax	void PCV_EnableInterlace()

This function enables interlaced output.  Interlaced output is dependent on the type of 	VGA and the VGA MODE selected. The Win/TV card must be programmed for the 	same type of field display output as the VGA card.

Parameters	None

Return Value	None

Comments	Interlaced output tends to make the VGA "flicker"  This is especially true if large

areas of white, or other high-luminance images are being displayed. In general, it 			is preferable to use Non-interlaced display output if the VGA hardware and video

monitor can support it. Non-interlaced output generally requires a higher-quality

Video monitor.

See also	PCV_DisableInterlace

## PCV_EnableVideo

Syntax	void PCV_EnableVideo()

This function enables the display of video captured in the Win/TV frame buffer.

Parameters	None

Return Value	None

Comments	The video display window  must be set up before video will be displayed. If color 			keying is not selected (via PCV_DisableColorKey), the video will be displayed 			over any VGA information at the location specified by PCV_CreateWindow.

See also	PCV_DisableVideo, PCV_DisableColorKey

## PCV_Exit

Syntax	void PCV_Exit()

This function disables display of video and saves the current configuration information to 	the file PCVIDEO.INI.

Parameters	None

Return Value	None

Comments	This function is included for compatibility. Use WTV_Exit for all new software 			development.

See also	PCV_DisableVideo, PCV_Initialize, WTV_Exit

## PCV_FreezeVideo

Syntax	 Bool PCV_FreezeVideo()

This function disables video capture. The last frame captured is displayed in the VGA 	display window until video is unfrozen using the PCV_UnFreezeVideo function.

Parameters	None

Return Value	None

Comments	This function does not affect how video in the video frame buffer is displayed. It 			only disables video capture. The image must be frozen prior to any access by 			the CPU to the frame buffer. The image will remain in the display buffer until

cleared by the CPU, or video capture is re-enabled. By changing the capture

address (using PCV_SetCaptureAddress), and if the capture window is small

enough, video can be captured to different portions of the Win/TV frame buffer.

See also	PCV_UnFreezeVideo, PCV_SetCaptureAddress

## PCV_GetColor

Syntax	BYTE PCV_GetColor(iColorIndex)

This function returns the setting of the specified color level.

Parameter		Type	Description

iColorIndex		Word	Color index  (Valid indices below)

Index

Brightness

Saturation

Contrast

Hue

Red

Green

Blue

Return Value	Value of the specified color level (0-255)

Comments	Returns the current working values for the listed video parameters. Upon 				software initialization, these values are preserved in PCVIDEO.INI, and

loaded as the defaults.

See also	PCV_SetColor, PCV_ResetColors, PCV_Initialize

## PCV_GetInputFormat

Syntax Word PCV_GetInputFormat()

This function returns the currently selected input video format.

Parameters	None

/* Input Formats */

#define	PAL_BGHI	0

#define	NTSC_M	1

#define	SECAM	2

#define	PAL_M		3

#define	PAL_N		4

#define	NTSC_443	5

#define EXTERNAL_SRC	6

Comments	Video input format refers to the encoding standard used by the video source. 			Encoding standards vary throughout the world. In general Western Europe 			uses the PAL standard; France, Russia, and other East European countries 			use SECAM. North America, Japan and parts of South America use the NTSC 			standard.

See also	PCV_SetInputFormat

## PCV_GetIRQUsed

Syntax	int PCV_GetIRQUsed(int)

This function sets/returns the hardware interrupt used by the software.  The default 	interrupt number is loaded from PCVIDEO.INI.

Parameters	  0 - Returns the current interrupt level being used for VBLANK

Int		¹0 -  Set new interrupt level to iCurIRQ

Return Value	Current IRQ level

Comments	Interrupts are not required for most applications. Must match IRQ level selected 			by interrupt jumper.

## PCV_GetPortAddress

Syntax	 Word PCV_GetPortAddress()

This function returns the Win/TV I/O port address.  This is the address set by default in 	the PCVIDEO.INI file. It does NOT reflect the hardware jumpered address.

Parameters	None

Return Value	Port address

Comments	The Win/TV Celebrity card has a default factory I/O address of 0x51C. The 			default board address is specific to the type of WinTV board you are using. The 			Celebrity card can be assigned the following alternate addresses:

0x53C  0x55C  0x57C  0x59C 0x5BC  0x5DC 0x5FC

Consult the Win/TV manual for jumper details.

See also	PCV_SetPortAddress

## PCV_GetProfileInt

Syntax	WORD FAR PASCAL PCV_GetProfileInt(LPSTR lpSectionName,

LPSTR lpKeyName)

This function returns the value of a key from the pcvideo.ini file.

Parameter		Type	Description

section			LPSTR	section header name within the .ini file

keyword		LPSTR	key name of parameter

Return Value

-1	Key does not exist

key	Key value

Comments

See also	WTV_WriteProfileInt, WTV_WriteProfileString, PCV_GetProfileString

## PCV_GetProfileString

Syntax	int FAR PASCAL PCV_GetProfileString(LPSTR lpSectionName,

LPSTR lpKeyName,

LPSTR lpReturnedString, int nSize)

This function returns the string value of a key from the pcvideo.ini file.

Parameter		Type	Description

lpSectionName		LPSTR	section header name within the .ini file

lpkeyName		LPSTR	key name of parameter

lpReturnerString	LPSTR Returned string information

nSize			Word 	Buffer size of returning string

Return Value

0	Key does not exist or no entry for selected key

¹0	Length of the profile string

Comments	These parameters are loaded during PCV_Initialize. They may be				individually interrogated for reference at any time after the PCV_Initialize

call has been executed. This call is similar to PCV_GetProfileInt, but returns

the actual string value entered in the .ini file. See windows getprivateprofilestring 			windows call for better description.

See also	PCV_Initialize, PCV_GetProfileInt, WTV_WriteProfileInt and

WTV_WriteProfileString

## PCV_GetRegister

Syntax	BYTE PCV_GetRegister(wIndex)

This function returns the value of the specified overlay controller register or I2C bus Peripheral.

Parameter	Type	Description

wIndex		word	If the high byte is 0, read VXP500.  Otherwise, this byte is the I2C 				address of one of the other Digital Video chips. The low byte is the 				subaddress of the register to be read.

15                8     7                   0

| 0 0 0 0 0 0 0 0 | VXP500 Reg |
| --- | --- |

15                8     7                   0

| I2C Address | Subaddress |
| --- | --- |

Return Value	Register value

In Visual Basic:

Declare Function PCV_GetRegister Lib "pcvideo.dll" (ByVal index As Integer)As Integer

SyncPol = SyncMode((PCV_GetRegister(&H4D) And &H30) \ 16)

Comments	For detailed descriptions of the overlay chip set registers, consult the Data book 			of the overlay chipset your WinTV board is using.

See also	PCV_SetRegister

## PCV_GetSkewFactor

Syntax	WORD PCV_GetSkewFactor(Word iAlignmentFactor)

This function returns the specified alignment setting.

Parameter		Type	Description

iAlignmentFactor	Word	Valid index values below

Index				Description

SF_DISPWINSKEWX		Display window X skew

SF_DISPWINSKEWY		Display window Y skew

SF_DISPADDRSKEWX		Display address X skew

SF_DISPADDRSKEWY		Display address Y skew

SF_SHIFTCLOCKSTART	Shift clock start

SF_PALETTESKEW		Palette skew

SF_DISPLAYWIDTH		Display Width (Horizontal Total) */

SF_PLLVCOGAIN		Display Width (Horizontal Total) */

SF_PLLPHDETGAIN		Display Width (Horizontal Total) */

Return Value	Value of the specified skew factor

Comments	These  timing-related parameters vary depending on the graphics mode and 			VGA card installed. The alignment settings should only have to be set once, but 			may vary on the same VGA card from mode to mode.

Once the parameters are set, PCV_Initialize will reload these values from

PCVIDEO.INI

See also	PCV_SetSkewFactor, PCV_ResetSkewFactors

## PCV_GetSystemMetrics

Syntax	WORD PCV_GetSystemMetrics(Word wMetric)

This function returns the information specified by the wMetric parameter.

Parameter	Type	Description

wMetric		word	Specifies which information is required.  Valid types are listed below.

Index				Description

sm_videowidth		Width of captured video

sm_videoheight		Height of captured video

SM_XRESOLUTION		Width of full source video resolution

SM_YRESOLUTION		Height of full source video resolution

SM_BOARDTYPE		BT_YUV411 or BT_YUV422

sm_version			Silicon revision number

sm_interlace		If interlaced output is on, the return

value=Interlace_on.  Otherwise, it is 0.

sm_replicate		If field replication is enabled, the return 								value=replicate_on.  Otherwise, it is 0.

sm_imagewidth		Width of image loaded with the

PCV_LoadImageRect command.

sm_imageheight		Height of image loaded.

sm_imagetype		Type of image loaded.

## PCV_GetVideoAddress

Syntax	 Word PCV_GetVideoAddress()

The function returns the integer megabyte number of the location of the Win/TV frame 	buffer. 	 The Following addresses are Valid video buffer addresses.

B000, B800, C800, D000, D800, E000, E800, 12, 13,  14, 15.

Parameters	None

Return Value	Video buffer address

Comments	If using upper memory address I.E. B800, please exclude the range (x=b800-			b7ff) in your EMM386.exe statement in your config.sys.  Some systems may 			require special consideration in advance cmos setup to disable the caching or 			shadowing from the region used. The Win/TV frame buffer location must not 			conflict with system RAM.

The buffer location megabyte value is stored as a parameter in PCVIDEO.INI.

Hauppauge Computer's PCVIDEO.DLL allows the Win/TV's frame buffer

to be placed at any megabyte number above system RAM.

See also	PCV_SetVideoAddress

## PCV_GetVideoSource

Syntax	 Word PCV_GetVideoSource()

Returns the currently selected video source.

Parameters	None

Return Value	Integer between 0 and 2

Comments	Video Source 0 is External Composite Video input 1.

Video Source 1 is External Composite Video input 2.

Video Source 2 is Tuner Video (If Equipped)

If video source 0 is S-VIDEO (S-VHS), then video source 1 may not be

selected (Video source 1 carries the chrominance information for the S-VIDEO 			channel)

See also	PCV_SetVideoSource

## PCV_HorizontalZoom

Syntax	void PCV_HorizontalZoom(iZoomFactor)

This function sets the output horizontal zoom factor.  Captured video can be scaled up by 	a factor of 2, 4, or 8 on output. Negative scaling is not possible. To decimate (scale 	down) an image, see PCV_SetVideoScaling.

Parameter	Type	Description

iZoomFactor	int	Zoom factor can range from 0 to 3.

0=1x,1=2x, 2=4x, and 3=8x.

Return Value	None

In Visual Basic:

Declare Sub PCV_HorizontalZoom Lib "pcvideo.dll" (ByVal Zoom As Integer)

Sub m_zoom_Click ()

If m_zoom.checked = NOT m_zoom.checked Then

m_zoom.Checked = 1

Call PCV_VerticalZoom(1)

Call PCV_HorizontalZoom(1)

Else

m_zoom.Checked = 0

Call PCV_VerticalZoom(0)

Call PCV_HorizontalZoom(0)

End If

End Sub

Comments	Since video information is inherently low-resolution, zoom factors higher than

2x tend to present blocky images.

See also	PCV_VerticalZoom, PCV_SetVideoScaling.

## PCV_Initialize

Syntax	int PCV_Initialize()

This function loads configuration information from PCVIDEO.INI, detects the current 	graphics mode, detects the Win/TV board implementation, and initializes the Digital 	Video chip set registers accordingly.

Parameters	None

Return Value	1=Success

0=Failure

In Visual Basic:

Declare Function PCV_Initialize Lib "pcvideo.dll" () As Integer

e = PCV_Initialize()

If e <= 0 Then

Display_WinTV_Error = MsgBox("Win/TV Initialization Error",

MB_STOP, "Sample Application")

Unload WinTV

Exit Sub

End If

Comments	If the card will not initialize, check to make sure that it is properly seated in the 			I/O connector, that it is NOT plugged into an 8-bit slot, and that the I/O address 			matches the one supplied in PCV_SetPortAddress.

Use WTV_GetErrorCode to get more detailed information about the failure.

See also	PCV_Exit, WTV_Exit, WTV_GetErrorCode, PCV_LoadConfiguration

## PCV_LoadConfiguration

Syntax	int PCV_LoadConfiguration()

This function loads the current configuration from the configuration file. This file includes 	information such as the assigned I/O port and video buffer addresses, registers settings, 	video alignment(skew) parameters, and color levels, etc.

Parameters		None

Return Value	 1	Success

0	File not found.

-1	File is missing data.

-2	File is not a PC Video configuration file.

In Visual Basic:

Declare Function PCV_LoadConfiguration Lib "pcvideo.dll" () As Integer

Init =  PCV_LoadConfiguration()

Comments	Under Windows: PCVIDEO.INI contains the configuration information

Under DOS:	The filename specified by the WINTVINI environment

variable, or PCVIDEO.INI in the current directory contains

the configuration information.

PCV_Initialize  loads the configuration file. It is not generally necessary 				to use PCV_LoadConfiguration. PCV_LoadConfiguration is included for 				compatibility with older DLLs. For all new software development, use 				WTV_LoadConfiguration.

See also	PCV_Exit, WTV_Exit, PCV_SaveConfiguration, PCV_Initialize

## PCV_LoadClipboardFormat

Syntax	int PCV_LoadClipbordFormat(hWnd, xLeft, yTop)

This function retreives the image from the clipboard, detects the data format, converts 	data to the video buffer format and writes it to the video buffer at position (xLeft, yTop).

Parameter	Type	Description

hWnd		HWND	Handle identifying the window to be associated with the open clipboard

xLeft		Word	Pixel coordinate of left side of block

yTop		Word	Pixel coordinate of top side of block

Return Value 	 1	Success

0	Error writing Win/TV frame buffer

-1	Clipboard is not available

-2	Unsupported clipboard format

-3	Clipboard data error

-4	Memory could not be allocated

-5	Memory could not be locked

-6	Unsupported file format

In Microsoft C:

err_code= PCV_LoadClipboardFormat(hWnd, org.x, org.y)

Switch (err_code)

{

case 0

MyMessage("Error writing video buffer.");

case -1

MyMessage("Clipboard is not available.");

case -2

MyMessage("Unsupported Clipboard format.");

case -3

MyMessage("Clipboard data error.");

case -4

MyMessage("Memory could not be allocated.");

case -5

MyMessage("Memory could not be locked.");

case -6

MyMessage("Unsupported file format.");

}

Comments	Make sure that the PC Video buffer is not located where it will conflict with 			system memory. This command works in Windows only.

See also	PCV_SaveClipboardFormat, PCV_SetVideoAddress

## PCV_LoadImageInRect

Syntax	int FAR PASCAL PCV_LoadImageInRect(LPSTR lpFile, RECT rc)

This function reads the specified image file, detects the format, converts the image data 	to the video buffer format, and writes it to the video buffer at location (xLeft, yTop). The

function is similar to PCV_LoadImageRect, but uses the Windows RECT construct to

pass the size of the bounded area.

Parameter	Type	Description

pFile		char *	Pointer to a file name string

RECT    	struc					/* From windows.h */

word	rcLeft

word	rcTop

word	rcRight

word	rcBottom

RECT    ends

Return Value	 1	Success

0	Error writing video buffer

-1	File open error

-2	Memory could not be allocated

-3	Insufficient data

-4	Unrecognized file format

In Microsoft C:

int FAR PASCAL PCV_LoadImageInRect(LPSTR lpFile, RECT rc)

status = PCV_LoadImageInRect(lpLoad->lpfilename,rc);

Comments	Make sure that the PC Video buffer is not located where it will conflict with 			system memory.

See also	PCV_SaveImageRect, PCV_SetVideoAddress, PCV_LoadImageRect

## PCV_LoadImageRect

Syntax	int PCV_LoadImageRect(pFile,xLeft,yTop)

This function reads the specified image file, detects the format, converts the image data 	to the video buffer format, and writes it to the video buffer at location (xLeft, yTop).

Parameter	Type	Description

pFile		LPSTR	Pointer to a file name string

xLeft		Word	Pixel coordinate of left side of block

yTop		Word	Pixel coordinate of top side of block

Return Value	 1	Success

0	Error writing video buffer

-1	File open error

-2	Memory could not be allocated

-3	Insufficient data

-4	Unrecognized file format

In Visual Basic:

Declare Function PCV_LoadImageRect Lib "pcvideo.dll"  ( ByVal FileName As String,

ByVal xpos As Integer,

ByVal ypos As Integer) As Integer

Call PCV_FreezeVideo

Load_it = PCV_LoadImageRect(fn$, 0, 0)

Comments	Make sure that the PC Video buffer is not located where it will conflict with 			system memory.

See also	PCV_SaveImageRect, PCV_SetVideoAddress

## PCV_PanWindow

Syntax	void PCV_PanWindow(xLeft, yTop)

This function positions the video display within the VGA window.

Parameter	Type	Description

xLeft		Word	Position in the video buffer of the left side of the region to be displayed 				(in pixels).

yTop		Word	Position in the video buffer of the top of the region to be displayed (in 				pixels)

Return Value	None

In Visual Basic:

Declare Sub PCV_PanWindow Lib "pcvideo.dll" (ByVal X As Integer, ByVal Y As Integer)

Call PCV_UnFreezeVideo()

Call PCV_PanWindow(0, 0)

Call PCV_EnableVideo()

Comments	This has no affect on the size or position of the display window on the VGA.  It 			only changes the portion of the Win/TV frame buffer displayed in that window.

See also	PCV_CreateWindow, PCV_SetWindowSize, PCV_SetWindowPosition

Syntax	 void PCV_ReadImageRect (fpBuffer,xLeft, yTop, xWidth, 								      yHeight,BM_Type,fpWorkspace)

This function reads the image data from the video buffer (4:1:1 YUV) and converts it to the specified format.

Parameter	Type	Description

fpBuffer		LPSTR	Pointer to output buffer

xLeft		Word	Position in the video buffer of the left side of the region to be displayed 				(in pixels).

yTop		Word	Position of the top of the region to be saved (in pixels)

yWidth		Word	Width of the display region to save to the buffer  (in pixels)

yHeight		Word	Height of the display region to save to the buffer  (in pixels)

BM_Type	Word	Type of bitmap to translate to. See PCV_SaveImageRect

for valid types.

fpWorkspace	char far * Pointer to palettization workspace; 64K bytes should be sufficient for

most palettized file formats. If the format does not require palettization, 				this parameter can be set  to 0 (NULL).

Return Value	 1	Success

0	Error reading video buffer

In Visual Basic:

Declare Function PCV_ReadImageRect Lib "pcvideo.dll" (Buffer%,

ByVal Xpos%, ByVal Ypos%,

ByVal Xsize%, ByVal Ysize%,

ByVal FType%, ByVal ptr&)

AS INTEGER

iStatus = PCV_ReadImageRect (BUFF, 0, 0, 100, 100, BM_YUV411, 0);

Comments	This function both reads and converts the data in the frame buffer. The

conversion process may require a pointer to a workspace if a palettization

is required. Large images may require multiple calls to assemble the image

because the buffer size is restricted to 64k. A simple way to accomplish this

is by calling this procedure once per scanline and assembling the resulting

image into a file or a HUGE declared buffer.

See also	PCV_LoadImageInRect, PCV_WriteImageRect

## PCV_ResetColors

Syntax	void PCV_ResetColors()

This function resets all color levels to default values.

Parameters	None

Return Value	None

In Visual Basic:

Declare Sub PCV_ResetColors Lib "pcvideo.dll" ()

Call PCV_ResetColors()

See also	PCV_GetColor, PCV_SetColor

## PCV_ResetSkewFactors

Syntax	void PCV_ResetSkewFactors()

This function resets all alignment factors to default values.

Parameters	None

Return Value	None

See also	PCV_GetSkewFactor, PCV_SetSkewFactor

## PCV_SaveConfiguration

Syntax	int PCV_SaveConfiguration()

This function saves the current configuration to the PCVIDEO.INI file. This file includes 	information such as the assigned I/O port and video buffer addresses, registers settings, 	video alignment(skew) parameters, and color levels, etc.

Parameters		None

Return Value	 1	Success

0	File create error, disk may be full.

-1	File write error, disk may be full.

In Visual Basic:

Declare Function PCV_SaveConfiguration Lib "pcvideo.dll" () As Integer

iExit =  PCV_SaveConfiguration()

Comments	Under Windows: PCVIDEO.INI contains the configuration information

Under DOS:	The filename specified by the WINTVINI environment

variable, or PCVIDEO.INI in the current directory contains

the configuration information.

See also	PCV_Exit, WTV_Exit

## PCV_SaveClipboardFormat

Syntax     int PCV_SaveClipboardFormat(hWnd, xLeft, yTop, width, height, 								   wDataType, wOptions)

This function reads a rectangular block of pixels from the video frame buffer, converts the 	data, and writes this to the clipboard in the specified data format.

Parameter	Type	Description

hWnd		HWND	Identifies the window to be associated with the open clipboard

xLeft		Word	Pixel coordinate of left side of block

yTop		Word	Pixel coordinate of top side of block

width		Word	Width of block in pixels

height		Word	Height of block in pixels

wDataType	Word	Contains the internal data type in its low-order word and clipboard data 				type in its high-order word

Data Type		Description

Clipboard data type

CLP_BITMAP		The device-specific memory bitmap

CLP_DIB		The device-independent bitmap

Internal data type

bm_dib24		Windows 3.0 24 bit color

bm_dib8g		Windows 3.0  8 bit gray scale

bm_dib8p		Windows 3.0  8 bit color (palettized)

wOptions	word	Miscellaneous options (must be 0)

Return Value	>0	Success

0	Error reading the frame buffer

-1	File creation error

-2	Memory allocation error

-3	File write error

-4	Palette creation error

In Visual Basic:

Declare Function PCV_SaveClipBoardFormat Lib "pcvideo.dll"

(ByVal hWnd As Integer,

ByVal xpos As Integer, ByVal ypos As Integer,

ByVal xsize As Integer, ByVal ysize As Integer,

ByVal filetype As Integer,

ByVal Options As Integer) As Integer

iClip = PCV_SaveClipBoardFormat(hWnd, 0, 0, w, h, Clipboard_Format, 0)

Comments	Make sure that the PC Video buffer is not located where it will conflict with 			system memory. This function works only under WINDOWS.

See also	PCV_LoadClipboardFormat, PCV_SetVideoAddress

## PCV_SaveImageRect

Syntax     int PCV_SaveImageRect(pFile, xLeft, yTop, width, height, wFileType, wOptions)

This function reads a rectangular block of pixels from the video frame buffer, converts the 	data, and writes this to the specified  file in the specified image format.

Parameter	Type	Description

pFile		LPSTR	Pointer to a file name string

xLeft		Word	Pixel coordinate of left side of block

yTop		Word	Pixel coordinate of top side of block

width		Word	Width of block in pixels

height		Word	Height of block in pixels

wFileType	Word	File type

File Type		Description

bm_yuv411		IBM MMotion (4:1:1 YUV) format

bm_dib24		Windows 3.0 24 bit device-independent bitmap format (DIB)

bm_dib8g		Windows 3.0 DIB 8 bit gray scale

bm_dib8p		Windows 3.0 DIB 8 bit color (palettized)

BM_DIB4D		Windows DIB 4 bit dithered

BM_DIB1P        		BMP monochrome

BM_DIB4P        		BMP 16 colors palettized

bm_trg24		Targa 24 bit format

bm_tiff24		Tiff 24 bit format

bm_tiff8p		Tiff 8 bit color (palettized)

BM_TIFF8G		TIFF 8 bit gray-scale

bm_pcx8p		PCX 8 bit color (Palettized)

bm_pcx8g		PCX 8 bit gray scale

BM_PCX4D		PCX 4 bit dithered */

BM_GIF8P        		GIF 8 bit palettized

BM_GIF8G        	GIF 8 bit gray-scale

BM_JPEG		JPEG

wOptions	word

if wFileType == BM_DIB24 or BM_DIB8P or BM_DIB8G or BM_DIB4D or BM_DIB1P or 				BM_DIB4P

wOptions	0	uncompressed

1	compressed

if wFileType == BM_TIFF24 or BM_TIFF8P or BM_TIFF8G

wOptions	0	uncompressed

1	pack bits

2	modified huffman

3	Lempel Ziff Welch (LZW)

if wFileType == BM_JPEG

wOptions	0 through 100

0 = lowest quality, smallest file size

100 = highest quality, largest file size

Return Value	>0	Number of colors found in the original image

0	Error reading the video buffer

-1	File creation error

-2	Memory allocation error

-3	File write error

In Visual Basic:

Declare Function PCV_SaveImageRect Lib "pcvideo.dll" (ByVal FileName As String,

ByVal xpos As Integer,

ByVal ypos As Integer, 					 					ByVal xsize As Integer,

ByVal ysize As Integer,

ByVal filetype As Integer,

ByVal Options As Integer) As Integer

iSave = PCV_SaveImageRect(fn$, 0, 0, w, h, File_Type, 0)

Comments	Make sure that the Win/TV frame buffer is not located where it will conflict with

system memory.

See Also	PCV_LoadImageRect

## PCV_SetAcquisitionWindow

Syntax	void PCV_SetAcquisitionWindow(xLeft, yTop, xWidth, yHeight)

This function specifies what portion of the input video image to capture. By default, the 	acquisition window is set according to the values specified in the PCVIDEO.INI file.  	Changing the width or height of the acquisition window may require video scaling to be 	re-adjusted.

Parameter	Type	Description

xLeft		int	x coordinate of left side of acquisition window

yTop		int	y coordinate of top side of acquisition window

xWidth		int	x coordinate of right side of acquisition window

yHeight		int	y coordinate of bottom side of acquisition window

Return Value	None

In Visual Basic:

Declare Sub PCV_SetAcquisitionWindow  Lib "pcvideo.dll" (ByVal xLeft As Integer,

ByVal yTop As Integer,

ByVal xWidth As Integer,

ByVal yHeight As Integer)

Call PCV_SetAcquisitionWindow(X1, Y1, X2, Y2)

Comments	Changing the width or height of the acquisition window will affect scaling.  For 			example, if the width and height are set to 200 and the display window width and 			height are 100, input video must be scaled down by a factor of 2. If the 				acquisition window width and height were changed to 100, and the display 			window remained the same, no scaling would be needed. This function is

usually called when using video formats (I.E. PAL) that contain more information

than the frame buffer is capable of storing. The input buffer is only capable

of capturing 512 lines. This is not a problem with NTSC signals, since they only

contain 485 displayable lines; the PAL format contains 585 displayable lines.

See Also	PCV_SetVideoScaling

## PCV_SetCaptureAddress

Syntax	void PCV_SetCaptureAddress(xLeft, yTop)

This function sets the location in the Win/TV frame buffer where captured video should 	be stored.  By default this is set to 0,0.  To display video captured at a location other than 	the default, you need to pan to 	this part of the video buffer using the PCV_PanWindow 	function.

Parameter	Type	Description

xLeft		int	x coordinate of left side of captured video in buffer

yTop		int	y coordinate of top side in the capture buffer.

Return Value	None

In Visual Basic:

Declare Sub PCV_SetCaptureAddress Lib "pcvideo.dll" (ByVal X As Integer,

ByVal Y As Integer)

Call PCV_FreezeVideo()

Call PCV_SetCaptureAddress(0, 0)

Call PCV_CreateWindow(WinPos.x, WinPos.y, Xsize, Ysize, Fit)

See Also	PCV_SetPortAddress, PCV_PanWindow

## PCV_SetColor

Syntax	void PCV_SetColor(iColorIndex, iColorValue)

This function sets the specified color level.

Parameter		Type	Description

iColorIndex		word	Color index (Valid indices below)

Index

brightness

Saturation

Contrast

Hue

iColorValue		byte	Color value between 0 and 255.

Return Value	None

In Visual Basic:

Declare Sub PCV_SetColor Lib "pcvideo.dll" (ByVal index As Integer, ByVal value As Integer)

call PCV_SetColor(Index, color[index]);

In Microsoft C:

#define NO_COLOR_CONTROLS 7		/* Actual number of color controls */

BYTE bColor[NO_COLOR_CONTROLS];	/* Current color settings (B,S,C,H,R,G,B) */

PCV_SetColor(index,bColor[index]);

See also	PCV_GetColor, PCV_ResetColors

## PCV_SetColorKey

Syntax	void PCV_SetColorKey(wColorIndex)

This function sets the color index used as the color key for video display.

Parameter	Type	Description

wColorIndex	Word	Specifies the color index to be used as the key for selecting between the 				video frame buffer and the VGA display.

Return Value	None

Comments	Any pixel within the VGA generated portion of the display window set to this color  		will display video from the Win/TV frame buffer.

The Win/TV, in professional video terminology, functions as a downstream 			keyer. The image displayed on the VGA monitor is actually a composite of the 			original VGA image, with a scaled live video image superimposed over it. The  			Win/TV board	provides the circuitry to scale the input video image, store it, and  		generate a "key	signal" to indicate where to insert the live video image.

The  VXP500 video controller IC has a built-in digital comparator. This				logic compares the internally stored "key" color with every pixel on the 				screen ).  If the 	comparison matches, an analog switch is activated to display the 		live video image for the duration of the comparison being equal. Once the key	 		color no longer matches, the video stream reverts to the VGA image.

Recommendation

PCV_Initialize will establish a default key color that should work with the

VGA installed. Use WTV_GetKeyColor after PCV_Initialize to get the

default color.

See also	PCV_EnableColorKey

## PCV_SetDisplayWindow

Syntax	void PCV_SetDisplayWindow(xLeft, yTop, xWidth, yHeight)

This function sets the size and location of the video display window without affecting 	scaling.

Parameter	Type	Description

xLeft		Word	x coordinate of left side of display window.

yTop		Word	y coordinate of top side of display window.

xWidth		Word	Display window width

yHeight		Word	Display window height

Return Value	None

In Visual Basic:

Declare Sub PCV_SetDisplayWindow Lib "pcvideo.dll" (ByVal X As Integer,

ByVal Y As Integer,

ByVal xWidth As Integer,

ByVal yHeight As Integer)

call PCV_SetDisplayWindow(x1, y1, width, height)

Comments	Do not confuse this function with PCV_PanWindow which positions the

video image within the display window.

See also	PCV_PanWindow

## PCV_SetInputFormat

Syntax	void PCV_SetInputFormat(iVideoFormat)

This function sets the input video format.

Parameter	Type	Description

iVideoFormat	Word	Video standard

PAL_BGHI

NTSC_M

SECAM

PAL_M

PAL_N

NTSC_44

Return Value	None

In Visual Basic:

Declare Function PCV_SetInputFormat Lib "pcvideo.dll" (ByVal iForm As Integer) As Integer

Current_Format = PCV_SetInputFormat(1)	' Set input format to NTSC

Comments	Video input format refers to the encoding standard used by the video source. 			Encoding standards vary throughout the world. In general Western Europe uses 			the PAL standard; France, Russia, and other East European countries use 			SECAM. North America, Japan and parts of South America use the NTSC 			standard.

See also	PCV_GetInputFormat, WTV_HWOptions

## PCV_SetPortAddress

Syntax	 Bool PCV_SetPortAddress(iPortAddr)

This function sets the PC Video port address.

Parameter	Type	Description

iPortAddr	Word	Win/TV I/O port address

Return Value	None

In Visual Basic:

Declare Function PCV_SetPortAddress Lib "pcvideo.dll" (ByVal iPort As Integer) As Integer

Set_Port = PCV_SetPortAddress(&H5D6) 	' Commonly used as the third Win/TV

' card address location

Comments	The Win/TV Celebrity card has a default factory I/O address of 0x51C (This 			address is board specific, please  check your Win/TV manual for the proper 			setting on your board).   The card can be assigned the following alternate 			addresses:

0x53C  0x55C  0x57C  0x59C

Consult the Win/TV manual for jumper details.

See also	PCV_GetPortAddress

## PCV_SetRegister

Syntax	Byte PCV_SetRegister(WORD wIndex, BYTE bValue)

This function sets the value of the specified Digital Video chip register.

Parameter	Type	Description

wIndex		word	If the high byte is 0, write to the Overlay Video Chip. Otherwise, this byte 			specifies the I2C address of the I2C peripheral to be written to. The low 				byte is the index of the register to be written to.

bValue		byte	Value to be written

15                8    7                   0

| 0 0 0 0 0 0 0 0 | overlay chip |
| --- | --- |

15                8    7                   0

| I2C Address | Subaddress |
| --- | --- |

Return Value	None

See also	PCV_GetRegister

## PCV_SetSkewFactor

Syntax	void PCV_SetSkewFactor(iAlignmentFactor, wValue)

This function sets the specified video alignment factor.  These are timing related 	parameters that varies depending on the graphics mode and VGA card installed.

Parameter		Type	Description

iAlignmentFactor	Word	Index specifying the parameter to set. Valid values are given 					below.

Index				Description

sf_dispwinskewx		Display window X skew

sf_dispwinskewy		Display window Y skew

sf_dispaddrskewx		Display address X skew

sf_dispaddrskewy		Display address Y skew

sf_shiftclockstart	Shift clock start

sf_paletteskew		Palette skew

SF_DISPLAYWIDTH		Display Width (Horizontal Total)

SF_PLLVCOGAIN		Display Width (Horizontal Total)

SF_PLLPHDETGAIN		Display Width (Horizontal Total)

wValue			Word 	New value of alignment factor

Return Value	None

In Visual Basic:

Declare Sub PCV_SetSkewFactor Lib "pcvideo.dll" (ByVal index As Integer,

ByVal value As Integer)

call PCV_SetSkewFactor (SF_PALETTESKEW,									   (PCV_GetSkewFactor(SF_PALETTESKEW) + 2 ),

Comments	These parameters may require adjust for each new VGA card, or new VGA mode

See also	PCV_GetSkewFactor, PCV_ResetSkewFactors

## PCV_SetVideoAddress

Syntax	 Bool PCV_SetVideoAddress(iVideoAddr)

This function sets the video buffer address.  The Following addresses are Valid and can be used when setting the video buffer address.

B000, B800, C800, D000, D800, E000, E800, 12, 13,  14, 15 .

Parameter		Type	Description

iVideoAddr		Word	Integer number in megabytes of the location of the Win/TV

frame buffer.

Return Value	None

In Visual Basic:

Declare Sub PCV_SetVideoAddress Lib "pcvideo.dll" (ByVal meg As Integer)

call PCV_SetVideoAddress(PCV_GetVideoAddress())

Comments	If using upper memory address I.E. B800, please exclude the range (x=b800-			b7ff) in your EMM386.exe statement in your config.sys.  Some systems may 			require special consideration in advance cmos setup to disable the caching or 			shadowing from the region used.

See also	PCV_GetVideoAddress

## PCV_SetVideoMode

Syntax	void PCV_SetVideoMode(WORD fCropVideo)

This function sets the acquired video either in cropping mode or in scaled mode (scaled 	to fit the frame buffer). This function is useful when the input video resolutions is larger 	than the frame buffer size.

Parameter	Type	Description

fCropVideo	word	Flag indicating whether to keep video full size and crop or to fit video to 				the frame buffer.

1 = crop

0 =  fit to buffer.

Comments	When the fCropVideo is set, the panning range for pcvideo will be the full video 			resolution set in PCV_GetSystemMetrics. The full input video resolution can 			be viewed by changing the position point in PCV_PanWindow.

See also	PCV_PanWindow, PCV_GetSystemMetrics

## PCV_SetVideoScaling

Syntax	void PCV_SetVideoScaling(xWidth, yHeight, fFitToWindow)

This function sets the input scaling of captured video without affecting the display window 	size.

Parameter	Type	Description

xWidth		Word	Width of scaled video

yHeight		Word	Height of scaled video

fScale		bool	Flag indicating whether to enable or disable scaling

Return Value	None

In Visual BASIC:

Declare Sub PCV_SetVideoScaling Lib "pcvideo.dll" (ByVal xWidth As Integer,

ByVal yHeight As Integer,

ByVal fit As Integer)

Call PCV_SetVideoScaling(width, height, 1)

In Microsoft C:

#define	WIDTH		32

#define	HEIGHT	32

#define	FIT		1

PCV_SetVideoScaling(WIDTH,HEIGHT,FIT);		/* Calculate scaling for 32 x 32 box */

PCV_SetDisplayWindow( 0, 0, WIDTH, HEIGHT);	/* Calculate size and position          */

PCV_PanWindow(0,0);					/* Position the video within the window */

Comments	This function only calculates the scaling values for the given size of box.

The window must still be positioned and panned to align the video properly.

See also	PCV_SetDisplayWindow, PCV_PanWindow, PCV_SetVideoScaling

## PCV_SetVideoSource

Syntax	void PCV_SetVideoSource(iSource)

Selects one of the three possible video input sources.

Parameter	Type	Description

iSource		Word	Source index (0-2)

Return Value	None

Declare Sub PCV_SetVideoSource Lib "pcvideo.dll" (ByVal SOURCE As Integer)

Call PCV_SetVideoSource(2)		' Set video source to TUNER

Comments	Video Source 0 is External Composite Video input 1

Video Source 1 is External Composite Video input 2

Video Source 2 is Tuner Video (If Equipped)

If video source 0 is S-VIDEO (S-VHS), then video source 1 may not be

selected (Video source 1carries the chrominance information for the S-VIDEO 			channel)

See also	PCV_GetVideoSource

## PCV_SetWindowPosition

Syntax	void PCV_SetWindowPosition(xLeft, yTop)

This function sets the position of the top left corner of the video display window on the 	VGA.

Parameter	Type	Description

xLeft		Word	Position in pixels of the left side of the video window on the VGA display

yTop		Word	Position in pixels of the top of the video window on the VGA display

Return Value	None

In Visual BASIC:

Declare Sub PCV_SetWindowPosition Lib "pcvideo.dll" (ByVal X As Integer, ByVal Y As Integer)

call PCV_SetWindowPosition ( 100, 200 )	' Set the Top-Left corner of the video..

' ..display to the VGA coordinates..

' .. X=100, Y=200.

Comments	The parameters are in terms of VGA pixels. The top-left corner of the

VGA screen is (0,0). The bottom right corner is (MAX_X, MAX_Y)

See also	PCV_CreateWindow, PCV_SetWindowSize, PCV_PanWindow

## PCV_SetWindowSize

Syntax	void PCV_SetWindowSize(xWidth, yHeight, fFitToWindow)

This function sets the width and height of video display window on the VGA.

Parameter	Type	Description

xWidth		Word	Width of window in pixels

yHeight		Word	Height of window in pixels

fFitToWindow	Bool	Flag indicating whether to fit video to the window.

1 = fit to window

0 = keep video full size and crop

Return Value	None

In Visual Basic:

Declare Sub PCV_SetWindowSize Lib "pcvideo.dll" (ByVal xsize As Integer,

ByVal ysize As Integer,

ByVal fit As Integer)

call PCV_SetWindowSize( 100, 100 )	' Bound the displayable video in a square..

' .. 100 pixels wide by 100 pixels high

Comments	This function defined the size of the displayed video window.

See also	PCV_CreateWindow, PCV_SetVideoScaling, PCV_PanWindow

## PCV_UnFreezeVideo

Syntax	void PCV_UnFreezeVideo()

This function re-enables video capture.

Return Value	None

In Visual Basic:

Declare Sub PCV_UnFreezeVideo Lib "pcvideo.dll" ()

Call UnfreezeVideo()

Comments	This function re-enables video capture only. It does not affect how or whether 			captured video is displayed.  Display of captured video is	controlled with the 			PCV_EnableVideo, PCV_SetWindowSize, and PCV_SetWindowPosition 			functions.

See also	PCV_FreezeVideo

## PCV_VerticalZoom

Syntax	void PCV_VerticalZoom(iZoomFactor)

This function sets the vertical zoom factor.  Captured video can be scaled up by a factor 	of 2, 4, or 8 on output.

Parameter	Type	Description

iZoomFactor	Word	Zoom factors can range from 0 to 3.

0 =1x,1=2x, 2=4x, and 3=8x.

Return Value	None

Declare Sub PCV_VerticalZoom Lib "pcvideo.dll" (ByVal Zoom As Integer)

Call PCV_VerticalZoom(1)

Comments	Since video information is inherently low-resolution, zoom factors higher than

2x tend to present blocky images.

See also	PCV_HorizontalZoom

## PCV_WaitVGARetrace

Syntax	void PCV_WaitVGARetrace()

This function waits until the start of VGA vertical retrace is detected.  This is useful when 	trying to make smooth updates to the VGA display.

Return Value	None

In Visual Basic:

Declare Sub PCV_WaitVGARetrace() Lib "pcvideo.dll" ()

call PCV_WaitVGARetrace()

Comments	The code will loop until a VGA VERTICAL RETRACE occurs, or an internal 			time-out has elapsed. The loop polls the VGA retrace bit in the 82C9001A. The

vertical retrace time varies from VGA mode to VGA mode.

Syntax	void PCV_WriteImageRect (fpBuffer,xLeft, yTop, xWidth, yHeight, 							      BM_Type,fpWorkspace )

This function converts the data to the specified format, and loads the image into the frame buffer in 4:1:1 YUV format.

Parameter	Type	Description

fpBuffer		char	Pointer to input buffer

xLeft		int	Position in the video buffer of the left side of the region to be displayed 				(in pixels).

yTop		int	Position in the video buffer of the top of the region to be saved (in 				pixels)

yWidth		int	Width of the display region to save to the buffer  (in pixels)

yHeight		int	Height of the display region to save to the buffer  (in pixels)

BM_Type	word	Type of bitmap to translate to. See PCV_SaveImageRect

for valid types.

fpWorkspace	char	Pointer to palettization workspace; 64K bytes should be sufficient for

most palettized file formats. If the format does not require palettization, 				this parameter can be set  to 0 (NULL).

Return Value	 1	Success

0	Error writing video buffer

In Visual Basic:

Declare Function PCV_WriteImageRect Lib "pcvideo.dll" (Buffer%,

ByVal Xpos%, ByVal Ypos%,

ByVal Xsize%, ByVal Ysize%,

ByVal FType%, ByVal ptr&)

AS INTEGER

iStatus = PCV_WriteImageRect (BUFF, 0, 0, 100, 100, BM_YUV411, 0);

Comments	This function both converts and writes the data from the buffer BUFF into the 			frame buffer. The conversion process may require a pointer to a workspace if a 			palettization is required. Large images may require multiple calls to assemble the 		image since the buffer size is restricted to 64k. A simple way to accomplish this

is by calling this procedure once per scanline and assembling the resultant

image into a file or a HUGE declared buffer.

See also	PCV_LoadImageInRect, PCV_ReadImageRect

# Win/TV Extended functions

## WTV_CheckQFactor

Syntax	 void FAR PASCAL WTV_CheckQFactor( Void )

This function provides a stablizing effect when moving the image around the screen.  	Without	this call installed, the screen in certain resolutions may be distorted.  Insert this 	call into your Move Timer routine.

Parameters	None

Return Value	None

In Visual Basic:

Declare Sub WTV_CheckQFactor Lib "PCVideo.dll" ()

WTV_CheckQFactor

Comments	This routine may not be needed in future DLL versions.

## WTV_CreateDIB

Syntax	 HGLOBAL  FAR PASCAL WTV_CreateDIB( WORD xleft, WORD ytop, WORD width, 								WORD height, WORD wfiletype, WORD 							wOptions).

WTV_CreateDIB captures a frame from the Win/TV buffer and copies it directly to memory in the file format requested ( this call is similiar to the PCV_SaveImageRect call, except it captures to memory instead of the disk).  This call returns a handle to the memory location where the image is saved,   to access the buffer do a globallock with the handle to get the pointer to the buffer.  This call can be used with DLL version 3.67 or greater.

Parameter	Type	Description

xLeft		Word	Pixel coordinate of left side of block

yTop		Word	Pixel coordinate of top side of block

width		Word	Width of block in pixels

height		Word	Height of block in pixels

wFileType	Word	File type

File Type		Description

bm_dib24		Windows 3.0 24 bit device-independent bitmap format (DIB)

bm_dib8g		Windows 3.0 DIB 8 bit gray scale

bm_dib8p		Windows 3.0 DIB 8 bit color (palettized)

BM_DIB4D		Windows DIB 4 bit dithered

BM_DIB1P        		BMP monochrome

BM_DIB4P        		BMP 16 colors palettized

wOptions	word	Miscellaneous options (must be 0)

Return Value

xxx = WTV_CreateDIB( WORD, WORD, WORD, WORD, WORD, WORD )

xxx   = 	the handle returned from the call,   to access the buffer do a 					globallock with the handle to get the pointer to the buffer.

In Visual Basic:

Declare Sub WTV_CreateDIB Lib "PCVideo.dll" (WORD, WORD, WORD, WORD, WORD, 							WORD)

Comments

## WTV_Exit

Syntax	void FAR PASCAL WTV_Exit( BYTE save )

This function will shut-down the video display window, mute the audio, and optionally update the 	configuration file, PCVIDEO.INI

Parameters	0 - Do NOT update the configuration file

¹ 0 - Update the configuration file

Return Value	None

In Visual Basic:

Declare Sub WTV_Exit Lib "pcvideo.dll" (ByVal save As Integer)

Save_State = 0			' Do NOT update the configuration file

Call WTV_Exit(Save_State)

Comments	This routine is similar to PCV_Exit except that it allows the caller to

optionally save the configuration file.

See also	PCV_Exit

## WTV_GetAudio

Syntax	WORD FAR PASCAL WTV_GetAudio(BYTE index)

This routine returns the value assigned to the selected audio channel

Parameters	None

Return Value	Current value programmed for the selected audio parameter

In Visual Basic:

DAC Channel:

Global Const VOLUME

Global Const BALANCE

Global Const BASS

Global Const TREBLE

Global Const SEPERATION

Global Const SOURCE

Global Const MUTE

Declare Function WTV_GetAudio Lib "pcvideo.dll" (ByVal index As Integer) As Integer

vol = WTV_GetAudio(Volume)	' Read the current volume setting

vol = vol - 1			' Lower volume by one click

Call SetVolume(vol)		' Output new DAC value for volume

Comments	See the WTV_SetAudio function for details on programming the audio subsystem

See also	WTV_SetAudio

## WTV_GetBoardType

Syntax	 void FAR PASCAL WTV_GetBoardType( Void )

This function returns a value that represents the type of WinTV board installed in your system.  It can be very useful in determining which pcvideo.dll to install.

Parameters	None

Return Value

BT_WINTV

BT_WINTVCELEBRITY

BT_WINTVCINEMA

BT_WINTVCINEMA202

In Visual Basic:

Declare Sub WTV_GetBoardTypeLib "PCVideo.dll" ()

Comments

## WTV_GetCaptureColor

Syntax	 Byte WTV_GetCaptureColor(Word index)

This function gets the various Capture color control parameters on the Win/TV card.

Index

BRIGHTNESS

SATURATION

CONTRAST

HUE

BW

If BW = 0 Black and White mode off  (color on),   If BW = 1 Black and White mode is on.

Return Value	None

Value (Valid Range)

BRIGHTNESS		1....255

SATURATION		1....255

CONTRAST		1....255

HUE			1....255

BW   			0....1

Comments	The GetCaptureColor call gets the current values for brightness, saturation, contrast, hue and color.

See also	WTV_SetCaptureColor, WTV_ResetCaptureColors

## WTV_GetErrorCode

Syntax	WORD FAR PASCAL WTV_GetErrorCode()

This function returns an error code for the last operation that did not complete normally.

Parameters	None

Return Value

E_NONE		“Unknown Error”

E_DRIVER		“Incompatible Windows display driver detected”

E_NOTPRESENT	“Hardware not present or not responding”

E_WIDTH		“illegal dislpay width detected”

E_HEIGHT		“illegal display height detected”

E_INIFILE		“Cannot find PCVIDEO.INI”

E_BOARDTYPE	“PCVIDEO.INI specifies illegal board type”

E_WINTV		“Hardware initialization error”

E_NOVGACLK		“Check feature connector cable connection”

E_NODECCLK		“Video decoder clock is missing”

E_PNP_INT		“Plug ‘N Play Initialization Error”

E_PNP_NEED_STATIC_CONFIG		“Plug ‘N Play Needs Static Configuration”

E_AUDIO_UNSPECIFIED			“Audio Initialization Error”

E_POWER_UP_FAILURE			“Hardware PowerUp Test Failure”

E_PNP_CONFIG_ATTEMPTED			“Hardware Reconfigured. Please restart 								computer”

E_CAP_DRV_ALREADYLOADED		“Drivers Already Loaded. Win/TV shutting down”

E_UNSUPPORTED_DIRECTDRAW_DRV	“Your VGA drivers need to be updated to 							support Microsoft direct draw, see the manual or 						VGA driver diskette for more information.  							Win/TV Shutting Down.

E_NOTPRESENT_TRY_RECONFIG		Hardware Not Present or Not Responding.  Try 							re-configuring this Plug 'N Play product since it 							is in a Non PNP OS.

E_NOTPRESENT_CONFIG_ATTEMPTED	Hardware Not Present or Not Responding.  							Restart computer to see if your re-configuration 							attempt solved your problem.

E_NT_KERNEL_DRVR				NT Kernel Driver Initialization Error.

In Visual Basic:

Declare Function WTV_GetErrorCode Lib "pcvideo.dll" () As Integer

Display_WinTV_Error = MsgBox(s + Chr$(13) + errtxt(WTV_GetErrorCode()), t)

## WTV_GetKeyColor

Syntax	 COLORREF FAR PASCAL WTV_GetKeyColor()

This function returns the color value to be used to paint the Video overlay region of the 	video display.

Parameters	None

Return Value	Type	Description

lKeyColor	long	Windows representation of KeyColor:

31                24  23              16 15                 8 7                  0

| 0 0 0 0 0 0 0 0 | Blue | Green | Red |
| --- | --- | --- | --- |

In Visual Basic:

Declare Function WTV_GetKeyColor Lib "pcvideo.dll" () As Long

KeyColor = WTV_GetKeyColor()	' Default Key Color is &HFF00FFL

Comments	See PCV_SetColorKey for a brief discussion on keying.

Key color should not be confused with Key color Index. Key color is a

color being painted by the VGA controller, and exists in the VGA's display

buffer. Key color index is the number loaded in the comparator latch

inside the overlay video controller.

See also	WTV_SetColorKey, PCV_SetColorKey, PCV_GetColorKey,

PCV_EnableColorKey, PCV_DisableColorKey

## WTV_GetStatus

Syntax	WORD FAR PASCAL WTV_GetStatus(Word)

This call returns a status of true or false for the selected item.

Parameters

GS_VIDEOSIGNAL

GS_TUNED

GS_INIT

Return Value

Get Video Signal Status TRUE = Valid Video

Get Tuner Status TRUE = Tuner PLL Locked

Get Initialization Status (True if PCV_Initialize already called).

In Visual Basic:

Declare Function WTV_GetStatus Lib "pcvideo.dll" (word) As Integer

Call WTV_GetStatus(GS_TUNED)

Comments

See also

## WTV_GetSVideo

Syntax	WORD FAR PASCAL WTV_GetSVideo()

This routine returns the status of the S-VIDEO bit

Parameters	None

Return Value	0 = Channel 0 is normal composite video

1 = Channel 1 is S-VIDEO

In Visual Basic:

Declare Function WTV_GetSVideo Lib "pcvideo.dll" () As Integer

Call WTV_SetSVideo(1)

Comments	See WTV_SetSVideo for a discussion of using S-VIDEO in the Win/TV board

See also	PCV_SetVideoSource, PCV_GetVideoSource, WTV_SetSVideo

## WTV_GetTunerChannel

Syntax	WORD FAR PASCAL WTV_GetTunerChannel()

This function returns the currently set channel and region

Parameters	None

Return Value

15                     8  7                         0

| Region | Channel |
| --- | --- |

Region:

Const USA 			' North America

Const US_CATV 		' North American Cable-TV

Const CCIR 			' Western Europe

Const CCIR_CATV 		' Western Europe

Const AUSTRALIA 		' Australia

Const JAPAN		 	' Japan

Const JAPAN_CATV 		' Japan Cable-TV

Const FRANCE 		' France

Const OIRT 			' Eastern Europe

Channel:

Min and Max channels vary by region and broadcast standard.

In Visual Basic:

Declare Function WTV_GetTunerChannel Lib "pcvideo.dll" () As Integer

Get_Tuner= WTV_GetTunerChannel()

Current_Region = (Get_Tuner \ 256) And 255

Current_Channel = Get_Tuner And 255

See also	APPENDIX A

## WTV_GetTunerType

Syntax	WORD FAR PASCAL WTV_GetTunerType()

This function returns the type of tuner currently configured.

Parameters	None

Return Value	const M_TUNER 		' North America

const BG_TUNER 		' Europe

const I_TUNER 		' UK

In Visual Basic:

Declare Function WTV_GetTunerType Lib "pcvideo.dll" () As Integer

TunerType = WTV_GetTunerType()

Comments	Tuners are specific to regions of the world. M, BG and L refer to broadcast

standards used to transmit the video and audio portions of a program.

See also	WTV_SetTunerType, WTV_SetTunerChannel, WTV_GetTunerChannel

## WTV_GetVersion

Syntax	 DWORD FAR PASCAL WTV_GetVersion()

This function returns the DLL version number

Parameters	None

Return Value

Bits 0-15  Version number.

Bits 16-23 Minor release number.

Bits 24-31 Flags (If bit 24=1 then its considered a debug version,  if bit 25=1 then it is considered a prerelease).

In Visual Basic:

Declare Function WTV_GetVersion Lib "pcvideo.dll" () As Integer

Dim Ver As Long

Ver = GetVersion()

DLLVersion = "PCVIDEO.DLL Version " + Format$(Loword(ver) / 100, "0.00")

## WTV_GetVerticalInterpolation

Syntax	 Bool FAR PASCAL  WTV_GetVerticalInterpolation( void );

This function returns a value that indicates theVeritcal interpolation mode.   If Vertical 	interpolation is set to 0 then vertical interpolation is off.  If set in this mode a sharper 	image is provided, but with motion artifacts.  If set to 1 it 	indicates Vertical interpolation 	mode is on.  This provides a less sharp image, but with 	the advantage of no motion 	artifacts.

Parameters	None

Return Value	Verical Interpolation off 	= 0

Vertical Interpolation on 	= 1

In Visual Basic:

Declare Sub WTV_GetVerticalInterpolation Lib "pcvideo.dll" ()

see also	WTV_SetVerticalInterpolation

## WTV_HWOptions

Syntax	WORD FAR PASCAL WTV_HWOptions()

This function returns the state of various hardware option bits.  The return value is a 	combination of one or more of the following bits:

Parameters	None

Return Value

HW_TunerPresent

HW_SECAMPresent

HW_AudioPresent

HW_TeletextPresent

HW_SVideoPresent

HW_External2

HW_RAMAccess

In Visual Basic:

If WTV_HWOptions() And HW_TunerPresent Then ...

## WTV_LoadConfiguration

Syntax	int FAR PASCAL WTV_LoadConfiguration()

Alternate name for PCV_LoadConfiguration.

Parameters	None

Return Value	 1 - Success

-1 - File does not exist

In Visual Basic

Declare Sub WTV_WTV_LoadConfiguration() Lib "pcvideo.dll" ()

init = WTV_LoadConfiguration()

Comments	PCV_Initialize already performs this function.

## WTV_ReadI2C

Syntax	int FAR PASCAL WTV_ReadI2C(BYTE, int, BYTE FAR *)

See also  WTV_WriteI2c.

Parameter	Type	Description

I2C Address	byte	Address of specified I2C device.

No. of Bytes 	int	Number of bytes to receive from the I2C device.

Pointer		byte	Pointer to buffer which contains I2C data to be transfered.

Return Value	 1 - Success

-1 - File does not exist

In Visual Basic

Declare Function WTV_ReadI2C Lib "PCVideo.dll"

(ByVal addr%, ByVal count%, Buffer%) As Integer

init = WTV_ ReadI2C(addr, count, buffer)

Comments	Hauppauge Computer Works does not provide register level information concerning specific I2C devices, such as the Teletext chip. For information on specific I2C devices please refer to the devices data book.

## WTV_ResetAudio

Syntax	void FAR PASCAL WTV_ResetAudio()

This function resets the audio parameters to their default settings.

Parameters	None

Return value	None

In Visual Basic

Declare Sub WTV_ResetAudio Lib "pcvideo.dll" ()

Call WTV_ResetAudio()

Comments	The default audio parameters are:

Volume =	10	' 10% of full volume

Balance =	50	' Centered audio

Bass =		50	' 50% Bass applied

Treble =	50	' 50% Treble applied

Source =	2	' Source 2 (Tuner, if present)

Mute =		0	' Non-muted

Seperation =	100	' Full stereo (MPX stereo if tuner present)

See also	WTV_SetAudio, WTV_GetAudio

## WTV_ResetCaptureColor

Syntax	 Void FAR PASCAL  WTV_ResetCaptureColors( void );

This function resets the Capture color values to there default values.

Parameters	None

Return Value	None

In Visual Basic:

Declare Sub WTV_ResetCaptureColors Lib "pcvideo.dll" ()

see also	WTV_SetCaptureColors,  WTV_GetCaptureColors

## WTV_SetAudio

Syntax	void WTV_SetAudio(BYTE index, BYTE value)

This function sets the various audio control parameters on the Win/TV card.

Index			Value (Valid Range)

Volume			0..100

Separation 		0..100	' 0% = full mono, 100% = full stereo

Bass			0..100

Treble 			0..100

Balance		0..100	' 0% = full left, 100% = full right

Audio Channel Select	0..2

Mute			0..1

Return Value	None

Comments	The Audio Channel Select parameter should follow the selected video

channel. IE: Video Channel 0 has audio on audio channel 0, etc..

The values for volume, bass, treble are in terms of percent of full

scale.

See also	WTV_GetAudio, WTV_ResetAudio

## WTV_SetCaptureColor

Syntax	void WTV_SetCaptureColor(Word index, BYTE value)

This function sets the various color control parameters on the Win/TV card.

Index			Value (Valid Range)

BRIGHTNESS		1....255

SATURATION		1....255

CONTRAST		1....255

HUE			1....255

BW   			0....1

Hue is controlled only in the front end so setting Hue with SetColor, calls SetCaptureColor.

BW is used to set Black & White mode  -  0 = BW is off (color on),   1 = BW on

Return Value	None

Comments	The SetCaptureColor call changes the saved image color values as well as what 			is displayed.

See also	WTV_GetCaptureColor, WTV_ResetCaptureColors

## WTV_SetFreezeResolution

Syntax	void WTV_SetFreezeResolution(Width, Height, Flags)

This function sets Max Res freeze mode on the Win/TV card for best possible image 	saves.

Parameters	Type	Description

Width		Word	Width of image to save.

Height		Word	Height of image to save.

Flags		Word 	Set save mode in Viewable Freeze or Best Save mode

Flags

0 = Viewable Freeze

3 =  Best Save

Return Value	None

Declare Sub WTV_SetFreezeResolution Lib "pcvideo.dll" (ByVal W%, ByVal Height%, ByVal Mode%)

example:

WTV_SetFreezeResolution ScaleWidth, ScaleHeight, &H3  	' Best Save

Call PCV_FreezeVideo

I = PCV_SaveImageRect(holdfn, 0, 0, ScaleWidth, ScaleHeight, holdopt, 0)

Select Case I

Case 0: Msg = "ERROR 0: Error Reading the Video Buffer."

MsgBox Msg

Case -1: Msg = "ERROR -1: File Creation error."

MsgBox Msg

Case -2: Msg = "ERROR -2: Memory allocation error."

MsgBox Msg

Case -3: Msg = "ERROR -3: File Write error."

MsgBox Msg

End Select

Call PCV_UnFreezeVideo

Syntax	void FAR PASCAL WTV_SetDAC(BYTE index, BYTE value)

This function allows the user to adjust the values associated with the various video

DACs on the Win/TV board.

Parameter	Type	Description

Index						Value

0		byte	Red Bias cutoff level	0..63

1		byte	Green Bias cutoff level	0..63

2		byte	Blue Bias cutoff level	0..63

3		byte	Filter bit - Set to 0	0, 63

4		byte	Filter bit - Set to 0	0, 63

5		byte	IBias			0..63

6		byte	Separation		0..63

7		byte	Not Used - set to 0	0

Return Value	None

In Visual Basic

Declare Sub WTV_SetDAC Lib "pcvideo.dll" (ByVal index As Integer,

ByVal value As Integer)

Call WTV_SetDAC(IBias, 14)

Comments	Set both filter bits to 0

See also	WTV_ResetDAC, WTV_GetDAC

## WTV_SetSVideo

Syntax	void FAR PASCAL WTV_SetSVideo(BYTE enable)

This subroutine puts the Win/TV into S-Video (S-VHS) mode.

Parameters	0 = Channel 0 is normal composite video

1 = Channel 0 is an S-VIDEO input

Return Value	None

Comments	The function is only valid for video channel 0.

See also	WTV_GetSVideo

## WTV_SetTunerChannel

Syntax	WORD FAR PASCAL WTV_SetTunerChannel(	WORD region,

WORD channel,

int finetune,

WORD options )

This subroutine sets the tuner to the selected channel for the selected region. A fine 	tuning parameter is also included.

Parameter	Type	Description

region		word	Region specifier

channel		word	Channel for the region specified

USA			' North America

US_CATV 		' North American Cable-TV

CCIR			' Western Europe

CCIR_CATV		' Western Europe

AUSTRILIA		' Australia mispelled in wintv.h

JAPAN			' Japan

JAPAN_CATV 		' Japan Cable-TV

US_HRC		' US CATV HRC

options		word	Not used - set to 0

Return Value	CHANNEL_OK

CHANNEL_LOW

CHANNEL_HIGH

CHANNEL_SKIP

In Visual Basic

Declare Function WTV_SetTunerChannel Lib "pcvideo.dll" (ByVal Region As Integer,

ByVal Channel As Integer%,

ByVal Fine As Integer,

ByVal Options As Integer) As Integer

channel = 3	' Channel 3 is assigned to 55.25MHz

' In some parts of Europe, there are channels "in between" channels

' For Example: Channel 3+ is at the frequency 56.25MHz. This

' frequency is exactly one Mega Hertz above channel 3, or

' 16 increments of 62.5 Kilo Hertz (See COMMENTS below)

finetune = 16

errorChannel = WTV_SetTunerChannel(CCIR, channel, finetune, 0)

if errorChannel > 0 then					' If s <> 0 the channel did not..

' ..tune properly

if channelError = 1 then channel = channel + 1 	' channel number was too low

if channelError = 2 then channel = channel - 1	' channel was too high

if channelError = 3 then channel = channel + 1	' We hit a skipped channel

.

.

.

Comments	Verify that the region and range are valid for any channel that is selected.

The on-board tuner can be tuned to any of the center frequencies listed by 			simply sending the channel and the region as parameters. The frequency can be 			fine-tuned by passing a non-zero finetune parameter with the call.

The finetune parameter is a signed integer with a range of -128 to +127. Each 			fine-tune increment adds or subtracts 62.5 KHz to/from the indicated center 			frequency.  Based on the width of the frequency channel, the fine-tuning range	 		may cross over into adjacent channels. The frequency spread for any given 			channel in the United States is ±300 increments (which covers the 6MHz channel 		bandwidth).

See also	WTV_GetTunerChannel, WTV_GetTunerType, WTV_SetTunerType,

APPENDIX A

## WTV_SetTunerType

Syntax	void FAR PASCAL WTV_SetTunerType(BYTE value)

This subroutine sets the type of tuner on the Win/TV board.

Parameters	M_TUNER 		' North America

BG_TUNER 		' Europe

I_TUNER 		' UK

Return Value	None

Comments	This function configures the type of tuner present on the board.

See also	WTV_SetTunerChannel, WTV_GetTunerChannel

## WTV_SetVerticalInterpolation

Syntax	 Void FAR PASCAL  WTV_SetVerticalInterpolation( Bool );

This function sets a value that determines Veritcal interpolation mode.  To disable	Vertical 	interpolation set to 0, in this mode a sharper image is provided ( fast motion may produce 	motion artifacts).  If set to 1 Vertical interpolation mode is on.  This provides a less sharp 	image, but without motion artifacts.

Parameters	Verical Interpolation off 	= 0

Vertical Interpolation on 	= 1

Return Value	None

example:

Sub mnuVertInterp_Click ()

mnuVertInterp.Checked = Not mnuVertInterp.Checked

WTV_SetVerticalInterpolation (mnuVertInterp.Checked)

End Sub

See also	WTV_GetVerticalInterpolation

## WTV_WriteI2C

Syntax	int FAR PASCAL WTV_WriteI2C(BYTE, BYTE, int, BYTE FAR *)

See also  WTV_ReadI2c.

Parameter	Type	Description

I2C Address	byte	Address of specified I2C device.

I2C SubAddr.	byte	Sub Address of specified I2C

No. of Bytes 	int	Number of bytes to xfer to the I2C device.

Pointer		byte	Pointer to buffer which contains I2C data to be transfered.

Return Value	 1 - Success

-1 - File does not exist

In Visual Basic

Declare Function WTV_WriteI2C Lib "PCVideo.dll" (ByVal addr%, ByVal Subaddr%, ByVal count%, DataBytes As Any) As Integer

init = WTV_ WriteI2C(addr, subaddr,  count, buffer)

Comments	Hauppauge Computer Works does not provide register level information concerning specific I2C devices, such as the tuner or teletext chip. For information on specific I2C devices please refer to the specific devices data book.

## WTV_WriteProfileInt

Syntax	WORD FAR PASCAL WTV_WriteProfileInt(LPSTR lpSectionName,

LPSTR lpKeyName, int NKey)

This function writes the value of a key to the pcvideo.ini file.

Parameter		Type	Description

section			LPSTR	section header name within the .ini file

keyword		LPSTR	key name of parameter

integer			INT	Integer value to be written to .ini file

Return Value

-1	Key does not exist

Comments

See also	 WTV_WriteProfileString, PCV_GetProfileInt

## WTV_WriteProfileString

Syntax	int FAR PASCAL WTV_WriteProfileString(LPSTR lpSectionName,

LPSTR lpKeyName,

LPSTR lpReturnedString)

This function writes the string value of a key to the pcvideo.ini file.

Parameter		Type	Description

lpSectionName		LPSTR	section header name within the .ini file

lpkeyName		LPSTR	key name of parameter

lpReturnerString	LPSTR String Value to be written to INI file

Return Value

0	Key does not exist or no entry for selected key

¹0	Length of the profile string

Comments

See also	PCV_GetProfileString

# Win/TV Extended Accessory functions

NOTE: The accessory functions are only available under WINDOWS

## WTVacc_AdjustColors

Syntax	void FAR PASCAL WTVacc_AdjustColors(HWND);

This function activates a dialog box to allow the user to adjust various color parameters.

Parameter	Type	Description

HWND		HWND	Identifies the window to be associated with the dialog box

Return Value	None

In Visual Basic

Declare Sub WTVacc_AdjustColors Lib "WinTVacc.dll" (ByVal hWnd%)

WTVacc_AdjustColors(hWnd)

Comments	This subroutine provides the user with a complete color adjustment dialog

box.

See Also	WTVacc_AlignVideo, PCV_SetRegister, PCV_GetRegister

## WTVacc_AlignVideo

Syntax	void FAR PASCAL WTVacc_AlignVideo(HWND);

This function activates a dialog box to allow the user to align the video display

Parameter	Type	Description

HWND		HWND	Identifies the window to be associated with the dialog box

Return Value	None

In Visual Basic

Declare Sub WTVacc_AlignVideo Lib "WinTVacc.dll" (ByVal hWnd%)

WTVacc_AlignVideo(hWnd)

Comments	This subroutine provides the user with a complete video alignment dialog

box.

See Also	WTVacc_AdjustColors, PCV_SetRegister, PCV_GetRegister

## WTVacc_PrinterSetup

Syntax	BOOL FAR PASCAL WTVacc_PrinterSetup(HWND hWnd)

This function call the Windows Printer Setup dialog box. It is used to prepare the printer

for printing a Win/TV Image.

Parameters	Handle to the current window

Return Value	0 - Setup was unsuccessful

1 - Printer setup was successful

In Visual Basic

Declare Function WTVacc_PrinterSetup Lib "wintvacc.dll" (ByVal hWnd As Integer) As Integer

P_Setup = WTVacc_PrinterSetup(hWnd)

Comments	This call requires Windows

See also	WTVacc_PrintImage

## WTVacc_PrintImage

Syntax	BOOL FAR PASCAL WTVacc_PrintImage(HWND hWnd, HDC hdc, struct 								    PageSetupStruct FAR *,

WORD wxPos,WORD wyPos,

WORD width,WORD height)

This function prints the current image in the Win/TV capture buffer

Parameter	Type	Description

hWnd		hWnd	Handle to current window

hdc		HDC     Handle to Printer Device

PageLayout

Type PageSetup

LeftMargin As Single

RightMargin As Single

TopMargin as Single

BottomMargin As Single

PrintScale As Integer

Flags As Integer

End Type

(Flags  		F_FitToPage			0x01)

(Flags		F_PreserveAspectRatio		0x02)

wxPos, wyPos  word	Top, Left corner of image to print

width, height	word	X, Y size of window

Return Value	0 - Print was unsuccessful

1 - Printer setup was successful

Comments	This call requires Windows

See also	WTVacc_PrinterSetup

## WTVacc_ReadCmos

Syntax	BYTE PASCAL FAR WTVacc_ReadCmos( int loc )

This function returns the value at the location loc in CMOS memory.

Parameter	Type	Description

loc		int	Location to read in CMOS RAM

Return Value	Contents of the addressed location

In Visual Basic

Declare Function WTVacc_ReadCmos Lib "WinTVacc.dll" (ByVal location As Integer) As Integer

ExtendedMemory = Format$(	WTVacc_ReadCmos(&H18) * 256 + 								WTVacc_ReadCmos(&H17)	      ) + " KB"

Comments	This is a general purpose function.

## WTVacc_ConfigureHardware

Syntax	 void PASCAL FAR WTVacc_ConfigureHardware( HWND )

This function activates a dialog box to allow the user to configure the WinTV hardware.

Parameter	Type	Description

HWND		HWND	Identifies the window to be associated with the dialog box.

Return Value	None

Comments	This subroutine provides the user with a complete hardware configuration.

# Windows Dynamic Link Libraries

A Windows dynamic link library(DLL) is a collection of routines that can be called by Windows applications or by routines in other dynamic link libraries (DLLs). There are several advantages to having routines packaged in a DLL.

+ Routines residing in dynamic link libraries can be easily replaced by replacing the 	  	    dynamic link library.

+ Executable programs take less disk space since code is not linked into a single 	  	    executable image.

+ Routines in a DLL share the same data segment and can communicate with each 	   	    other to act as a unified interface to some physical resource.

## Calling functions in PCVIDEO.DLL

Functions in the PCVIDEO.DLL are called just like any routine residing in another module.  Include WINTV.H in the source to inform the compiler that these entry points are external. If functions residing in the accessory DLL are required, be sure to add an include

statement for that DLL as well.

#include  "pcvideo.h"		/* Procedure declarations */

int xpos=0,ypos=0;		/* Top left corner of window */

.

int width=100, height=100;		/* Window width and height */

PCV_Initialize();

PCV_EnableVideo();			/* Enable display of video */

PCV_CreateWindow(xpos,ypos,width,height,TRUE);

## Linking with PCVIDEO.DLL

Applications that call functions in PCVIDEO.DLL should include PCVIDEO.LIB in their link statement.

link /NOD module1+module2,,,slibcew libw pcvideo.lib module.def

The file PCVIDEO.LIB is created using the IMPLIB.exe utility from the PCVIDEO.DEF definition file. The utility extracts the names of all entry points exported by the DLL and writes them into the .LIB file which the linker refers to when linking your application.

implib pcvideo.def pcvideo.lib

## How the loader resolves references to the DLL

At load time, the loader searches the program code for dynalink records; (dynalinks are references to routines in DLLs).  Whenever one is found, it checks whether the library has been loaded from the disk, and loads it if necessary.  It then writes the address of the routine into the calling statement.

## Using the PCVideo dynamic link library with Asymetrix ToolBook

When writing a ToolBookÒ script that will call functions in a DLL, you must precede the references with C-like statements informing ToolBookÒ of the function name, parameters, and return type of the function(s) to be called.  These statements must be preceded by a linkdll statement with the name of the library and followed by an end linkdll statement.  Below is an example.

linkdll "pcvideo.dll"

int PCV_Initialize()

int PCV_EnableVideo()

int PCV_CreateWindow(word,word,word,word,word)

.

.

.

end linkdll

get PCV_Initialize()

get PCV_EnableVideo()

get PCV_CreateWindow(0,0,100,100,TRUE)

# DOS Demonstration Program

The DOS example represents a simple implementation of the Win/TV card. The software has been designed to exersize all the basic functions of the Win/TV board.  It differs from the Windows based application mainly in how it determines the video mode of the VGA card.

The Windows application uses the wealth of system calls provided by the Windows environment to determine the type and mode of video card in the system. The DOS based

library will attempt to determine the video card and mode using calls to a VESA BIOS on the

VGA card. If the VGA card does not contain a VESA BIOS, the software will attempt to

determine, as best it can, what kind of card is connected to the system.

# Win/TV  Frame Buffer Format

The Win/TV board maps a  1 MB area for frame storage. This memory is accessible in a linear format to the CPU.  The base address for this memory is programmable through the 82C9001A registers.  Since all of the 1MB is accessible in a linear format, the memory must be mapped above the 1 MB space so as not to conflict with the main system memory. BIOS interrupt 15h can be used to access memory beyond the first megabyte.

It may be useful to allocate a small buffer in system memory, and copy parts of the Win/TV frame buffer into this working buffer for any processing.  A good size is 2 KB, which is sufficient to store 1 scan line of the image. The software included in this package maps the memory at 14MB in the CPU address space by default.  You can change this in the Windows and DOS sample programs.

The image is stored in the YUV color space where Y is luminance (or intensity) and U and V are blue and red chrominance, respectively.  These components are defined in terms of red, green and blue in the following formulas.

Y 	= 	0.299R	+	0.587G	+	0.114B

V 	=	(R-Y)

=	R - 0.299R - 	0.587G	-	0.114B

=	0.701R	-	0.587G	-	0.114B

Vmax	=	0.701(255)	-	0.587(0)	-	0.114(0)	= 178.76

Vmin	=	0.701(0)	-	0.587(255)-	0.114(255)	= -178.76

V must be scaled by 127/179 to fit into 8 bits

U 	=	(B-Y)

=	-0.299R 	- 	0.587G	+	0.114B

=	-0.299R		-	0.587G	+	0.866B

Umax	=	-0.299(0)	-	0.587(0)	+	0.866(255)	= 225.93

Umin	=	-0.299(255) -	0.587(255)-	0.114(0)	= -225.93

U must be scaled by 127/226 to fit into 8 bits

Deriving the RGB equations from the above is a matter of simple substitution:

R 	= 1.409V	+ Y

B 	= 1.779U	+ Y

G 	= 1.706Y	-	0.509R - 0.194B

All the three components Y, U and V are stored at 7 bit resolution, resulting in a color quality of 21 bits/pixel (2 million colors).  U and V are signed quantities and are scaled  by 127/226 and 127/179 respectively to fit in seven bits.  Luma (Y) is a 7 bit unsigned magnitude.  The YUV format used is 4:1:1. There is one sample of luminance per pixel and chrominance is sub sampled so that one value of U and one of V are stored for each four pixels. This exploits the fact that natural life images require lower bandwidth for chrominance than for luminance.

The net result is that you get 21 bits/pixel quality by actually using only 42 bits/4 pixels == 10.5 bits/pixel. However, due to the memory packing issues this actually is equivalent to 11 bits/pixel.  The Win/TV board has 1 MB of frame buffer memory organized as 512 Kwords (pixels) of 16 bits each.  Only 11 bits of each word are actually used.  Bit 0 and bits 8:11 of every word are not used.

Each scan line in the memory begins at a 2kb boundary.  A scan line in a standard video image consists of about 700 pixels and can thus be stored within this 2 KB (1 K word) space.

From the CPU point of view, the image can be accessed in chunks of 4 pixels. This  4 pixel chunk is important because the image is stored in a 4:1:1 luma/chroma compressed format.  The CPU view of these 4 pixels is as follows:

Win/TV  YUV Image File format

For RGB data, the Targa 16 file format is used.  The Targa format is nearly the same as that of the RGB board video buffer.  The only difference is that in the RGB video buffer the most significant bit (D15) is used for the least significant bit of the 6-bit green component.  In the Targa 16 file format, this bit is used for the alpha channel.  The PCV_SaveImageRect routine of the function library stores the low bit of green here instead.

# APPENDIX A

## USA Tuner Frequency List

| Ch | Frequency (x 10KHz) | Ch | Frequency (x 10KHz) | Ch | Frequency (x 10KHz) |
| --- | --- | --- | --- | --- | --- |
| 2 | 5525 | 24 | 53125 | 47 | 66925 |
| 3 | 6125 | 25 | 53725 | 48 | 67525 |
| 4 | 6725 | 26 | 54325 | 49 | 68125 |
| 5 | 7725 | 27 | 54325 | 50 | 68725 |
| 6 | 8325 | 28 | 55525 | 51 | 69325 |
| 7 | 17525 | 29 | 56125 | 52 | 69925 |
| 8 | 18125 | 30 | 56725 | 53 | 80525 |
| 9 | 18725 | 31 | 57325 | 54 | 71125 |
| 10 | 19325 | 32 | 57925 | 55 | 71725 |
| 11 | 19925 | 33 | 58525 | 56 | 72325 |
| 12 | 20525 | 34 | 59125 | 57 | 72925 |
| 13 | 21125 | 35 | 59725 | 58 | 73525 |
| 14 | 47125 | 36 | 60325 | 59 | 74125 |
| 15 | 47725 | 37 | 60925 | 60 | 74725 |
| 16 | 48325 | 38 | 61525 | 61 | 75325 |
| 17 | 48925 | 39 | 62125 | 62 | 75925 |
| 18 | 49525 | 40 | 62725 | 63 | 76525 |
| 19 | 50125 | 41 | 63325 | 64 | 77125 |
| 20 | 50725 | 42 | 63925 | 65 | 77725 |
| 21 | 51325 | 43 | 64525 | 66 | 78325 |
| 22 | 51925 | 44 | 65125 | 67 | 78925 |
| 23 | 52525 | 45 | 65725 | 68 | 79525 |
|  |  |  |  | 69 | 80125 |

# APPENDIX B

## Media Control Interface (MCI)

### Introduction

MCI (Media Control Interface) is high-level control software that provides a device-independent interface to multimedia devices and media files. MCI supports a command-message and a command-string interface. These interfaces are supported by both IBM and Microsoft.

MCI command-strings are sent to MCI device drivers with the mciSendString and mciExecute functions. These functions translate the command strings into command-messages.

Many popular Multimedia presentation packages allow the user to communicate with media devices at a command-string level. Others communicate with the hardware directly using the command-message interface.

This document describes the Hauppauge Computer Works Win/TV command-string interface to MCI, its command syntax and expected results. The response to related command-messages can be derived from this information.

### About MCI Device Commands

MCI command strings are sent to the MCI device driver with the mciSendString and mciExecute functions. The commands are divided into the following categories:

System Commands	Interpreted directly by MCI rather than being

relayed to a device.

Required Commands	Are supported by all MCI devices.

Basic Commands	Are optional commands.

Extended Commands	Are specific to a device type or device class.

These extended commands contain both unique commands and extensions to the required and basic commands.

This document will discuss only those Required, Basic and Extended commands that relate to the video overlay class of device. It is assumed that an understanding of Multimedia programming for Windows and of MCI System commands already exist.

### Driver Installation

To install the driver software you must have Multimedia Windows already installed on your system.  Start up Windows and make sure that the driver diskette is in a floppy drive.

Go to the Windows Control Panel and double click on the DRIVERS icon.  Select ADD and ADD Unknown. Select the driver to be installed and click on OK. The MCI driver is then installed. If multiple boards are installed in your system, you may want to install the corresponding MCI drivers by repeating the above process once for each driver.

The installation under Windows 3.0 will not use the new files on the

floppy but will (quietly) use the (old) files on the hard disk if available.

The installation program under Windows 3.1 will ask if you would prefer to use the older version of the driver, etc. on your system.

To avoid these problems, if you have installed a previous version of

the MCI driver, please remove the driver first using Control Panel,

exit to DOS, and run the MCICLEAN.BAT provided on the install floppy.

MCICLEAN requires the drive that your current Windows is located on and the name of the Windows directory e.g

A:\MCICLEAN C WIN31.

If no drive and/or directory are specified, MCICLEAN.BAT will assume C:\WINDOWS as the Windows directory. This will erase all the files from the WINDOWS and WINDOWS\SYSTEM directory that need to be copied fresh from the install floppy. The installation (as described above) will then proceed smoothly. Please note that MCICLEAN.BAT assumes that there are no duplicate files in the WINDOWS and WINDOWS/SYSTEM directories and that none of the required files are found in other directories in your current path.

### Video Overlay Device Command Overview

Video overlay devices support the following set of commands which are a subset of those supported for digitalvideo devices. Video overlay devices also support notify, test and wait as optional flags. You can add either or all of these flags to any digital-video command.

Command	Brief Description

capability	Obtains the capabilities of a device.

close	Closes the device.

freeze	Disables video acquisition to the frame buffer.

info	Obtains textual information from a device.

load	Recalls data from a disk file.

open	Initializes the device.

put	Defines the source, destination and frame

windows.

save	Saves data to a disk file.

set	Sets the operating state of a device.

setaudio	Sets Audio source & levels.

setvideo	Sets Video source & display levels.

status	Obtains status information from the device.

unfreeze	Enables the frame buffer to acquire video data.

where	Obtains the rectangle specifying the

source,destination or frame area.

window 	Sets the window for the display of overlay video.

### Creating Command Strings

In order to assemble a command string you need to be familiar with some basic rules of syntax.  A command string is made up of three pieces of information: the command element, the device_id element, and the arguments element.

A command string takes the form:

command     device_id     arguments

Example:

capability  overlay  can freeze

where:

capability 	is the command element
overlay 	is the device_id element
can freeze 	is the arguments element

This command instructs the overlay device to return information about its ability to freeze live video.

The Command Component

Every MCI command string must begin with an MCI command. The command includes a command from the system, required, basic or extended command set. Examples of commands includes open, close, and play.

The Device ID Component

Every MCI device command string includes a device_id element that tells MCI where you want to direct the command. MCI accepts the names of MCI device types and names of device elements for the device_id. An example of a device type is overlay.

Depending on the device you are using, the device_id element may have to specify different information. Some devices (such as a CD audio player) are known as simple devices because they require only the name of the device as the device_id element. Other devices (such as animation players) are known as compound devices because they require the name of the device and the name of the filename used by the device.

The Arguments Component

Arguments are the flags and parameters required for a given command.

As mentioned earlier, flags are keywords supported by an MCI command.  Parameters consist of additional numeric or string data that you must supply to either the command or a flag that you are using.  For example, one of the basic commands is the save command. You can use the save command with the at flag and the rectangle parameter to specify the block of data that is to be saved. The command would appear as follows:

save overlay xxx.bmp at 0 0 50 50

where:

save	is the command
overlay	is the device
xxx.bmp 	is the filename parameter (24 bit DIB by default)

at 	is the optional flag
0 0 50 50 	are the parameters (rectangle).

When you use MCI commands, commands requiring parameters typically need the following kinds information::

Integers - Make sure to use a blank space before and after an integer.  Unless otherwise specified, integers can be positive or negative.  If you are using a negative integer do not leave a space between the negative sign and the first digit.  As an example,  an integer must be specified when using the samplespersec command; this command sets the sample rate for playing and recording wave audio.

Rectangle - Some commands will require that you specify four integer values that describe a rectangular area.  The first two integers are the coordinates of the top left corner of the rectangle and the second two integers are the width and height of the rectangle.  For example, you specify a rectangle when you use the freeze command to freeze a video image via an overlay device.

### About MCI Device Types

The following table gives a description of the MCI devices, the name of the device as it would appear in a command string, and an indication of whether the device is simple or compound

:

Device Description	Device Name	Simple or Compound

Animation player	animation	compound

CD audio player	cdaudio	simple

Overlay device	overlay	compound

MIDI sequencer	sequencer	compound

Videodisk player	videodisk	simple

Audio device that plays

digitized waveform files	waveaudio	compound

The [mci] section of the SYSTEM.INI file lists the device types installed on your system.  If you have a device type installed more than once, the device type names will have numbers attached to them.  For example, if you have two cdaudio type devices installed on your system they will be identified as cdaudio1 and cdaudio2.

Example 1: Simple Devices

CD audio devices and videodisc players are simple devices. When simple devices are used,  the device name alone is sufficient as the device_id element of the command string. Before a device can be accessed, it must be initialized with the open command. The following command illustrates initializing a simple device:

open cdaudio

where:

open is the command element
cdaudio is the device_id element.

Example 2: Compound Devices

MIDI sequencers, waveform audio devices, animation players, and overlay cards are compound devices. When using a compound device, such as a MIDI sequencer, additional information must be specified in addition to the device_id element of the command string.  The full path and filename for the MIDI file ,the type flag, and  the actual device name must be included.

Note: Flags are keywords supported by an MCI command.  For example, 	when you use the play command with a CD audio player, you can

also use the from and to flags to play a particular portion of the

CD. When you initialize a MIDI sequencer, you might specify the

command string:

open d:\music\sonata1.mid type sequencer

where:

open 	is the command element

d:\music\sonata1.mid 	is the device_id element
type 	is the flag that precedes the name of the device
sequencer 	is the device type

Note: If you use the open command with a compound device type and

specify only the device type, for example, open animation, only

limited commands for that device type are available.  As an

example, once you issue the open animation command you can

use the capability command to find out the capabilities of that

device, but you cannot use a play command to play that device.

### Opening and Closing MCI Devices

Before using a device, it must be initialized with the open command.  The number of devices that can be open depends on the amount of available memory.  The close command releases access to a device (or a file used by a device).  To help MCI manage the devices, your application must explicitly close each device (or file) when it is finished with it.

### Obtaining Information from MCI Devices

Every device responds to the capability, status, and info commands.  These commands obtain information about the device.

Example:

capability overlay freeze

This command returns true to indicate that the device is capable of freezing video in the frame buffer.

### Using Flags

Using the Alias Flag

The preceding section described how a compound device requires that you specify a full path and filename, the type flag, and the device type.  Typically, you will issue a number of command strings in an MCI-aware application that all have to include this information as the device_id element.

Because it would be tedious to keep repeating that same lengthy portion of the command string, MCI lets you take all of the information in the device_id element and use the alias flag to assign it an alias (an alternative, shorter name). If your application creates an alias it must use the alias as the device_id for all subsequent references.The following series of commands strings show how you might set and use an alias:

open overlay alias wintv
	window wintv

.
	.
	.
	close wintv

In this example, the first command string uses the alias flag to assign the alias wintv to the video overlay device, path, and filename.  From that point on, the expression wintv is used as the device_id element in command strings.

Using the Type Flag

See "Creating Command Strings."

Using the Shareable Flag

The shareable flag lets multiple applications access the same device concurrently. Different applications can use the same device (such as a CD audio player).  If your application opens a device without the shareable flag, no other application can access it simultaneously.  If your application opens a device as shareable, other applications can access the device by also opening it with this parameter.

The shared device gives each application the ability to change the parameters governing the operating state of the device.  If you make a device shareable, your application should not make any assumptions about the state of that device.  When you work with shared devices your application might need to compensate for changes made by other applications using the same services.

Using the All Flag

You can specify all as a device name for any command that does not return information.  When you specify the all flag, MCI sequentially sends the command to all devices opened by the current application.

Example:

close all

closes all open devices.

MCI sequentially sends the commands to the MCI devices. There is a delay between when the first device receives the command and when the last device receives the command.

Using the New Flag

MCI lets you use command strings to create new files, but you don't have to name the file until you use a save command to actually save it.  To create a new file without specifying a filename, you can specify the new flag.  If you use the new flag, you must also use an alias flag.

Using the Wait Flag

Normally MCI commands return to the application immediately, even if it takes several minutes to complete the action initiated by the command.  One way to modify the default behavior is to use the wait flag that is supported by MCI.

Using the Notify Flag

The notify flag  directs the device to post an MM_MCINOTIFY message to a specified window when the requested action is complete.

Normally MCI commands return to the user immediately, even if it takes several minutes to complete the action initiated by the command. Either the wait flag or the notify flag can be used to modify this default behavior.

Using the Test Flag

This flag will be supported in a later revision.

### Supported Video Overlay Commands

The following tables describe the MCI commands supported by this release of the Win/TV Media Control Interface Driver.  To construct a command, follow the basic syntax:

command device_id arguments.

The device_id for all video overlay device commands is overlay. If multiple drivers of the same type are installed, a numeral will be added to the device_id argument. For example, if three Win/TV boards were installed in your system, the first would have device_id overlay, the second would have device_id overlay1 and the third would have device_id overlay2.

If a command takes any arguments, the command description states whether the arguments are optional or required.  Arguments are made up of flags (keywords) and/or parameters (variable information).  Flags appear in bold type, for example, can eject. Parameters appear in italicized type, for example, rectangle, where you are expected to substitute a value for the parameter rectangle.

For detailed information on how to construct command strings, see "Creating Command Strings".

capability

Requests information about the capabilities of a video overlay device.  This command requires an arguments element.

can eject	Returns false.

can freeze	Returns true, since the device can freeze data in the frame buffer.

can play	Returns false.

can record	Returns false.

can save	Returns true, since the device can save the current contents of the frame buffer to disk.

can stretch	Returns false.

compound device	Returns false since multiple windows are not supported, and the device does not support an element name.

device type	Returns overlay device type.

has audio	Returns true, although the device does not support audio playback, it can pass audio info.

has video	Returns true.

uses files	Returns false.

windows	Returns the number of simultaneous display windows the device can support, 1.

close

Closes a video overlay element and any resources associated with it. This command does not use an arguments element.

freeze

Disables video acquisition to the frame buffer.  This is supported only if capability can freeze returns true.

info

Fills a user-supplied buffer with information.  This command requires an arguments element.

file	Returns the name of the file used by the video overlay device. This argument is valid only if capability uses files returns true.

product	Returns the product name and model of the current device, Hauppauge Computer Works Win/TV Video Overlay Device.

window text	Returns the caption of the window used by the device, Hauppauge Computer Works Win/TV.

load

Loads the contents of the video buffer in a device specific format.  This command requires an arguments element.

filename	Specifies the file and pathname used to load the data.

at rectangle	Specifies the rectangle relative to the video buffer origin.  The rectangle is specified as X1 Y1 X2 Y2.  The coordinates X1 Y1 describe the top, left corner of the rectangle and X2 Y2 describe the width and height.

open

Initializes the video overlay device.  This command requires an arguments element.

alias device_alias	Specifies an alternate name for the video overlay device.  If specified, it must also be used for subsequent references.  (See "Using Flags" earlier in this manual)

parent hwnd	Specifies the window handle of the parent window.

shareable	Initializes a device element as shareable.  Subsequent attempts to open it fail unless you specify shareable in both the original and subsequent open commands.  MCI returns an invalid device error if it is already open and not shareable.  (See "Hints for Using Flags" earlier in this manual.)

style style_type	Indicates a window style.

style child	Opens a window with a child window style.

style overlapped	Opens a window with an overlapped window style.

style popup	Opens a window with a popup window style.

type device_type	Specifies the device type of the device element.  MCI reserves overlay for the video overlay device type.  As an alternative to type, MCI can use the [mci extension] entries in the SYSTEM.INI file to select the controlling device based on the extension used by the device element.

put

Defines the source, destination, and frame windows.  This command requires an arguments element.

destination	Sets the whole window as the destination window.

This completed command defines the VGA window.

destination at rectangle	Specify a rectangle for the area of the window used to display the image.  The rectangle coordinates are relative to the window origin and are specified as X1 Y1 X2 Y2.  The coordinates X1 Y1 describe the top left corner of the rectangle and the coordinates X2 Y2 describe the width and height of the rectangle.  When an area of the display window is specified and the device supports stretching, the source image is stretched to fill the destination rectangle.

frame

Specifies that the whole video buffer is used to capture the video image. This completed command defines the capture rectangle in the video buffer.

frame at rectangle	Specify a rectangle for the area of the video buffer used to capture the video image.  The rectangle coordinates are relative to the video buffer origin and are specified as X1 Y1 X2 Y2.  The coordinates X1 Y1 describe the top left corner of the rectangle and the coordinates X2 Y2 describe the width and height of the rectangle.  If the device supports stretching, the video image is stretched to fill the rectangle.

source	Selects the whole video buffer for display in the destination window. This completed command defines the source rectangle in the frame buffer.

source at rectangle	Specifies a rectangle for the video buffer area used to obtain the image for display.  The rectangle coordinates are relative to the video buffer origin and are specified as X1 Y1 X2 Y2.  The coordinates X1 Y1 describe the top left corner of the rectangle and the coordinates X2 Y2 describe the width and height of the rectangle.  When an area of the source image is specified and the device supports stretching, the source image is stretched to fill the destination rectangle.

video	Selects the whole video source for display in the destination window. This completed command defines the acquisition window.

video at rectangle	Specifies a rectangle for the input video source area used to obtain the image for display.  The rectangle coordinates are relative to the video buffer origin and are specified as X1 Y1 X2 Y2.  The coordinates X1 Y1 describe the top left corner of the rectangle and the coordinates X2 Y2 describe the width and height of the rectangle.  When an area of the source image is specified and the device supports stretching, the source image is stretched to fill the destination rectangle.

save

Saves the contents of the video buffer in a device specific format.  This command requires an arguments element. A 24 bit DIB format is used for the saved image.

filename	Specifies the file and pathname used to save the data.

at rectangle		Describes a rectangle relative to the video buffer origin, specified as X1 Y1 X2 Y2.  The coordinates X1 Y1 describe the top left corner of the rectangle and the coordinates X2 Y2 describe the width and height of the rectangle.

type x  	 	type x values 0-18

The type setting is an extended command. It 			allows you to choose one of nineteen different file 			formats 	for saving frames.  The following are 			ordinal numbers and there file types.

0	/* Windows DIB 24 bit true color */

1	/* Windows DIB 8 bit palettized */

2	/* Windows DIB 8 bit gray-scale */

3	/* Windows DIB 4 bit dithered */

4	/* Targa 32 bit true color */

5	/* Targa 24 bit true color */

6	/* Targa 16 bit true color */

7	/* IBM MMotion format 4:1:1 YUV */

8	/* TIFF 24 bit true color */

9	/* TIFF 8 bit palettized */

10	/* TIFF 8 bit gray-scale */

11	/* PCX 8 bit palettized */

12	/* PCX 8 bit gray-scale */

13	/* PCX 4 bit dithered */

14       	/* GIF 8 bit palettized

15      	/* GIF 8 bit gray-scale

16     	/* JPEG compressed 4:2:2 YUV

17     	/* BMP monochrome

18      	/* BMP 16 colors palettized

set

The SET command has only two functions with Win/TV,  Time Format and Audio.

ON/OFF			Turns Video Overlay On or OFF.

Audio	The Audio command sets the volume level from 0 to 100.  It may not be audible until a setting of 50 or higher.

Time Format	Time Format is used in conjunction with the Tuner and is used for changing channels from 1-125.  This command is hard coded for a particular region (USA CATV), if you need another region set please call out technicial support department.

Source to xx  		values 0-6

This function selects the video format. Dont 			confuse this

call with the setvideo source call.

The ordinal values are as follows:

0	PAL_BGHI

1	NTSC_M

2	SECAM

3	PAL_M

4	PAL_N

5	NTSC_443

6	EXTERNAL_SRC

channel to xxx  		values 1-125

This function set the channel number.  This call 			is functional only if the WinTV board has a tuner.

tuner region to x          		values 0-7

This function sets the tuner for the proper RF 			Region.  The ordinal values are as follows:

0	USA

1	US_CATV

2 	CCIR	/* Western Europe	*/			3	CCIR_CATV	/* Western Europe   */

4	AUSTRILIA	/* Austrilia		*/

5	JAPAN	/* Japan		*/

6	JAPAN_CATV	/* Japan		*/

7	US_HRC	/* US CATV HRC	*/

setaudio

SETAUDIO has full command of the audio spectrum on the Win/TV board,  with access to all six different audio functions.

Bass to xx	Sets the Bass level, Values 0-100.

On / Off 	This function turns the audio on or off.    SetAudio Overlay On will UnMute the audio.  SetAudio Overlay OFF will Mute the audio.

Source to xx	This Function selects the audio source, Values from 0-2.    Setting  to 2 selects the tuner, 0 selects video input 1 and 1 selects video input 2.

Stream to xx	Sets the Balance between left and right. Values 0-100.  Where 0 = full left and 100 = full right.

Treble to xx	This function sets the treble level,  Values 0-100.

Volume to xx	The Audio command sets the volume level from 0 to 100.  It may not be audible until a setting of 50 or higher.  This commands works like "SET Overlay Audio" command.

setvideo

SETVIDEO controls the color, brightness, contrast, tint and video source selection.

ON/OFF			Turns Video Overlay On or OFF.

Brightness to xx	Sets the Brightness level, Values 1-255.

Color to xx 	Sets the Color level, Values 1-255.

Contrast to xx	Sets the Contrast level, Values 1-255.

Source to xx	This Function selects the Video source, Values from 0-2.   A Value of 2 selects the tuner, 0 selects video input 1 and 1 selects video input 2.

Tint to xx	Sets the tint,  Values 1-255.

status

Obtains status information for the device.  This command requires an arguments element.

media present	Returns true.

mode	Returns stopped.

ready	Returns true, indicating that the device is ready.

window handle	Returns the handle of the window whose client area is used for the destination video in the low-order word of the return value.

stretch	Returns false.

unfreeze

Enables the acquisition of video data into the frame buffer. This command is usually preceeded by a freeze command.

where

Obtains the rectangle specifying the source,

destination, or frame area. This command requires an arguments element.

destination	Requests the offset and extent of the destination rectangle.

frame	Requests the offset and extent of the frame rectangle.

source	Requests the offset and extent of the source rectangle.

video	Requests the offset and extent of the video rectangle.

window

The window command can be used to tell the video overlay device to use a given window for display instead of the default window created by the driver.  By default, these devices should create a window when opened but should not display it until they receive the play command.  Applications providing window handles should manage the display issues that result when the window is sized or when the window handle is switched during play.  Several flags manipulate the window.  Since the status command can obtain the handle to the current display window, you can use the standard window management functions to manipulate the window. This command requires an arguments element.

handle window_handle	Specifies the handle of the destination window used as an alternate to the default window.

handle default	Specifies that the video overlay device should create and manage its own destination window.  This flag can be used to set the display back to the driver's default window.

state hide	Hides the current display destination window.

state iconic	Displays the destination window as an icon.

state maximized	Maximizes the current destination  window.

state minimize	Minimizes the specified window and activates the top-level window in the window-manager's list.

state minimized	Minimizes the current destination  window.

state no action	Displays the destination window in its recent size and state.  The window that is currently active remains active.

state normal	Activates and displays the current display window in its original size and position, as it was created.

state show	Shows the current destination window, in response to a hide.

text caption	Specifies the caption for the display window.