*** NETWARE 3.12 SERVER DRIVER INSTALLATION ***

Three sections are covered in this information as below:

1.INTRODUCTION

2.DRIVER PARAMETER DEFINITION

3.DRIVER INSTALLATION

INTRODUCTION

============

This documentation covers the information for installing the 100VG NetWare

Server drivers for 100VG LAN adapters and the operating modes respectively:

Driver            100VG Adapters          Operating modes supported

---------------   -------------------   ----------------------------------

KVGNW.LAN         DP16I,DP16I2          I/O-mapped mode, Memory-Mapped mode

KVGNW.LAN         DP32E,DP32E2          I/O-mapped mode, Memory-Mapped mode

KVGNW.LAN         DP32PCI,DP32PCI2      I/O-mapped mode, Memory-Mapped mode

KVGNWBM.LAN       DP32E,DP32E2                Bus-Master mode

DP16I, DP16I2     : KTI 100VG ISA LAN Adapters (referred to as "ISA adapters")

DP32E, DP32E2     : KTI 100VG EISA LAN Adapters (referred to as "EISA adapters")

DP32PCI, DP32PCI2 : KTI 100VG PCI LAN Adapters (referred to as "PCI adapters")

Both LAN adapters are shipped with factory default setting as below:

EEPROM Setting          ISA adapter     EISA adapter     PCI adapter

-------------------     -----------     --------------   ------------

Base I/O address          300h               n/a             n/a

Interrupt                  3                  3              n/a

Interrupt type             n/a            edge-trigger       n/a

Operating mode          I/O-mapped         I/O-mapped     I/O-mapped

Memory-mapped address     D0000              D0000           n/a

Those settings can be changed by using a software setup program called

KVGSETUP.EXE. The program is included in this adapter driver diskette.

If any configuration needs to be changed, run KVGSETUP program first before

you start up your Netware server.

DRIVER PARAMETER DEFINITION

===========================

DRIVER          : KVGNW.LAN

ADAPTER         : ISA, EISA, PCI adapter

OPERATING MODE  : I/O-mapped mode and Memory-mapped mode

Parameter  Description          Valid Values and Notes

-------------------------------------------------------------------------------

PORT       Base I/O Address     300, 340, 380, 3C0, 200, 240, 280, 2C0

for ISA adapter

If only one ISA adapter with one frame type is installed, it is

not necessary to specify PORT for the adapter. The driver will

search the adapter automatically. If multiple ISA adapters are

installed, PORT needs to be specified for the driver to identify

a specific ISA adapter.

-------------------------------------------------------------------------------

SLOT       EISA slot number     1,2,3,4,5,6,7,8,9,10,11,12,13,14,15

in which the EISA

adapter is installed

If only one EISA adapter with one frame type is installed, it is

not necessary to specify this option. The driver will search the

adapter automatically. If multiple adapters are installed, the

driver needs this option to identify a specific EISA adapter.

-------------------------------------------------------------------------------

PCIBOARD   PCI board number     1,2,3,4

This parameter specifies a specific PCI adapter when multiple

PCI adapters are installed in one system. If only one PCI adapter

is installed, it is not necessary to specify this option. The

driver will search the adapter automatically.

-------------------------------------------------------------------------------

INT        Interrupt (IRQ)      3, 4, 5, 6, 7, 9, a, b, c, f

When this option is specified, the driver will use it for the

adapter operation and ignore the Adapter's EEPROM setting.

If this option is not specified, the driver will use the

interrupt setting stored in the Adapter's EEPROM.

-------------------------------------------------------------------------------

IRQTYPE    Interrupt trigger    EDGE  : edge-triggered interrupt type

type for the EISA    LEVEL : level-triggered interrupt type

adapter

This option is only valid for the EISA adapter. If the option is

not specified, the driver uses the adapter default setting.

-------------------------------------------------------------------------------

BUSTYPE    Adapter's bus type    1 : ISA, 2 : EISA, 3 : PCI

This option is applied only when "PORT" or "SLOT" or "PCIBOARD"

parameter is not specified in the command line when loading the

driver and when more than one type of the adapters are installed

at the same time. Under such circumstances, the driver needs this

information to prompt the correct message for more input from the

installer.

-------------------------------------------------------------------------------

OPMODE     Operation Mode for    MM, IO

the adapter operation

If MM is specified for this option, the driver will use

Memory-Mapped mode for the adapter operation. If IO is specified,

I/O-mapped mode is used. In above both cases, the driver will

ignore the operating mode setting in the Adapter's EEPROM.

If this option is not specified, the operating mode setting in

the Adapter's EEPROM is used. Be aware that a memory map address is

required for memory-mapped mode. See MEMADDR for more information.

-------------------------------------------------------------------------------

MEMADDR    Memory map address    C0000 - EE000 (2000 increament)

for memory-mapped     or above 1M (on 8K boundery)

mode operation

A memory-mapped address block is required for memory-mapped mode

operation. You may use this option to specify the memory address.

If not specified, the driver will use the memory-mapped address

setting stored in the Adapter's EEPROM.

The specified location must be at 2000 boundary and have no

physical system memory module installed. For an ISA adapter, do

not specify a location above FFE000 (16M). For an EISA adapter,

do not specify a location above FFFFE000 (4G).

This driver does not verify whether the location is in system

memory or not. Please avoid using a location in system memory.

If the map adrress is below C0000 or, above EE000 and below 100000,

the driver uses D0000 as default.

Depending on the combination of OPMODE and MEMADDR, the driver

takes different action as below:

OPMODE    MEMADDR          Driver action

--------  --------------   ---------------------------------------

IO        igonred          I/O-mapped mode is used

MM        (none)           Memory-Mapped mode, use EEPROM address

(none)    valid address    mamory-mapped mode, use MEMADDR address

(none)    invalid address  memory-mapped mode, use D0000

(none)    (none)           use EEPROM setting for mode and address

-------------------------------------------------------------------------------

FRAME      Packet Frame type    Ethernet_802.2, Ethernet_II, Ethernet_802.3

Ethernet_SNAP

If not specified, the driver uses Ethernet_802.2 as default.

-------------------------------------------------------------------------------

PRIORITY   Priority level for   NORMAL, HIGH

packet transmission

If HIGH is specified, the driver will force all packet transmission

as high priority. If NORMAL is specified, use normal priority for

packet transmission. If this option is not specified, normal

priority is used. See PRIORITY.DOC for more information.

===============================================================================

DRIVER          : KVGNWBM.LAN

ADAPTER         : EISA adapter only

OPERATING MODE  : Bus-Master mode

Parameter  Description          Valid Values and Notes

-------------------------------------------------------------------------------

SLOT       EISA slot number     1,2,3,4,5,6,7,8,9,10,11,12,13,14,15

in which the EISA

adapter is installed

This option is valid for EISA adapter only. When only one adapter

is installed, it is not necessary to specify this option. The

driver will search the adapter automatically. If multiple adapters

are installed, the driver needs this option to identify a specific

EISA adapter.

-------------------------------------------------------------------------------

INT        Interrupt (IRQ)      3, 4, 5, 6, 7, 9, a, b, c, f

When this option is specified, the driver will use it for the

adapter operation and ignore the adapter's EEPROM setting.

If this option is not specified, the driver will use the

interrupt setting stored in adapter EEPROM.

-------------------------------------------------------------------------------

IRQTYPE    Interrupt trigger    EDGE  : edge-triggered interrupt type

type for the EISA    LEVEL : level-triggered interrupt type

adapter

This option is only valid for the EISA adapter. If the option is

not specified, the driver uses the adapter default setting.

-------------------------------------------------------------------------------

FRAME      Packet Frame type    Ethernet_802.2, Ethernet_II, Ethernet_802.3

Ethernet_SNAP

If not specified, the driver uses Ethernet_802.2 as default.

-------------------------------------------------------------------------------

PRIORITY   Priority level for   NORMAL, HIGH

packet transmission

If HIGH is specified, the driver will force all packet transmission

as high priority. If NORMAL is specified, use normal priority for

packet transmission. See PRIORITY.DOC for more information.

===============================================================================

DRIVER INSTALLATION

===================

USING "LOAD KVGNW ..." COMMAND

You may load the server driver directly under Netware console prompt. However,

make sure you have copied all drivers and the required NLM files from the

driver diskette into your proper direcotry in the server before loading the

driver.

To load KVGNW.LAN driver or KVGNWBM.LAN, follow the commands as below:

:load kvgnw [parameter=value] .....[parameter=value]

or

:load kvgnwbm [parameter=value] .....[parameter=value]

The supported parameters for each driver are summarized as below:

Driver supported       Valid values

Parameter  KVGNW.LAN   KVGNWBM.LAN  (none) means parameter is not specified.

---------  ----------  -----------  -----------------------------------------

PORT          yes          no       200,240,280,2c0,300,320,340,380,3c0

(none) -- auto search

SLOT          yes          yes      1,2,3,4,5,6,7,8,9,10,11,12,13,14,15

(none) -- auto search

PCIBOARD      yes          no       1,2,3,4

(none) -- auto search

INT           yes          yes      3,4,5,6,7,9,a,b,c,f

(none) -- use adapter's EEPROM setting

IRQTYPE       yes          yes      EDGE, LEVEL

(none) -- use adapter default setting

BUSTYPE       yes          no       1,2,3  (1:ISA, 2:EISA, 3:PCI)

OPMODE        yes          no       IO, MM

(none) -- use adapter's EEPROM setting

MEMADDR       yes          no       ISA adapter: C0000-EE000, 100000-FFE000

EISA adapter: C0000-EE000, 100000-FFFFE000

See parameter definition section.

FRAME         yes          yes      Ethernet_802.2, Ethernet_802.3,

Ethernet_SNAP, Ethernet_II

PRIORITY      yes          yes      NORMAL, HIGH

(none) -- use normal priority

-----------------------------------------------------------------------------

Note : All parameter names and values are caps insensitive.

Examples:

* Single adapter using EEPROM settings with frame Ethernet_802.2

(a) command:          load kvgnw

(b) command:          load kvgnw

* Single adapter using memory-mapped mode with memory-mapped address D0000

(ignoring EEPROM settings) and supporting frame ethernet_802.3

load kvgnw opmode=mm memaddr=d0000 frame=ethernet_802.3

* Single adapter supporting multiple frame types

(a) one ISA adapter (base I/O address = 300) supporting frame

ethernet_802.2 and ethernet_802.3:

load kvgnw port=300 frame=ethernet_802.2 name=kvg_300_802.2

load kvgnw port=300 frame=ethernet_802.3 neme=kvg_300_802.3

bind IPX to kvg_300_802.2 net=xxx

bind IPX to kvg_300_802.3 net=xxx

(b) one EISA adapter (in EISA slot 3) supporting frame ethernet_802.2

and ethernet_802.3:

load kvgnw slot=3 frame=ethernet_802.2 name=kvg_s3_802.2

load kvgnw slot=3 frame=ethernet_802.3 name=kvg_s3_802.3

bind IPX to kvg_s3_802.2 net=xxx

bind IPX to kvg_s3_802.3 net=xxx

(c) one PCI adapter supporting frame ethernet_802.2

and ethernet_802.3:

load kvgnw pciboard=1 frame=ethernet_802.2 name=kvg_s3_802.2

load kvgnw pciboard=1 frame=ethernet_802.3 name=kvg_s3_802.3

bind IPX to kvg_s3_802.2 net=xxx

bind IPX to kvg_s3_802.3 net=xxx

(d) one EISA adapter (in EISA slot 3) using bus-master driver and

supporting frame ethernet_802.2 and ethernet_802.3

load kvgnwbm slot=3 frame=ethernet_802.2 name=kvg_s3_802.2

load kvgnwbm slot=3 frame=ethernet_802.3 name=kvg_s3_802.3

bind IPX to kvg_s3_802.2 net=xxx

bind IPX to kvg_s3_802.3 net=xxx

* Multiple adapters, single frame type

(a) two ISA adapters with port 300 and 340 respectively to support

frame ethernet_802.3

load kvgnw port=300 frame=ethernet_802.3 name=kvg_300_802.3

load kvgnw port=340 frame=ethernet_802.3 name=kvg_340_802.3

bind IPX to kvg_300_802.3 net=xxx

bind IPX to kvg_340_802.3 net=xxx

(b) two EISA adapters (one is in slot 3, the other is in slot 4) to support

frame ethernet_802.3

load kvgnw slot=3 frame=ethernet_802.3 name=kvg_s3_802.3

load kvgnw slot=4 frame=ethernet_802.3 name=kvg_s4_802.3

bind IPX to kvg_s3_802.3 net=xxx

bind IPX to kvg_s4_802.3 net=xxx

(c) two PCI adapters to support frame ethernet_802.3

load kvgnw pciboard=1 frame=ethernet_802.3 name=kvg_pci1_802.3

load kvgnw pciboard=2 frame=ethernet_802.3 name=kvg_pci2_802.3

bind IPX to kvg_pci1_802.3 net=xxx

bind IPX to kvg_pci2_802.3 net=xxx

(d) one ISA adapter with port 300 and and one EISA adapter in slot 3

to support frame ethernet_802.3

load kvgnw port=300 frame=ethernet_802.3 name=kvg_300_802.3

load kvgnw slot=3 frame=ethernet_802.3 name=kvg_s3_802.3

bind IPX to kvg_300_802.3 net=xxx

bind IPX to kvg_s3_802.3 net=xxx

* Multiple adapters, multiple frame types

(a) one ISA adapter with port 300, and one EISA adapter in slot 3

supporting frame ethernet_802.2 and ethernet_802.3

load kvgnw port=300 frame=ethernet_802.2 name=kvg_300_802.2

load kvgnw port=300 frame=ethernet_802.3 name=kvg_300_802.3

load kvgnw slot=3 frame=ethernet_802.2 name=kvg_s3_802.2

load kvgnw slot=3 frame=ethernet_802.3 name=kvg_s3_802.3

bind IPX to kvg_300_802.2 net=xxx

bind IPX to kvg_300_802.3 net=xxx

bind IPX to kvg_s3_802.2 net=xxx

bind IPX to kvg_s3_802.3 net=xxx