This directory contains files:

pcntnd.dos - The NDIS 2.0.1 DOS driver

pcntnd.nif - NIF file for NDIS installation under PCNFS

protocol.ini - A sample protocol.ini file for NDIS driver

This directory is provided for the quickndis installation utility

from PCNFS. Please make a directory called "pcntnd" on your quickndis

floppy and copy the contents of this directory in the pcntnd directory

on the quickndis floppy. Run the PCNFS quickndis utility and choose the

PCNet family installation from the menu.

How to Install NDIS 2.0.1 Drivers

===================================

NDIS (Network Driver Interface Specification) 2.0.1 is a driver

specification developed by Microsoft Corporation to offer a standard

driver interface for all adapter cards.  A standard driver interface

provides flexibility to support not only multiple network adapter

cards but also multiple protocol stacks.  As a result, many operating

systems have incorporated the NDIS interface to make configuration

and installation of network adapter cards easy and independent of

network adapter card.

Included in this manual are instructions to install the ETHER-32V/P family

NDIS 2.0.1 driver for the following environments:

Microsoft LAN Manager 2.x

Microsoft Windows for Workgroups 3.1

Microsoft Windows for Workgroups 3.11

Banyan VINES

IBM LAN Server 2.0, 3.0

Sun Microsystems PC-NFS 5.0

How to Install NDIS 2.0.1 for PC-NFS

========================================

The ETHER-32V/P family NDIS 2.0.1 driver (PCNTND.DOS) may also be installed for

the PC-NFS environment.

The directory structure provided on the ETHER-32V/P driver diskette has been

setup for automatic OEM installation.  The OEM Installation section

in this manual provides a brief explanation regarding OEM driver

installation.  Follow the guidelines in the Sun Microsystems PC-NFS

Manuals for more details to install the family driver from the

ETHER-32V/P driver diskettes.

Note: Some of the custom installation procedures provided here are

either an abbreviated or a duplication of the OEM installation

guidelines.

To install the NDIS 2.0.1 driver, follow the instructions

given below:

1. Insert the PC-NFS QUIKNDIS disk into the system's floppy

drive. Change the drive letter containing the

QUIKNDIS disk.

2. Type QUIKNDIS at the prompt and press Enter.

3. A message will appear asking for the current boot drive.

Enter the current boot drive and press Enter.

4. In the next dialog box, press "Y" to save the current

PROTOCOL.INI as PROTOCOL.OLD.  Ignore this step if this

dialog box does not appear.

5. In the adapter card list box, select the Custom option

to install the NDIS driver for PC-NFS.

6. If necessary, remove the QUIKNDIS disk from the floppy drive.

Insert the ETHER-32V/P driver diskette into a floppy drive.

7. Now, follow the dialog boxes and enter the correct driver

and path on the ETHER-32V/P driver diskette to install the NDIS

driver. For example, A:\PCNFS.

8. Next, enter the PCNTND for the NIF file name and press Enter.

9. Follow the dialog boxes and enter the correct parameters.

10. Installation of the family NDIS 2.0.1 driver for

PC-NFS is now complete.