CHGBIOS.DOC        UPDATING THE T3300SL BIOS TO VERSION 1.30         7/16/92b

The T3300SL's BIOS is in Flash EPROM so can be updated electrically

without changing any hardware. CHGBIOS.EXE, a DOS program, is the

software tool used to install BIOS code from a DOS data file into the

Flash EPROM BIOS of the T3300SL.

���������������������������������������������������������������ķ

� NOTE: CHGBIOS.EXE will not run in V86 mode!  This will be     �

�       the case if you are using a 386 memory manager,         �

�       running Windows, or using any other software which      �

�       puts the system in virtual-8086 mode. If you get the    �

�       message "This utility is not available on V86 mode!"    �

�       reboot from a "clean" floppy, or rename your            �

�       CONFIG.SYS file and reboot, and run CHGBIOS.EXE again.  �

���������������������������������������������������������������Ľ

Extracting T33SL31.ZIP should result in producing the following files:

BOOTBIOS.COM    114944 bytes    2/26/92 2:49p

T33SL130.COM    114944 bytes    6/19/92 3:05p

CHGBIOS.EXE      18653 bytes     2/2/92 5:33p

CHGBIOS.DOC       1735 bytes    7/16/92 1:30p

��������������������������������������������������������������������������

To update your BIOS follow these five easy steps.

1. Run CHGBIOS.EXE

2. Insert a scratch diskette into drive A.

3. When prompted for BIOS file name enter "T33SL130.COM"

4. BOOTBIOS.COM is written to floppy disk.

5. Remove diskette and press reset switch to reboot.

Your BIOS version has now been updated.  Enter "BIOSVER" at the DOS

prompt to verify that you now have a BIOS version of 1.30.

-Jim