VideoLogic Apocalypse 3D for Windows 95

January 30th 1997

Contents

1. Introduction

2. Matrox update

3. Installing Apocalypse 3D

4. Hints and tips

5. Troubleshooting

6. PowerVR D3D HAL Setup

7. System modifications made by Apocalypse 3D

1. Introduction

This file contains late-breaking information, hints and tips, system details and troubleshooting information for Apocalypse 3D.

You can print this file by clicking the File menu at the top of the window, then Print. The page

setup is for US Letter sized paper, but will also print to A4 size.

2. Matrox update

Introduction

At the time of the launch of the 1.0 PowerVR drivers there were two known problems with drivers running on the Matrox Millennium and Mystique graphics cards. These affected general full-screen Direct3D (D3D) operation and the running of certain applications which performed 2D/3D compositing, and were related to the operation of the D3D drivers only - PowerSGL applications were unaffected.

Full Screen Operation

In the initial release, there were problems with D3D titles hanging when they went into full screen. This was a synchronisation issue with the Matrox controller being unable to cope with PowerVR and host access at the same time. This has been fixed in the latest drivers.

2D/3D Compositing

2D/3D compositing affects D3D SDK samples like Tex4, Tex5, Oct1 and retail applications such as Hellbender and Monster Truck Madness. For these applications, the PowerVR drivers use the transparent Blit function of the  2D controller to render the images correctly. Unfortunately, Matrox cards behave differently from other graphics cards when perfoming transparent Blits. The side-effect in the initial release of the Apocalypse 3D driver is that images were rendered with magenta blocks.

This driver release provides an interim fix when running on Matrox cards, in which the PowerVR D3D HAL disables itself for those applications requiring 2D/3D compositing.

Work is still in progress to improve the operation of the PowerVR HAL with these Matrox graphics cards and VideoLogic are discussing a permanent technical solution with Matrox .

To overcome these problems, follow the steps below:

1.	To correct the hanging application problem, run the SGLHW.INI file by clicking Start, 	clicking Run, typing SGLHW.IN and clicking OK.

There is a section in the file that looks like this:

[default]

StrictLocks=0

Change the section to look like this:

[default]

StrictLocks=1

2.	To enable the 2D/3D compositing solution, add this line to the [PVRHAL] section:

MatroxFix=1

3.	Save SGLHW.INI file under the same name.

3. Installing Apocalypse 3D

Installing the hardware and drivers

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer's documentation.

Before installation, make sure you are using a graphics card with drivers that support Microsoft DirectDraw, and that your PC is running Microsoft Windows 95 and DirectX III.

1. Switch off your computer, monitor, and any hardware devices such as printers connected to your computer. Leave the power cable connected to the power outlet so that your computer is grounded.

2. Remove the computer's cover. You may need to refer to the documentation supplied with your computer for instructions on how to do this.

3. Locate an empty PCI bus expansion slot and remove the slot cover. Keep the screw, you will need it later to secure the card.

4. Touch the bare metal chassis of your computer with the back of your hand to discharge any static that may have built up on you or your clothes.

5. Align Apocalypse 3D with the slot you have chosen. Firmly press it into the slot. You will need to use some pressure to push the card into position, but do not use excessive force.

6. Secure the card using the screw that you removed previously.

7. Replace the cover of your computer and switch on your computer and your computer monitor.

8. When your computer has restarted, a dialog box is displayed, telling you that Windows has found new hardware. Click the check box for the driver provided by the hardware manufacturer. Click OK.

9. Another dialog box is displayed, asking for the location of the software. Browse to [disk]:\  where [disk] is your CD drive. After browsing, click OK.

10. You are now ready to install the drivers. Click OK.

Two sets of drivers are installed, for PowerSGL and Direct3D. PowerSGL is VideoLogic's proprietary 3D language.

Direct3D is Microsoft's 3D graphics language. These drivers enable you to optimally run applications based on either language.

11. When the drivers have been installed, a message is displayed asking whether you want to restart Windows. Click Yes.

When your computer has restarted Apocalypse 3D is ready for use.

Upgrading the drivers

From time to time, VideoLogic may issue upgrades to the drivers. If this happens, follow the instructions below to install them on your PC.

1. Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2. From the Control Panel, double-click System. The System Properties dialog box is displayed.

3. Click the Device Manager tab. A list of devices available on your PC is displayed.

4. From the list, double-click Sound, Video and game controllers. Under Sound, Video and game controllers, the driver for Apocalypse 3D is displayed.

5. From the list, double-click Apocalypse 3D. The PCI Properties dialog box is displayed.

6. Click the Driver tab. A list of driver files is displayed.

7. Click Change driver. The Select Device dialog box is displayed.

8. Click Have Disk. The Install from Disk dialog box is displayed.

9. Browse to [disk]:\  where [disk] is the drive containing the Apocalypse 3D updates. When you have browsed, click OK.

10. At the Install from Disk dialog box, click OK. At the Select Device dialog box, click OK. At the PCI Properties dialog box, click OK. The drivers are upgraded on your PC, which may take a short while.

11. When the drivers have been upgraded, a message is displayed asking whether you want to restart Windows. Click Yes.

When your computer has restarted Apocalypse 3D is ready for use.

4. Hints and Tips

Introduction

This section gives late-breaking information on how to get the most out of your Apocalypse 3D.

Installing games

The majority of DirectX games currently on the market are supplied with DirectX II. When installing these games you are often offered the opportunity to install DirectX II. Do not install DirectX II, as Apocalypse 3D requires DirectX III. If the game's installation forces DirectX II to be installed, reinstall DirectX III immediately afterwards.

Microid Research BIOS

Some PCs with an MR BIOS can give a much lower frame rate with Apocalypse 3D. Switching to another BIOS can greatly enhance performance.

5. Troubleshooting

Introduction

This section describes common problems you may encounter when installing or running Apocalypse 3D.

Games graphics display a 'halo' around objects

This problem can be rectified by running the D3D Setup utility, as described in section 5. For other graphics problems, access our web site where the latest version of our D3D drivers registry update can be found.

With the D3D demos I am not given the D3D Hal option

Ensure that you have the DirectX III graphics drivers installed, and that your PC is in a display mode greater or equal to 16 bits per pixel.

The application display is corrupted

Your PC may not be fully DirectDraw compliant. This can be tested using the DirectDraw Compliance utility provided on the drivers disk.

The application looks fuzzy, with lines appearing alongside the active window

The resolution of the window in which the application is running is too high, or the refresh rate of the screen is too high. Lower the resolution or the refresh rate in Display Properties.

The Start menu and Windows menus contain corrupted text, or text is missing

After playing a game in 8-bit display mode, the PC has changed from 8-bit to 16-bit display mode automatically, which the display driver may not be able to support. Restart the PC.

To avoid this problem in future, your PC should be in a 16-bit display mode before playing a game.

When running an application that uses PowerSGL or Direct3D, the system freezes or restarts

Apocalypse 3D may be installed in a PCI bus slot that does not have bus-mastering capability.

When running an application that uses PowerSGL the system restarts

The display driver may not support buffer flipping. To support flipping the amount of free off-screen memory must be greater than the current on-screen memory. A lower screen resolution may free up sufficient memory, however, due to other graphics memory requirements (e.g. hardware cursors), 1 MB cards may not support flipping even in a resolution of 640x480 pixels with 65 thousand colors. This is why we recommend a minimum graphics memory of 2 MB.

The Use Hardware Accelerator option within an application is greyed out

Solution 1:	The application is currently set to run in a graphics resolution that cannot be accelerated by Apocalypse 3D. Changing the application's graphics resolution setting to a suitable mode will make the check-box available.

Solution 2:	The Apocalypse 3D board is not being recognised by the system. Check that it is in System Properties, check BIOS, IRQ allocation etc.

Problem with the D3D test utility (d3dtest.exe)

This program performs simple benchmarks for D3D. After each test session the last frame of the window display remains on-screen. This feature of d3dest.exe does not detract from Apocalypse 3D's useability.

6. PowerVR D3D HAL Setup

You can access the setup utility by clicking the PowerVR icon at the bottom right corner of your screen.

When you select PowerVR D3D HAL Setup, this window is displayed:

This utility enables the graphics of some D3D (PowerVR-enhanced) games to work optimally with PowerVR technology. You must be in 16 bits per pixel to run the utility.

Some games display a halo of solid color around objects, such as in the example below:

If you are experiencing this problem, follow the steps below.

If your game is listed in the dialog box

1.	Click the game's description to turn the 2D/3D Compositing setting on.

2.	Click Exit.

3.	Restart the game.

If your game is not listed in the dialog box

1.	Click Add. The add new game dialog box is displayed.

2.	In the Application column, type in the name of the executable file for the game, in the form app_name.exe

If you are not sure of this filename, the best way to find it is through the Find facility (click the Start button and point to Find). The filename usually has the extension .EXE or .COM. For example, if your game were called Chess, you would search for Chess.EXE on your hard drive. If this did not work, you would search for Chess.COM

3.	In the Description column, type in a brief description of the game, in capitals or lower-case including symbols and digits, up to a maximum of 35 characters.

4.	Click OK. The PowerVR D3D Setup dialog box is displayed.

5.	Click Exit.

6.	Restart the game.

If you want to modify an existing game's entry

1.	Highlight the game's application name, and click Edit. The edit game dialog box is displayed.

2.	In the Description column, type in a brief description of the game, in capitals or lower-case including symbols and digits, up to a maximum of 35 characters.

3.	Click the 2D/3D Compositing box to turn the utility on.

4.	Click OK. The PowerVR D3D Setup dialog box is displayed.

5.	Click Exit.

6.	Restart the game.

7. System modifications made by Apocalypse 3D

Introduction

This section gives a listing of the files resident on your system after installing the Apocalypse 3D hardware and software.

PowerSGL

\WINDOWS\SGL.DLL  		main DLL to which applications link (hides 	board specific details from applications)

\WINDOWS\SGLMID4.DLL 		PowerSGL/PowerSGL Direct DLL for 	Midas 4

\WINDOWS\SYSTEM\VSGL.VXD 	generic Midas virtual device drive

\WINDOWS\INF\MIDAS.INF		generic installer

D3D

\WINDOWS\SYSTEM\PVRHAL32.DLL 	main D3D HAL

\WINDOWS\SYSTEM\PVRILOAD.EXE 	toolbar application for enabling/disabling 	PowerVR HAL

\WINDOWS\SYSTEM\PVRWIN32.DLL	DLL for PVRILOAD

\WINDOWS\SYSTEM\PVRTHK16.DLL  	DLL for PVRILOAD on Compaq systems

\WINDOWS\SYSTEM\PVRHELP.HLP	PowerVR D3D help file