VideoLogic GrafixStar 410 for Windows 95

4th April 1997

Contents

1. Introduction

2. The GrafixStar 410 drivers

3. Installing GrafixStar 410

4. Removing GrafixStar 410

5. Removing SmartTools

6. Troubleshooting

7. Field notes

1. Introduction

This file contains installation instructions, late-breaking information, troubleshooting information, and notes on GrafixStar 410.

If you are reading this file outside of the VideoLogic Interactive Installation Guide and want to install GrafixStar 410 manually, refer to section 3 below or the User Guide for complete instructions on how to do this.

2. The GrafixStar 410 drivers

Introduction

This section describes the display modes available with video acceleration, and how GrafixStar 410 interacts with DirectDraw.

Display modes with video acceleration

The modes below support video acceleration:

Display size	Colors	Refresh rate (Hz)

640 x 480	256	60, 72, 75, 85, 100, 120

800 x 600	256	56, 60, 72, 75, 85 ,90, 100, 120

1024 x 768		256	43, 60, 70, 72, 75, 80, 85, 90,100

640 x 480	65,536	60, 72, 75, 85, 90, 100, 120

800 x 600	65,536	56, 60, 72, 75, 85

1024 x 768		65,536	43, 60, 70, 72, 75, 80, 85, 90, 100, 120

640 x 480	16.7m	60, 72, 75, 85, 90

800x600	16.7m	56, 60, 72, 75, 85

All other modes display AVI files using software only, which gives poorer quality playback. If you want to playback MPEG files without special hardware, you can obtain a FREE copy of Xing MPEG Player by returning your GrafixStar 410 registration card.

Using games with DirectDraw

GrafixStar 410 display drivers for Windows 95 are compatible with DirectDraw.

SmartTools automatically installs DirectVideo with DirectDraw 2.0. To manually install DirectVideo with DirectDraw, run GS410\WIN95\DISK3\REDIST.IHV\SETUP.EXE from the VideoLogic Software Library CD-ROM, after which Windows 95 needs to be restarted. To remove DirectDraw from your PC, delete the following files from the windows\system directory on hard disk on which you are running Windows 95.

| ddhelp.exe | dsound.vxd |
| --- | --- |
| dsound.dll | ddraw.dll |
| ddraw16.dll | d3drm.dll |
| d3dxof.dll | d3dim.dll |
| d3drampf.dll | d3drgbf.dll |
| d3drm8f.dll | d3drm16f.dll |
| d3drm24f.dll | d3drm32f.dll |
| d3drg8f.dll | d3drg16f.dll |
| d3drg24f.dll | d3drg32f.dll |
| d3dhalf.dll | dplay.dll |
| dpserial.dll | dpwsock.dll |
| dvideo.dll |  |

3. Installing GrafixStar 410

Introduction

The easiest way to install your GrafixStar 410 is to install the hardware first, referring to the User Guide, and then install the software using the Interactive Installation Guide which runs automatically when you insert the VideoLogic CD-ROM into your PC. If you don’t have a User Guide, or you want to install the software manually, refer to the following instructions.

Handling GrafixStar 410

Follow these guidelines to minimize the risk of electrostatic charges damaging the card:

Keep the card in its protective packaging until you are ready to install it.

Just before handling the card, touch the metal frame of your computer with the back of your hand to discharge any static electricity which may have built up on you or your clothes.

Hold the card by the adapter bracket or by its edges; do not touch the printed circuit board.

Do not place the card on top of your computer or on any other metal surface.

Make sure that the card is not accidentally touched by anyone else.

Handle the card with care to avoid damage to the fragile electronic components mounted on the circuit board.

Installing the hardware

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

1.	Switch off your computer, monitor, and any hardware devices such as printers connected to your computer. Leave the power cable connected to the power outlet so that your computer is grounded.

2.	Disconnect your monitor cable from your computer.

3.	Remove the computer’s cover. You may need to refer to the documentation supplied with your computer for instructions on how to do this.

4.	Remove your existing display card or disable your built-in graphics controller. Refer to the documentation supplied with your computer for instructions on how to disable a built-in graphics controller.

5.	Locate an empty PCI bus expansion slot and remove the slot cover. Keep the screw, you will need it later to secure the card.

6.	Touch the bare metal chassis of your computer with the back of your hand to discharge any static that may have built up on you or your clothes.

7.	Align GrafixStar 410 with the slot you have chosen. Firmly press the card into the slot. You will need to use some pressure to push the card into position, but do not use excessive force.

8.	Secure GrafixStar 410 using the screw that you removed previously.

9.	Replace the cover of your computer.

10.	Connect the monitor cable to GrafixStar 410. All cables connected to GrafixStar 410 should be properly shielded and comply with radio frequency interference standards.

11.	Switch on your computer and your computer monitor. You are now ready to install the software.

Installing the software

The easiest way to install the software is to use the Display Properties window. Installation will vary slightly depending upon which build of Windows 95 you are using. Use the following steps to find out which build of Windows 95 you are using, then follow the instructions for that version.

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the System icon to open the System Properties Window.

3.	Click the General tab. The build number is located in the System section, under the ‘Microsoft Windows 95’ line.

Software installation for Windows 95 build 4.00.950 or 4.00.950a

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the Display icon to open the Display Properties Window.

3.	Click the Settings tab. The Settings page is displayed.

4.	Click the Change Display Type button.

5.	Make sure that the Monitor Type line contains the name of your monitor. If not, click the Change button and click the name of your monitor on the list.

6.	Click the Change button for the Adapter Type.

7.	Insert the VideoLogic CD-ROM into your computer (close the Interactive Installation Guide if it autoruns at this point) and click Have Disk from the Select Device menu. Browse to the folder \gs410\win95\disk1 on your CD-ROM drive.

8.	Click GrafixStar 410 GrafixMAX from the list. Click OK.

9.	When the GrafixStar files have been copied to your PC, the Change Display Type window is displayed again. Click Close.

10.	The Display Properties windows is displayed again. Click Apply.

11.	A message appears asking whether you want to restart your computer. Click Yes. Windows 95 restarts with the new GrafixStar Windows 95 drivers.

12.	When your computer has restarted a dialog box is displayed asking whether you want to install SmartTools with DirectVideo, or just SmartTools, or neither.

SmartTools is desktop control software that enables you to configure and customize your display, set a virtual desktop, and build your own toolbars. Click SmartTools to install SmartTools and DirectVideo.

DirectVideo is a set of drivers that optimize your computer’s performance when running DirectDraw video applications. If you only want to install DirectVideo, click DirectVideo.

If you don’t want to install either SmartTools or DirectVideo, click Cancel. Note that video playback performance will not be optimized without DirectVideo.

13.	Follow the instructions that appear on the screen.

Software installation for Windows 95 build 4.00.950 B (otherwise known as OSR2)

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the Display icon to open the Display Properties Window.

3.	Click the Settings tab. The Settings page is displayed.

4.	Click the Advanced Properties button.

5.	Click the Monitor tab. The Monitor page is displayed.

6.	Make sure that the Monitor page contains the name of your monitor. If not, click the Change button and click the name of your monitor on the list.

7.	Click the Adapter tab. The Adapter page is displayed.

8.	Click the Change button to change display driver.

9.	Insert the VideoLogic CD-ROM into your computer (close the Interactive Installation Guide if it autoruns at this point) and click Have Disk from the Select Device menu. Browse to the folder \gs410\win95\disk1 on your CD-ROM drive.

10.	Click GrafixStar 410 GrafixMAX from the list. Click OK.

11.	When the GrafixStar files have been copied to your PC, the Advanced Display Properties window is displayed again. Click Close.

12.	The Display Properties windows is displayed again. Click Apply.

13.	A message appears asking whether you want to restart your computer. Click Yes. Windows 95 restarts with the new GrafixStar Windows 95 drivers.

14.	When your computer has restarted a dialog box is displayed asking whether you want to install SmartTools with DirectVideo, or just SmartTools, or neither.

SmartTools is desktop control software that enables you to configure and customize your display, set a virtual desktop, and build your own toolbars. Click SmartTools to install SmartTools and DirectVideo.

DirectVideo is a set of drivers that optimize your computer’s performance when running DirectDraw video applications. If you only want to install DirectVideo, click DirectVideo.

If you don’t want to install either SmartTools or DirectVideo, click Cancel. Note that video playback performance will not be optimized without DirectVideo.

15.	Follow the instructions that appear on the screen.

4. Removing GrafixStar 410 from Windows 95

You remove the card from Windows 95 in two stages: first the hardware, then the software.

To remove GrafixStar 410 from your Windows 95 system:

1. 	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2. 	Double-click the System icon to open the System Properties Window.

3.	Click the Device Manager tab.

4.	Double-click Display Adapters.

5.	Click GrafixStar 410 GrafixMAX, and click Remove.

6.	A dialog box is displayed asking you if you want to restart Windows. Click No.

7.	Shut down Windows 95.

8.	Switch off your computer, monitor, and any hardware devices such as printers 
connected to your computer. Leave the power cable connected to the power outlet 
so that your computer is grounded.

9.	Disconnect the monitor cable from your computer.

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

10.	Remove the computer’s cover. You may need to refer to the documentation supplied 
with your computer for instructions on how to do this.

11.	Remove the GrafixStar card from its PCI slot and replace it with a different graphics card.

12.	Replace the cover of your computer.

13.	Switch on your computer and your computer monitor. Windows 95 detects the new graphics card and installs drivers for it.

14.	When Windows 95 is running with the new graphics card’s drivers you can delete the following files from the disk on which you are running Windows 95:

\windows\vl410dd.ini

\windows\system\s3dtkw.dll

\windows\system\s3kernel.dll

\windows\system\vlenable.sys

\windows\system\vlgx410a.drv

\windows\system\vlgx410a.vxd

\windows\system\vlgxcpl.dll

\windows\system\vlregio.dll

\windows\system\vlspdd32.dll

5. Removing SmartTools

Note: Save any open documents before removing SmartTools, as you will need to restart Windows.

To remove SmartTools:

1.	Click the Start button, and point to Settings.

2.	Click Control Panel.

3.	Click Add/Remove Programs.

4.	From the list of programs displayed, click VideoLogic SmartTools, and click Add/Remove.

5.	Follow the instructions that appear on the screen.

6. Troubleshooting

Introduction

This section describes problems you may encounter when using SmartTools or running GrafixStar 410.

Display Properties

Errors when running virtual desktops and/or changing modes

Some applications, especially those that display video, report errors when running a virtual desktop or changing modes. Every effort is made to prevent such problems but if they do occur we recommend that you change to a non-virtual desktop mode, or close the problematic application before changing modes.

Modes

16-color modes

SmartTools is not designed to work with a 16 color palette (4 bits per pixel). You must change to a higher palette via the Display Settings dialog box of Control Panel to enable SmartTools to work.

Monitors

Display sizes

Windows 95 does not restrict the generic SVGA monitor settings to a given set of display sizes, so SmartTools allows any display size to be selected. If you have a standard monitor selected, there is therefore a possibility that you could zoom out to a display size that your monitor does not support.

Corrupt or blank display

Windows 95 does not restrict the generic SVGA monitor settings to a given set of refresh rates, so SmartTools allows any refresh rate to be selected. If you have a standard monitor selected, there is therefore a possibility that you could restart your system with a refresh rate that your monitor does not support. This usually results in a corrupted or black display in Windows 95. To rectify this problem, restart Windows 95, holding down the CTRL key on your keyboard. This starts Windows 95 in the lowest possible refresh rate for the resolution selected.

Toolbars

Disappearing toolbars

When using SmartDesktop to zoom into the desktop, you may lose sight of the SmartTools toolbars. You can relocate a toolbar within your current display using the Find Now feature. This is located by clicking the Properties button of the toolbar on the Toolbar Properties page of SmartTools. Click the Find Now button to move the active SmartTools toolbar to a visible location within the display.

Toolbar Properties page of SmartTools

Maximize in Display

The SmartDesktop Maximize on Display utility maximizes your applications to the full size of your display, so part of your application window may disappear behind the Windows 95 taskbar. If you use this feature, we recommend that you set the taskbar to auto-hide.

Moving and deleting buttons from toolbars

You can move buttons around on the toolbars by holding down the SHIFT key and dragging them to the desired location. To remove a button, drag it off the toolbar while holding down the SHIFT key.

HotKeys

DOSBox

The SmartDesktop hotkeys will not function when a DOSBox is active.

Connectivity - Feature connector

Feature connector

The Feature connector is supported in 640 x 480 and  800 x 600 display sizes in 256 colors.

If you select Feature Connector from the Connectivity page of SmartDisplay, your CONFIG.SYS file is modified to include the TSR VLENABLE.SYS. VLENABLE is a utility that enables DAC snooping when you turn on your PC.

If your system does not have a CONFIG.SYS file, or it has one that is zero bytes in size, you must create a file called CONFIG.SYS at the root of your hard disk and make sure that it contains some data (a rem line is sufficient), before you select Feature Connector.

When you have selected Feature Connector you must restart your system to run VLENABLE.

If you uncheck the Feature Connector option, the VLENABLE.SYS line in the CONFIG.SYS file will be disabled by changing it to a rem line.

Drivers

QuickTime for Windows Version 2.X

If you experience problems when viewing QuickTime movies, run the QuickTime Control Panel from the Windows 95 Control Panel, and follow the steps below:

1.	Run the QuickTime Control Panel, click the More button and click the Video tab.

2.	From within the Optimization section, the Draw Method setting will be set to Video Hardware. Change this to Video Driver.

3.	Close the QuickTime Control Panel.

Playing AVI with MS PLUS! pack

If you are using a display mode which is capable of video acceleration and you have the Show Window Contents While Dragging option enabled, you will see the accelerated video displaced from the video window while dragging the video window around the desktop. Disable the Show Window Contents While Dragging on the PLUS page in Display Properties.

Monitor screen centering

Some advanced monitors create a display setting by detecting the horizontal line frequency, and this may change the size and positioning of the Windows 95 screen when task switching from a DOS full-screen session to a Windows 95 session. This problem occurs when the monitor is at the borderline between two settings. To remedy this, use the controls on your monitor to adjust its screen settings.

Xing 3.X

MPEG clips replayed in modes that are not video accelerated may cause Windows to become unstable. If this problem occurs, change the display mode to one that supports video acceleration.

Laplink for Windows 95

When using Laplink for Windows 95 and the Client system has a GrafixStar 410 installed, the Client system might lock-up when the host is connected.

This problem can be fixed by disabling bitmap caching in the GrafixStar 410 driver. There are two ways of disabling bitmap caching.

Disabling bitmap caching in systems with SmartTools installed:

SmartTools v2.0 or later comes with a feature called SmartMAX. SmartMAX allows you to disable certain parts of the GrafixStar driver. To use SmartMAX, right-click the Windows 95 desktop and click Properties while holding down the CTRL key. In the Display Properties window click the SmartMAX tab and select the checkbox next to “Disable Bitmap Caching”. Click OK and restart Windows 95.

Disabling bitmap caching in systems without SmartTools:

Add the following entry to the bottom of your SYSTEM.INI file:

[VideoLogic SVGA]

TuningControl=27

RealizeControl=16

Save the changes and restart Windows 95.