| System Unit | Dimensions (HxWxD) | 5 x 18.25 x 15.69 in/12.7 x 46.36 x 39.85 cm |  |
| --- | --- | --- | --- |
|  | Weight (approximate) | 28.3 lbs/12.85 kg |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |
| Power Supply |  |  |  |
|  | Operating Voltage Range | 90 - 132 VAC/180 - 264 VAC |  |
|  | Rated Voltage Range | 100 - 120 VAC/220 - 240 VAC |  |
|  | Rated Line Frequency | 50 - 60 Hz |  |
|  | Rated Input Current | 5A/3A |  |
|  | Maximum Rated Power | 200 W |  |
|  | Temperature |  |  |
|  | Operating | 50° to 95°F/10° to 35°C |  |
|  | Nonoperating | -4° to 140°F/-20° to 60°C |  |
|  | Humidity (noncondensing) |  |  |
|  | Operating | 8% to 90% |  |
|  | Nonoperating | 5% to 95% |  |
|  | Maximum Altitude (unpressurized) |  |  |
|  | Operating | 10,000 ft/3048 m |  |
|  | Nonoperating | 30,000 ft/9144 m |  |
| Enhanced Keyboard | Dimensions (HxWxD) Weight | 1.5 x 18.0 x 6.5 in/3.81 x 45.72 x 16.51 cm 3.5 lbs/1.59 kg |  |
| 1.44-MB Diskette Drive | Diskette Size | 3.5-in |  |
|  | LED Indicators | Green |  |
|  | Read/Write Capacity/Diskette (high/low density) | 1.44 MB/720 KB |  |
|  | Drive Height | 1 (in) |  |
|  | Drive Rotation | 300 rpm |  |
|  | Transfer Rate (high/low density) | 500/250 Kb/s |  |
|  | Bytes/Sector | 512 |  |
|  | Sectors/Track (high/low density) | 18/9 |  |
|  | Tracks/Side (high/low density) | 80/80 |  |
|  | Seek Time |  |  |
|  | Track-to-Track (high/low density) | 3/6 ms |  |
|  | Average (high/low) | 94/173 ms |  |
|  | Settling Time | 15 ms |  |
|  | Latency Average | 100 ms |  |
|  | Cylinders (high/low) | 80/80 |  |

Read/Write Heads2

| Ultra ATA Hard Drives |  |  |  | 2.4-GB |  | 3.2-GB |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Formatted Capacity |  |  | 2429.7 MB |  | 3228.6 MB |  |  |  |  |  |  |  |  |
|  | Drive Type |  |  | 65 (soft) |  | 65 (soft) |  |  |  |  |  |  |  |  |
|  | Transfer Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Media |  |  | 132 MB/s |  | 87.8 MB/s |  |  |  |  |  |  |  |  |
|  | Interface |  |  | 33 MB/s |  | 33 MB/s |  |  |  |  |  |  |  |  |
|  | Seek Time (typical including settling) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Single Track |  |  | 3.0 ms |  | 2.0 ms |  |  |  |  |  |  |  |  |
|  | Average |  |  | 10 ms |  | 10 ms |  |  |  |  |  |  |  |  |
|  | Full Stroke |  |  | 20 ms |  | 20 ms |  |  |  |  |  |  |  |  |
|  | Disk Rotational Speed |  |  | 5400 rpm |  | 4500 rpm |  |  |  |  |  |  |  |  |
|  | Cylinders |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Physical |  |  | 7066 |  | 5691 |  |  |  |  |  |  |  |  |
|  | Logical |  |  | 4708 |  | 6218 |  |  |  |  |  |  |  |  |
|  | Data H . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . eads |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sectors per Track |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Physical |  |  | 160-273 |  | 114-234 |  |  |  |  |  |  |  |  |
|  | Logical |  |  | 63 |  | 63 |  |  |  |  |  |  |  |  |
|  | Buffer Size |  |  | 128 KB |  |  |  | 128 KB |  |  |  |  |  |  |
| 16X Max1 CD-ROM Drive | Disk |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IDE (slot-load) | Applicable Disk |  |  | CD-ROM (Mode 1 and 2), CD-DA, CD-XA Ready, Photo CD (single and multi session), Mixed Mode (audio and data combined) |  |  |  |  |  |  |  |  |  |  |
|  | Capacity |  |  | 540 MB (Mode 1, 12 cm) 640 MB (Mode 2, 12 cm) |  |  |  |  |  |  |  |  |  |  |
|  | Block Size |  |  | 2048 bytes (Mode 1) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2340, 2336 bytes (Mode 2) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2352 bytes (CD-DA Mode) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2328 bytes (CD-XA Mode) |  |  |  |  |  |  |  |  |  |  |
|  | Rotational Speed |  |  | 1840 - 4240 rpm |  |  |  |  |  |  |  |  |  |  |
|  | Diameter |  |  | 4.7 in, 3.15 in/12 cm, (8 cm may require adapter) |  |  |  |  |  |  |  |  |  |  |
|  | Center Hole |  |  | .6 in/15 mm diameter |  |  |  |  |  |  |  |  |  |  |
|  | Thickness |  |  | .05 in/1.2 mm |  |  |  |  |  |  |  |  |  |  |
|  | Track Pitch |  |  | 1.6 µm |  |  |  |  |  |  |  |  |  |  |
|  | Interface |  |  | IDE |  |  |  |  |  |  |  |  |  |  |
|  | Access Time (typical) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Random |  |  | < 100 ms |  |  |  |  |  |  |  |  |  |  |
|  | Full Stroke |  |  | < 200 ms |  |  |  |  |  |  |  |  |  |  |
|  | Audio Output Level |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Line Out |  |  | 0.7 VRMS at 47 kOhms |  |  |  |  |  |  |  |  |  |  |
|  | Headphone |  |  | 0.6 VRMS at 32 Ohms (maximum volume) |  |  |  |  |  |  |  |  |  |  |
|  | Data Transfer Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sustained |  |  | 150 kB/s |  |  |  |  |  |  |  |  |  |  |
|  | Variable (8x to 16x) |  |  | 1200 to 2400 kB/s |  |  |  |  |  |  |  |  |  |  |
|  | Burst |  |  | 4 MB/s |  |  |  |  |  |  |  |  |  |  |
|  | Cache Buffer Startup Time |  |  | 128 KB Less than 7 seconds |  |  |  |  |  |  |  |  |  |  |
|  | Stop Time |  |  | Less than 4 seconds |  |  |  |  |  |  |  |  |  |  |
| 16X Max1 CD-ROM Drive IDE (slot-load) (cont’d) | Laser Parameters Type Wave Length |  |  | Semiconductor Laser GaAIAs 780 ± 25 nm |  |  |  |  |  |  |  |  |  |  |
|  | Divergence Angle |  |  | 53.5° ± 1.5° |  |  |  |  |  |  |  |  |  |  |
|  | Output Power |  |  | 0.14 mW (at objective lens) |  |  |  |  |  |  |  |  |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operating Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Temperature |  |  | 41-113° F/5-45° C |  |  |  |  |  |  |  |  |  |  |
|  | Humidity |  |  | 10-80% |  |  |  |  |  |  |  |  |  |  |
|  | Dimensions (HxWxD) Weight |  |  | 1.69 x 5.91 x 8.19 in/4.29 x 15.01 x 20.8 cm 2.09 lbs/.95 kg |  |  |  |  |  |  |  |  |  |  |
| 24X Max2 CD-ROM Drive IDE (slot-load) | Disk Applicable Disk |  |  | CD-ROM (Mode 1 and 2), CD-DA, CD-XA |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | (Mode 2, Form 1 and 2), Photo CD (single and multi session), Mixed Mode (audio and data combined) |  |  |  |  |  |  |  |  |  |  |
|  | Capacity |  |  | 650 MB |  |  |  |  |  |  |  |  |  |  |
|  | Block Size |  |  | 2048, bytes (Mode 1) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2040, 2336 bytes (Mode 2) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2352 bytes (CD-DA) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2328 bytes (CD-XA) |  |  |  |  |  |  |  |  |  |  |
|  | Rotational Speed |  |  | 5200 rpm |  |  |  |  |  |  |  |  |  |  |
|  | Diameter |  |  | 12 cm, 8 cm |  |  |  |  |  |  |  |  |  |  |
|  | Center Hole |  |  | N/A |  |  |  |  |  |  |  |  |  |  |
|  | Thickness |  |  | 1.2 mm |  |  |  |  |  |  |  |  |  |  |
|  | Track Pitch |  |  | 1.6 µm |  |  |  |  |  |  |  |  |  |  |
|  | Interface |  |  | IDE |  |  |  |  |  |  |  |  |  |  |
|  | Access Time (typical) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Random |  |  | < 100 ms |  |  |  |  |  |  |  |  |  |  |
|  | Full Stroke |  |  | < 200 ms |  |  |  |  |  |  |  |  |  |  |
|  | Audio Output Level |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Line Out |  |  | 700 MV (.7 volts RMS) |  |  |  |  |  |  |  |  |  |  |
|  | Headphone |  |  | 0.6 VRMS |  |  |  |  |  |  |  |  |  |  |
|  | Data Transfer Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sustained |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Burst |  |  | 3.6 MB/s (variable) |  |  |  |  |  |  |  |  |  |  |
|  | Cache Buffer |  |  | 128 KB |  |  |  |  |  |  |  |  |  |  |
|  | Startup Time |  |  | Less than 7 seconds |  |  |  |  |  |  |  |  |  |  |
|  | Stop Time |  |  | Less than 7 seconds |  |  |  |  |  |  |  |  |  |  |
|  | Laser Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Type |  |  | Semiconductor Laser GaAIAs |  |  |  |  |  |  |  |  |  |  |
|  | Wave Length |  |  | 780 +/- 25 nm |  |  |  |  |  |  |  |  |  |  |
|  | Divergence Angle |  |  | 53.5° +/- 1.5° |  |  |  |  |  |  |  |  |  |  |
|  | Output Power |  |  | 0.14 mW |  |  |  |  |  |  |  |  |  |  |
|  | Operating Conditions Temperature |  |  | 41-113° F/5-45° C |  |  |  |  |  |  |  |  |  |  |
|  | Humidity |  |  | 10-80% |  |  |  |  |  |  |  |  |  |  |
|  | Dimensions (HxWxD) |  |  | 1.69 x 5.91 x 8.2 in/4.29 x 15.01 x 20.8 cm |  |  |  |  |  |  |  |  |  |  |
|  | Weight |  |  | 2.65 lbs/1.2 Kg |  |  |  |  |  |  |  |  |  |  |
| Compaq Netelligent 10/100 TX PCI UTP Controller (embedded) | Network Interface Expansion Bus Architecture Network Transfer Rate |  |  | 10/100 Base TX 32-Bit PCI bus-master |  |  |  |  |  |  |  |  |  |  |
|  | 10Base-T (Half-Duplex) |  |  | 10 Mb/s |  |  |  |  |  |  |  |  |  |  |
|  | 10Base-T (Full-Duplex) |  |  | 20 Mb/s |  |  |  |  |  |  |  |  |  |  |
|  | 100Base-TX (Half-Duplex) |  |  | 100 Mb/s |  |  |  |  |  |  |  |  |  |  |
|  | 100Base-TX (Full-Duplex) |  |  | 200 Mb/s |  |  |  |  |  |  |  |  |  |  |
|  | Connectors |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ethernet |  |  | RJ-45 |  |  |  |  |  |  |  |  |  |  |
|  | Standards Compliance |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ethernet |  |  | IEEE 802.3 |  |  |  |  |  |  |  |  |  |  |
|  | 100 Mb/s Ethernet |  |  | IEEE 802.3 |  |  |  |  |  |  |  |  |  |  |
|  | I/O Address |  |  | Auto-configured |  |  |  |  |  |  |  |  |  |  |
|  | Environmental Requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Operating |  |  | 10° to 35°C; 10% to 90% humidity |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | (noncondensing); 3048 m altitude |  |  |  |  |  |  |  |  |  |  |
|  | Nonoperating |  |  | 10° to 50°C; 5% to 90% humidity |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | (noncondensing); 9144 altitude |  |  |  |  |  |  |  |  |  |  |
|  | Shipping |  |  | 30° to 50°C; 20% to 80% humidity |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | (noncondensing); 3084 m altitude |  |  |  |  |  |  |  |  |  |  |
|  | Communication Processor |  |  | Texas Instruments ThunderLAN 3.1 |  |  |  |  |  |  |  |  |  |  |
|  | OS Driver Support |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | MS Windows NT Server and Workstation v. 3.51 |  |  | NDIS 3.0 Compliant |  |  |  |  |  |  |  |  |  |  |
|  | MS Windows NT Workstation 4.0 |  |  | NDIS 4.0 Compliant |  |  |  |  |  |  |  |  |  |  |
|  | MS Windows 95 (OSRO) |  |  | NDIS 3.0 Compliant |  |  |  |  |  |  |  |  |  |  |
|  | MS Windows 95 (OSR2) |  |  | NDIS 4.0 Compliant |  |  |  |  |  |  |  |  |  |  |
|  | WFW v.3.11 |  |  | NDIS 3.0 Compliant |  |  |  |  |  |  |  |  |  |  |
|  | MS LAN Manager |  |  | NDIS 2.0 Compliant |  |  |  |  |  |  |  |  |  |  |
|  | Novell NetWare Server v. 3.x, 4.x |  |  | ODI Compliant |  |  |  |  |  |  |  |  |  |  |
|  | Novell NetWare DOS |  |  | ODI Compliant |  |  |  |  |  |  |  |  |  |  |
|  | IBM OS/2 v. 3.x, 4.x, WARP, SMP | OS/2 NDIS 2.0 Compliant |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IBM OS/2 NetWare Requestor v.2.x, 3.x | ODI compliant |  |  |  |  |  |  |  |  |  |  |  |  |
| S3 ViRGE/GX Enhanced |  | 2-MB SGRAM | 4-MB SGRAM |  |  |  |  |  |  |  |  |  |  |  |
| 64-bit Graphics with 3D Acceleration (PCI) | Resolution | Colors Supported | Refresh Rate |  | Colors Supported |  | Refresh Rate |  |  |  |  |  |  |  |
|  | 640 x 480 | 16.7M (32 bpp) | 85 Hz |  | 16.7M (32 bpp) |  | 85 Hz |  |  |  |  |  |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 800 x 600 | 65K (32 bpp) |  |  | 85 Hz |  |  |  |  | 16.7M (32 bpp) |  |  |  |  |  |
| 85 Hz85 Hz | 1024 x 768 |  |  | 65K (16 bpp) |  |  |  |  | 85 Hz |  |  |  |  |  |

16.7M (24 bpp)1280 x 1024256 (8 bpp)85 Hz65K (16 bpp)85 HzAcceleration3D GUI, 170 MHz RAMDACMemory2 MB SGRAM, 75 MHz standard
(2 MB SGRAM Upgrade to total 4 MB)

| Matrox MGA 1064/SG Performance Graphics | Resolution | 2-MB SGRAM Colors Supported | 4-MB SGRAM Colors Supported | Refresh Rate |
| --- | --- | --- | --- | --- |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |
| 640 x 480 800 x 600 1024 x 768 1280 x 1024 1600 x 1200 | 256, 65K, 16.7M 256, 65K 256, 65K 256 256 |  | 16.7M 16.7M 16.7M 16.7M 65K |  |
| 85 Hz 85 Hz 85 Hz 85 Hz 75 Hz | Acceleration Memory |  |  |  |
| 2D GUI, 220 MHz RAMDAC 2-MB SGRAM standard with 2-MB SGRAM option available (2 + 2 MB total) |  |  |  |  |
|  | 1Data transfer rate will vary from 150 KB/s to 2400 KB/s. 2Data transfer rate will vary from 150 KB/s to 3600 KB/s. |  |  |  |

Compaq, Compaq Insight Manager, Deskpro, registered United States Patent and Trademark Office. Netelligent, SmartQ, MultiLock, QuickBlank, QuickLock, are trademarks and /or service marks of Compaq Computer Corporation.

Novell and NetWare are registered trademarks of Novell, Inc. Microsoft, Windows, Windows NT and SQL Server are trademarks of Microsoft Corporation. IBM and OS/2 are registered trademarks of IBM Corporation. SCO is a registered trademark of the Santa Cruz Operation, Inc. UNIX is a registered trademark of the Open Group in the US and other countries, Inc. ORACLE is a registered trademark and ORACLE7 is a trademark of Oracle Corporation. Cheyenne and ARCserve are registered trademarks of Cheyenne Software, Inc. JETserve is a trademark of Cheyenne Software, Inc.

Other product names mentioned herein may be trademarks and/or registered trademarks of their respective companies.

©1998 Compaq Computer Corporation. All rights reserved. Printed in the U.S.A.