| System Unit | Dimensions (HxWxD) | 3.56 x 12.50 x 14.6 in/9.00 x 31.80 x 37.10 cm |  |  |
| --- | --- | --- | --- | --- |
|  | Weight (approximate) | 20 lb/9.08 kg |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |
| Power Supply |  |  |  |  |
|  | Operating Voltage Range | 90 - 132 VAC/180 - 264 VAC |  |  |
|  | Rated Voltage Range | 100 - 120 VAC/220 - 240 VAC |  |  |
|  | Rated Line Frequency | 50 - 60 Hz |  |  |
|  | Rated Input Current | 5A/3A |  |  |
|  | Maximum Rated Power | 76 W |  |  |
|  | Temperature |  |  |  |
|  | Operating | 50° to 95°F/10° to 35°C |  |  |
|  | Nonoperating | -4° to 140°F/-20° to 60°C |  |  |
|  | Humidity (noncondensing) |  |  |  |
|  | Operating | 8% to 90% |  |  |
|  | Nonoperating | 5% to 95% |  |  |
|  | Maximum Altitude (unpressurized) |  |  |  |
|  | Operating | 10,000 ft/3048 m |  |  |
|  | Nonoperating | 30,000 ft/9144 m |  |  |
| EIDE Hard Drive |  | 1.6-GB | 2.1-GB |  |
|  | Formatted Capacity | 1.6 GB | 2.1 GB |  |
|  | Drive Type | 65 (soft) | 65 (soft) |  |
|  | Transfer Rate |  |  |  |
|  | Media | 94.0 Mb/s | 27.2 - 55 Mb/s |  |
|  | Interface | 16.7 MB/s | 16.6 MB/s |  |
|  | Seek Time (typical including settling) |  |  |  |
|  | Single Track | 2.0 ms | 2.0 ms |  |
|  | Average | < 11 ms | < 12 ms |  |
|  | Full Stroke | 250 ms | < 20 ms |  |
|  | Disk Rotational Speed | 5400 rpm | 4500 rpm |  |
|  | Cylinders |  |  |  |
|  | Physical | 7066 | 5691 |  |
|  | Logical | 4708 | 4092 |  |
|  | Data Heads | 16 | 16 |  |
|  | Sectors per Track |  |  |  |
|  | Physical | 160-273 | 114-234 |  |
|  | Logical | 63 | 63 |  |
|  | Buffer Size | 256 K |  | 128 K |
| Compaq Netelligent 10/100 TX Embedded UTP Controller | Network Interface Expansion Bus Architecture Network Transfer Rate | 10/100 Base TX 32-Bit PCI bus-master |  |  |
|  | 10Base-T (Half-Duplex) | 10 Mb/s |  |  |
|  | 10Base-T (Full-Duplex) | 20 Mb/s |  |  |
|  | 100Base-TX (Half-Duplex) | 100 Mb/s |  |  |
|  | 100Base-TX (Full-Duplex) | 200 Mb/s |  |  |
|  | Connectors |  |  |  |

EthernetRJ-45 and AUI

| Compaq Netelligent | Standards Compliance |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 10/100 TX Embedded | Ethernet |  | IEEE 802.3 |  |  |
| UTP Controller (cont’d) | 100 Mb/s Ethernet |  | IEEE 802.3 |  |  |
|  | I/O Address |  | Auto-configured |  |  |
|  | Environmental Requirements |  |  |  |  |
|  | Operating |  | 10° to 35°C; 10% to 90% humidity |  |  |
|  |  |  | (noncondensing); 3048 m altitude |  |  |
|  | Nonoperating |  | 10° to 50°C; 5% to 90% humidity |  |  |
|  |  |  | (noncondensing); 9144 altitude |  |  |
|  | Shipping |  | 30° to 50°C; 20% to 80% humidity |  |  |
|  |  |  | (noncondensing); 3084 m altitude |  |  |
|  | Communication Processor |  | Texas Instruments ThunderLAN 3.1 |  |  |
|  | OS Driver Support |  |  |  |  |
|  | MS Windows NT Server and Workstation v. 3.51 |  | NDIS 3.0 Compliant |  |  |
|  | MS Windows NT Workstation 4.0 |  | NDIS 4.0 Compliant |  |  |
|  | MS Windows 95 (OSRO) |  | NDIS 3.0 Compliant |  |  |
|  | MS Windows 95 (OSR2) |  | NDIS 4.0 Compliant |  |  |
|  | WFW v.3.11 |  | NDIS 3.0 Compliant |  |  |
|  | MS LAN Manager |  | NDIS 2.0 Compliant |  |  |
|  | Novell NetWare Server v. 3.x, 4.x |  | ODI Compliant |  |  |
|  | Novell NetWare DOS |  | ODI Compliant |  |  |
|  | IBM OS/2 v. 3.x, 4.x, WARP, SMP |  | OS/2 NDIS 2.0 Compliant |  |  |
|  | IBM OS/2 NetWare Requestor v.2.x, 3.x |  | ODI compliant |  |  |
| S3 Trio 64V2/GX Integrated PCI | Resolution |  | 2-MB SGRAM Colors Supported |  | Refresh Rate |
| Grap . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . hics Controller |  |  |  |  |  |
| 640 x 480 | 16.7M (32 bpp) |  | 85 Hz |  |  |
|  | 800 x 600 |  | 16.7M (32 bpp) |  | 85 Hz |
|  | 1024 x 768 |  | 64 K (16 bpp) |  | 85 Hz |
|  | 1280 x 1024 |  | 256 (8 bpp) |  |  |
| 75 Hz | Acceleration Memory | 2D GUI, 170 MHz RAMDAC 2-MB SG DRAM not upgradeable |  |  |  |

Compaq, Compaq Insight Manager, Deskpro, registered United States Patent and Trademark Office. Netelligent, SmartQ, MultiLock, QuickBlank, QuickLock, are trademarks and /or service marks of Compaq Computer Corporation.

Novell and NetWare are registered trademarks of Novell, Inc. Microsoft, Windows, Windows NT and SQL Server are trademarks of Microsoft Corporation. IBM and OS/2 are registered trademarks of IBM Corporation. SCO is a registered trademark of the Santa Cruz Operation, Inc. UNIX is a registered trademark of the Open Group in the US and other countries, Inc. ORACLE is a registered trademark and ORACLE7 is a trademark of Oracle Corporation. Cheyenne and ARCserve are registered trademarks of Cheyenne Software, Inc. JETserve is a trademark of Cheyenne Software, Inc.

Other product names mentioned herein may be trademarks and/or registered trademarks of their respective companies.

©1997 Compaq Computer Corporation. All rights reserved. Printed in the U.S.A.