Section 0	:	Details of this document

Chapter 1	:Introduction

(1.1)  :	How the problem arose

This end product of this project is a generalised microcode simulation system. The simulation engine provides a representation of the internal workings of a "user defined" microprocessor. Microcode is the internal controlling software used by the microprocessor that decodes assembler level instructions and activates the various processing units. See Chapter 2 for further details.

The problem arose because of the need to convey the internal workings of a microprocessor to students studying relevant first and second year modules at the University of Wolverhampton. In addition to this there was a need for a generalised simulator that could accept differing microprocessor architectures, in essence meaning that the microprocessor's architecture can be defined by the user.

(1.2)  :	Aims and objectives

The major aims of the completed system would at the most simple level allow the user to:

a) 	View the microcode processing required for certain macroinstructions (assembler level instructions),

and

b)	View the effect on the processor as a whole, via a ("standard") graphical representation of the chosen

microprocessor.

And at a more complex level:

c)	Experiment by creating new microarchitectures of his own, within restrictions see chapter 5,

and observing the results via the means described above.

d)	Experiment by changing existing microarchitectures' microinstructions (the instructions which make up 		the microcode program) and structure and observing the advantages and disadvantages, and overall 		effects of these alterations.

Thus the simulation system contains features useful for users with differing levels of experience in design and operation of microprocessors.

The major academic aim of the project, with respect to microprocessor architecture, lies within the features available at the "complex level" within the generalised microcode engine. For this level, and to a lesser extent the "simple level", to succeed the end product needs to contain a virtual microprocessor, with software models of certain elements common to all microprocessors, most notably the ALU (Arithmetic Logic Unit which performs the arithmetic and logic functions of the microprocessor. See sub section 8.4). These internal workings of the microprocessor need to therefore need to expressed in a detailed as possible manner to allow for accurate simulation.

(1.3)  :	Intended approach

This sub section briefly describes the path that will be taken leading to the microcode simulator.

The research that is required to be carried out falls into the following categories. Firstly research the overall design of microprocessor archtectures need to be carried out, with emphasis on simple 8 bit models such as the Zilog Z80 (see chapter 10). Design and operation of ALUs will be important here.

Following this, the use and various examples of microcode will need to be researched. Various existing microcode simulators need to be looked at, as do the microprogrammable WISC (Writable Instruction Set Chips, which allow their microcode to be user defined, see Chapter 4) based machines. Other issues regarding microcode include the use of microcode within RISC (Reduced Instruction Set Chip. See sub section 2.3) chips, microprocessors which combine both microcode and hardwired parsing methods (to a large extent), and other areas which have relevance to this project.

In addition to research of microprocessor architectures, "research" into the Object Oriented Borland Turbo Vision libraries needs to be carried out, as this is the chosen user interface for the microcode simulator.

Armed with this research, the Object Oriented design of the microcode simulator will take place in earnest. The first part of the system to be designed will be the virtual processor, then the macroinstruction parsing, and then fitting in the processing units into this virtual processor. The design notation / paradigm used is that used by the Select OMT CASE Tool (although the tool was not used to produce code outlines) with pseudo English used for the system's main algorithms. The chosen language of implementation is Borland C++.

As each of the above elements of the simulator are implemented they will be tested via a test harness for each of the relevant classes. A system wide testing programme will take place when the system is completed.

Of course, the timing  of some elements of research and design / implementation will overlap. The proposed and actual time plan are in Appendix A.

(1.4)  :  Overview of the framework of the report

The report is split up into five sections, each with a number of chapters and sub sections. Section 1 and Section 2 include the survey / literature review, theoretical discussion, and description and design of the design of the microcode simulator.  Section 3 deals specifically with the main example processor which was used to demonstrate the functionality of the simulator, the Zilog Z80. Section 4 is the critical evaluation, and Section 5 contains the conclusions reached. Following these sections are the required References, Bibliography and Appendices.

A description of each section and its chapters follows.

Section 1  :  Introduction to microcode

This section provides background material to the rest of the report in that it introduces the reader to microcode, its function in a microprocessor, and then looks at how it is used in past and contemporary microprocessors. This section contains some of the details of the survey / literature review. It should be emphasised that the information in this section is primarily an introduction to the description of the microcode simulator in Section 2.

Chapter 2  :  What is microcode ?

This is a chapter whose purpose is to give an introduction to microcode. It gives a definition to the term microcode and describes its function in microprocessors to provide background material for the remainder of the report.

In addition this chapter contains the much of the results and findings of the research carried out, and provides a theoretical basis for the discussion in Section 2. Details of research into WISC systems and other microcode simulators, which are directly relevant to the actual design of the microcode simulator, are contained in Section 2.

So this chapter gives an overview of the use of microcode in current processors, introduces the concept of hardwired processors and compares them to the microcoded approach, looks at processors that make use of both hardwired and microcoded techniques and looks at the use and spread of microcode in RISC machines. The chapter concludes with a review of the spread and success of microcoded processors.

Section 2  :  Implementation of the microcode simulator

The chapters in this section discuss the design of the generalised microcode simulation system.

Chapter  3:  Background to the microcode simulation system

The purpose of this chapter is to give an overview of the issues that need to be considered when developing a microcode simulation system. It discusses microprocessor simulation in general, microcode simulation, level of user definition, and user interface considerations.

Chapter  4:  Microprogrammable chips and alternative microcode simulators

The purpose of this chapter is to describe WISC and various previous software approaches to microcode simulation to provide a base for this new microcode simulation system to build upon. It contains details of research into WISC chips and various previous software approaches to microcode simulation.

Chapter  5:  Outline of the design of the system

The purpose of this chapter is to give the microcode simulation system a specification and overview of its major points. It will describe the different engines used and how these are integrated into the system's object oriented approach. Further it will describe the interaction between the major objects within the system.

Chapter  6:  The user interface of the system

This chapter describes the way in which the microcode simulator interacts with the user. It describes the major Borland Turbo Vision and Borland Graphics Interface features used, and how these are used for assembler code programs and microprocessor configuration files.

Chapter  7  Preconditioning program code for use by the microcode engine

Before a macroinstruction is passed to the simulator, it must be preconditioned. The purpose of this chapter is to explain how the system achieves this. The chapter describes how the simulator "assembles" programs, it describes the structure of the macroinstruction database, and explains how the pre processor distinguishes between different addressing modes within macroinstructions.

Chapter  8:  The generalised microcode engine

The purpose of this chapter is to fully describe the workings of the microcode engine. Briefly the chapter gives a description of the parts of microprocessor emulated, details of registers, description of the user customisable features of the processor, details of the instruction decoder, description of the ALU, details of how the engine handles instruction flow and the microinstruction database.

Chapter  9:  The graphics engine

The purpose of this chapter is to describe the graphics engine, which is an object that processes messages from the microcode engine described in the previous chapter, and converts them to graphic images.

Section  3 :  Simulation of specific real world processors

When developing the microcode simulator, the Zilog Z80 processor was chosen to be the major example configuration file used. This section devotes a chapter to the simulation processor, and a further section to the attempts at simulation of other microprocessors.

Chapter  10:  The Zilog Z80

As already mentioned in the previous paragraph, the purpose of this chapter is to describe and evaluate the success of simulating the Zilog Z80 processor. The chapter contains a very brief overview of the real world processor and then goes on to describe how closely the elements of the Z80 are able to be emulated by the virtual software processor, and it describes the instructions emulated via the example configuration file. To sum up the success of this simulation is evaluated. The possibilities for simulation of other processors are also discussed.

Section 4 :  Review of the microcode simulation system

This section is the main critical evaluation of the success of the project as a whole.

Chapter  11:  Evaluation of system

The purpose of this chapter is to evaluate the project. Both an evaluation of practices involved in the implementation of the system, and an evaluation of the microcode simulator are carried out.

Section  5 :  Conclusions

The purpose of this chapter is to give a summary of findings and comments on the project. Possible future extensions to the project are also described.

Section 1	:	Introduction to microcode

Chapter 2	:	What is microcode ?

(2.1)  :	Definition of microcode

A basic definition of microcode has already been given in the previous section. To reiterate, microcode is the controlling program of a microprocessor. This microcode contains processing techniques to decode the assembler level macroinstructions and then to send control sequences to the corresponding parts of the microprocessor, such as the ALU, and may be held internal (more commonly) or external to the main processor. The macroinstructions are composed of a set of microinstructions.

An alternative approach for internal microprocessor control is by hardwiring the decoding, where a logic circuit is used to decode the assembler instructions. Such processors are described as being of hardwired control. This section discusses and compares these two approaches and provides a literature review of the information researched on this subject. More details of the other main areas of research of the project are contained in Chapter 4.

(2.2)  :	Comparison between hardwired and microcoded processors

The IBM 360 machine, created in 1964, was probably the first major microcoded processor system and was built out of the need for flexibility. Hardwired control which was the norm at the time was recognised and accepted to be required because of speed, but this form of control had a much higher development overhead due to the lack of flexibility. It is not too difficult to see that the control of a processor whose chip is controlled by logic circuits is more difficult to maintain and upgrade than a chip that is controlled through a software program. Another important advantage of hardwired approaches is that they are much less complex in terms of implementation when used with non complex instructions like RISC, for example they take up far less decoding area of a microprocessor that uses microcoded control.

Koopman (1987) compares the importance of hardwired and microcoded control. The article first discusses the importance of hardwired control for fast single memory cycle operation of RISC (see sub section 2.3), but then goes on to say generally:

"But it is not clear whether hardwired control is an absolute requirement. Since a designer can

make a small amount of microcode memory extremely fast in relation to large amount of microcode

memory (while achieving a reasonable cost / performance trade off), there is no reason why a

microcoded processor cannot achieve single memory reference cycle operation for most

operations."

This statement suggests that as technology improves and microcoded memory access times and speed of processing increases, the speed advantages of hardwired systems decrease. It is interesting to note however that with contemporary processors such as the Motorola MC68040 and the Intel Pentium some speed critical processing functionality is of hardwired control, which shows an interesting hybrid approach where the flexibility of microcode is for the most retained with only the parts of the functionality that need to be hardwired are. Johnson (1987) provides an insight into such hybrid approaches.

Very importantly the general consensus is that hardwired control is necessary for RISC processors (see sub section 2.3).

(2.3)  :	The use of microcode in RISC microprocessors

RISC is an acronym for Reduced Instruction Set Chip. RISC first begin in 1975 at IBM, as Robinson (1987) describes:

"John Cook, an IBM fellow, was working with a team to make a very large telephone switching

system. Such a large system needed a fast controller. The team experimented along many lines,

including slashing the instruction set. Last the team considered using the controller itself as

a computer. The outgrowth of their efforts was the 801 mini computer built in 1971."

Defining exactly what a RISC chip must constitute is quite a contentious issue. Briefly:

o	Few instructions

o	Instructions are the same size and in a fixed format

o	Execution on a single cycle of a processor

o	Load / Store architecture (data is loaded into internal registers, manipulated, then stored)

It is perhaps important to remember that if a chip has few instructions it does not automatically qualify as a RISC chip.

As researched continued into RISC during the 1970's and 1980's it became clear that the factors described above resulted in a speed increase over CISC (Complex Instruction Set Chip. This is the norm in processor designs.) designs. A comparison between CISC and RISC, and the design of RISC processors is outside the scope of this report. However a design feature of RISC that is common to most processors of this type, and not common to CISC, is that hardwired techniques are the major control method that is used in most of RISC processors.

Koopman (1987) makes the following debatable statement:

"RISC machines must use hardwired control. The intent of using hardwired control is to allow for

fast single memory cycle operation of opcodes and (when combined with a very small instruction

set) reduce the amount of silicon area required for implementation on a single chip."

So RISC systems use hardwired control for optimum performance, and are able to use hardwired control because of the non complex nature of the required decoding logic (because of the small number of instructions and fixed instruction format and load / store architecture).

This is not to say that all RISC processors use hardwired control, or that the ones that do use hardwired control do not include some microcoded elements. The Intel Pentium chip has some RISC elements (for example, in its pipelined dual processing architecture). In the Pentium Intel defines some instructions as "simple instructions" which don't require microcode and are therefore hardwired. Ryan (1993) discusses this further. Taking this concept a stage further the Intel P6, AMD K5 and Cyrix M1 processors contain a RISC core with a microcoded instruction decoder to translate the i8086 format macroinstructions into the RISC instructions required by this core.

(2.4)  :	Review of the acceptance and success of microcoded processors

As mentioned in the previous sub sections of this chapter, microcoded control was first used in the mid 1960's out of the need for flexibility. As technology improved with regard to speed the spread of microcode increased and is widely used today in modern processors such as the Intel Pentium.

However the growing trend seems to be going to RISC processors which are in the most hardwired. The main reason for this hardwired approach in RISC is speed of decoding. The RISC paradigm allows for the hardwired approach because of the non complex nature of the decoding needed when compared to CISC (for example, RISC chips use fewer and less complex instructions). Where microcode is used in RISC it most often tends to be of a supplementary nature with the large slice of decoding logic being hardwired. These facts may suggest that for reasons of production costs and optimum speed that hardwired control will become more and more important in the near future as RISC becomes the norm in desktop machines. What could happen as implementation technology increases that, as happened with the introduction of microcode, hardwired implementation will have less and less of an advantage in speed of microcode implementation, which could make these microcoded systems more popular as time goes on.

Section 2	:	Implementation of microcode simulation

Chapter 3	:	Background to the microcode simulation system

(3.1)  :	An overview of microprocessor simulation in general

There are many microprocessor simulation packages that are available and these have been documented well. Basically these simulators fall into three main categories.

o	Educational use

o	Detailed simulation, for example those used by microchip designers

o	Simulation, used as a basis for simulating one machine's architecture on another

The microprocessor simulation system discussed in this report falls clearly into the educational category.

A feature common to most microprocessor simulators available is that they are very closely linked to a single architecture, for example there are simulators of varying degrees of complexity for most common processors. It is not difficult to realise that when simulating hardware at such a low level then it is mostly essential that simulators are for one single architecture as any generalisation would undermine the simulator's integrity as a viable means of simulation.

It is now useful to look at how people inexperienced in the field of microprocessors may use a microprocessor simulation package. The aim of such a package must be to teach these people the fundamentals of microprocessor operation and because of this they often use a trivial simulation of a "simple" 8 bit architecture such as the Motorola 6502 or Zilog Z80.

For all of the types of simulation the type of display of the internal processing of the microprocessor must also be considered, although this will most often be dependent on which one of the three pre mentioned uses (described at the beginning of this chapter) the simulator will be aimed at. It is not a trivial manner however, and is discussed more fully in chapter 9. Also for all types of simulator it would not be unreasonable to expect that it would be capable of running complete assembler programs with jumps as opposed to macroinstructions that run independently of other macroinstructions.

What the rest of this chapter will attempt to show is how a simple architecture may be designed by "user" (the "user" in this case most often being somebody with some microprocessor experience) defining the basic characteristics of this architecture, for example the number of registers and their sizes, together with a definition of the chosen microprocessor's instruction set by its microcode operations.

(3.2)  :	An overview of the issues involved in microcode simulation

So general microprocessor simulation has been introduced as has the generalised microcode engine. What needs to be better explained here is exactly how microcode can be emulated. Some ideas are discussed in this sub section.

As has been defined in chapter 1, microcode is the system software of a microprocessor. It controls the execution of instructions by fetching the instruction from memory, and then this instruction is decoded and processed by the microcode. It can be seen that all of this processing is a function of the microcode, therefore it is reasonable that this functionality is emulated at a microcode level rather than being hardwired into the program code of the simulator.

The former approach is not supported by the microcode simulator described in this report, and only the idea that the macroinstructions are broken down into microcode operations is supported. These microinstructions are processed by a processing engine. The functionality of parts of the ALU within the simulator is implemented at a level similar to a real world microprocessor. This is described in chapter 8.

There is also the format of both macroinstructions and microinstructions to be considered. Again it all depends on the level of abstraction that the simulator's engine is removed from the physical microprocessor. A complex and detailed non abstract engine would require that these instructions be fed into the simulator in raw data form, but the engine described in this report is not able to support this. Microoperations are sent to the virtual processor in a text format, such as for an addition command to the ALU:

b+1->a

Would mean add the contents of register 'b' to value 1 and place the result in register 'a'. Detailed description of the formats of instructions are in chapter 11.

It may be now worth considering some advantages and disadvantages of simulation at a microcode level. Perhaps the most important advantage is that it gives the user more and better information when simulating the processor because educational simulation tools that do not use microcode simulation, or do not show the microcode operations related to each macroinstruction, can quite often only convey the movement and processing of data within a microprocessor in a limited and possibly ambiguous manner without some highly complex and clever graphical display.

Williams & Stewart (1982) gives some notes when simulating at a microcode level., although the article was based around simulation at a more detailed level than this report describes. The most important of the paper's observations was the one that when simulating at a microcode level, better timing can be achieved because each macroinstruction is broken down to the children microinstructions. This works because in most (all) CISC designs not all macroinstructions take the same number of cycles to execute.

A list of useful articles on the subject of microcode simulation can be found in the bibliography of this report.

(3.3)  :	Considerations that need to be taken for a generalised simulation

An overview of microcode simulation in general has now been given. This sub section discusses the further areas that need to be considered for the design of a generalised microcode simulator.

As has been described in the previous sub sections of this chapter, the microcode simulator described in this report simulates the microcode microinstructions that constitute the assembler macroinstruction. How this can be achieved is by having two databases. One database of macroinstructions (which are defined by the user), and one database of microinstructions that are linked to a parent macroinstruction. When a macroinstruction is sent to the simulator's engine, the children microinstructions are accessed and then processed. While the format of the macroinstructions are open for customisation, the microinstructions are not however as these are required to be in the format expected by the simulator's engine.

Looking at the simulator as a whole and not just from the microcode simulation, a reduction of complexity needs to be made when creating a general architecture. Much emphasis has been made throughout this report of the virtual processor engine being "generalised", when it is not too difficult to see from a viable point of view this is a facade, as the operation of the ALU, in particular the fetch execute cycle, and microcode structure is fixed and is not open to customisation by the user. A very complex and detailed microcode simulation engine would be needed to facilitate such facilities as these (see sub section 4.2). So, some sacrifices need to be made. These "sacrifices" include such things as the page:segmentation addressing structure of the Intel 80x86 family of microprocessors.

The design of the simulator described in this report is fully detailed in chapter 7 through chapter 9.

(3.4)  :	User interface considerations

The issue of a functional user interface is one of great importance with most types of microprocessor simulation. As is described in Chapter 6 the type of user interface is dependent on the type of simulation system.

The considerations for the user interface for a microprocessor simulation that deals specifically with microcode simulation are not too different from those for all microprocessor simulation systems. The major point is that the microcode microoperations need to be shown as they are being executed, as a back up to some form of possibly animated graphical representation of the chosen microprocessor.

When dealing with a generalised engine the restrictions imposed on the processing engine are also imposed on the graphical display. For example if the processing engine has a fixed structure of two input registers feeding the ALU with one output register, then the graphical display must reflect this.

Chapter	 4	:	Microprogrammable chips and alternative microcode simulators

(4.1)  :	Introduction to WISC (Writable Instruction Set Chip) architectures

WISC is an acronym for Writable Instruction Set Chip, thus meaning that the instruction set, and sometimes the decoding logic, of the processor is user definable.

WISC is a concept that was researched mainly in the 1970's where the main concern was to improve flexibility in design of a processor and improve the ease of programmability from an macroassembler viewpoint. For example a WISC chip may be able to be programmed to accept Pascal statements as its native instructions, with the decoding logic providing the function similar to a programming language interpreter but with considerable speed advantages. Considering the age of much of the research, the size of memory required for such decoding may have been a deciding factor in limiting its growth. Another important factor for improvement in flexibility of design for the processor is cost. Testing and redesigning of a chip will be a costly factor, especially with errors which are found after the chips are in circulation, like the floating point problem with the Intel Pentium which was as a result of an error downloading control logic. More information on the subject of WISC is in the paper "Flexible High Level Microcode Generation" (Petzold, Rohrs) which discusses the design of the microcode on WISC processors.

Koopman (1987) in the article "The WISC Concept: A proposal for a writable instruction set computer" proposes the use of WISC in "modern" applications. The basis of this article, and indeed of the WISC argument, is that it should be possible for the user to be able to write high level statements in the microcode embedded in the processor.

From this discussion it can be seen that the major points of WISC are not one of evaluation and teaching as is the case with the simulation system described in this report but it is useful to discuss WISC as a method of implementing user definable instruction sets.

(4.2)  :	A review of existing software approaches to microcode simulation

An overview of various approaches to microprocessor simulation was given in chapter 2. This sub section describes various other approaches to simulation, mostly with specific reference to microcode simulation, that were found while researching. The approaches mentioned here concentrate on the simulation of a processor and not on the various issues regarding the design of a suitable user interface required for a teaching system, and there are also no solutions for a generalised engine. Therefore this makes it quite difficult to directly compare these approaches the solution given in this report. So what is discussed in this sub section are the particular features of the simulators that are of direct relevance to the simulator in this report.

Wilson P. (1988). "CPU Architectures: Inner loop of an instruction set simulator, a complex integer expression evaluator and matrix addition", Byte vol.13 no.5

This approach does not focus on simulation at microcode level but provides an insight into a design of a virtual processor (which has some features based on the Motorola MC68000 processor) and some ALU functionality.

The virtual processor accommodates for 1024 bytes of register memory and only contains the PC (program counter which points to the next executable instruction to be fetched from memory) and accumulator fixed format registers. The matrix addition capabilities are not of great relevance to the simulator described in this report.

This solution mentions about gaining performance by not using an intermediate accumulator (like the MC68000) but implements separate data and address registers for simplicity, as different types of operations need to be carried out on the different types of register (unlike the MC68000). Overall the simulator is quite simple and does not provide any solutions for feedback to the user, but does show how a simulator can be written to be small and efficient, which is rather the opposite of the simulator in this report.

Lane A. (1987). "Simulating a Microprocessor", Byte vol.12 no.9

Discusses a Simple Architecture Microprocessor (SAM) based on a subset of the Intel i8085, which has a similar architecture to the simulator in this report. The simulator is implemented in the Prolog language. The article gives an example of grouping register pairs where the simulator maintains storage space for each of the two registers, and a separate storage space for the combined registers. For example if the processor had registers B and C, there would be storage space allocated for B, C, and BC. When any of the three storage areas are modified this will result in modification of the other corresponding registers. This is the method used by the simulator in this report. The simulator also describes a similar mechanism for updating the flags register. As with the previous simulator the package is simple, and in common to all of the simulators discussed here there, is no provision for different processor architectures (indeed the simulator is rather closely tied with the i8085).

VanDerLinden F.W. (1978) "New Generation of Microsystem simulators", Microprocessing and Microprogramming vol 4. no.1

This paper discusses the problem that most microprocessor evaluators are provided by the chips' manufacturers, and provides a solution named "ISPC" which is a system that facilitates simulator development. This system therefore provides some form of generalisation, although it is at a rather complex level as opposed to simple editing of a configuration file as with the solution in this report. The ISPC generation system is much less abstract however. What should be remembered is that at the time that the paper was written, 1978, many microprocessors had a similar architecture to one another.

ISPC works at a machine code level rather than an assembler level, so the engine receives the instructions in the native numerical form of the target processor rather than in textual form, and has distinct fetch / decode / execute cycles. The paper sadly does not describe any simulation of arithmetic and logic operations, but does go into some depth of interrupt simulation which is an area which the simulator in this report makes no attempt to address.

To summarise, ISPC is (was) a system for highly trained engineers which provides a much closer simulation of the internal workings of a processor but is not of much use as a teaching aid which is the main reason for the simulator described in this report.

Williams M.H. & Stewart A.J. (1982). "Machine cycle simulator for a microprocessor", Microprocessing and Microprogramming vol.6 no.3

This paper describes a simulator, based on the Motorola MC68000, that works at a microinstruction level, and although the simulator is tied to a particular architecture, it shows some similarities to the simulator in this report. The paper discusses simulation at both a microinstruction and macroinstruction level, arguing for the former. The major advantage that the paper gives for simulation at a microinstruction level is for more realistic timing. This is true but with the simulator described in this report, the major advantage for simulation at a microinstruction level is that the macroinstructions are able to be user defined as this simulator is more of a teaching aid and less of a diagnostic tool.

There are distinct advantages for realistic timing however, the paper mentions:

"The first benefit to be obtained is that of realistic timings for the processes involved. This is

important in the study of the performance of an operating system since the timings enable one

to make accurate assessments of the proportion of time spent in the operating system as

opposed to that spent executing other programs..."

The above statement shows that this simulation is at a deeper level than the simulator in this paper, and this could be an area of expansion in the future for the simulator mentioned in this report.

Marczynski R.W, Pulczyn W.T. & Sochacki J.M. (1975).

"3SM Computer Simulation and some problems of microprogrammed computer simulation". Microprocessing and Microprogramming

Some elements of this system are similar to the ISPC system described in VanDerLinden (1978) which has been discussed earlier in this chapter. The paper describes a language called 3SM ( The Structure Simulation System for Microprogramming) for describing the simulated system. As with the ISPC system simulation is at a non abstract level, and emulates the following parts of a processor: all the internal storage devices, functional devices, and a main memory. The paper goes on to discuss some forms of automated documentation that are required for microcode simulation development which are outside of the scope of the simulator in this report.

In terms of evaluation features 3SM contains some features similar to the simulator in this report. One of these similarities is that 3SM can run in step (after each microinstruction) or continuous mode. 3SM goes into more depth however as it provides the user with the facility to stop the simulation and then alter the values of internal storage and main memory. As with most of the simulators described in this sub section, it is rather heavyweight and not suited to teaching naive users.

Chapter 5 	:	Outline of the design of the system

(5.1)  :	Introduction

The purpose of this chapter is to give the microcode simulation system a specification and overview of its major points. It will describe the different engines used and how these are integrated into the system's object oriented approach. Further it will describe the interaction between the major objects within the system.

The various areas of the system can be seen in the diagram below. These areas are discussed later in this chapter.

Diagram 5.1 : Elements of the simulation system

(5.2)  :	The Object Oriented model

From the information contained in the previous chapters of this report, the reader should now have a sufficient idea of the type of microprocessor evaluation attempted here, and the scope of the simulator's approach. The language that was chosen for implementation of the simulator was the Borland implementation of the C++ language (version 3.1 running under MS-DOS), and uses some concepts concurrent with the Object Oriented (OO) paradigm. A C based language is used because it is uses a mainstream syntax and therefore is able to be read and understood by many people who will read this report and study the simulator. There is also an advantage for developing projects with a number of source files in the Borland IDE (Integrated Development Environment) in terms of ease of compilation and maintainability. The OO language C++ was specifically chosen for its ability to bind together associated functions together with their state, and this neatly mapped on to the design of the various individual parts of the virtual processor.

For the purposes of this discussion of the design of the virtual processor simulator just a few principles of OO design need to be understood here.

Briefly an 'object' is an instance of a 'class'. A class is a structure the defines the data and functions that make up that object. For example, a system may include an abstract representation of a computer screen. The data ('state') could be a boolean variable representing the on / off switch and integer variables representing the value of brightness and contrast. The functions ('member functions' or 'methods') would be a function to switch the screen on / off, and functions to modify the brightness and contrast controls. When objects communicate with each other, they do so by sending 'messages' which are simply function calls.

The microcode simulator does not use OO methods at a sophisticated level, meaning such concepts as inheritance (passing of data types through a hierarchy) and strict adherence to the paradigm (for example standard imperative programming methods are still used, such as not all functions are members of a class, and the use of the standard ANSI C libraries) are not used. Therefore the level of description given in the above paragraph should be sufficient for an understanding of the following chapters.

Apart from the various objects related to the OO Borland Turbo Vision libraries used for the user interface objects, which need not be described in the main of this report (see the technical documentation), these are the main objects and groups of objects used within the simulation system.

(5.3)  :	The Processor Configuration File

This is the configuration file that contains the identity of the virtual microprocessor. Below is an outline of the design of the configuration file for a virtual processor. The diagram uses a form of JSD (Jackson Structured Design) notation which is hierachial and uses the "*" sign to mean "repetition of".

Diagram 5.2 : Processor Configuration File Structure

This is the format of a sample configuration file in the textual format that is processed by the simulator.

#===========================================================================

# Name of processor

#===========================================================================

!title

[Name of processor]

!end

#===========================================================================

#===========================================================================

# Some useful information about the processor

#===========================================================================

!description

[Some free format lines describing information about the processor]

[...]

!end

#===========================================================================

#===========================================================================

# Processor instructions

#===========================================================================

!instructions

# An instruction

size=size in bytes of instruction + operands

[Instruction name plus operands]

!program

[microcode operations that make up above macro instruction]

[...]

!end

[...]

!end

#===========================================================================

#===========================================================================

# Mapping of register names to virtual processor storage

# Format of record is array_offset:name:size

#

# A $ prefix specifies the accumulator

# A œ prefix specifies the temporary accumulator

# A @ prefix specifies the flags register

# A ! prefix specifies this is a compound register which is held in the

#   format:         array_offset:name:size:reg1_index:reg2_index

#===========================================================================

!mappings

[register number]:[register name]:[register size in bits]

[...]

!end

#===========================================================================

Firstly a line with '#' as the first non whitespace character is a comment and is ignored by the simulator. The file is split into a number of sections. A section is defined as follows.

!description of section

<body of section>

...

!end

All of the sections of the file are mandatory. These sections are:

Name. This is a single line text string defining the processor's name.

Description. This section is just an unformatted description of the processor over a user defined number of lines.

Instructions. This is the specification for the instruction set of the processor. A number of macroinstructions are

contained here and are further broken down to their microinstruction functionality. The size of the instruction in bytes is also specified here to allow for calculation of branch instructions within assembler programs. Parameters entered as part of the macroinstruction are passed through to the belonging microinstructions.

Mappings. This section defines all of the registers of the microprocessor. The internal index used by the engine, the name and size in bits of the register are held. The special registers: the accumulator, temporary accumulator and flags register are noted by a special prefix. Compound registers can be made up of two defined single registers.

(5.4)  :	The application object

The application object is the object that contains the general purpose functions required by the simulator, together with the pre processing required for preparing assembler programs together with the means for editing these programs, and configuration of the simulator.

(5.5)  :	The virtual processor and generalised microcode engine

The object which represents the virtual processor and generalised microcode engine is the processor object. This object contains a number of subclass objects and related objects.

(5.6)  :	The graphics engine

The object which receives messages from the application object to show the effects of the current state of the microprocessor object, is the graphics engine object.

Chapter	 6	:	The user interface of the system

This aim of this chapter is to provide an overview of the user interface considerations that needed to be taken when developing the microcode simulator.

(6.1)  :	User interface for the environment

As mentioned in chapter 5, the microcode simulator uses the Borland Turbo Vision OO library as the primary user interface. Turbo Vision is used as the main program shell for accessing required sections of the simulator, and for editing of both the microprocessor configuration file and assembler programs.

(6.2)  :	Graphical representation of the engine

The Borland Graphics Interface (BGI) library was used to provide a graphical representation of the internals of the microprocessor. Some of the ideas of the required complexity of this graphical representation are discussed in chapter 6, and a detailed discussion of the internal workings of the graphics engine are discussed in chapter 9.

In brief, the graphical representation for the simulator is non too complex. The screen is mapped from a set of graphics primitives, so making it easier adjustable although the user does not have the facility to do this. The representation includes the following major parts as can be seen from the diagram.

Diagram 6.1 : The simulator's graphical representation of the virtual microprocessor

Registers. These simply show the register name and numerical contents.

Buses. Link the various components together.

The ALU. Contains two input registers and one output register.

Arrows. The function of these can be seen from the diagram above.

Chapter	 7	:	Preconditioning program code for use by the microcode engine

This chapter describes the stages that need to be gone through before the macroinstructions reach the microcode engine.

This, and chapters 8 and 9, are not meant to be a program design. C++ syntax code extracts showing data structures, are only used to highlight the steps used.

(7.1)  :	The structure of the macroinstruction database

The macroinstructions that define the microprocessor are included in the processor configuration file, and these need to be read into a memory area that is accessible by the virtual processor. Viewed from an abstract level this memory area would form part of the decoder of a real world microprocessor.

It is useful to show the format of instructions held in the configuration files here. This a load immediate instruction for a Z80.

# Load register with immediate data [2]

size=2

LD [1] [2]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

dbb->[1]

[2]->[1]

pc+1->pc

!end

This information is held in the format specified by the following C++ class definition.

// Instruction class defintion

class instruction{

// Data members

char name[STRSIZ]; 			// Name of the instruction. i.e. ADD

operation op[MAX_OPS]; 		// Microcode program for this instruction

int no_op; 				// Number of microcode operations for this instruction

int instType; 				// Type of instruction

int instSize; 				// Size of instruction in bytes

}

The data members which require further explanation are operation op[MAX_OPS] and int instType.

op is an array containing the microinstructions for the instruction. The structure of this array is simple.

// Microcode operation class definition

class operation{

// Data members

char line[STRSIZ]; // The operation as it is held in the configuration file

};

instType records if the instruction is of any special type, for instance an unconditional jump that will require specific processing by the microcode engine.

The instructions are held in an array of objects of the class definition instruction.

The virtual processor itself (see chapter 8) contains the register locations, but the details of the mappings between these locations and the user defined registers are held in a separate storage location. For example part of this mapping information that needs to be held for a Z80 would be what location in the virtual processor does register 'D' represent.

The details can be better explained by showing the mapping section of the configuration file whose design was given in chapter 5.

#===========================================================================

# Mapping of register names to virtual processor storage

# Format of record is array_offset:name:size

#

# A $ prefix specifies the accumulator

# A œ prefix specifies the temporary accumulator

# A @ prefix specifies the flags register

# A ! prefix specifies this is a compound register which is held in the

#   format:         array_offset:name:size:reg1_index:reg2_index

#===========================================================================

!mappings

1:$a:8

2:b:8

3:c:8

4:d:8

5:e:8

6:h:8

7:l:8

8:!bc:16:2:3

9:!de:16:4:5

10:!hl:16:6:7

11:œa':8

12:b':8

13:c':8

14:d':8

15:e':8

16:h':8

17:l':8

18:!bc':16:12:13

19:!de':16:14:15

20:!hl':16:16:17

21:@f:8

22:f':8

23:p:8

24:q:8

25:!pq:16:23:24

!end

This information is held in the format specified by the following C++ class definition.

// Mapping class definition

class map{

// Data members

int no; 				// No. of reg as used by engine

char name[REG_SIZ]; 		// Register name as used by user

int size; 			// Size of the register in bits

int map_reg; 			// Set on if register is made up of two others

int map1,map2; 			// The index of those two registers

};

A line of the mapping file maps on to a map object like:

no : name : size

If the name field has a '!' prefix specifying a compound register, then the map_reg, map1 and map2 fields are filled in accordingly. If the name field has any other prefix (except for no prefix at all) then this is a special register and the number of this register is held in the virtual processor object.

(7.2)  :	The structure of assembler programs and pre processing techniques

The assembler programs that the engine runs are in plain ASCII format and may be edited using the simulator's built in facilities or via an external text editor. What is discussed here are the techniques used to read these programs into the virtual microprocessor.

What should be stressed is that the program does not assemble in a traditional sense as the microcode engine receives instructions in their mnemonic format. The major task that needs to be done is the replacement of the parameter in branch instructions. For example, in the following assembler program:

JP main

:main

The main parameter needs to be replaced with the address that the main label resides at.

The assembler program is held in a single object of the following class definition.

// Class for lines of program code

class ProgramCode{

int line_no[PROG_SIZ]; 		// This is the "address"

char line_det[PROG_SIZ][80];		// Line details

int no_lines;				// Number of lines in program

int instJump[PROG_SIZ]; 		// If instType==JUMP then this is the line to jump to

};

The line_no field contains the "address" of the current line. This is incremented the number of times specified by instruction.instSize.

What needs to be achieved is to search for all branch instructions held in the array of instruction objects. When a branch instruction is found, the label (in the standard :label format) in the assembler program is searched for. When this is found the instJump field of the current line of the ProgramCode object is updated to be this line number. Thus the microprocessor engine will be able to set the program counter to this line number when it encounters a branch.

Chapter	 8	:	The generalised microcode engine

The purpose of this chapter is to fully describe the workings of the microcode engine. Briefly the chapter gives a description of the parts of microprocessor emulated, details of registers, description of the user customisable features of the processor, details of the instruction decoder, description of the ALU, details of how the engine handles instruction flow and the microinstruction database.

(8.1)  :	Overview of the parts of a microprocessor emulated

The relationship between the system objects that compose the virtual processor can be seen in the diagram. The circles represent an object. A line interconnecting objects represents a relationship (for example a macroinstruction contains many microinstructions).

Diagram 8.1 : The relationship between the objects in the simulation system

As can be seen from the diagram only the macroinstruction objects and microinstruction objects are linked. It would have been natural for both macroinstructions and microinstructions to be held within the virtual processor but this design was thwarted by implementation restrictions (Borland C++ allows objects of only a certain size). The assembler program entered by the user together with the mapping of registers defined in the processor configuration file are more naturally held external to the processor object.

The class definition of the virtual processor is as follows.

// Microprocessor class definition

class upro{

public:

// Data members

// Registers, storage areas

unsigned long pr[PR]; 				// Processor registers

unsigned long sp; 				// Stack pointer

unsigned long pc; 				// Program counter

unsigned long ir; 				// Instruction Register

unsigned long dbb; 				// Data bus buffer

unsigned long abb; 				// Address bus buffer

unsigned long cbb; 				// Control bus buffer

unsigned long mar; 				// Memory address register

unsigned long alutmp; 				// ALU temporary register

unsigned char pmemory[MEM_SIZE];		// External memory

// Registers, storage areas as strings used for holding binary values

char Spr[PR][32]; 				// Processor registers

char Sgp[GP][32]; 				// General purpose registers

char Ssp[32]; 					// Stack pointer

char Spc[32]; 					// Program counter

char Sir[32]; 					// Instruction Register

char Sdbb[32]; 					// Data bus buffer

char Sabb[32]; 					// Address bus buffer

char Scbb[32]; 					// Control bus buffer

char Smar[32]; 					// Memory address register

char Salutmp[32]; 				// ALU temporary register

// Flags register details

unsigned char 	carry,

subtract,

parity_overflow,

half_carry,

zero,

sign;

// Useful information

int no_pr; 					// Number of processor registers

int no_inst; 					// Number of instructions

int time_delay; 			 		// Milliseconds to pause for each instruction

unsigned int flag_ind; 				// Index of the flags register

unsigned int a_ind; 				// Index of the accumulator

unsigned int act_ind; 				// Index of the temporary accumulator

char runningInstruction[80]; 			// The instruction currently running

// Member functions

void gdisplay(char *,int); 			// Display of current state of the processor

void drawpro(); 					// Actual base drawing of processor

void process(char *,int);				// Process an operation

void run(int,int);					// Run a program produced by process()

void binof(int,char *,int);				// Denary to binary conversion

int testflag(int);					// Test that certain bit of flags register is set

void setflag(int,int);				// Set on/off certain bit of flags register

};

The above declaration defines the structure of the virtual processor. The pr[] array holds the general purpose registers of the processor which are defined by the processor configuration file (see sub section 5.3). So with a Z80 processor configuration file, pr[1] may hold the contents of the 'a' register,  pr[2] may hold the contents of the 'b' register, and so on. The following set of registers with the 'S' prefix are for holding the contents of registers in textual form for display. The textual form of register contents may be held in binary, octal, decimal or hexadecimal form. The processor's external memory is held in pmemory and is MEM_SIZE bytes in size;

Details of the class definitions for the register mappings, macroinstructions and microinstructions have been described in sub section (5.3).

(8.2)  :	The Arithmetic Logic Unit

The functionality of the ALU is embedded within the simulator's control structure, meaning there is no separate ALU object communicating with the processor object. When a microoperation is fed into the processor engine and the simulator finds an ALU command, it executes the required section of code. Description of the instruction decoding and execution sequence is described in sub section (8.3).

The processor's ALU performs the functions Add, Subtract, Compare, Call, Return, and logical operators.

(8.21)	:	Add and Subtract

It is perhaps outside of the scope of this document to discuss in detail the area of addition and subtraction in microprocessors. To summarise briefly the virtual processor uses the following format for its registers that are used for arithmetic operations (when talking about an 8 bit processor, of course the size of a register is configurable by the simulator).

Carry	Bit 7 (Sign Bit)  Bit 6   Bit 5   Bit 4   Bit 3   Bit 2   Bit 1    Bit 0

x	x	           x        x        x         x        x        x         x

Using this format the range of decimal numbers the register can hold is between -127 and +127 as bits 6 to bit 0 contain the number and bit 7 specifies the sign. If the sign bit = 1 then the number is negative, if it is = 0 then the number is positive. Where two positive decimal numbers are added if the result > +127 then this generates an overflow condition as the sign bit has been wrongly changed. The overflow condition is retained in the overflow bit of the flags register. Where the result is greater than can be represented within the 8 bits then the carry bit in the flags register is set on and retained.

The microprocessor does not do specific processing for subtracting numbers, to subtract numbers it uses the same method as with addition. To allow for this, the second parameter in the subtract command is conditioned using a tecnique called Two's Complement. This is performed by inverting all of the bits of the number and then adding '1'. The two numbers can now be subtracted using the normal addition manner, and similarly the overflow condition is set if two negative numbers are "added" and the result is < -127.

It should be noted that if two numbers of unlike signs are added / subtracted then overflow is not possible.

Quite often, a processor's ALU add / subtract functionality is implemented through simulation of a ripple carry adder. A diagram of this type of adder circuit is shown below.

Diagram 8.2 : A full adder circuit

(taken from Horvath.R. (1992) "Introduction to Microprocessors". Mc-Graw Hill. p51)

This circuit is composed of a set of full adder circuits, 8 in the above 8 bit adder shown above. The A and B lines specify the two binary arguments, S specifies the sum produced by this binary addition. The connection line between the full adders is the carry from the previous additions.

The test for overflow using such a circuit is as follows. Firstly it is useful to reiterate that overflow can only occur if two numbers of like signs are added together. Upon adding the two operands, if overflow occurs then the sign of the sum is different than the common sign of the two input numbers. For example, if two positive numbers are added, if overflow occurs then the sign bit of the sum will be 1. So we can test for overflow via comparison of these signs, but there is a simpler manner that is commonly used in microprocessors.

This test is that if the two most significant carries from addition of two positive numbers are 01, or the two most significant carries from addition of two negative numbers are 10, then overflow has occurred. So we can use the virtual processor's exclusive OR function to detect overflow. If the result is positive then overflow has occurred.

This can be seen in the following diagram.

Diagram 8.3 : Using Exclusive OR to detect the overflow condition

The format for addition in the microcode engine is:

[1]+[2]->[3]

Which means add [1] to [2] and place the result in location [3].

The format for subtraction in the microcode engine is:

[1]-[2]->[3]

Which means subtract [2] from [1] and place the result in location [3].

(8.22)	:	Compare

The virtual processor performs compare operations in a simple manner. Upon receipt of an ALU microoperation the processor uses the built in comparison techniques of the C++ compiler (for example the operator "== "which means "equal to", and the operator "!=" which means "not equal to) and then adjusts the flags register accordingly. This microinstruction adds sufficient functionality to the user defined macroinstruction to allow conditional jumps (where an address is jumped to depended on the result of previous operations) to be used.

The microinstruction for compare is similar to the macroinstruction (except the decoder will have converted the relevant parameters to numbers from addresses or register names). The format is:

cmp [1] [2]

The processor subtracts [2] from [1]. For an explanation on the results on the flags register see sub section (8.21) on addition / subtraction.

(8.23)	:	Call and Return

The implementation of call and return instructions is quite similar to that of a real world processor. Much of the functionality of implementing this mechanism has been described in Chapter 7 that describes the assembly of programs.

There are not call and return commands as such and implementation of this is left down to the user's definition of a correct macroinstruction in the processor configuration file. A call command is executed by storage of an address in the virtual processor's PC register and the current address being placed on the memory location specified by the current address held in SP. SP is subtracted by the variable defining its size. Here is an example JP command from a processor configuration file where this functionality can be seen:

JP [1]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

pc->sp

sp-1->sp

[1]->pc

!end

The implementation of a RET macroinstruction is just the opposite of the above procedure. The storage in the memory pointed to by SP is placed on the program counter, and the SP is incremented. A macroinstruction to do this could be:

RET

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

sp->pc

sp-1->sp

!end

(8.24) :	Logical functions

The logical functions which the simulator implements are: AND, OR, NOT, and EXCLUSIVE OR. They are all two input and one output. The general format of each of these operations (except NOT) is:

[1] <function> [2] -> [3]

where [1] and [2] are the two inputs and [3] holds the result.

AND			[1]/*[2]->[3]

OR			[1]/+[2]->[3]

NOT			/![1]->[2]

EXCLUSIVE OR	[1]/:[2]->[3]

(8.3)  :	The Instruction Decoder and execution of instructions.

This is the section of the virtual processor that receives the assembler macroinstructions and then performs the operation on this processor's state.

The flow of commands through the virtual processor can be summarised by the following diagram.

Diagram 8.4 : The flow of commands through the virtual microprocessor

The production of the preconditioned microinstruction is described in chapter 7, and graphics operations in chapter 9. What is described here is the functionality of the two member functions of the processor class,  decode() which sends microinstructions to execute() which executes these operations.

(8.31) :	Decode

The decode() function receives a macroinstruction, and must interrogate the instructions and operations objects (for the structure and descriptions of these see Chapter 7) to find the microinstructions that are associated to this macroinstruction. What it then must do is to replace the parameters held specified in the macroinstruction with the parameters specified in the microinstruction.

For example, with the register load operation from a Z80 processor configuration file:

LD [1] [2]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

dbb->[1]		(1)

[2]->[1]			(2)

pc+1->pc

!end

If the macroinstruction sent to display() was LD B,C then the macroinstructions (1) and (2) would need to be converted to dbb->B and C->B.

Another task which is performed is determining whether the parameters in the macroinstruction are registers, immediate data or addresses. To determine if the parameter is a register, the register map table is searched, and an address is specified by a '$' prefix.

Processing of these tasks is quite simple and not of direct relevance to processor design. A more full discussion is in the technical documentation.

(8.32) :	Execution

The execute() member function executes the set of microinstructions produced by decode(), providing the functionality of the ALU described in sub section (8.2).

The type of the microinstruction needs to be determined. All microcode operations are in the following format (see Chapter 7):

lhs -> rhs

Where lhs may be split into two parts separated by an operator, such as add.

Thus the execution logic of the simulator must first test if lhs is split into two parts. If it is then the operation must be performed on these two parts, and if any operands are registers then the value stored in these registers is used and the resultant value stored. If lhs is in only one part then this is the value stored. The success of these operations is dependent on the ALU operation, these operations being described in sub section (8.2).

The rhs parameter must always be a register location. If memory or an address is to be accessed then these values must be placed upon the  DBB (Data Bus Buffer) or MAR (memory address register) as appropriate.

As with the decoding logic, refer to the technical documentation for a description of the algorithm used.

Chapter	 9	:	The graphics engine

(9.1)  :	Description of the workings of the graphics engine

The operation of the graphics engine is quite simple. The graphics engine object simply receives one message from the virtual processor object: a message to redraw. The graphics object reads the internal state of the processor: the register contents, and displays the values accordingly. A display of the simulator's main evaluation screen is Diagram 6.2 in sub section 6.2.

The structure for the graphics engine class is as follows:

class graphics_engine{

// Data members

void far *last_scr[4]; // Pointer to the saved last screen

int last_active_page,last_visual_page; // The last of these pages (0||1)

// Member functions

// Member functions to draw ALU components

void drawarrow(int,int,int,int); // Draw arrow at position

void drawvbox(int,int,int,int); // Draw "V" ALU type box

void drawreg(int,int,char *,int); // Draw a register

void drawflag(int,int); // Draw flags register

// These are for the following shapes

void drawA(int,int,int); // Top left hand side corner

void drawB(int,int,int); // Horizontal

void drawC(int,int,int); // Top right hand side corner

void drawD(int,int,int); // Vertical

void drawE(int,int,int); // Bottom right hand side corner

void drawF(int,int,int); // Bottom left hand side corner

void drawG(int,int,int); // Top junction

void drawH(int,int,int); // Right junction

void drawI(int,int,int); // Bottom junction

void drawJ(int,int,int); // Left junction

};

The instance of the above class reacts from a gdisplay() message from the virtual processor object. When it receives this it starts to draw the screen from new. It receives this gdisplay() message after each cycle of the processor. When this message is received, the graphics object's draw_pro() member function is called, and this does the following processing.

Firstly the relevant drawing screen is established. The engine works by using two screens, a foreground and a background. Drawing takes place on one screen while the other screen is being displayed. The tecnique (often called double buffering) works by maintaining a two pointers, a pointer to the active screen ( last_active_page ) which is the screen for drawing on, and a pointer to the visual screen ( last_visual_page ) which is the screen currently being displayed. So firstly these two pointers are swapped and the drawing screen is cleared. Next all of the graphics objects are drawn on to the screen. When registers are drawn, the contents of these registers in the processor object is interrogated. These objects are displayed in the following sub section (9.2). When drawing has completed, the screen pointers are swapped again in preparation for the next screen redraw.

Graphics drawing is implemented using the Borland Graphics Interface library using the VGAMED screen mode (640 x 350 pixels).

(9.2)  :	Description of the various graphics objects

These graphics objects are the various elements which constitute the graphical representation of the processor. The initiating message and a graphical representation for each object is described below. Each of the objects are displayed on a grid which is 40 x 20 blocks in size with each block being 16 x 16 pixels in size. The grid system was used for flexibility making the design of the processor more easy to modify and allowing for different screen modes to be used with little effort. However only VGA 640 x 350 is used by the simulator. The colour of each object can be defined through the parameter list together with the x , y grid position. Some objects have various other configurable features.

Diagram 9.1 : The various graphics objects that the graphics engine can display

Section 3	:	Simulation of specific real world processors

Chapter 10	:	The Zilog Z80

(10.1)  :	A brief overview of the Zilog Z80 microprocessor

The real world microprocessor simulated by the simulator is the Zilog Z80. This section gives a very brief introduction to the Z80 for the purpose of understanding the remaining sub sections in this chapter. The simulation of the Z80 is non too complex, so this description will only highlight the major design features.

The Z80 is a processor with an 8 bit data bus and a 16 bit address bus, and contains 6 general purpose 8 bit registers. These registers: B, C, D, E, H and L are also grouped into register pairs: BC, DE, HL to allow for 16 bit storage.

The processor has a fixed accumulator where the ALU outputs to. The ALU has two input registers, TMP and ACT. The Z80 has an 8 bit flags register with 6 bits used by the processor. These bits are: SIGN, ZERO, HALF_CARRY, PARITY_OVERFLOW, SUBTRACT and CARRY.

The registers A, B, C, D, E and F all have an alternate set denoted by a ' suffix (for example, A', B' and so on).

The Z80 has 2 16 bit index registers, a 16 bit stack pointer (the processor uses external memory for stack storage), and a 16 bit program counter.

(10.2)  :	Simulation of the Z80 by the microcode simulator

The report so far has emphasised that the simulator described in this report is neither detailed or complex, therefore only a limited amount of instructions are simulated. The major elements missing from this simulation are described below.

There are no shift operations.

External storage space has been allocated has part of the virtual processor, but the ALU has no facilities for accessing this memory.

The instruction set is incomplete (see 10.3 following).

Only the carry and overflow bits of the flag register are affected by operations, although the implementation of the engine contains functions to interrogate and modify the entire range of bits. This restriction is in part due to the generalised nature of the engine as the structure and order of the bits of a flag register differs between different processor architectures.

The above description means only part implementation of a compare mechanism, and therefore non complete implementation of conditional jumps.

(10.3)  :	Z80 instructions emulated by the microcode simulator

This section gives a listing of the Z80 instructions simulated by the processor configuration file given as an example for the simulator described in this report.

When looking at the structure of the microinstructions for each macroinstruction, it is very important to realise that the FETCH section refers to fetching the operands from memory. For example when a load immediate LD [1] [2] instruction is decoded, the two fetch cycles refer to fetching of the two operands that constitute the remaining part of the instruction. This is opposed to fetching of the instruction opcode itself, or any references to external memory, which as mentioned in (10.2) is not supported by the engine.

Another feature of the macrooperations is that implicit operations are not supported. For example 'LD A 1' on a Z80 would contain the information 'load the second operand into register A' embedded in the opcode. This is understandable when considering the generalised nature of the simulator, and means that there are not many 'LD', or other similar cases, instructions in the configuration file.

Details on the structure of the processor configuration file is in sub section (5.3).

# Unconditional jump instruction to address

size=3

JP [1]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

[1]->pc

!end

# Unconditional jump instruction to register pair

size=2

JP [1]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

[1]->pc

!end

# Pop from stack to register

size=2

POP [1]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

sp->[1]

sp+1->sp

pc+1->pc

!end

# Push to stack

size=2

PUSH [1]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

sp-1->sp

[1]->sp

pc+1->pc

!end

# Return

size=1

RET

!program

EXECUTE

pc->mar

sp->pc

sp-1->sp

!end

# No operation

size=1

NOP

!program

EXECUTE

pc+1->pc

!end

# Compare

size=2

CP [1]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

[1]->tmp

a#tmp->a

pc+1->pc

!end

# Load immediate

size=2

LD [1] [2]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

dbb->[1]

[2]->[1]

pc+1->pc

!end

# Load from register

size=2

LD [1] [2]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

dbb->[1]

[2]->[1]

pc+1->pc

!end

# Add immediate

size=2

ADD [1] [2]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

[2]->tmp

[1]->a'

a'+tmp->a

pc+1->pc

!end

# Add from register

size=2

ADD [1] [2]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

[2]->tmp

[1]->a'

a'+tmp->a

pc+1->pc

!end

# Subtract immediate

size=2

SUB [1]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

[1]->tmp

a#tmp->a

pc+1->pc

!end

# Logical AND

size=2

AND [1]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

[1]->tmp

a/*tmp->a

pc+1->pc

!end

# Logical OR

size=2

OR [1]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

[1]->tmp

a/+tmp->a

pc+1->pc

!end

# Logical XOR

size=2

XOR [1]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

[1]->tmp

a/:tmp->a

pc+1->pc

!end

# Complement accumulator

size=2

CPL [1]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

[1]->tmp

/!tmp->a

pc+1->pc

!end

# Decrement register

size=2

DEC [1]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

[1]->tmp

tmp#1->a

pc+1->pc

!end

# Increment register

size=2

INC [1]

!program

FETCH

pc->mar

readmem dbb

dbb->ir

pc+1->pc

EXECUTE

pc->mar

readmem dbb

[1]->tmp

tmp+1->a

pc+1->pc

!end

(10.4)  :	Other processors

From reading the issues involved in generalised simulation (3.3) and the complexity of the simulator (chapter 5),  the main problem with emulation of different microprocessor architectures is that the design of the internal processing units of the virtual processor is, to a large extent, fixed. This means that emulation of very different microarchitectures is impossible at anything but a superficial level. For example, it is possible for the user to create a processor configuration file which contains register definitions and the (partial) instruction set of a MC68000, but the operation of the processor's internal units would be quite different.

Emulation of microarchitectures similar to the Z80 is possible however, for example the Intel i8086. The major trouble with implementing the i8086 would be the lack of a facility within the engine for the segment:offset addressing mechanism of such a chip.

Section 4	:	Review of the microcode simulation system

Chapter 11 	:	Evaluation of system

(11.1)  :	Evaluation of the practices involved in the implementation of the system

Overall I feel that I attempted the project reasonably well. I designed the system firstly getting the engine mechanics working, giving me time to develop the low level processor functionality such as ALU simulation, and to develop the graphics engine. I feel that I gave myself enough time and planned my work sufficiently well to allow for a reasonable completion of my aims and a workable end product. The design and subsequent implementation of the system is quite modular and hopefully is composed of  "good" C++ code.

There are however a number of factors that could have been improved when producing this project. An important factor was that at times I did not understand the aim of the problem fully, for example I did not realise the level of simulation that was needed with regard to ALU implementation, so this was completed rather later and less fully than I would have hoped for. This point also meant a deviation from my time scale meaning that additional research was required after it should have been completed. I also would have liked to begin work on the final report a bit sooner and spend more time on this report than I actually did. This was down to some bad planning and under estimating the scale of work required.

The system was not defined using rigid software engineering practices. The system was designed firstly and quite comprehensively (remembering the late addition of non abstract ALU simulation as described above), but there was a tendency to not to design every small aspect of the system before implementation. Also, while the system was developed in C++, I did not embrace the OO paradigm fully as it contained many procedural programming features, nor was any CASE design method used when such facilities were available.

(11.2)  :	Evaluation of the end product

When considering that the major aim of the project was to provide a generalised microcode simulation package as a teaching and educational aid, I feel that the end product is quite successful. The Borland Graphics Interface and Turbo Vision libraries provided the basis for a clean user interface that allows for multiple text editor windows and an unambiguous graphical representation of the processor. Within the time scale of the project I feel that an acceptable number of features have been incorporated into the system as a whole.

The scope of the research carried out for the project is adequate for the problem at hand, especially when considering the relatively small amount of material on microcode in text books, articles and papers.

The simulator is user configurable to the state that the user can define his own processor architectures. This is only correct with severe limitations. The user can only define the name, size, and number of registers within the processor, the instruction set, and can only define the series of microcode operations that make up the macroinstructions that compose this instruction set.

We can see from this that there are a number of features of a microprocessor's architecture that the user is not able to define. The major limitations are that the user is not able do define the workings of the microprocessor's internal processing units such as the ALU, for example the user cannot decide between a ripple carry or carry lookahead type of adder (these are differing methods of implementing the way a processor handles addition / subtraction. See chapter 8), the microprocessor is always of a fixed design in the manner in which the registers and processing units are arranged, for example there is always a memory address register (see chapter 8), and the format of the microinstructions are fixed.

When considering that the aim of the project is of  microcode simulation and not to produce an open ended and exhaustive microprocessor design simulation system, these discrepancies can hopefully be appreciated. It is not difficult to realise however that with these limitations, creation of RISC type processors, or other processors that are not too radically different from a Z80 type model, and successful evaluation of these processors would lead to some unsatisfactory emulations from the simulation engine. It is perhaps helpful to view the virtual processor as being similar to some WISC systems which allow the instruction set to be modified whilst the internal workings and hardware configuration is not allowed to be modified (However as already mentioned the software virtual processor has user definable general purpose registers).

There are further limitations to the front end of the system where the microcode engine sits. As the system stands the user is able to edit the microprocessor configuration file which contains the details of a processor's architecture, and edit an assembler program for the engine to run. The system allows the user to enter in individual macroinstructions directly into the microprocessor engine and to instantly see the results on the screen, or to send an assembler program to the engine. The graphics engine shows the important processing units of the system and their contents where relevant, and communication routes between these elements.

The most important limitation here is a side effect of the limitations of the microcode engine, which as already mentioned is limited in the number of ways it can be defined, so for the graphical display this means that the registers and internal units of the virtual processor will always be shown in the same way. For example, there will always be shown on the screen two registers acting as input to the ALU and one register acting as output from the ALU, even if the user's microprocessor architecture does not support this configuration.

Another limitation of the simulation environment is the lack of any benchmarking features. Other limitations are more pedantic, such as the lack of specialised editor for configuration files (the standard Borland Turbo Vision edit window is used), and no on line help.

Section 5	:	Conclusions

To conclude the report, it is felt that the completed microcode simulation and evaluation system is a worthwhile attempt at a teaching learning aid, that provides some help in learning the general operation of a microprocessor and fundamentals of machine code instructions. The limitations of the simulation package are not too great if it is treated mainly as a teaching aid, although there are severe restrictions when developing a complex simulation of a processor, especially those which do not conform to the processor architecture on which the virtual processor engine is based.

The graphical display and general ease of use of the interface are seen to be good points for educational systems. The environment provided Borland Turbo Vision library was useful as it allowed the user to open text editor windows to edit assembler programs and processor configuration files. The screen display of the internal representation of the virtual processor is functional, but not overly complex. This screen display should be able to convey the workings of the virtual processor engine at a reasonable level.

While researching for this project is was quite difficult to compare this work with others, as there was no microprocessor simulation package that conformed had the three features of:

It was user configurable

It was based on microinstructions

It was a teaching / educational package mostly aimed at naive users

Establishing links between this work and others was only possible on specific individual features. The information obtained from this research was of use when building specific elements of the simulator described in this report (such as the implementation of compound registers).

There is much that can be done to improve and expand the simulator, based on the limitations of the system mentioned in sub section (11.2).

The complexity of the microprocessor simulation aspect of the system could be improved, such as more addressing modes, more commands in the processor configuration files, more processors emulated by producing more of these files, and the possibility of more commands being able to be included in these files (the simulator is not able to simulate all types of instruction).

The system could be made more generalised, thus it is possible for the user to create more different types of processor, such as accurate simulation of RISC machines. This would require the user determining differing types of functionality within the processor such as number of integer units, pipelines, and so on. The graphics engine could be expanded to allow for the user to "paint" the screen, positioning the graphics object where he or she wishes.

More evaluation tools may be added to the system, such as cycle timing, allowing more than one processor to be running on the system at one time and debugging facilities such as with modern programming integrated development environments.

To summarise, the major problems with the simulator described in this report is that it does not provide a complex simulation of any single microprocessor, and the customisation features available make complex implementations of different microprocessor architectures impossible.

The structure of the simulator is such that it is possible to reasonably easily improve on the ALU facilities that are currently provided. It is hoped that this package will provide a useful teaching tool, and will be interesting and helpful for users to have some level of control over defining processor architectures of their choice.

References

Articles

Johnson T.L. (1987). "The RISC / CISC melting pot", Byte vol.12 no.4

Koopman P. (1987). "The WISC Concept: A proposal for a writable instruction set computer", Byte vol.12 no.4

Lane A. (1987). "Simulating a Microprocessor", Byte vol.12 no.9

Marczynski R.W, Pulczyn W.T. & Sochacki J.M. (1975). "3SM Computer Simulation and some problems of microprogrammed computer simulation". Microarchitecture of computer systems: Euromicro Workshop Nice June 1975

Robinson P. (1987). "How much of a RISC ?", Byte vol.12 no.4

Ryan B. (1993). "Inside the Pentium", Byte May 1993

VanDerLinden F.W. (1978) "New Generation of Microsystem simulators", Microprocessors and Microsystems vol 4. no.1

Wilson P. (1988). "CPU Architectures", Byte vol.13 no.5

Books

Horvath. R. (1992). "Introduction to Microprocessors". McGraw-Hill.

Hartenstein R. & Zaks R. (1976). "Microprocessing and microprogramming: Venice Symposium October 1976"

Hartenstein R. & Zaks R. (1975). "Microarchitecture of computer systems: Euromicro Workshop Nice June 1975"

Bibliography

Articles

Cadiou J.M, Armite R., Laberdene A. & Prevost D. (1975). "Implementation of a macroprocessor on a microprogrammable mini computer", Microarchitecture of computer systems: Euromicro Workshop Nice June 1975

This reference was useful for the description of alternative approaches to simulation packages.

Chroust G. (1976). "Parameters of microprogrammed interfaces",  Microprocessing and microprogramming: Venice Symposium October 1976

This reference was useful for the description of alternative approaches to simulation packages.

Dzilkowski J. & Rounce P. (1994). "Using databases to support the development of microcode", Microprocessors and Microsystems vol.18 no.9

This reference was useful for the description of microcode structure.

Edwards M.  (1977). "Microprogrammable microprocessors", Microprocessors and Microsystems vol.1. no.7

This reference was useful for the description of microcode structure.

Edwards M. & Dagless E. (1977) "LSI microprogrammable microprocessors", Microprocessors and Microsystems vol.1 no.3

This reference was useful for the description of alternative approaches to simulation packages.

Ivansi G. & Hubert B. (1975). "A microcomputer and its simulation system", Microarchitecture of computer systems: Euromicro Workshop Nice June 1975

This reference was useful for the description of alternative approaches to simulation packages.

Lane A. (1987). "Instruction Set Strategies", Byte vol.12 no.4

This reference was useful for the description of microcode and macroinstruction structure.

Marczynski R.W. & Tudris M.S. (1976). "Microprogrammed control units towards modularity in microprogramming", Microprocessing and microprogramming: Venice Symposium October 1976

This reference was useful for the description of alternative approaches to simulation packages.

Petzold R. & Rohrs H. (1975). "Flexible High Level Microcode Generation", Microarchitecture of computer systems: Euromicro Workshop Nice June 1975

This reference was useful for the description of alternative approaches to simulation packages.

Rietti G. & Tiriticco V. (1976). "An approach to a microprogramming language compilaton scheme,"  Microprocessing and microprogramming: Venice Symposium October 1976

This reference was useful for the description of alternative approaches to simulation packages, and microcode structure in general.

Vanreschi M. (1976). "Implementation of microprograms and processing speed",  Microprocessing and microprogramming: Venice Symposium October 1976

This reference was useful for the description of alternative approaches to simulation packages, and microcode structure in general.

Wendt S. (1976). "Models and Structures for Microprogramming",  Microprocessing and microprogramming: Venice Symposium October 1976

This reference was useful for the description of alternative approaches to simulation packages, and microcode structure in general.

Williams M.H. & Stewart A.J. (1982). "Machine cycle simulator for a microprocessor", Microprocessors and Microsystems vol.6 no.3

This reference was useful for the description of alternative approaches to simulation packages.

Books

Boyce J.C. (1979). "Microprocessor and Microcomputer Basics". Prentice-Hall

This reference was useful for the description of microprocessor architecture, especially ALU design.

Brunner H. (1982). "Introduction to Microprocessing"

This reference was useful for the description of microprocessor architecture, especially ALU design.

Carr J.J. (1980). "Z80 Users manual"

This reference was useful for the detailed description of the Zilog Z80 microprocessor.

Coffernon J.W. (1983). "Programming the 8086 / 8088"

This reference was useful for the detailed description of the Intel i8086 microprocessor.

Foster C.C. (1970). "Computer Architecture". Van Nostrand Reinhold Company

This reference was useful for the description of microprocessor architecture, especially ALU design.

Gorsline G.W. (1984). "16 Bit Modern Microcomputers: The Intel i8086 Family"

This reference was useful for the detailed description of the Intel i8086 microprocessor.

Hayes. J.P. (1988). "Computer Architecture and Organisation". McGraw-Hill.

This reference was useful for the description of microprocessor architecture, especially ALU design.

Hieserman D.L. (1979). "Microprocessor Instructions Sets and Software Principles"

This reference was useful for the description of microprocessor architecture, especially microcode design and functionality.

Khambuta A.J. (1976). "Microprocessors / Microcomputer Architecture, Software and Systems"

This reference was useful for the description of microprocessor architecture, especially ALU design.

Klingman E.E. (1977). "Microprocessor Systems Design: Vol.2. Microcoding, array logic, and architectural design"

This reference was useful for the description of microprocessor architecture, especially microcode design and functionality.

Rector.R. & Alexy G. (1980). "The 8086 Book". McGraw-Hill

This reference was useful for the detailed description of the Intel i8086 microprocessor.

Robinson P.R. (1985). "Mastering the 68000 Microprocessor". Tab Books

This reference was useful for the detailed description of the Motorola MC68000.

Short K.L. (1987). "Microprocessors and programmed logic"

This reference was useful for the description of microprocessor architecture, especially microcode design and functionality.

Zaks.R. (1979). "Programming the Z80", Sybek.

This reference was useful for the detailed description of the Zilog Z80 microprocessor.