RealMagic OS/2  Driver(PDD)  Beta Release  1.10

Date  : June 5,1995

The following Drivers and files are  included  in  the Beta Release 1.10

1. VIDRMS.SYS                    copy this into the C:|MMOS2

2. VIDVCI.DLL                     copy this into the C:\MMOS2\DLL

3. RMLBASE.SYS                 copy this into C:\OS2\BOOT

4. Add the following line in the CONFIG.SYS:

BASEDEV=RMLBASE   /io:260   /i:11  /p:340  /sa:0000 /ct:2

As you see the iobase address and interrupt number , the CD-ROM address , SCSI

Address, CD-ROM type can be set/modified as desired.

5. RMAUDIO.SYS        	copy this into the c:|MMOS2

6. DSP1.BIN and DSP2.BIN   copy these into c:|MMOS2\DSP

7. MPGDEV1.SYS                  copy this into c:\REALMAGC

Load this through  DOS-DEVICE  setting in DOS/WIN-OS2

session.

8. WINADOS.SET                  copy this to c:\spool

NOTE: C is the drive that OS/2 warp is installed at.

This Release(1.10) has the followings features and updates:

Native OS/2:

1. Following resolutions are supported:

640x480x256    or 16  colors   @ 60 and 56 refresh rates

800x600x256    or 16  colors   @ 60 and 56 refresh rates

1024x768x256  or 16  colors   @ 60 and 56 refresh rates

(Support for higher frequencies(Refresh Rates) have not been  provided by OS/2 Warp Graphic Engine

yet. Support for higher frequency could be added through. an application with a dialog box to get the

desired refresh rate  from the user. This could be done upon customers(Beta sites) request.)

2. Support for higher color depth is now possible.

This driver will support 16,256, and 64k colors for all the above resolutions.

3. Provided a Base driver (RMLBASE.SYS) for supporting  the CD-ROM interface.

Real Magic LITE:      NO    CD-ROM interface

Real Magic 1      :       Sony CD-ROM interface

Real Magic 2      :       Sony,Philips,IDE

Load the following device  driver in config.sys :

DEVBASE=RMLBASE.SYS /io:260 /i:11 /p:340 /sa:0000 /ct:2

/io:    io port address for realmagic card

/i:      interrupt value(even for SONY we need to have an interrupt set)

/p:     base address for Sony CD-ROM interface

/sa:    setting for SCSI address

/ct:    CD-ROM type

2   install  SONY

5   install IDE

6   Panasonic

4. Can Playback the encrypted MPEG files.

5. Added the support for RealMagic 2

6. Modify the RMAUDIO driver(the Audio PDD) to support the audio for the RealLMagic 2

7. Added the support to be able to load different DSP.bin files based on the different types of the

board.

REALMAGIC LITE    DSPLITE.BIN

REALMAGIC 1          DSP1.BIN

REALMAGIC 2          DSP2.BIN

Problems/Open issues:

1. Problem with stopping MPEG files after running the  files for few hours or more.

This  is a KNOWN BUG and is related to the audio timing which is handled by the stream handler in

the subsystem on the top of REALMAGIC PDD.  This is reported to IBM and they are working on

this  problem.

2. They do have the support for the Video-CD (White-Book) which works fine with MOST of the

CD-ROMS(TOSHIBA,IBM,MITSUMI)

The current SONY DRIVER (SONY31A.ADD) which is provided by IBM does not play VIDEO-CD.

This is been reported to IBM. Meanwhile, we need to make sure we have the latest driver and set the

interrupt jumper(even thought we do not use interrupt for Sony CD-ROM, the OS/2 Sony driver expect

the SONY drive to be set to one.)

3. OS/2 WARP does NOT have the support for CD-I movie(Green-Book file format)

4.  Digital Player error message MM-MCINOTIFY(MCI_PLAY)....

This is due to not being able to allocate memory. Go to DOS and run DEFRAG. This will fix the

problem.

5. Sound emulation is NOT supported  in the current  Native OS/2 driver.

6. The message regarding the RMAUDIO.SYS can not be loaded is due to the interrupt conflict.

Check the interrupt numbers(IRQ) , make sure there is not any conflict between the one

used by RMAUDIO and the one used by SoundBlaster. Remember the IRQ 15 which is the

default IRQ selected by OS/2 WARP setup for RMAUDIO does NOT work for realmagic 2.

For RealMagic 2 change the interrupt to other available IRQ.

WIN-OS/2 and DOS:

The Drivers we use in these sessions are the ones we are currently using under native DOS and

Windows 3.1.

1. Most MPEG files can be played with the proper settings under these two sessions.

2. The main problem in these sessions is due to the fact that OS/2 WARP does NOT handle the

interrupt properly under WIN-OS2/DOS. They Buffer the interrupts and sometimes(after 8-10

interrupt) they pause and then they burst few interrupts(1-4) to the driver at once which the driver

can not handle. This is one of  the cause of the stopping MPEG file. Some other companies also

reported this problem(interrupt latency) with their audio drivers.

This Problem is been reported to IBM.

Some of the problems we are experiencing are related to settings(Incorrect Settings) under these

two sessions.  I  mentioned all of the relevant settings with the proper values that I had to set in order

to get just the MPEG files running under WIN-OS2/DOS  in  OS2SPEC.DOC. Also I included a

file(WINADOS.SET) which could be loaded by LOAD command in DOS/WIN-OS2 session; This

file will set the proper values for the current session(It is like an installer). However, the WINaDOS

does not have the optimum/final values. We  need  to spend more time investigating this and going

through all the settings and try to customize it   for our particular application and /or system. For

example, I changed the size of my extended   memory from  48 to 412 and that  increased the

performance of  files running under CD-STATION a great deal  and reduced   the number of

interruptions(stopping).  I will recommend concentrating on ONE  DOS title and  readjusting the

settings,  especially  the size of the  Expanded  and Extended Memory. When we get one of them

working, there  is GOOD possibilities the  rest would work. Then we could work on making this

easier on the END-USER by providing an interactive  installer that helps them to set the proper

values.