PCOPY Version 1.01 Sep 29 1993    IBM Internal Use Only

By: Joseph H. Nord

Send files between two OS/2 machines using parallel port connection.

The program requires the LPTIO.SYS device driver be loaded.  For details

ask for verbose help by running "pcopy -help > pcopy.doc"

One machine acts as server, the other as client with files being sent

from the client PC to the server - to send files in the other direction,

run the server on the other machine.

Wildcards, long file names and extended attributes are supported.

Syntax:

pcopy                                    (no parms, server mode)

pcopy -switches <SourceFile> [DestDir]   (Send a file)

[switches]

-H, -?  Help

-HELP   Verbose help, includes cable requirements

Examples "pcopy"  (start server)

"pcopy c:\startup.cmd d:\" (send file startup.cmd)

----------------------------------------------------------------------

The program consists of two parts:

1) The device driver (LPTIO.SYS):

Provide read-write access to the parallel port.

OS/2 based printing remains supported, though printing is disabled

while pcopy is executing.

2) The application, PCOPY.exe, acts as both client and server.

The server watches the parallel port.  When requested, it performs

operations commanded by the client (notably file transfer).

When executed with parms (filenames), pcopy is the client and acts

to send files to the other PC.

----------------------------------------------------------------------

The device driver supports two popular "NULL" parallel cables.

- LapLink Brooklyn Bridge and DOS 6.x all use the same cable.

- JFACOMM and TWCOPY formats, popular inside IBM, are also supported.

The above cable types are identified to the device driver using a

parameter on the config.sys device= line.  For this identification,

the cable types are titled "LAPLINK" and "JFACOMM".

The LapLink cable type is the default.

LPTIO.sys does not support the JFACOMM "Full parallel" cable.

Syntax: (config.sys)

device=d:\path\lptio.sys [switches] [CableType]

[switches]

-T:num  Change timeout limit for read/write - Default, 3 seconds

[CableType] - "JFACOMM" or "LAPLINK"

Examples:

device=d:\path\lptio.sys             (Use Laplink, Booklyn bridge cable)

device=d:\path\lptio.sys JFACOMM     (Use IBM JFACOMM 1/2 cable)

device=d:\path\lptio.sys -t6 LAPLINK (LapLink cable, 6 second timeout)

The device driver uses parallel port LPT1.

----------------------------------------------------------------------

If you don't have a cable and want to build one, there are several

valid constructions.  The only requirement is that the correct pin

mappings occur between the two machines.

The complete construction calls for 25-pin to 25-pin D-Shell.

The required pin connections can be achieved via several methods.

The simplist is a single 25-pin to 25-pin cable which looks like a

normal printer cable, but has 25-pin parallel connection on both sides.

(LapLink cables have this configuration)

I prefer a modified centronics printer cable and an A-B printer switch box.

This approach allows switching between file transfer and printing with

minimal effort.

----------------------------------------------------------------------

For reference, here's the layout of a centronics

parallel printer cable (before modification):

25-pin   Printer side

1 ------- 1

2 ------- 2

3 ------- 3

4 ------- 4

5 ------- 5

6 ------- 6

7 ------- 7

8 ------- 8

9 ------- 9

10 ------- 10

11 ------- 11

12 ------- 12

13 ------- 13

14 ------- 14

15 ------- 32

16 ------- 31

17 ------- 36

18 ------- 19..30

19 ------- 19..30

20 ------- 19..30

21 ------- 19..30

22 ------- 19..30

23 ------- 33

24 ------- 19..30

25 ------- 19..30

(25-pin D shell pin assignments - PC parallel port pin assignments)

Pin I/O Name             Pin  I/O  Name

1  I/O -STROBE          14  O    -AUTO FD XT

2  I/O Data 0           15  I    -ERROR

3  I/O Data 1           16  O    -INIT

4  I/O Data 2           17  O    -SLCT (out)

5  I/O Data 3           18       Ground

6  I/O Data 4           19       Ground

7  I/O Data 5           20       Ground

8  I/O Data 6           21       Ground

9  I/O Data 7           22       Ground

10  I   -ACK             23       Ground

11  I   -BUSY            24       Ground

12  I   PE (paper end)   25       Ground

13  I   SLCT (in)

----------------------------------------------------------------------

LapLink cables are 25-pin to 25-pin direct cables.

The pin layout is listed below.  DOS 6.x, Fasylynx and

Brooklyn Bridge use this same format.

OneSide    OtherSide   (LapLink PC to PC pin cionnections)

2  ----->  15

3  ----->  13

4  ----->  12

5  ----->  10

6  ----->  11

15  <-----   2

13  <-----   3

12  <-----   4

10  <-----   5

11  <-----   6

25  ------  25  Ground

OneSide     OtherSide  (JFACOMM, TWCOPYII pin connections)

5  ----->  15

6  ----->  13

7  ----->  12

8  ----->  10

9  ----->  11

15  <-----   5

13  <-----   6

12  <-----   7

10  <-----   8

11  <-----   9

25  ------  25  Ground

If using a printer A-B switch, a standard centronics printer cable can be

modified by swapping 5 sets of wires.  This is usually easiest on the

25-pin side of the cable.  Modifications are below:

LapLink  2 - 15  (cut wires at pin 2 and pin 15, swap, solder)

3 - 13

4 - 12

5 - 10

6 - 11

JFACOMM  6 - 13

7 - 12

9 - 11

10 -  8

5 - 15

----------------------------------------------------------------------

To support cable testing and program debug, PCOPY includes a

test facility.  On the client machine, PCOPY is is executed

with the -TEST option.

The other machine is brought up as the server.

Examples

"PCOPY"          (server machine).

"PCOPY -TEST"    (client, user I/O machine)

You will be presented with a test menu.

Issue a 'O' (open) command to establish communication between the application

and the LPTIO.sys device driver.  If this fails, information is displayed.

Normally, opens only fail when the device driver hasn't been loaded or some

other instance of PCOPY.exe is already using the device.

With the server executing on the second machine, you can verify complete

connection by using the 'P' (ping) command.

Here, the test facility sends a ping packet to the server.

The server receives the packet, and replies with a pong.

With successful Ping-Pong, you know the device driver and application are

configured properly on both machines.  This also indicates that your cable

is probably good.

To assist development, PCOPY's test facility includes data

throughput testing (menu item 'T').

The client (machine with the test menu) writes data packets to the

parallel port.  The server receives and discards.

When transfer completes, statistics are displayed.

These numbers reflect the throughput of the PC-PC connection

from ring-3 on the client through ring-3 on the server.

Using a PS/2 Model 95 (25 Mhz) and a 33Mhz 80486 DX2 Value Point

throughput is measured at about 30,000 Bytes/second.

File transfer speeds have been measured at 29980 bytes/sec, but

the 47 seconds elapsed time was measured with a wrist watch so your

mileage may vary.

----------------------------------------------------------------------

Version History:

Version    Date     Notes

1.00   22-Sep-93  Initial development complete

1.01   29-Sep-93  Update help, prepare for OS2TOOLS release.

For questions and support, please use PCOPY FORUM on IBMPC.

Joe Nord