I used to have a lot of problems with the keyboard on my DTK 386

SX20 computer: arrow keys would print out as numbers, function keys

would not work properly, everything I typed in would come out

capitalized, etc.  My problems were solved when I came across the

following article.  Although it focuses on PC-KWIK Power Pak and

WordPerfect, the problems it discusses, and the solutions it suggests,

seem to go well beyond these two programs.  In particular, even non-

WordPerfect users may wish to try out the WP files discussed below.

These utilities may solve problems with programs besides WordPerfect,

and they come with additional documentation on the causes and cures of

keyboard problems.  I hope other users find this information as helpful

as I did!

Richard Williams, University of Notre Dame

INTERNET:  F67TEL@IRISHMVS.CC.ND.EDU

--------------------------------------------------------------------

(The following is copied verbatim from Multisoft Corporation's PC-Kwik

Power Report, Volume 3, Number 1, Winter 1992, page 9.)

"Sticking Shift Key" Reports

Some of our Power Pak users have reported incorrect keyboard data

when using enhanced (101-key) keyboards.  There are two manifestations

of this problem:

1.    Every key on the keyboard is suddenly in a shifted state, as

if a shift key was locked down.  This state can usually be cleared by

pressing and releasing the LEFT shift key.

2.    When using either the standard arrow keys or the numeric

keypad arrow keys, the corresponding number from the numeric keypad is

sometimes inserted, as if the shift key or the "Num Lock" key had also

been pressed.  Pressing the arrow keys results in the corresponding

number:  the "down arrow" is entered as a 2, the "up arrow" as an 8,

and so on.

This is especially annoying if numbers are unknowingly placed in a

spreadsheet or other file, when you intended to simply move the cursor

between cells.

These problems are not really caused by the Power Pak utilities,

although the presence of the Keyboard Accelerator increases the

possibility that the problem will occur.  Similar problems have also

been experienced by WordPerfect and LANtastic users, even without

Power Pak being installed.

The problem is caused by the way the keyboard sends information to

the BIOS and how the BIOS handles that information.  It is most likely

to occur if there are too many keyboard handlers looking at Interrupt

09H, like the one found in the Power Pak Keyboard Accelerator and

other programs.

The keyboard emits multibyte sequences for extended keys, such as

the arrow keys.  The TIMING OF THE KEYBOARD determines when each byte

will be read.  If the chain of keyboard handlers takes too long, one

or more of the handlers could read the byte AFTER the byte that caused

the interrupt.

This means that part of the information sent by the keyboard could

be overwritten and lost.  For the case of the arrow keys, the code

that tells the keyboard handler that the key is an extended key is

lost and the key is interpreted as whatever the non-extended code

denotes.

Since the problem is inherent to the way keyboard information is

handled, there is nothing that could be changed about Power Pak that

would eliminate it.  The following list includes solutions that some

users have applied to correct the problem.  Starting with the first

one, try one or more of these solutions until your problem is

corrected.

1.    If you are using Quarterdeck's QEMM-386, try adding one or

both of these parameters to the QEMM line in your CONFIG.SYS file:

IA for "IGNOREA20"

U8 for "UNUSUAL8042"

Both of these parameters have been found to reduce or eliminate

keyboard problems.  Refer to your QEMM manual for more information.

2.    Try a different keyboard.  Certain keyboard/BIOS

combinations appear to be particularly susceptible to the problem.

3.    Unload the Power Pak Keyboard Accelerator (PCKKEY) and/or

other utilities which use the INT09 interrupt.

4.    Disable the Power Pak Print Spooler (PCKSPL) Configurable

Hot-Key Monitor by using the /H- parameter.

5.    Unload the Power Pak Screen Accelerator (PCKSCRN), which

uses the INT09 interrupt.

6.    If you are a WordPerfect user, some utilities are available

on their bulletin board, archived under the name: INT09FIX.ZIP.  This

.ZIP file includes the following:

KBFIX.EXE

KBDFIX.EXE

INT9FIX.EXE

KBFIX.DOC

The telephone number for the WordPerfect BBS is: (801)225-4414.

The KBFIX.DOC file includes instructions on how to use these

utilities.

These utilities are also available on Compuserve in the NOVA forum

library #14, under the name KBDFIX.ZIP.

7.    Another possibility for WordPerfect users is to start

WordPerfect with the /NC or /NK parameters.  These parameters have

been found to eliminate some conflicts with TSR programs.  Refer to

your WordPerfect manual for additional information on these startup

options.