# Linux MPT SCSI OS Driver Release Notes

# Driver Version: mptlinux-3.01.14.23

# 8/05/2004

Hardware Supported:

- 53C1020, 53C1030, 53C1035 (Scsi Mode)

- FC909, FC919, FC929

- FC919X, FC929X, FC929XL

Bundling Information:

- No update available at this time.

- Official Kernel:

  - No update available at this time.

OS Release Restrictions:

For restrictions with other Linux releases, please see the

NOTES, ISSUES and DETAILS

section located at the end of this document.

Driver Release Restrictions:

- Recommended FW:

  - 1.0 series:  Initiator: Version 1.0.16 or later

  - 1.1 series:  Initiator: Version 1.1.46 or later

  - 1.1 Series:  Mirroring: Version 1.1.26 or later

- The isense driver and associated code has been removed. To enable the printing of sense data that is available in the kernel do the following:

sysctl –w dev.scsi.logging_level=0x1000

#### Driver Release Package Contents

mptlinux-xxx-src.tar.gz:

Full driver source suitable for inclusion with any publicly available kernels.  The flash utility is included in the utils/ subdirectory.

mptlinux-xxx-rpms.zip:

RPM images are named as:

mptlinux-vendor-<driver version>-release.arch.rpm

where:

vendor = {suse, redhat, turbo}

release = release tag for this rpm

arch = ia64		 Itanium Processor

x86_64	 Opteron Processor, w/ x86_64 Install

i686		i686  or later processor (Red Hat)

i586		x86 installations (SuSE)

i586		i586 or later processor (Turbo)

The kernels supported are listed in the Readme_rpms.txt included in the rpms.

SuSE:

i586

9.1

2.6.4-52	(kernel-default, kernel-smp,kernel-bigsmp)

x86_64

9.1

2.6.4-52	(kernel-default, kernel-smp)

Major Changes For Version 03.01.14.23

Release Date:  08/05/2004

Driver General Changes

Functionality

Defect fixes

- A fix for the module parameter “mptscsih”; which was not being exported.

Major Changes For Version 03.01.13.23

Release Date:  07/27/2004

Driver General Changes

Functionality

Defect fixes

- Fix panic for IM config when driver is loading.  This was due to a NULL pointer in mptscsih_initTarget.

Major Changes For Version 03.01.12.23

Release Date:  07/14/2004

Driver General Changes

Functionality

- The port of the 2.05.22 thru 2.05.23 driver.

- Patch provided by Christoph Hellwig : remove the isense code.

Defect fixes

- None

Major Changes For Version 03.01.11.22

Release Date:  07/07/2004

Driver General Changes

Functionality

- The port of 2.05.17 thru 2.05.22 driver

Defect fixes

- None

Major Changes For Version 03.01.10

Release Date:  07/02/2004

Driver General Changes

Functionality

- Patch provided by Masao Fukuchi: mptctl updates for hot plug.

- Patch provided by Christoph Hellwig: mptlan updates for hot plug; remove CONFIG_LBD ifdefs from fusion mptscsih

Defect fixes

- None

Major Changes For Version 03.01.09

Release Date:  06/18/2004

Driver General Changes

Functionality

- Patch provided by Masao Fukuchi: Remove limit on number of support hosts.

- Patch provided by Christoph Hellwig: kill scsi_to_pci_dma_dir usage, both pci and scsi use the same bits; kill mptscsih_io_direction and always trust the midlayer; kill usage of Scsi_Foo typedefs; use <scsi/*.h> headers; avoid list search in fusion ->proc_info.

- Patch provided by Jeremy Higdon: Add readX_relaxed .

Defect fixes

- None

Major Changes For Version 03.01.08

Release Date:  05/26/2004

Driver General Changes

Functionality

- The port of 2.05.13 thru 2.05.16.01 driver

Defect fixes

- None

Major Changes For Version 03.01.07

Release Date:  05/24/2004

Driver General Changes

Functionality

- Patch provided by Christoph Hellwig: clean up backword compatibility with 2.4 kernel and older

Defect fixes

- None

Major Changes For Version 03.01.06

Release Date:  05/04/2004

Driver General Changes

Functionality

- Change in mptscsih_io_done. For MPI_IOCSTATUS_SCSI_PROTOCOL_ERROR return DID_SOFT_ERROR.

Defect fixes

- None

Major Changes For Version 03.01.05

Release Date:  04/22/2004

Driver General Changes

Functionality

- Added FC909 support back.

Defect fixes

- None

Major Changes For Version 03.01.04

Release Date:  04/14/2004

Driver General Changes

Functionality

- Additions to loginfo debug info.

- Added IOC debug error code info.

- Dump FCP ResponseInfo.

- Added element "segmentID" in get_iocinfo IOCTL.

Defect fixes

- Fix a Oops in mptscsih_flush_running_cmds().

Major Changes For Version 03.01.03

Release Date:  03/23/2004

Driver General Changes

Functionality

- Request from Jeff Garzik: Replace wrappers for CHIPREG_XXX with defines.

- Remove support for FC909.

- Remove PortIo modules parameter.

- Move procmpt_destroy function before pci_unregister_driver, as the memory allocated for the proc was not being deallocated.

- Remove mptscshi_reset_timeouts function. The timer was already expired when mod_timer is called.

- Fix small bug in slave_destroy, which could prevent domain validation on hidden drive in a RAID volume.

Defect fixes

- Fix fifo memory allocation under 64bit systems.

Major Changes For Version 03.01.02

Release Date:  03/17/2004

Driver General Changes

Functionality

- Request from Andi Kleen: put warning "Device (0:0:0) reported QUEUE_FULL!" into debug messages.

- Request from Alexander Stohr: fix warnings from mptscsih_setup and get_setup_token, when driver isn't compiled as module.

- Request from Randy Dunlap: Remove unnecessary min/max macros and change calls to use kernel.h macros instead.

Defect fixes

- None

Major Changes For Version 03.01.01

Release Date:  03/17/2004

Driver General Changes

Functionality

- None.

Defect fixes

- IM fix in slave_destroy, poor RAID1 performance.

Major Changes For Version 03.01.00

Release Date:  03/05/2004

Driver General Changes

Functionality

- The port of the 2.05.03 thru 2.05.13 driver.

Defect fixes

- None

Major Changes For Version 03.00.04

Release Date:  02/24/2004

Driver General Changes

Functionality

- Kconfig cleanup.

- Merged LAN driver updates from Kernel.org.

Defect fixes

- Fix bug with driver loading and no devices attached.

Major Changes For Version 03.00.03

Release Date:  02/12/2004

Driver General Changes

Functionality

- None.

Defect fixes

- Fix for discarded symbol in mptscsih.c, as seen in ia64. { mptscsih_exit() }

Major Changes For Version 03.00.02

Release Date:  02/02/2004

Driver General Changes

Functionality

- Hot plug fix's requested from Christoph Hellwig.

- mptbase_remove cleanups.

- mptbase_shutdown - calls the sub drivers entry points.

- __init, __ exit cleanups

Defect fixes

- None

Major Changes For Version 03.00.01

Release Date:  01/13/2004

Driver General Changes

Functionality

- Hot plug updates.

- Define shutdown entry point to replace old reboot notifier call.

- mptbase_pci_table clean up.

- Change naming of MPTSCSIH_DISABLE_DOMAIN_VALIDATION to MPTSCSIH_ENABLE_VALIDATION.

- CONFIG_PM defined incorrectly.

- probe/release cleanup

- replace do_div with sector_div

Defect fixes

- None

Major Changes For Version 03.00.00

Release Date:  01/06/2004

Driver General Changes

Functionality

- Added new PCI API support.

- Added ACPI support.

- Added CONFIG_LBA, READ16, WRITE16 support.

- Reduce task management (abort=2sec,reset bus=5sec, timeout=10sec).

Defect fixes

- Underun fix.

- Chain buffer free list not being init properly.

Major Changes For Version 02.05.00.06

Release Date:  01/16/2004

Driver General Changes

Functionality

- None.

Defect fixes

- IOMMU fix for SUSE.

Major Changes For Version 02.05.00.05

Release Date:  11/12/2003

Driver General Changes

Functionality

- None.

Defect fixes

- Error handling fixes, e.g. use of host_lock.

Major Changes For Version 02.05.00.04

Release Date:  11/04/2003

Driver General Changes

Functionality

- Removed __init from mptscsih_setup.

- Removed __init from get_setup_token.

- Changed copyright from 2002 to 2003.

- Added new mailto, and removed Pam.Delaney.

Defect fixes

- Added fix for 32bit emulation when unloading mptctl module.