|  | Enterprise Platform & Services Marketing  Intel Corporation  2800 Center Drive  DuPont, WA 98327 |
| --- | --- |

| DATE: | July 28, 2006 |
| --- | --- |
| TO: | Intel® SR870BN4 customers |
| SUBJECT: | Intel® SR870BN4 BIOS Release Notes PR (Production Release) 1.5 Release Build # 1002 |
| Information in this document is provided in connection with Intel products. No license, express or implied, by estoppel or otherwise, to any intellectual property rights is granted by this document. Except as provided in Intel's Terms and Conditions of Sale for such products, Intel assumes no liability whatsoever, and Intel disclaims any express or implied warranty, relating to sale and/or use of Intel products including liability or warranties relating to fitness for a particular purpose, merchantability, or infringement of any patent, copyright or other intellectual property right. Intel products are not intended for use in medical, life saving, or life sustaining applications. Intel may make changes to specifications and product descriptions at any time, without notice. Intel is a registered trademark of Intel Corporation. *Other names and brands are the property of their respective owners. Copyright © 2002 Intel Corporation. |  |

# About This Release

Release #	: 1002.PR1.5 (Production Release 1.5 Montecito BIOS )

Build Stamp	: S870BN4A.86B.1002.P01.0607211008

Build Date	: July 20, 2006

# BIOS Components/Contents

PAL Ver.			: 7.46

###### SAL-A / SAL-B Ver.		: 3.0/3.0 (SAL3.20 spec update changes)

EFI Spec. Ver.		: 1.10.14.62

FPSWA Ver.			: 1.18

EFI GIGUNDI			: 2.16

SCSI BIOS Ver. (LSI1030)	: 01.03.10

ATI  BIOS			: 0101145700000112

Processor stepping(s) supported: Montecito Processors, B, C0, and C1 stepping

System hardware configurations supported: Production Released systems

# System FW Requirements/Revisions

BMC FW	:  BMC34

FRU/SDR	:  SDR21

# Important Installation Notes

To eliminate the chance of flashing an incompatible BIOS binary, please use IFLASH64.efi Version 1.02 or greater with P04.  Starting with Version 1.02, Iflash64.efi is capable of verifying that the selected BIOS binary is compatible with the system's Processors type, by matching the Processor's  CPU ID.  If an incapability is detected, then an error message is displayed.  Refer IFLASH64.EFI Ver1.02 Release notes for details.  Note that the IFLASH64.efi ver 1.02 is not backward compatible with RC3 BIOS or any of the previous releases, in which case the older version of IFLASH64.efi should be used.

Other instructions:

- CLEAR CMOS after updating to this BIOS.

- Preferably clear the Processor history, by going to the Setup and issuing a Re-arm.

- Have the BMC jumper in the Update mode

# Known Issues / Workarounds

- None

# Features added in this release

- MT personality support

- DBS Support

- PAL 7.46, 7.44, 7.43, 7.33 related changes

# Issues Fixed

Changes in PR1.5 BIOS for Itanium 2 Processor 9000 series:

- 4P system failing with FRB2 hang (NVRT AC cycling)

- Tiger BIOS not setting LastBlock for DVD block device in EFI block IO protocol

Changes in earlier BIOS for Itanium 2 Processor 9000 series:

- Type 4 Entries in SMBIOS Table Report Incorrect Actual Speed

- ASSERT FAILED error when copy certain size file using USB FDD

- Power off the system will cause WHQL test error

- VSD-SAL: Montecito: post screen shows 2.3GHz instead of 1.6GHz when BMC is disabled

- SMBIOS cache entries present and enabled for empty proc sockets

- Hurdle test hang at ‘testing SAL’

- Wrong calue for BogoMIPS in Linux /proc/cpuinfo on Montecito

- Performance issue: 5x slowdown when running Raccoon tests on 8 cores

- “Multi-Threading” in BIOS setup must be changed to “Hyper-Threading Technology”

- Added support for HPC Processors (removed MT option, fixed cache display)

- EBC fix for Emulex, Qlogic EFI configuration utility hang issue.

- "Proc speed mismatch" error occurs with other errors.

- SMBIOS type 11 (OEM String) not properly terminated, has garbage characters.

- Different speed DIMM do not cause mismatch

- Lexar 256MB USB2.0 Keyfob does not work in EFI

- System hangs and doesn't log error when dSERR is injected

- Added Localization for Multi-threading and Multi-threading Help

- Updated Multi-threading feature so power cycle not needed when going from disabled to enabled

- BIOS console redirection fails intermittently

- BIOS hangs if console redirection is enabled

- Duplication keyboard instructions for "Enter" in BIOS Setup

- Extraneous BIOS Setup keyboard instructions in Processor Info Sub-Menus

- Missing keyboard instructions for right arrow

- Processor speed incorrect in Type 4 SMBIOS Structure

- Serial Redirection over MODEM does not function

- Quiet boot stays enabled when Service Partition Boot is enabled

- Montecito BIOS Setup items don't reset to default when NVRAM is cleared by DIP

- Mad6M/Mad9M Transition BIOS hangs SUT at 0x07E3 using Mad6M/Mad9M Procs

- SUT will not auto-recover to Montecito BIOS from Mad9M/Mad6M BIOS

- Multi-Threading is Disabled in BIOS Setup by default

- SMBIOS changes for Montecito

- Fixed the wrong mismatch L3 cache error and stepping error in the POST

- Changed the code to select thread 0 as BSP

# Helpful Reference Material

- Intel Server System SR870BN4 Product Guide

# BIOS Recovery Instructions

To recover the SR870BN4 platform BIOS follow the procedure outlined below:

Package Contents:

This package contains the BIOS Recovery file S870BN4D.REC used for recovering the system to BIOS 1002.P01. Instructions on using Recovery are described below.

Getting Started:

Copy the S870BN4D.rec file to a blank LS-240 diskette from a Win32 system.  Alternatively, the S870BN4D.rec file may be burned to a CDROM using the El Torito format.

BIOS Recovery Procedure:

- With the system switched off and AC power disconnected:

- Place I/O Riser board jumper switch #1 to the ON position for BIOS recovery

- Insert the LS-240 diskette or CDROM containing ONLY the file S870BN4D.REC

- Reconnect the AC power and switch server power on.

- LS-240 diskette/CDROM activity will start and one full beep is heard (start load SR870BN4D.REC from disk to memory).

- Wait two minutes.  No indication of LS-240/CDROM activity will be seen until two full beeps are heard that indicate the BIOS recovery has completed successfully

- Remove the LS-240 recovery diskette or CDROM, switch system power off, and disconnect AC power.

- Replace I/O Riser jumper switch #1 to the OFF position

- Reconnect AC power and switch the system on.

- When the system reboots, go into BIOS setup by hitting ‘F2’ function key.

- Tab to the ‘Exit’ menu. Select ‘Load Setup defaults’. Press ‘yes’ to ‘Load Setup Defaults’?

- Make any changes that you need to do in setup. Press F10 to save and exit.

- System will automatically reboot to EFI.

Verification of State:

Determine your BIOS version. Read the version in the BIOS banner string.

Final Configuration:

After the update, the following should be the configuration of the system.

S870BN4A.86B.1002.P01

# Safety Notice

THE INTERNAL WORKINGS OF OEM UNITS CARRY HIGH STANDBY VOLTAGES DESPITE BEING POWERED DOWN BY FRONT PANEL. USERS SHOULD UNPLUG BOTH POWER CORDS TO THE UNIT BEFORE ACCESSING THE SYSTEM OR REMOVING ANY BOARDS.