## SR870BH2 Transition BIOS T01 Release notes

Revision: 221.T01 (for transition from Itanium® 2 Processor with up to 6M L3 cache to Itanium® 2 Processor 9000 Series)

Build ID: S870BH2A.86B.0221.T01.0601191402

Build Date: January 19, 2006

# Components/Contents

BIOS				86B.0221.T01*

PAL				5.73 (Itanium® 2 Processor with up to 6M L3 cache) *

SAL-A/SAL-B		3.0/3.0 (SAL3.20 spec update changes)

FPSWA			1.18

EFI 				1.10.14.62

SCSI BIOS (LSI1030) 	1.03.00.00 + F/W 1.03.10 (IME)

ATI  BIOS			114F

EFI GIGUNDI		2.16

(* indicates new for this release)

# System configuration supported

Gold system with Itanium® 2 Processors with up to 6MB of L3 cache

# Special Instructions

This BIOS is only to be used to transfer from Itanium® 2 Processor with up to 6M L3 cache to Itanium® 2 Processor 9000 Series. Other uses are not supported.

Please refer to TA-0789-03 for detail steps on how to upgrade to Itanium® 2 Processor 9000 Series

USE LATEST VERSION OF IFLASH64.EFI

To eliminate the chance of flashing incompatible BIOS binary, please use IFLASH64.efi Version 1.03 or greater. Starting with Version 1.03, Iflash64.efi is capable of verifying that the selected BIOS binary is compatible with the system's Processors type, by matching the Processor’s CPUID.  If incapability is detected, then an error message is displayed. Refer IFLASH64.EFI Ver1.03 Release notes for details.

USING THE BIOS CD IMAGE TO CREATE BIOS RECOVERY-CD

As the systems have only the IDE-CD as the recovery media device, this BIOS release includes a bootable (El-Torito) CD-image file. This CD image can be burned into blank CD-R or CD-RW media to create the BIOS recovery-CD. To recover BIOS from the CD, put the BIOS recovery-CD into the IDE-CD drive and set the recovery jumper and reset the system, the BIOS recovery will start.

INSTRUCTIONS

Using the SUP batch file to do the BIOS update or you can do manually as following:

1. Copy S870BH2B.bin and IFlash64.efi to a CDR, or a USB Keyfob.

2. Boot the system to EFI.

3. Change the current directory to the FS drive that corresponds to the media with the above files.

4. Type the command <Iflash64> and press Enter

5. Once the program loads, move the cursor to FILE, and select OPEN...

6. Navigate to the folder that contains the S870BH2B.bin file and press Enter. This will cause the file to be loaded onto the system.

7. Move the cursor to BIOS, press Enter, and select Update System BIOS. Select OK, and the BIOS will be updated.

8. When the message "BIOS Updated Successfully" is displayed, choose if you want the NVRAM to default values from CMOS on the next boot.

9. Select ok to reset the system.

10. Verify the system boots to EFI. During POST verify there is a message stating that the BIOS is only for transition.

Verification of State:

Determine your BIOS version. Read the version in the BIOS banner string.