[ XALL.DOC ]

Version 1.4.7[a3] of XALL.SYS is an OS/2 device driver for the

following DigiBoard intelligent multichannel cards:

PC/Xi,e         PC/Xem          PC/2e       EPC/X (ISA)

MC/Xi           MC/Xem          MC/2e       EPC/X (EISA)

COM/Xi          COM/Xi-422                  EPC/X (MCA)

C/X (ISA)       C/X (EISA)      C/X (MCA)   AccelePort

This document is intended for programmers developing software who need

to be aware of the advantages and possible problems of using DigiBoard

hardware and this provided device driver.

Included in the release are several binary data files.  They are the

BIOS and On-Board Operating system (Front End Processor; i.e. FEP) code

for the different boards.

sxbios sxfep                -- for the PC/Xem, MCXem and Acceleport

xxbios xxfep                -- for the PC/Xi,e and MC/Xi(fep only)

mxfep                       -- for the COM/Xi

cxbios cxfep cxconc         -- for the C/X series

fxbios fxfep fcconc         -- for the EPC/X series

Almost all of the standard Device IO commands are supported in full and

behave exactly as the standard OS/2 COM01.SYS driver.  There are however

a few expections as well as undefined or undocumented pieces of how

a 'well behaved' serial driver is to behave. Programmers should in

general be aware that these are INTELLIGENT cards with large on-board

memory buffers for data.  Calls to the driver that expect information

related to byte-by-byte activity of the driver are at best going to

return approximations and at worst will be simply missing some information.

Please read the sections below on the setting in the Device Control Block,

and GETCOMMSTATUS, GETLINESTATUS, GETCOMMERROR.

FILES:

xall.sys      -- the executable device driver file.

xall.doc      -- this document.

install.doc   -- some installation notes for PC bus boards

relnotes.doc  -- upgrade notes.

dmode.exe     -- a MODE workalike with extended DEVICE capabilities.

The device driver requires a number of command-line arguments

described in the installation guide (INSTALL.DOC).

STANDARD OS/2 DEVICE DRIVER COMMANDS

READ    (4)     Fully Supported

NDREAD  (5)     Fully Supported

WRITE   (8)     Fully Supported

Writes will return to the caller AS SOON as the data has

cleared the card -- unless the request size indicates that

the transmission should take less than a few seconds -- in

which case it will return immediately.  If it is important to

wait until the data has actually ALL been transmitted be sure

to check with GETOUTQUEUECOUNT (see IOCtls below) to see how

many bytes remain in the transmit queue.

WRITEVFY(9)     Supported.

We have discovered that even though undocumented in the

Programmers Reference, this call gets generated by some

implementations of OS/2 if DISKVERIFY is set to ON in the

'config.sys' file EVERY TIME an application program calls

DosWrite.  Hence this will be treated like a DosWrite.

INSTAT  (6)     Fully Supported

OUTSTAT (10)    Fully Supported

OPEN    (13)    Fully Supported

Shared opens are supported and permissions enforced by the

system.  Device Baud rates, data types, DCB information, etc.

are set per system defaults if this is the first 'open' since

system initialization.

NOTE: This means that before the first open, the channel

is NOT enabled and will NOT accept any data into the

receive buffer.

CLOSE   (14)    Fully Supported

CLOSE will decrement the number of opens on this port.  If

this is the final close, CLOSE will flush any outstanding

READ requests.  If there are still some outstanding WRITEs

AND some form of output handshaking is enabled then

CLOSE will block until they complete or time out.

NOTE: If the process that currently owns the port dies or

is killed (eg. Cntl-C/Brk), CLOSE will NOT wait for

data to clear before dropping the lines.  However, note

that OS/2 does NOT pass on signals to the threads

within a process; thus if a spawned thread is ALSO

blocked on IO when the process tries to die the

process will hang for whatever timeout is in effect

on the IO block in question.

*GEN_IOCTL --   See subsequent part of this document.

*PRT_ACS(21)    UNSUPPORTED.  The IO port associated with the card is

not used by application programs for communications, but

by the driver for control of the card as a whole.  Will

return Error.

STANDARD OS/2 ioctl COMMANDS

Below are the functions defined in the Programmers Ref. manual.

Where behavior might not be exactly what you would expect, the function

name is preceded by an asterisk.

Category 01:

SETBAUDRATE     (41)    Supported in Full

Additionally, in order to support higher baud rates within the

limitations of a 16-bit word, the following non-standard values

will be interpreted thus:

value passed:   768             will set 76.8Kbaud

1152            will set 115.2Kbaud

SETLINECTRL     (42)    Supported in Full

SETXBAUDRATE    (43)    Supported in Full

The baud rate is passed as a 4-byte long and if supported then is set.

Note that the CX cards only support up to 57Kbaud.

TRANSMITIMM     (44)    Supported

Since commands must be issued to the card to temporarily suspend

software flow control if such is in effect it is possible that

if other data is in the Transmit queue that a byte or so of that may

also be transmitted.  In general this function is NOT recommended

if any software flow control is in effect as results on the

receiving end may be unpredictable.

SETMODEMCTRL    (46)    Supported in Full

Attempts to set/clear modem lines that are currenly invloved in

flow control as per DCB.fbCtlHndShake (flag1) will fail.

SETBREAKON      (4b)    Supported

The length of time of the transmitted break is 1 min. max !

SETBREAKOFF     (45)    Supported in Full

STOPTRANSMIT    (47) (4c)       Supported in Full

STARTTRANSMIT   (48) (4d)       Supported

SETDCBINFO      (53)    Supported

Setting Flag2.bit6 AND Flag2.bit7 is supposed to enable the RTS-toggle

mode on Transmit.  This is not supported.  Setting BOTH of these bits

will be interpreted simply as RTS handshake on Input.

DCB.fbTimeout bits 3-7 are documented in IBM's Device Driver guide

as relating to Extended Hardware Buffering.  Since the settings are

so tightly related to the particular hardware used by IBM on

their MCA machines and since setting these bits may have caused

application programs to assume the presence of a raw 8530 UART,

this driver will always clear these bits.

Character Replacement/Stripping:

NULL-stripping and BREAK- and ERR-char-replacement are supported

in this version of the driver but it is important to note that

this is done by the driver code, not the on-board FEPOS.  Turning

these options ON will thus slow things down.  Also, if NULL-stripping

is enabled, there can be discrepancies between how many bytes

GET_INQUEUE_COUNT says are in the buffer and how many bytes are

returned to the user by a read call.

GETBAUDRATE     (61)    Supported in Full

Beyond the normal limits of 19200 (IBM) or 38400 (MS), this driver

supports settings of 57600.  Settings beyond 19200 are NOT recommended

on the COM/Xi.

GETLINECTRL     (62)    Supported in Full

GETXBAUDRATE    (43)    Supported in Full

The fractional bit rates returned are always zero since the rates are

quantized by the FEPOS on the card and not calculated by the driver

GETCOMMSTATUS   (64) (6b)       Supported

The information in this byte about hardware signal REASONS for Rx or

Tx being paused is extracted from comparing the current line

signals with the handshake ones requested in DCB.fbCtlHndShake (flag1).

No explicit information is available from the DigiCHANNEL hardware

concerning those pauses/reasons but the above analysis will result

in accurate information barring strange cabling or severely ill

on-board operating system.  If Tx is paused for reasons of Software

Flow control (i.e. an XOFF was received), that condition is accurately

reflected in the COMMSTATUS byte.

The TX_WAITING_TO_SEND_XON will never be set since the time between

the transmitter sending an XON and proceeding (i.e. no longer paused)

to subsequent data is not trappable by the Device Driver.

Other bits are all set as per the MS OS/2 documentation.

Note that on the C/X, information about the transmitter being paused

can be inaccurate if all the data in the TX buffer is small enough to

have LEFT the host adapter but is still paused on the remote

Concentrator.  In such instances the above analysys ofcourse fails.

Likewise on the GETLINESTATUS call.

GETLINESTATUS   (65)    Supported

The HARDWARE_TRANSMITTING bit is set by cheating.  The call actually

pauses for the time increment to transmit 1 byte at the current

baud rate then checks to see if the transmit buffer head pointer has

moved.

The WAITING_TO_SEND_XON and WAITING_TO_SEND_XOFF bits will never be

set as the batch nature of the intelligent card does not permit

that information to be reliable.

GETMODEMOUTPUT  (66)    Supported in Full

GETMODEMINPUT   (67)    Supported in Full

GETINQUECOUNT   (68)    Mostly Supported

This count only accounts for the data that is on the actual host

adapter and not on C/X and EPC/X concentrators.

GETOUTQUECOUNT  (69)    Supported in Full

On the C/X there can be up to 2K of data out on the remote box

that the driver is [almost] oblivious of.  If the buffer on the

host adapter is empty, then the driver checks a boolean to see if

there is still activity on the remote box -- returning a character

count of 1 of there is.

GETCOMMERR      (6d)    Supported

Errors in the input data are only detected as the data is read

from the card into user space.  Thus the COMMERROR byte will

accurately indicate Parity, framing, and hardware-overrun errors on

data that has been read off the card into user-space.  We remain

oblivious of COM errors in the data out on the as yet un-read

(i.e. un-requested) buffers of the card.

GETCOMMEVENT    (72)    Supported in Full

*GETDCBINFO     (73)

Again, note quirks in SETDCBINFO

Category 0x0b

FLUSHINPUT      (01)    Not Supported at this time

FLUSHOUTPUT     (02)    Not Supported at this time

********************************************

Category 0xD1

********************************************

This is a non-standard category of IOCtl custom for the XALL driver.

This section of code will ENABLE altpin processing on the given device.

The _QUICKWRITE bit permits the writing of data to the card to happen

much more efficently -- utilizing ALL the buffer space on the card before

resorting to BLOCKING the process.  Without this bit set (default=0), the

driver will mimick the Standard MS/IBM COM driver and wait for EACH write

request to clear the card before continuing with the next or returning to

the user program.

/* Enable-Disable reversed interpretations of DSR and CD for RJ45, and

* method of waiting for WRITE requests. */

#define IOCTL_CATEGORY_DIGI     0xd1

#define DIGI_CUSTOM             0x07

#define DIGI_QUERY              0x00    /* non-destructively READ settings */

#define QUICKWRITE_ENABLE       0x10    /* if set, wait only on last close */

#define ALTPIN_ENABLE           0x01

#define ALTPIN_DISABLE          0x02    /* default Value */

#define DIGI_BLOCK_EVENT        0x08

unsigned char   Current, Want;

...

Want = DIGI_QUERY ;

...

/* Query current status without change */

if  ( DosDevIOCtl (     &Current,

&Want,

DIGI_CUSTOM,

IOCTL_CATEGORY_DIGI,

handle) )

{

printf ("This driver isn't XALL, won't do DIGI processing");

}

else

{       /* If need CD on 8-wire RJ45 and not already set up */

if ( 8_WIRE_RJ45 && ((Current & ALTPIN_ENABLE) == 0))

{

Want = (Current & (~ALTPIN_DISABLE)) | ALTPIN_ENABLE;

if  ( DosDevIOCtl (     &Current,

&Want,

DIGI_CUSTOM,

IOCTL_CATEGORY_DIGI,

handle) )

printf ("Something ill in State of DIGI");

...

...

/* If I want to have QuickWrite processing on this port */

if ( (Current & QUICKWRITE_ENABLE) == 0))

{

Want = Current | QUICKWRITE_ENABLE;

if  ( DosDevIOCtl (     &Current,

&Want,

DIGI_CUSTOM,

IOCTL_CATEGORY_DIGI,

handle) )

RETURN VALUE:

The call will always return 0.  Current Status (0x1,0x2,0x11,0x12) returned in

'Current' byte.

A new  function has been added to the Digi custom IOCTL's. This function

provides a mechanism by which a user program can block until a specific

DigiBoard event occurs. These events are defined below. The driver will

only block on one event and will return an error if more than one event is

specified.

#define DIGI_BLOCK_EVENT        0x08            // IOCTL Function Number

#define DeltaCTS                0x0008          // Change in CTS

#define DeltaDSR                0x0010          // Change in DSR

#define DeltaDCD                0x0020          // Change in DCD

#define StartRXBreak            0x0040          // Start of a received BREAK

#define DeltaRI                 0x0100          // Change in RI

unsigned char   Want;

...

Want = DeltaCTS;

...

/* Query current status without change */

if  ( DosDevIOCtl (     NULL,

&Want,

DIGI_BLOCK_EVENT,

IOCTL_CATEGORY_DIGI,

handle) )

{

printf ("Error In Call To XALL, Check Parameter\n");

return(ERROR)

}

printf("Thread Done Blocking on CTS Event\n");

...