1.0 What this is about

------------------------------------

This document describes how to write OS/2 V1.X device drivers in C.  While they

do require a small amount of assembler code, the majority of the device driver

can be coded in C.  Only the interfaces to OS/2 and the functions that need

to access the CPU directly need be written in assembler.

When writing device drivers in assembler, the programmer has complete control

over his code.  He can control the order of segments, what goes in which

segment, and in what order.  He can pass parameters using any mechanism he

wishes.  He can control exactly what gets linked in and what doesn't.  A C

programmer, however, does not have quite that level of control over his final

executable.  The compiler has made many decisions for him, or provides fairly

remote control of these parameters.  The problem is how to regain the control

that the assembler programmer enjoys, yet keep the efficiencies afforded by

programming in a high level language.  This document describes how to gain

that control.

Also included in this package is a sample device driver with source code.  The

sample does not do anything useful, except display an installation message and

install.  The only commands it accepts are INIT, OPEN, CLOSE, WRITE, OUTPUT

FLUSH, IOCTL, and DEINSTALL.  On all of these, except INIT, it does nothing,

but return SUCCESS.  INIT installs the device driver.  It does demonstrate the

techniques described here.  Anyone receiving this package is free to use the

source code in any way they see fit.

The text will reference the sample code as part of the explanation of the

techniques.

1.1 Scope

------------------------------------

This document assumes the reader is familiar with the organization and design

of OS/2 device drivers.  It does not attempt to teach how to write device

drivers, but how to implement them in C.  It also assumes a fairly solid

background in Intel 80286 and 80386 CPU architecture and assembler language

programming.  Finally, it assumes that the reader understands the use of

Microsoft programming tools - LINK, LIB, and MAKE, and IBM's C/2 compiler.

1.2 Tools used

------------------------------------

The tools used to build the sample code are:

IBM C/2  V1.1

IBM MAKE V2.0

OS/2 Library Manager/2 V1.01

OS/2 Linker/2 V1.20

Microsoft MASM V5.1

The sample code will also compile with Microsoft C6.00A.

2.0 Problems to be solved

------------------------------------

Segment Control - How do you control the order of segments in the final

executable?  How you do control which segments get grouped together, and

in what order?  How do you control which segments are kept loaded after

INIT time?  How do you control which segment library functions end up in?

How do you make sure the first thing in the Data Segment is the Device

Driver header?

Compiler and Library Control - How do you make sure stack probes are disabled

in library functions?  What library functions can you use and why? What

compiler memory models do you use and why?  How do you guarantee access to

variables?

Passing Parameters - How do you get the pointer to the request packet into a

C variable?  How do you load registers when calling the DevHelp facility?

DOS Box Memory Usage - How do you minimize the amount of memory the device

driver consumes of the Real Mode DOS Compatibility session?  How do you

gain access to memory above the 1Meg line when entered in Real Mode?

3.0 Segment Control

------------------------------------

Unlike programs, where you can organize your segments anyway you see fit, OS/2

requires a specific organization for its device drivers.  The first segment

in the executable (.SYS), must the main DATA segment.  The second segment must

be the main CODE segment - the one that contains all the device driver's entry

points.  All other segments, both CODE and DATA, will be loaded at INIT time,

but they will be discarded after INIT is complete unless you have marked them

as having IOPL privilege at LINK time and LOCKed them down at INIT time.

Finally, the first thing in the main DATA segment must be the device driver

header, which points to the strategy entry point, defines the name and

characteristics of the device driver and has a pointer to the next device

driver in the chain.

The Demo Device driver uses 5 methods, in concert, to control segment

organization and placement.  These are:

1. The MASM .SEQ directive

2. The MASM GROUP directive

3. Knowledge of the compiler's naming conventions

4. The SEGMENTS statement in the LINKer's definition (.DEF) file

5. The /NT option of the compiler to name the code segment created

The .SEQ directive tells the linker to place the segments in the executable in

the order encountered.  This allows, along with knowledge of the compiler's

naming conventions, allows us to place the main DATA segment before the main

CODE segment.  We do this by defining empty segments with the same name and

class as the segments defined by the compiler.  Combine this with the MASM

GROUP directive and we now have our logical segments combined into physical

segments in the order we want, and the physical segments placed in the

executable in the order we need.  The use of this method can be seen in

DEMO.ASM.

We use the same idea for the CODE segments, but with a twist.  We are not

constrained to using the compiler's default segment names.  The /NT option

when invoking the compiler allows us to give the CODE segment generated any

name we wish.  The names you will see in the sample code are: MAINSEG, INITSEG,

and KEEPSEG.  Their purpose is obvious.  This step is seen in DEMO.MAK, in

the definitions of the optXXXX variables.

The next step to segment control is done by using the SEGMENTS statement in the

definition file (.DEF) of the linker.  The logical segment names are placed in

the order we want them to appear.  It is optional whether you wish to specify

IOPL for the main DATA and CODE segments, but required for any others you wish

to remain accessible after INIT time.  The ordering must match that

encountered by the .SEQ statement, or extra segments with the same name and

class will be generated and you will have no control over which of the 2

segments stuff will go into.  This is seen in the file DEMO.DEF.

One final step is needed.  At INIT time, a device driver is required to tell

OS/2 how much memory the CODE and DATA segments require after INIT is done.

Essentially, this means finding the offset of the last instruction in the

CODE segment and the offset of the last byte used in the DATA segment.  It

is easier to just create a new segment with a single global variable and force

this logical segment to be the last in the GROUP that comprises the physical

segment.  It is a simple matter, then, at INIT time to get the offset of these

variables and store them in the INIT request packet on returning to OS/2.

This is seen in the files DEMO.ASM, for the definition of these segments -

LAST_D and END_TEXT.  The loading of the offsets into the request packet is

done (in C) in INIT.C.

4.0 Compiler and Library Control

------------------------------------

Other problems with using a compiled language compared to assembler are that

you have less control over the 'memory model' used.  Essentially, this means

what kind of pointers - NEAR or FAR are used as well as the way functions are

called and how they return - again NEAR or FAR.  You also have less control

over the inclusion of startup code and which segment holds the library

functions linked in.

The first thing is to decide on memory model.  My feeling is that LARGE is the

way to go.  This lets you place your functions in any segment that is

convenient, or that makes sense for your design, and be able to reach them

from any other segment.  This means that functions that are used ONLY at

INIT time can reach utility functions that are used later.  It means that

library functions can be placed in a segment of their own, and called by any

functions.

The LARGE model also implies FAR data pointers.  This is almost

a given.  All pointers passed by OS/2 to the device driver are FAR pointers.

Most of the pointers you will construct will be FAR pointers.  To handle these,

you need the LARGE or COMPACT model.

Next, there is the question of Stack Segment and Data Segment.  Most modern

compilers generate code that assumes SS equals DS.  In device drivers, this is

emphatically false.  Fortunately, the Microsoft family of C compilers has an

option that lets the programmer control whether the code generated makes this

assumption or not.  Just say no.  These compilers also have an option that lets

the programmer control whether DS will be loaded at the entrance to each

function or not.  Since OS/2 loads DS, SS and CS before entering the device

driver, just say no to this one as well.

The net of this is that the memory model to be used is LARGE code, FAR pointers

and don't assume DS=SS.  For IBM C/2, that translates to the option /Alfw.

This can be seen in the optXXXX definitions in DEMO.MAK.

Next is the problem of stack probes.  Microsoft compilers allow you to turn

off stack probes in your code and it needs to be done.  OS/2 controls your

stack size, not you.  The problem is what about library functions that were

compiled with the stack probes turned on.  To solve this problem, we replace

the stack probe function with a NULL function.  Unfortunately, Microsoft

included the adjustment of the stack pointer for local variables in the same

function.  To solve this problem, we need to write our own stack check

function that does the variable allocation, but doesn't do the stack pointer

comparisons.  This function is __chkstk, in DEMO.ASM.  The number of bytes

to reserve for local variables is passed in AX, and a FAR call to made to

__chkstk.  This function pops the return address, adjusts the stack pointer

for the local variables and the returns.  It needs to use a bit of trickery

in that adjusting the stack pointer causes the return address to be lost, so

it save the address and pushes it back on the stack after the adjustment is

made.

Now we come to variable access.  Microsoft compilers put STATIC variables in

different segments depending on whether they are initialized or not, when

using the LARGE model.  They then store the segment of that variable in a

static variable called $T2000n, where n is different for each static variable

being referenced.  This causes untold problems in accessing the variable

because the segment pointed to by $T2000n is valid at INIT time when the

device driver is at ring 3, but it is no longer valid later, when the device

driver is at ring 0.  To solve this problem, we force all the logical data

segments into one physical segment using the techniques described above.  Then,

we tell the compiler to use DS to access the variable with the near keyword.

In general, all global and static variables should be defined with this

keyword.  A simple hint - if you look into the .COD listing and see a $T2000n

variable defined, you didn't get what you wanted and need to put a near on

a variable.  A classic example of the use of the near keyword in this respect

can be seen in LOCK.C.

Finally, we come to the question of library functions.  Which ones can and

cannot be used?  The general answer is that you cannot use those that will

cause a call to the OS or will use the coprocessor.  These include all the

floating point operations, file I/O, memory management functions, process

control and time functions.  What remains are things like string and buffer

management, searching and sorting, character classification and some data

conversion (atoi(), for instance).  Fortunately, the kinds of things

forbidden are either not the kinds of things done by device drivers or are

provided by the DevHelp functions.

5.0 Passing Parameters

------------------------------------

OS/2 uses registers to pass parameters back and forth with device drivers, but

Microsoft compilers use the stack for most of these.  To solve this problem,

we need a layer of assembler between the device driver and OS/2.  All entry

points are in assembler.  All they do is push their parameters on the stack,

making a stack based parameter, and call the C code.  The C code returns its

results (usually an integer, in AX), and the assembler converts it to a form

suitable to OS/2 (a particular flag set or cleared, for instance).  This can

be seen in DEMO.ASM, in the form of the procedure _strategy.  It pushes the

request packet pointer onto the stack and calls the real strategy function

written in C.

Calling the DevHelp facility is a bit different.  One method is based on the

methods used for calling DOS in DOS based compilers.  A structure is defined

that holds all the values that will be loaded into the registers.  This

structure is loaded with the necessary values and a call made to an assembler

function that loads the registers from the structure and then calls DevHelp.

On return, it stores the register values back into the structure, allowing

the C code to examine the results.

There are at least 2 problems with this method that need to be overcome.

One is that not all values are allowed to be loaded into segment registers.

Put the wrong value in and you will get a TRAP 000D and the system will lock

up.  The only recourse is to power off.  This is not exactly user friendly.

Therefore, either a way must be divides to tell the assembler function to not

load a segment value from the structure, or the structure must always have

valid values in the segment register fields.  0 is always a valid value to

load into a segment register.  It cannot be used, but it can be loaded.  The

demo device driver uses the first method.  It is in DEVHLP.ASM.

The second problem is that some DevHelp functions use DS to hold a selector

and others use ES.  How can you make a general DevHelp caller when it has to

use a segment register to point to the DevHelp entry point to make the

indirect call?  The solution is to use CS.  This means that at INIT time,

the DevHelp function needs to create an alias to the CODE segment that is

writeable.  To do this, it needs the DevHelp functions.  The classic Catch-22.

The solution to this is to create 2 new DevHelp functions - one that uses

DS to call DevHelp and another that uses ES.  These live in the INIT segment

and are discarded after INIT time.  These functions are also in DEVHLP.ASM.

the C code to store the DevHelp entry point is in INIT.C.

6.0 DOS Box memory Usage

------------------------------------

The final problem addressed by this document is reducing the amount fo memory

used by the device driver in the DOS Compatibility Session (aka the DOS Box).

OS/2 loads the main CODE and DATA segments into memory below the 1Meg line,

using an already scarce resource - the amount of memory available for DOS

programs.  The key to solving this problem is the fact that all other segments

are loaded ABOVE the 1Meg line.  What this means is that you put only those

functions that absolutely need to be there in the main CODE segment.  These

include the DevHelp caller and all assembler entry points and the main

strategy function.  All other code can be moved to above the 1Meg line.

Moving the DATA segment is a little trickier.  The problem is that there is

no way to easily define a second DATA segment at compile/link time.  It needs

to be dynamically allocated.  All memory allocated via the AllocPhys DevHelp

call comes from above the 1Meg line.  All of your system level global variables

can be allocated and initialized at INIT time and accessed by pointer later.

This lets you reduce the amount of DOS Box memory consumed to around 1K or so.

The final problem with moving these memory blocks above the 1Meg line is that

when in REAL mode, the CPU cannot access memory above the 1Meg line.  How can

it get to the functions and data there?  The solution to this is don't run in

REAL mode.  At every entry point - there are only 5 possible, Strategy, Timer,

Interrupt, Notify, and IDC - check to see if the CPU is in REAL mode.  If it

is, make a call to the ReadToProt DevHelp function, and note in a local

variable that the mode change was made.  On exit, if the mode was changed,

go back via ProtToReal.  Also note, and this is REAL IMPORTANT - if you enter

in REAL mode, you cannot do a FAR call until you have changed to PROT mode;

and after you change back to REAL mode, you cannot do any FAR returns.  That

means that the mode changing functions must all be in the main segment.

Note that this path may be seldom taken, depending on the number of interrupts

taken and how much time the CPU spends in the DOS box.  The chances of hitting

it are reduced as OS/2 switches the CPU to PROT mode when entering the Strategy

function and Notify cannot be called fro a DOS program.  This leaves the Timer

and Interrupt and IDC calls as the only ones to worry about.  Since OS/2 is

always timeslicing into PROT mode, even when the DOS box is the active session,

the chances are reduced that the CPU will be in REAL mode when one of these

entry points are activated.  Regardless, that possibility needs to be accounted

for.

An example of this method can be seen in STRATEGY.C.  This call is not needed

here, but since the sample device driver has no other entry point, the

mechanism is shown here.

7.0 Some other ideas

------------------------------------

There are other ways to call DevHelp.  Some people write a separate assembler

level function for each DevHelp call.  Others pass the DevHelp entry point

on the stack instead of storing it in the CODE segment.  Some write several

generic DevHelper callers.  The point is that there are lots of solutions to

this problem.  You need to make sure that the one you use is suitable to your

needs.  You also need to make sure that they always call OS/2 from the main

CODE segment as any function that registers an entry point will use that CODE

segment as the selector portion of the entry point.

Another thing to watch out for is changing from REAL to PROT mode or when

going back.  After the mode change has been made, the only valid address

on the stack is the return address.  That is why those functions do not use

the normal DevHelper call in the sample code.  They can be found in

ASMUTILS.ASM.

It is useful to have the compiler generate combined listings - that is, the

listing should contain the ASM listing along with the C code.  The option

to make Microsoft compilers do this is /Fc.  Another thing should be to tell

the compiler to pack data structures.  OS/2 request packets have no extra

space in them.  Another useful item is the MAP file.  It will help you make

sure what is being generated is what you really want and it will help when

using some debuggers.

8.0 Putting it Together

------------------------------------

This section explains the sample device driver included in this package, in

light of the above discussion.

Put

DEVICE=<path>\DEMO.SYS

in your CONFIG.SYS and reboot.  You will see the loading message come up.

After re-booting is done, you can copy files to the $DEMO and the system will

say all went well.  If, however, you say copy $demo to some file, you'll get

an error saying that $DEMO doesn't like that command.  You are seeing the

difference between READ and WRITE.  READ is not recognized, while WRITE is

answered OK, we did it.

8.1 Directory Structure and the Make File

------------------------------------

The demo can be compiled by running MAKE against DEMO.MAK.  This make file

will generate the entire device driver, the MAP, and all listings or .COD

files.  It creates a library with all the .OBJ files stored in it, as well

as a MSG file used by INIT to display the loading commercial.

Currently, it stores the .LST and .COD files in a separate subdirectory off

the current one, the .OBJ in another, and all messages from the compile and

masm and link in a third.  These directories are defined in the variables

lst, obj and msg, defined at the beginning of the make file.  Note that they

use . as the current directory, making them relative references, not absolute.

Other variables are defined to tell MAKE where the library file is (and should

go) and where the source and include files can be found.  Also note that the

.ARF file has a reference to both the.\obj and .\lst subdirectories.  If you

change the .MAK file, you need to change this too.

There are 4 variables defined that put all the compiler options in one semi-

user friendly block.  The first is OPTMAIN.  This will cause the code from

this file to be included in the MAIN code segment (named MAINSEG).  The second,

OPTSIZE, does the same, except it has size optimization turned on instead of

none like the others.  OPTKEEP causes the code to go into the KEEPSEG segment

and OPTINIT puts it into the INITSEG segment (surprise!).

Each set of options has the link disabled (/c), stack probes disabled (/Gs),

structure packing (/Zp), warning level 3 (/W3), and the model as described

above (/Alfw).  It causes a .COD listing to be generated (/Fc), puts the .OBJ

file in a specific subdirectory (/Fo), renames the code segment generated

(/NT) and redirects the error messages to the msg subdirectory.

The rest of the MAKE file is standard stuff.  Each separate file is given

a separate set of dependencies so I can control the options on each.  They

then are added to the library.

8.2 Assembler files

------------------------------------

There are 4 assembler files:

ASMUTILS.ASM

BRKPOINT.ASM

DEMO.ASM

DEVHLP.ASM

These contain the entry points for the device driver, the DevHelp caller, the

segment controlling stuff described in Section 3.0, and a bunch of utility

functions.

ASMUTILS.ASM has a bunch of utility functions.  Not all are used by the DEMO

device driver, but they are useful as examples of the kinds of things you

resort to assembler for.  Included are the calls to DevHelp to change the CPU

to REAL mode a back.

BRKPOINT.ASM holds the code to do an INT3 on demand.  This function causes

a breakpoint in many debuggers.  When a call to this function is placed in

INIT code, you can trace your initialization code.  I also make it a habit

of creating an IOCTL call that invokes this function.  That lets me get into

my device driver with a debugger without recompiling.  This is a great boon

when you want to debug a particular version without re-compiling.

DEMO.ASM is the main assembler code file.  It holds the strategy entry point,

it does all the segment and group definition stuff described in Section 3, and

it holds the main DATA segment, with the device driver header.

DEVHLP.ASM has the DevHelp caller, along with the temporary one used at INIT

time to get the DevHelp entry point stored in the main CODE segment.  This

variable is also defined in this file.

8.3 C files

------------------------------------

There are 8 C files.  These are the meat of the function to the demo device

driver.  They include a bunch of utility functions as well as the necessary

stuff.  They are:

INIT.C

PRTMSG.C

STRATEGY.C

BADCMD.C

DDUTILS.C

GDTMEM.C

LDTMEM.C

LOCK.C

INIT.C holds the function to process the INIT command from OS/2.  Basically, it

sets up the DevHelp entry point, figures out the name of the message file,

prints the loading message, sets up a pointer to the milliseconds since IPL

timer, tells OS/2 how much code and data to keep loaded and exits.

PRTMSG.C is used for printing messages at INIT time.  It won't work at any

other time as it uses DOSGetMessage and DOSPutMessage.

STRATEGY.C is the main C function that figures out what OS/2 wants it to do

and calls the proper function to do it.  It also makes sure that the CPU is

in PROT mode before proceeding.  As explained above, this is unnecessary, but

it is instructive.  Most of the functions just set the status to Command Not

Recognized.  Others say 'Yes, we did it', when they really didn't.  INIT is

the only command that really calls another function.  After we get past all

this, dev_done() is called (except for INIT, when it isn't valid), to set the

request packet status.

BAD_CMD.C just returns the value needed to set into the request packet status

to reflect that fact that we don't recognize the command.

The rest of the files hold all sorts of utility functions.  There is stuff to

allocate and free memory, allocate GDT slots, Lock and unlock segments, block

a task and yield the CPU temporarily, and do all sorts of other things.  Some

of these are used by the demo device driver, others are included for

instructional purposes (actually, I was too lazy to take them out).

8.4 H files

------------------------------------

There are 4 include files.  Three that do the actual work and a fourth to

gather them all together in one include line.  The 3 are broken into constant

definitions, structure definitions and function prototypes.

8.5 Other files

------------------------------------

DEMO.ARF - Automatic Response file for the link stage.  Note that this file

assumes that the map file is to go to the .\lst subdirectory and

the main .obj file comes from .\obj.  If you change the subdir

structure, be sure to change these as well.

DEMO.DEF - Definition file for the link stage.  Here is where you set the IOPL

bit on for a segment.

DEMO.TXT - Source for the message file.

DEMO.LIB - Library file made of all the .OBJs

DEMO.MSG - The message file.  This is where the text for messages displayed

during INIT time are kept.

DEMO.SYS - The device driver (ta da!)