<-<-<-<-<-<------------------->->->->->->

<-<-<-<-<-< I M P O R T A N T >->->->->->

<-<-<-<-<-<------------------->->->->->->

Read this file carefully before using B008, B8LIB3, ISERVER or AFSERVER

-----------------------------------------------------------------------

---------

Objective

---------

This software package provides device driver and library support for

INMOS/SGS-THOMSON Transputer boards.

The software allows you to use an INMOS Transputer software development

system with OS/2. No additional components or porting necessary!!!

To use the software, you need:

- a host adapter compatible with the INMOS/SGS-THOMSON adapter type B004 or B008

- at least 1 Transputer

- for software development, you will need in addition:

- the Transputer language development system (must be iserver- or afserver-based)

- an OS/2 software development system if you want to build your own

server or Tansputer-boosted OS/2 application

-----------

Disclaimer:

-----------

I allow you to use and distribute B008, B8LIB3, ISERVER, AFSERVER  and the

sample freely under the condition that I am in no way responsible for any damage

or loss you may suffer.

Please note that this material has only been tested with OS/2 1.3,

OS/2 2.0 GA, OS/2 11/92 PDK, OS/2 2.1 GA.

------------------

Author/Copyrights:

------------------

The B008, B8LIB3, ISERVER and sample software has been written by:

D. Frieauff

Stockerholzstr. 27

7990 Germany

Tel: 049-7541-42305

CompuServe mail: 100115,1712

------------

Introduction

------------

The Transputer is a processor specially designed for use in multiprocessor

systems. It provides an on-chip floating point unit and 4 serial com-

munication links to interface with other processors (or I/O devices)

forming a network of processor nodes.

A Transputer provides all functions required for multitasking and

communication via on-chip serial links. This is the reason why you will

find Transputers in extremly powerful multiprocessor systems, but also in

embedded control applications since a Transputer system can be built with

very few components.

Transputer entry-level systems use a PC as front-end and have an adaptor

card which carries the processor nodes. The adaptor can be configured to

the user's needs by adding processor modules, called TRAMS.

TRAMs exist with various processor types and memory capacities ranging

between 64 K to 8 MB. Up to 10 processor modules can ride on one B008-type

adaptor card. Larger systems can be easily built by adding more cards or

extending to external subsystems.

Please note, that this software package only supports adapters compatible

with the INMOS/SGS-THOMSON adapter types B004 and B008.

All host-based software was compiled with the IBM C/Set++ PDK version 3/93.

Please contact me, if you have trouble with the installation on your

system or need assistance for special hardware.

Note on compatibility issues

============================

For a B008 compatible Transputer board the following register configuration

applies:

Addresses

---------

boardbase + 0:    input data reg

boardbase + 1:    output data reg

boardbase + 2:    input status reg

boardbase + 3:    output status reg

boardbase + 0x10: reset reg (write)

boardbase + 0x10: error reg (read)

boardbase + 0x11: analyse reg

boardbase + 0x12: DMA request reg

boardbase + 0x13: interrupt ctrl reg

Reset register

--------------

bit 0 to 1: assert RESET

bit 0 to 0: deassert RESET

Analyse register

----------------

bit 0 to 1: assert ANALYSE

bit 0 to 0: deassert ANALYSE

Error register

--------------

bit 0 == 1: ERROR not asserted

bit 0 == 0: ERROR asserted

DMA request register

--------------------

bit 0 to 1: trigger DMA transfer from B008 to PC

bit 0 to 0: trigger DMA transfer from PC to B008

Interrupt control register

--------------------------

bit 0: enable interrupt on DMA end

bit 1: enable interrupt on error

bit 2: enable interrupt on B008 ready-to-receive

bit 3: enable interrupt on B008 ready-to-send

For a B004 compatible Transputer board the register configuration

is identical to that of a B008 except of DMA and Interrupt functions

which are not supported by the card.

TRANSP.ZIP

==========

TRANSP.ZIP contains the following files:

B008.SYS     - the device driver for your Transputer board

B8LIB3.DLL   - the DLL with the a.m. functions, to be used by your

application and also by the iserver

B8LIB3.H     - include file for the library

B8LIB3.LIB   - a library with Transputer functions to manage the

processor system and to communicate with it

B8LIB3.DOC   - detailed description of the library functions

ISERVER.EXE  - the host-based server program to be used by all Trans-

puter development systems which do not use the afserver

AFSERVER.EXE - for those other development systems

IBMPC.ITM    - my INMOS toolset terminal configuration file, in case

yours doesn't work

SAMPLE       - subdirectory with a sample program (mandelbrot PM

application) to validate your environment and demonstrate

how to use the Transputer system under OS/2.

README.DOC   - this file

B008.SYS

--------

This is the B008 device driver for OS/2.

The driver supports OS/2 1.3, OS/2.0 and OS/2 2.1.

The driver does not support B008 access in DOS-sessions.

Please note that the driver's DMA functions have not been tested with

EISA machines. Disable DMA and/or interrupt support in case of problems.

Installation:

1. Copy B008.SYS into your OS/2 directory (usually c:\os2 or d:\os2)

2. Add the following line to your config.sys:

DEVICE=<os2_path>\B008.SYS /A<base_address> /I<interrupt> /D<dma_channel>

where

<os2_path> is your OS/2 directory (c:\os2 or d:\os2)

<base_address> is the B008 base address in hex according to your B008

DIP-switch configuration.

<interrupt> is the B008 interrupt according to your B008 DIP-switch

configuration. Omit this switch for B004-compatible cards.

<dma_channel> is the DMA channel according to your B008 DIP-switch

configuration. Omit this switch for B004-compatible cards.

Example for B008:

DEVICE=c:os2\b008.sys /A150 /I5 /D0

Example for B004:

DEVICE=c:os2\b008.sys /A150

Note: Use of DMA and interrupt is optional; all functions using DMA or interrupts

will be automatically re-configured to non-DMA functions if these switches are omitted.

The address switch is mandatory if the base address of your board is not set to the

default (hex 150). Interrupts should be enabled (by setting the interrupt switch)

if your board is of B008 type to optimize performance. Use of DMA is more experimental,

since for most ISA-based computers this offers no speed advantage.

B8LIB3

------

This is a support library providing the basic Transputer system management

and communication functions. It is used by the ISERVER and AFSERVER,

therefore the installation of the B8LIB3.DLL file in your LIBPATH is mandatory.

The intention of this library is to provide an intermediate layer between

hardware/device-driver and your application to achieve good portability

(yes, I intend to improve this software and possibly some device-driver

calls will change. It is less likely that your software has to

be changed if your application interfaces only with the B8LIB3 library).

Installation:

1. Copy the file 'B8LIB3.DLL' into a subdirectory which is in your LIBPATH,

or, alternatively, copy into the subdirectory where the programs are which

use this DLL and make sure to have a '.\' entry in your LIBPATH.

2. If you are developing your own server or host/Transputer application:

2.1. Copy the file 'B8LIB3.LIB' into a subdirectory on your LIB path

(so that the OS/2 linker can find it)

2.2. Copy the file 'B8LIB3.H' into a subdirectory on your INCLUDE path

(so that your compiler can find it)

2.3. Read the file 'B8LIB3.DOC'

ISERVER

-------

This is the OS/2 version of the INMOS iserver. It is totally rewritten

and much faster than the original. See the make-file in the SAMPLE

directory for how to use it.

Installation:

Copy the file 'ISERVER.EXE' into a subdirectory on your PATH and make

sure it is loaded instead of the original INMOS file. If this is a new

install, don't forget to add the toolset-specific environment settings

in your config.sys.

AFSERVER

--------

This is the OS/2 version of the 3L afserver. Up to now it has only been

tested with the 3L C-compiler version 2.0.

SAMPLE

------

This a (single) Transputer-based version of the famous mandelbrot set

generator (actually it will generate a julia set). It is a PM application

It has been tested with a 8514A graphic adapter only!

In addition to the mandelbrot generator this program provides a Transputer

link test and performance measurement function.

You need 'NMAKE.EXE' to recompile with the supplied make-files.

In case the sample won't compile with your development system, modify the make

file accordingly (no magic about it). In case it doesn't run, some IBM CSet/2

specific DLLs may be missing; in this case the only solution is to recompile.

The kernel software is written in OCCAM. Boot the Transputer with the file

'SAMPLE.BTL'; you do not have to re-compile if you have a T800 or T805.

Try out some interesting features of this program:

- zoom-in with the left mouse button

- start a CPU load meter in a second window and watch the influence of

the mandelbrot calculation on your CPU performance (to demonstrate

the real benefit of a Transputer system in your computer)

COMMENTS WELCOME

----------------

Please give me some feedback about this software and your application to help

further improvement and development of OS/2 Transputer software.