APAR= PJ12214

DATA IS CORRUPTED ON A: FLOPPY DRIVE ON RETURN FROM DOSDEVIOCTL

CALL TO LOCK FLOPPY DEVICE.

******************************************************************************

INSTALLK.EXE --

Please read the following information about the fix installation program BEFORE

you attempt to install the fixes provided. The ABSTRACT explains how the fixes

are applied to your system, and how they are managed so that additional fixes

can be applied in the future. Following the ABSTRACT are the installation

instructions, complete with examples of how to invoke the installation program.

Following the installation instructions are the descriptions of the fixes you

have received and the OS/2 system module being fixed.

ABSTRACT:

---------

The fix diskette you received contains the following files:

INSTALLK.EXE -- the fix installation program.

MODULE.LOG   -- input file that tells the installation program which OS/2 system

modules are being fixed, the file containing the fix for each

module, and the location of the modules following an OS/2 2.1

installation.

*.*          -- the modules containing the fix for each APAR described above.

README.1ST   -- Fix installation instructions.

README.DOC   -- this file, which contains the description of each APAR fixed in

the modules provided, an explanation of the files contained on

the diskette, an explanation of how fixes are applied to your

system, and instructions on how to invoke INSTALLF.EXE.

Presently, the fixes provided are new full-sized system modules, which will

replace the corresponding OS/2 2.1 GA-level system modules. Eventually, fixes

will be applied to your system modules using "patches" instead of replacing them

with new full-sized system modules. A patch can ONLY be applied to an OS/2 2.1

GA-level system module, so it's important that you follow the instructions

provided so that fixes can be applied in the future to both system modules that

have already been fixed and those that haven't. INSTALLF.EXE will "archive" the

system module being fixed to either your disk or a diskette. One of the

parameters you specify when invoking INSTALLF.EXE allows you to choose the drive

where you want the files archived, but whatever you choose, it is absolutely

necessary that you maintain these files as they will be needed each time you

want to apply fixes to your system. For example, if you specified D:, the

modules would be archived in D:\ARCHIVE.

When a fixed system module replaces the corresponding OS/2 2.1 GA-level system

module for the first time, the OS/2 2.1 GA-level system module is saved in the

\ARCHIVE directory of the drive you specified when invoking INSTALLF.EXE. This

is considered the "current" fix. If the same system module needs to be replaced

again, the current fixed system module is backed-up on the same drive you

specified to archive files to, and the new fixed system module is copied to the

directory where the current system module had been. For example, if you

specified D:, the current system module would be saved in D:\BACKUP.

**** NOTE 1 ********************************************************************

Presently, "true" Backout is not supported in INSTALLF.EXE. If you have a need

to return your system to the OS/2 2.1 GA-level (ie. you need to reinstall a fix

due to some failure), you will be required to copy the OS/2 2.1 GA-level module

you need from the \ARCHIVE directory into the directory where the corresponding

system module exists. If you have a need to return your system to a previous fix

level, you will be required to copy the system module you need from the \BACKUP

directory into the directory where the corresponding system module exists.

**** END NOTE 1 ****************************************************************

INSTALLF.EXE maintains a record of all fixes applied to your system in the log

file LOGU001.OS2. This log file is maintained on the same drive where you have

OS/2 2.1 installed, and in the directory \OS2\INSTALL. Another parameter you

specify when invoking INSTALLF.EXE is the drive where OS/2 2.1 is installed. For

example, if OS/2 2.1 is installed on the C: drive, the log file would be

maintained in C:\OS2\INSTALL.

**** NOTE 2 ********************************************************************

IT IS ABSOLUTELY NECESSARY that you maintain the location of the 2 items just

described to you: 1.) the archived OS/2 2.1 GA-level system modules, and 2.) the

log file LOGU001.OS2. The archived system modules and the log file are REQUIRED

each time you want to apply fixes to your system. If you specified the disk

drive on which you want to store the system modules, DO NOT delete the directory

\ARCHIVE, AND, DO NOT delete the directory \OS2\INSTALL where the log file is

stored. If you specified a diskette drive on which you want to store these

files, DO NOT erase the contents or lose the diskettes. If these files are not

available, you will have to re-install the OS/2 2.1 GA-level diskettes if you

want to apply new fixes.

**** END NOTE 2 ****************************************************************

KEY PROCEDURES PERFORMED BY INSTALLF.EXE:

-----------------------------------------

1.) ARCHIVING -- This is the process of storing an OS/2 2.1 GA-level system

module in the directory \ARCHIVE on the specified logical

drive.

2.) BACKING-UP -- This is the process of saving current fixed system modules

that exist from previous fixes in the directory \BACKUP on the

specified logical drive. If there is ever a need to return the

system to a previous fix level, then you can copy the system

module you need from this directory into the directory where

the corresponding system module exists.

3.) SERVICE LOGGING -- INSTALLF.EXE maintains a record of all service activity

performed on your system in the file LOGU001.OS2.

INSTALLF.EXE requires this file to determine if fixes

have ever been applied to your system, and if so, where

to locate the files that have been stored. If you are

installing multiple fixes from a pervasive 'pak',

INSTALLF.EXE will record the service information to the

temporary file LOGU001.BAK. When all fixes have been

applied, LOGU001.BAK will be written to LOGU001.OS2. If

you interrupt INSTALLF.EXE while it is applying fixes, do

not erase LOGU001.BAK. INSTALLF.EXE needs this file to

continue applying fixes when you re-run INSTALLF.EXE.

INSTALLATION INSTRUCTIONS:

--------------------------

**** NOTE 3 ********************************************************************

INSTALLF.EXE will attempt to copy the fixed system module into the path where it

was installed during the installation of OS/2 2.1. If the file does not exist in

the assumed path, you will be prompted for its location. For example, if the

file being replaced is actually located in \OS2\DLL_BAK, you would enter that

path-name when prompted.

**** END NOTE 3 ****************************************************************

1.) If the system module being fixed is being used by the operating system, it

will be necessary to do the following so that a patch can be applied:

a.) Start your system using the OS/2 2.1 Installation Diskette.

b.) When prompted, insert Diskette 1.

c.) When prompted to continue or escape, press the ESC key to get to a

command prompt. Continue to step 2.

2.) To install the fixes, type the following at the command prompt and then

press enter:

INSTALLF source dest arch-back

where:

source  is the source drive where the files from the fix diskette can be

found. INSTALLF.EXE can apply the fixes from either your disk

drive or diskette drive. If you choose to apply the fixes from

your disk drive, make sure you copy ALL files from the fix

diskette to the disk drive. For example, if you copied the files

from the fix diskette to C:\FIX, then source would be C:\FIX. If

you want to apply fixes from the diskette drive, then source

would be A:.

dest  is the drive letter of the system partition where OS/2 2.1 is

installed. For example, if OS/2 2.1 is installed on C:, then

dest would be C:.

arch-back  is the logical drive where you want the OS/2 2.1 GA-level

modules that are being fixed stored, AND, where fixed modules

from previous fixes are backed-up. The directories \ARCHIVE and

\BACKUP will be created. For example, if you want these files

stored on your D: drive, then arch-back would be D:. If you want

these files stored on a diskette in your A: drive, then

arch-back would be A:.

EXAMPLE:  If you choose to install the fixes from your A: diskette drive,

and OS/2 2.1 is installed on C:, and you want \ARCHIVE and \BACKUP

on your D: drive, then you would enter the following:

INSTALLF A: C: D:

As soon as INSTALLF.EXE has been started, it is important that you allow it to

finish installing all of the fixes that you have received. The message "Fix

successfully installed" will be displayed when the installation is complete. If

this message does not appear, or INSTALLF.EXE ended for some other reason,

please follow the error message and re-run INSTALLF.EXE if necessary.

3.) The service log maintained in \OS2\INSTALL\LOGU001.OS2 is stored in an

unreadable format. If there is a service requirement to view the service

log, you can view the log file by entering the following:

INSTALLF drive

where:

drive  is the drive where OS/2 2.1 is installed.

EXAMPLE:  If OS/2 2.1 is installed on the C: drive, you would enter the

following:

INSTALLF C:

If you would like to print the log file, simply redirect the output of the

previous command to a print device attached to your system. For example:

INSTALLF C: > lpt1:

**** NOTE 4 ********************************************************************

We suggest that you store INSTALLF.EXE on your disk drive so that if you ever

need to view the service log, the program is available to do so.

**** END NOTE 4 ****************************************************************