PRINTER TEST TOOL (PTT)

_______________________

The Printer Test Tool (PTT) is a Presentation Manager(*)

_______________________

application that enables the user to select one or more

tests and execute them.  A script interface permits the

PTT to run predefined test-case scripts automatically.

OVERVIEW

________

The PTT provides a mechanism for testing printer device

drivers by calling graphics programming interface (GPI)

functions.  Each call, on execution, produces quantifi-

able output by which the test can be measured.  As each

test is run, the PTT monitors the return codes from the

Gre calls and writes the results to a log file.  The

current log file can be displayed at any time during

manual testing.

PTT ARCHITECTURE

________________

The PTT is comprised of two major components:

o   A PM master program (PTT.EXE) that provides the user

interface for test selection and execution

o   Test-case dynamic-link libraries (DLLs) that contain

the actual test-case executable code.

PTT MASTER PROGRAM

PTT.EXE enables the user to do the following:

o   Select one or more tests from the test cases avail-

able

o   Execute the selected test cases on demand

o   Specify a test-case PM environment (PS units, PS

types, and so forth)

o   Display the PTT test log

o   Set the PTT test-logging level

o   Display the PTT release level

o   Exit PTT.

---------------

(*) Trademark of the IBM Corporation.

Printer Test Tool (PTT)  1

Command-line options enable the user to specify the

following:

o   Output log file name

o   Input script file name

o   Level of logging to capture

o   Search patterns for DLLs to be preloaded.

TEST-CASE DLLS

Each test-case DLL contains tests for a family of GPI

______

calls.  See "GPI Function Tests (32-Bit)" on page 23 for

descriptions of the supplied test-case DLLs and the GPI

functions called.

NOTE:  The test-case DLLs must reside in the same direc-

tory as the PTT executable.

OPERATING PTT

_____________

The PTT can be started from the command line in either an

OS/2(*) window or a full-screen session.  The PTT can

also be started from the OS/2 Desktop or any other folder

by using its icon.  When using an icon, command-line

options are specified in the OPTIONAL PARAMETERS field of

the Settings menu for the icon.

SPECIFYING COMMAND OPTIONS

All command-line options are specified with a hyphen (-),

followed by a case-sensitive alphabetic character.  The

alpha character is followed immediately by the parameter

value, if any, with no intervening spaces.  Following are

examples of PTT command lines.

Load all DLLs in the current directory and set the logging level to 8:

PTT -T*.DLL -L8

Preload the GRELINE DLL.

(Search LIBPATH to locate the DLLs.)

PTT -Tgreline.dll

---------------

(*) Trademark of the IBM Corporation.

2

Executing PTT with the -? command-line option displays a

list of the valid command-line options.

PTT -?

MENU SELECTIONS

The following options are available on the PTT menu:

TESTCASES

_________

TEST GROUPS           Lists all DLLs loaded.

TEST CASES            Lists all test cases available for

the selected Test Groups.

ADD MATCH             Adds the selected match pattern

from the chosen list.

SAVE                  Saves the entries selected.

CANCEL                Cancels this action and returns to

the main options.

DELETE MATCH          Deletes the selected match pattern

from the test case list.

HELP                  Not available.

ACTIONS

_______

DISPLAY TO SCREEN     Displays the output of the test

case to the screen.

START PRINTING        Prints the test case to an attached

printer.

OPTIONS

_______

CONFIGURATION         Configures printer, DC, data type,

PS Units, PS type.  See "Config-

uring the Device Context and Pres-

entation Space" for more

information about configuring PTT

DLLs.

DISPLAY LOG WINDOW    Displays PASS/FAIL logging window.

CHANGE LOG WINDOW     Selects a level (1 - 9).

HELP

____

HELP                  Not available.

ABOUT                 Displays an application message and

version number.  There is no

on-line Help with PTT.

EXIT

____

( NO DROP-DOWN )      Exits PTT.  (Available only when

using a mouse.)  Use the acceler-

ator key, ALT+F4.

Printer Test Tool (PTT)  3

LOCATING, PRELOADING, AND LOADING TEST-CASE DLLS

Each test case DLL passes a list of its test cases to PTT

when PTT initializes the DLL after loading.  A test-case

index file associates each test-case name with the DLL

that contains it.  When duplicate test cases are encount-

ered while loading DLLs and test-case index files, only

the first instance of a test case is retained.

PTT locates the test cases using the following sequence:

1.  DLLs specified using the -T command-line option are

preloaded during PTT startup.  Preloaded DLLs remain

resident for the duration of the PTT session.

The -T command-line option specifies a search pattern

for locating the test-case DLLs to be preloaded.  The

-T option value can contain global file-name charac-

ters (wildcards) in the file name.  Following is an

_________

example of the command used to preload DLLs:

Preload all DLLs in the PTT\DLL directory on the current drive.

PTT -T\PTT\DLL\*.DLL

The -T command-line option can be specified multiple

times (on the command line) with different paths to

the DLLs.

2.  At this point, if no test cases have been identified

to PTT, PTT attempts to preload all the test-case

DLLs it can locate using a *.DLL search pattern.

This searches the current directory.

3.  If PTT fails to locate any test-case DLLs in the

current directory, a message to that effect is

issued.

An OK check box in the informational window permits

the user to continue bringing up PTT.

PTT LOG FILE

____________

PTT writes information to a log file.  The information to

be logged is determined by the current logging level,

_____________

which is hierarchical.  At any given logging level, all

information at that level and below is logged.

4

SPECIFYING THE PTT LOG FILE NAME

A path and file name for the PTT log file can be speci-

fied on the PTT command line using the -F option.  If no

log file name option is given on the command line, the

default file name, PTT_LOG in the current directory, is

used.  If a log file does not exist when PTT is started,

the log file is created automatically.  If the log file

does exist, the new log is appended to the existing log

file.

SPECIFYING THE PTT LOGGING LEVELS

The PTT logging level can be specified on the PTT command

line or changed from the Options pull-down menu after PTT

has started.  To set the logging level on the command

line, specify a -Lx option, where x is the desired

_               _

logging level, using a number in the range 1-9.  To

change the logging level after PTT has started, select

Change logging level from the Options pull-down menu or

____________________

use the accelerator key, ALT+L.  The default PTT logging

level is 1, L_HDR

The following table describes the logging levels sup-

ported by PTT:

+-----------------------------------------------------+

| Table 1 (Page 1 of 2). PTT Logging Levels           |

+-------+-----------------+---------------------------+

| LEVEL | LEVEL NAME      | DESCRIPTION               |

+-------+-----------------+---------------------------+

| 1     | L_HDR           | Test Pass/Fail logging    |

|       |                 | only.                     |

+-------+-----------------+---------------------------+

| 2     | L_ABORT         | Test case failed.  Test   |

|       |                 | is incomplete.            |

+-------+-----------------+---------------------------+

| 3     | Reserved.       | Do not use.               |

+-------+-----------------+---------------------------+

| 4     | L_FAIL          | Internal test-case        |

|       |                 | failure.  An interface    |

|       |                 | other than the call cur-  |

|       |                 | rently being tested has   |

|       |                 | experienced a functional  |

|       |                 | or other type of failure. |

+-------+-----------------+---------------------------+

| 5     | Reserved        | Do not use.               |

+-------+-----------------+---------------------------+

Printer Test Tool (PTT)  5

+-----------------------------------------------------+

| Table 1 (Page 2 of 2). PTT Logging Levels           |

+-------+-----------------+---------------------------+

| LEVEL | LEVEL NAME      | DESCRIPTION               |

+-------+-----------------+---------------------------+

| 6     | L_WARN          | WARNING:  The presenta-   |

|       |                 | tion space is improperly  |

|       |                 | configured for the test   |

|       |                 | case (or a similar        |

|       |                 | error).                   |

+-------+-----------------+---------------------------+

| 7     | L_TRACE         | Trace script commands,    |

|       |                 | incorrect values, or      |

|       |                 | other unexpected results. |

+-------+-----------------+---------------------------+

| 8     | L_LOTRACE       | Location trace.  Log      |

|       |                 | entries and exits of      |

|       |                 | internal test-case rou-   |

|       |                 | tines.                    |

+-------+-----------------+---------------------------+

| 9     | L_DEBUG         | Debugging.  Test-case     |

|       |                 | debugging information,    |

|       |                 | internal test-case        |

|       |                 | results, errors, and      |

|       |                 | other information.        |

+-------+-----------------+---------------------------+

DISPLAYING THE PTT LOG FILE

To display the PTT log file for the current session,

select DISPLAY LOG WINDOW from the OPTIONS menu or use

ALT+D.

OPERATING PTT AUTOMATICALLY

___________________________

The following provides instructions for specifying a

script file, the script file structure, and the script

commands PTT supports.

SPECIFYING A SCRIPT FILE

If the name of a PTT script file is provided, PTT exe-

cutes the supplied script automatically.  To specify a

script file, use the -I command-line option, as in the

following example:

PTT -IC:\PTT\SCRIPTS\demo1.scr

6

SCRIPT COMMANDS

+--- USAGE NOTES ---------------------------------------+

|                                                       |

| 1.  Blank lines and comments can appear anywhere in a |

|     script file.                                      |

|                                                       |

| 2.  Comments must be preceded by an asterisk (*) as   |

|     the first non-blank character on the line.        |

|                                                       |

| 3.  Script commands are not case-sensitive, except    |

|     for the test-case name operand of the CHOOSE      |

|     script command.                                   |

|                                                       |

+-------------------------------------------------------+

CHOOSE (Test-Case Selection Command)

The operand of the CHOOSE script command specifies the

test case to be executed when a DISPLAY or PRINT script

command is encountered.  The test-case name operand is

_____________________________

case sensitive and must match the specified test-case

_____________________________________________________

name exactly.  In script files, test cases are selected

____________

by their test-case name only; the test-group name is not

used.  If the test-case name is not found, the CHOOSE

command is ignored.

DCTYPE (Device Context Selection Command)

The operand of the DCTYPE script command identifies the

device context to be used for subsequent test cases.

Valid operands are:

OD_QUEUED                     Output is to be queued.

(This is the default.)

OD_DIRECT                     Output is not to be queued.

OD_INFO                       The device context is to be

used only to retrieve

information.

OD_METAFILE                   The device context is to be

used to write a metafile.

OD_METAFILE_NOQUERY           Same as OD_METAFILE, except

querying of the attributes

is not permitted.

OD_MEMORY                     A device context that is

used to contain a bit map.

Printer Test Tool (PTT)  7

DELAY (Pause Script File Execution Command)

When a DELAY command is encountered in a PTT script file,

execution of the script is suspended for 5 seconds.  The

DELAY script command has no operand.

DISPLAY (Execute a Test Case Command)

The DISPLAY script command initiates immediate execution

of the test case selected by the most recent CHOOSE

command.  If the DISPLAY command is not preceded by a

CHOOSE command anywhere in the script file, the first

test case identified during PTT startup is executed.  The

DISPLAY script command has no operand.

PRINT (Execute a Test Case Command)

The PRINT script command initiates immediate execution of

the test case selected by the most recent CHOOSE command.

If the PRINT command is not preceded by a CHOOSE command

anywhere in the script file, the first test case identi-

fied during PTT startup is executed.  The PRINT script

command has no operand.

EXIT (Terminate PTT Command)

When an EXIT command is encountered in a PTT script file,

PTT terminates immediately.  The EXIT script command has

no operand.

PSTYPE (Presentation Space Type Selection Command)

The operand of the PSTYPE script command identifies the

presentation space type to be used for subsequent test

cases.  Valid operands are:

GPIT_MICRO          Micro-presentation space.

GPIT_NORMAL         Normal presentation space.  (This is

the default.)

PSUNITS (Presentation Space Units Selection Command)

The operand of the PSUNITS script command identifies the

presentation space units to be used for subsequent test

cases.  Valid operands are:

PU_ARBITRARY        Application-convenient units.

8

PU_PELS             Pel coordinates.  (This is the

default.)

PU_LOMETRIC         Units of 0.1 mm.

PU_HIMETRIC         Units of 0.01 mm.

PU_LOENGLISH        Units of 0.01 inch.

PU_HIENGLISH        Units of 0.001 inch.

PU_TWIPS            Units of 1/1440 inch.

QPROCESS

The QPROCESS command identifies a queue processor that is

associated with a queue driver.  Each queue has its own

queue processor that prints a spool file.  Queue drivers

must support the standard data types.

DATATYPE

The data type determines how the queue driver writes data

to the hardcopy device.  The operand to the DATATYPE

command is either of these two standard data types:

PM_Q_STD            The method used by the queue driver

to write data to the hardcopy device.

PM_Q_STD performs the following:

1.  Opens a DC for the hardcopy

device using DevOpenDC and

enables it as an OD_DIRECT

device.

2.  Calls DevEscape with

DEVESC_STARTDOC.

3.  Writes data that is in metafile

format using GpiPlayMetafile.

4.  Calls DevEscape with

DEVESC_ENDDOC.

5.  Closes hardcopy device DC using

DevCloseDC.

PM_Q_RAW            The method used by the queue driver

to write data to the hardcopy device.

PM_Q_RAW performs the following:

Printer Test Tool (PTT)  9

1.  Opens a DC for the hardcopy

device using DevOpenDC and

enables it as an OD_DIRECT

device.

2.  Calls DevEscape with

DEVESC_STARTDOC.

3.  Writes the data using DevEscape

with DEVESC_RAWDATA.

4.  Calls DevEscape with

DEVESC_ENDDOC.

5.  Closes hardcopy device DC using

DevCloseDC>

PRINTER_NAME

The PRINTER_NAME command is executed in a PTT script file

to change the selected device driver using the device

name.  This name corresponds with the printer name sup-

plied when the device was configured.  Device-driver

selection is based on the supplied operand.  The printer

name must be valid and associated with the device driver.

PRINTER_QUEUE

The PRINTER_QUEUE command is executed in a PTT script

file to change the selected device driver using the name

of the device queue.  This name corresponds with the

queue name supplied when the device was configured.

Device-driver selection is based on the supplied operand.

The queue name must be valid and associated with the

device driver.

PRINTER_PORT

The PRINTER_PORT command is executed in a PTT script file

to change the selected device driver using the physical

device assignment.  Device-driver selection is based on

the supplied operand.  The printer port must exist as a

valid port and must be associated with the device driver.

10

PTT COMMAND-LINE OPTIONS SUMMARY

________________________________

-F        Specifies the file name to use for logging.

(The file name can be fully qualified to

include the path.)

-I        Specifies the script file name for batch proc-

essing.

-L        Specifies the logging level to use.

-T        Specifies the default DLL search pattern.

-?        Displays the PTT command-line options.

NOTE:  All specified DLLS are loaded before the

Help window is displayed.

-G        Specifies the Testlog for the selected method

of logging.

-X        Used only by Testlog.

#B        Used only by Testlog.

#I        Used only by Testlog.

#S        Used only by Testlog.

#T        Used only by Testlog.

Printer Test Tool (PTT)  11

SCRIPT FILE STRUCTURE

A script file is a list of PTT commands that are exe-

cuted, one at a time, in the order that the commands

appear in the script file.  When the end of the script

file is reached, PTT terminates.  Any PTT script command

can appear anywhere in a script file.

PTT Sample Script File

Following is an example of a PTT script file.

*  PTT script file

* Select a Device Type.

DATATYPE PM_Q_STD

* Select a Device Context.

DCTYPE  OD_QUEUED

* Select a Presentation Space Type.

PSTYPE  GPIT_NORMAL

* Select a Presentation Space Unit.

PSUNITS PU_PELS

* Select several of the available test cases and execute them,

* pausing 5 seconds between the first and second test case.

*       Test Group ...: GPI ARC

*       Test DLL   ...: GPIARC.DLL

CHOOSE GpiArcApp

DISPLAY

DELAY

CHOOSE GpiArcPre2

DISPLAY

*  This is an example of a commented-out script command.

*

*       Test Group ...: GPI AREA

*       Test DLL   ...: GPIAREA.DLL

* CHOOSE GpiAreaApp

* DISPLAY

CHOOSE GpiBeginAreaExh

DISPLAY

CHOOSE GpiSetAndQryPatternSetExh

DISPLAY

12

*       Test Group ...: GPI BITMAP

*       Test DLL   ...: GPIBTMP.DLL

CHOOSE GpiBitApp2

DISPLAY

CHOOSE GpiQueryDeviceBitmapFormatsExh

DISPLAY

*       Test Group ...: GPI CHAR

*       Test DLL   ...: GPICHAR.DLL

CHOOSE GpiCharStringPosAtAndQryExh

DISPLAY

CHOOSE GpiCharStringPosAndQryShadeExh

DISPLAY

*       Test Group ...: GPI CLIP

*       Test DLL   ...: GPICLIP.DLL

CHOOSE GpiClipApp

DISPLAY

CHOOSE GpiClipPre

DISPLAY

*       Test Group ...: GPI COLOR

*       Test DLL   ...: GPICOLOR.DLL

CHOOSE GpiColorApp1

DISPLAY

CHOOSE GpiQueryColorIndexExh

DISPLAY

*       Test Group ...: GPI CORRELATION

*       Test DLL   ...: GPICORRE.DLL

CHOOSE GpiCorrelatePre

DISPLAY

*       Test Group ...: GPI ELEMENT

*       Test DLL   ...: GPIELMNT.DLL

CHOOSE GpiDeleteElementExh

DISPLAY

CHOOSE GpiElementAndQryExh

DISPLAY

*       Test Group ...: GPI FONTS

*       Test DLL   ...: GPIFONT.DLL

CHOOSE GpiCreateLogFontExh

DISPLAY

Printer Test Tool (PTT)  13

CHOOSE GpiQueryKerningPairsExh

DISPLAY

*       Test Group ...: GPI IMAGE

*       Test DLL   ...: GPIIMAGE.DLL

CHOOSE GpiImageApp

DISPLAY

CHOOSE GpiImagePre

DISPLAY

*       Test Group ...: GPI LINE

*       Test DLL   ...: GPILINE.DLL

CHOOSE GpiBoxExh

DISPLAY

CHOOSE GpiLineExh

DISPLAY

CHOOSE GpiLinePre

DISPLAY

*       Test Group ...: GPI Marker

*       Test DLL   ...: GPIMARK.DLL

CHOOSE GpiMarkerPre

DISPLAY

CHOOSE GpiMarkerExh

DISPLAY

CHOOSE GpiPolyMarkerExh

DISPLAY

*       Test Group ...: GPI PATH

*       Test DLL   ...: GPIPATH.DLL

CHOOSE GpiFillPathExh

DISPLAY

*       Test Group ...: GPI PCLIP

*       Test DLL   ...: GPIPCLIP.DLL

CHOOSE GpiSetClipPathExh1

DISPLAY

CHOOSE GpiSetClipPathExh2

DISPLAY

*       Test Group ...: GPI REGION

*       Test DLL   ...: GPIREGN.DLL

CHOOSE GpiCombineRegionExh1

DISPLAY

14

CHOOSE GpiQueryRegionBoxExh

DISPLAY

*       Test Group ...: GPI SEGMENT

*       Test DLL   ...: GPISEG.DLL

CHOOSE GpiDeleteSegmentExh

DISPLAY

CHOOSE GpiDrawChainExh2

DISPLAY

*       Test Group ...: GPI SHADE

*       Test DLL   ...: GPISHADE.DLL

CHOOSE GpiShadePelSpace

DISPLAY

CHOOSE GpiShadeOverlapCircle1

DISPLAY

CHOOSE GpiShadeOverlapCircle2

DISPLAY

*       Test Group ...: GPI STANDARD

*       Test DLL   ...: GPISTD.DLL

CHOOSE GpiStandardPre1

DISPLAY

*       Test Group ...: GPI View

*       Test DLL   ...: GPIVIEW.DLL

CHOOSE GpiSetDefViewingLimitsExh2

DISPLAY

CHOOSE GpiViewApp

DISPLAY

*       Test Group ...: GPI TRANSFORM

*       Test DLL   ...: GPIXFORM.DLL

CHOOSE GpiSetAndQryModelXformMatPre

DISPLAY

CHOOSE GpiSetAndQryViewingXformMatPre

DISPLAY

*       Test Group ...: GRE ARC

*       Test DLL   ...: GREARC.DLL

CHOOSE GreArcPre1

DISPLAY

CHOOSE GreArcExh

DISPLAY

Printer Test Tool (PTT)  15

*       Test Group ...: GRE ATTRIBUTE

*       Test DLL   ...: GREATTR.DLL

CHOOSE GreDeviceSetGetAttrExh

DISPLAY

CHOOSE GreSetLineAttrExh

DISPLAY

*       Test Group ...: GRE ATTRIBUTES2

*       Test DLL   ...: GREATT2.DLL

CHOOSE GreSetAndGetAttributesExh2

DISPLAY

CHOOSE GreSetAndGetAttributesExh3

DISPLAY

CHOOSE GreSetAndGetAttributesExh4

DISPLAY

CHOOSE GreSetAndGetAttributesExh6

DISPLAY

CHOOSE GreSetAndGetDefaultAttrsExh3

DISPLAY

CHOOSE GreSetAndGetDefaultAttrsExh6

DISPLAY

*       Test Group ...: GRE BITMAPS2

*       Test DLL   ...: GREBIT2.DLL

CHOOSE GreBitApp1

DISPLAY

CHOOSE GreBitApp2

DISPLAY

*       Test Group ...: GRE BITMAPS1

*       Test DLL   ...: GREBTMP.DLL

CHOOSE GreBitPre1

DISPLAY

CHOOSE GreBitBltExh

DISPLAY

CHOOSE GreSetBitmapBitsExh

DISPLAY

*       Test Group ...: GRE CLIP

*       Test DLL   ...: GRECLIP.DLL

CHOOSE GreClipPre

DISPLAY

16

CHOOSE GreSetXformRectExh

DISPLAY

*       Test Group ...: GRE FONT

*       Test DLL   ...: GREFONT.DLL

CHOOSE GreUnloadFontExh

DISPLAY

*       Test Group ...: GRE FUNCTIONS2

*       Test DLL   ...: GREGEN2.DLL

CHOOSE GreDisjointLinesPre

DISPLAY

*       Test Group ...: GRE LINE

*       Test DLL   ...: GRELINE.DLL

CHOOSE GreLinePre

DISPLAY

*       Test Group ...: GRE MARKER

*       Test DLL   ...: GREMARK.DLL

CHOOSE GreMarkerPre

DISPLAY

*       Test Group ...: GRE PATH

*       Test DLL   ...: GREPATH.DLL

CHOOSE GreBeginAndEndAreaExh1

DISPLAY

CHOOSE GreBeginAndEndPathExh

DISPLAY

CHOOSE GreCloseFigureExh

DISPLAY

*       Test Group ...: GRE REGION

*       Test DLL   ...: GREREGN.DLL

CHOOSE GrePaintRegionExh

PRINT

*       Test Group ...: GRE STANDARD

*       Test DLL   ...: GRESTD.DLL

CHOOSE GreStandardPre2

DISPLAY

*       Test Group ...: GRE TEXT

*       Test DLL   ...: GRETXTST.DLL

CHOOSE GreTextApp1

DISPLAY

*       Test Group ...: GRE VIEW

*       Test DLL   ...: GREVIEW.DLL

Printer Test Tool (PTT)  17

CHOOSE GreSetAndGetDefViewingLimitsExh

DISPLAY

*       Test Group ...: GRE TRANSFORM

*       Test DLL   ...: GREXFORM.DLL

CHOOSE GreSetAndGetPageViewportPre

DISPLAY

* Terminate PTT

EXIT

CONFIGURING THE DEVICE CONTEXT AND PRESENTATION SPACE

_____________________________________________________

PTT enables you to specify the device context,

presentation-space type, and presentation-space units to

use when executing test cases.  These parameters can be

changed during manual testing or through script commands

during automated testing.  To change the configuration

manually, select CONFIGURATION from the OPTIONS pull-down

menu or use ALT+C.

SETTING THE DEVICE DRIVER

To set or change the device driver, select one of the

optional printer device drivers available in the Printer

Queue, Printer Name, and Printer Port window by clicking

the mouse on the desired driver.  If a device driver is

installed after PTT is initiated, the user can select

Refresh Printer Driver List to list the other optional

___________________________

device drivers.

SETTING THE DEVICE CONTEXT

To set or change the device context, select one of the

following options from the Device Context section of the

PTT Configuration dialog panel:

OD_QUEUED                   Output is to be queued.

(This is the default.)  This

option is for hardcopy

drivers only.  The driver

opens a spool file and uses

the spooler's Spl interface

to send output to that file.

The driver must consider the

data type if this option is

chosen.  The following data

types are available:

18

PM_Q_STD       The hardcopy

driver uses

the spooler to

create a

device-

independent

spool file

using the Spl

interfaces.

See "Setting

_______

the Data

________

Type".

____

PM_Q_RAW       The hardcopy

driver proc-

esses the Gre

functions to

generate

device-

specific

output data.

This data is

written to a

spool file

using the

spooler Spl

interfaces.

See "Setting

_______

the Data

________

Type".

____

OD_DIRECT                   Output is not queued.  The

presentation driver processes

the Gre routines to generate

device-specific data.  Hard-

copy drivers use the Prt

interface routines to pass

the output to the physical

device driver.

OD_INFO                     The presentation driver does

not generate any output.  The

information device context is

used only to retrieve infor-

mation.

OD_METAFILE                 The device context is used to

write a metafile.  The option

is handled by PMGPI and never

is passed to the device

driver.

Printer Test Tool (PTT)  19

OD_METAFILE_NOQUERY         This is the same as

OD_METAFILE, except querying

of attributes is not per-

mitted.  The option is

handled by PMGPI and never is

passed to the device driver.

OD_MEMORY                   This is a device context that

is used to contain a bit map.

The device driver processes

the output data the same as

for an OD_DIRECT device type,

except the output is written

to a bit map that is compat-

ible with the physical

device.  (PTT creates the bit

map and associates it with

the device context.)

SETTING THE QUEUE PROCESSOR

To select the data type to be used when executing test

cases, use the mouse to click on the queue processor that

is available in the window.

SETTING THE DATA TYPE

To select the data type to be used when executing test

cases, select one of the following options in the DATA

TYPE check box of the PTT Configuration dialog panel:

PM_Q_RAW            Raw data.

PM_Q_STD            Metafile-formatted data.  (This is

the default.)

SETTING THE PRESENTATION-SPACE TYPE

To select the presentation-space type to be used when

executing test cases, choose one of the following options

from the PS TYPE section of the PTT Configuration dialog

panel:

GPIT_MICRO          Micro-presentation space.

GPIT_NORMAL         Normal presentation space.  (This is

the default.)

20

SETTING PRESENTATION-SPACE UNITS

To set the presentation page-size units for test-case

execution, select one of the following from the PS UNITS

section of the PTT Configuration dialog panel:

PU_ARBITRARY        Application-convenient units.

PU_PELS             Pel coordinates.  (This is the

default.)

PU_LOMETRIC         Units of 0.1 mm.

PU_HIMETRIC         Units of 0.01 mm.

PU_LOENGLISH        Units of 0.01 inch.

PU_HIENGLISH        Units of 0.001 inch.

PU_TWIPS            Units of 1/1440 inch.

SELECTING AND EXECUTING TEST GROUPS AND TEST CASES

__________________________________________________

The test cases to be executed are selected from the Test

Case Selection dialog panel. Select TESTCASE from the PTT

menu bar or use ALT+T.  One or more test cases can be

selected for execution during manual operation.  Use

script commands to select individual test cases during

automated operation.

TEST GROUP SELECTION

There is a 1:n relationship between test groups and test-

case DLLs.  Each test group represents a specific DLL.

Before a test case can be selected, the test group con-

taining the test case must be selected from the TEST

GROUPS section of the PTT Test Case Selection dialog

panel.  When you select a test group, the list of test

cases it contains is added to the Test Cases section of

the dialog panel.  De-selecting a test group removes the

list of test cases it contains from the Test Cases

section.

NOTE:  Test groups must be de-selected specifically.  PTT

never de-selects a test group automatically.

Printer Test Tool (PTT)  21

TEST CASE SELECTION

Once the test groups are selected, the individual test

cases can be selected from the TEST CASES section of the

PTT Test Case Selection dialog panel.  Test cases can be

selected and de-selected individually, or groups of test

cases can be selected with the match patterns provided

using the ADD MATCH and DELETE MATCH push buttons.

NOTE:  Test cases must be de-selected specifically unless

the user's test group is de-selected.  When that

happens, all test cases are de-selected and made

unavailable.  The PTT never de-selects a test case

automatically.

SAVING SELECTED TEST CASES

After selecting one or more test cases, use the SAVE push

button or ALT+S to close the Test Case Selection dialog

panel.

EXECUTING TEST CASES

After selecting and saving one or more test cases, select

Display to Screen from the ACTION topic of the PTT menu

_________________

bar, or use ALT+S to begin test-case execution to the

monitor or display output device.  To send the output of

the test case to the printer, select START PRINTING from

the ACTION topic of the PTT menu bar.  Each test case

selected is executed in the order it appeared in the Test

Cases section of the Test Case Selection dialog panel.

During test execution, "***** Printer Test Tool: Test

Running *****" appears in the PTT window title bar.  When

the test is complete, the name of the individual test

selected reappears in the title bar if a single test was

selected.  "Printer Test Tool: MULTIPLE TESTS SELECTED"

appears if more than one test is selected.

DISPLAYING THE PTT RELEASE AND REVISION LEVELS

______________________________________________

To display the current PTT release and revision levels,

select ABOUT from the PTT menu bar or use ALT+A.

22

EXITING PTT

___________

To terminate PTT, select EXIT from the PTT menu bar or

use ALT+F4 from the keyboard.

GPI FUNCTION TESTS (32-BIT)

___________________________

This section shows the GPI calls made by the Printer Test

Tool's test groups.  The following tables are organized

by test group.  Each table lists the test cases in the

group and the functions tested by each test case.  CON-

FIGURATION SETTINGS describes settings that must be

selected in the CONFIGURATION dialog box to run the test.

If no Configurations Settings are specified, use the

default settings of OD_QUEUED, PM_Q_STD, GPIT_MICRO, and

PU_PELS.

NOTE:  These tables represent GPI calls for the 32-bit

PTT.

GPIARC TEST GROUP

The GpiArc test case verifies printer drivers' arc-

related functionality.  This test case consists of preci-

sion, functionality, and exhaustive tests.

The precision tests verify the GPI Arc APIs for page

positioning and different arc shapes sizing.  The

functionality test verifies the GPI Arc APIs as they

function together and with other GPI APIs such as

GpiBeginPath and GpiEndPath.  Exhaustive testing of the

GPI Arc APIs involves verifying the functionality of a

given API with respect to the print driver.

This table lists the test cases and the functions called

by each test case in the GpiArc test group.  The GpiArc

test group is found in GPIARC.DLL.

+---------------------------------------------------------------------------+

| Table 2 (Page 1 of 3). Test Cases and GPI Calls - GpiArc Test Group.      |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiArcPre1                  | GpiFullArc                   |              |

|                             | GpiMove                      |              |

|                             | GpiPartialArc                |              |

|                             | GpiPointArc                  |              |

|                             | GpiSetArcParams              |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  23

+---------------------------------------------------------------------------+

| Table 2 (Page 2 of 3). Test Cases and GPI Calls - GpiArc Test Group.      |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiArcPre2                  | GpiMarker                    |              |

|                             | GpiMove                      |              |

|                             | GpiPolyFillet                |              |

|                             | GpiPolyFilletSharp           |              |

|                             | GpiPolySpline                |              |

|                             | GpiSetArcParams              |              |

+-----------------------------+------------------------------+--------------+

| GpiArcApp                   | GpiMove                      |              |

|                             | GpiPartialArc                |              |

|                             | GpiPointArc                  |              |

|                             | GpiPolyFillet                |              |

|                             | GpiPolyFilletSharp           |              |

|                             | GpiPolySpline                |              |

|                             | GpiQueryCurrentPosition      |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetArcParamsExh1         | GpiFullArc                   |              |

|                             | GpiMove                      |              |

|                             | GpiPartialArc                |              |

|                             | GpiPointArc                  |              |

|                             | GpiQueryArcParams            |              |

|                             | GpiSetArcParams              |              |

+-----------------------------+------------------------------+--------------+

| GpiSetArcParamsExh2         | GpiBeginArea                 |              |

|                             | GpiBeginArea                 |              |

|                             | GpiEndArea                   |              |

|                             | GpiFullArc                   |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiPartialArc                |              |

|                             | GpiQueryArcParams            |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetDefArcParamsExh1      | GpiFullArc                   |              |

|                             | GpiMove                      |              |

|                             | GpiPartialArc                |              |

|                             | GpiPointArc                  |              |

|                             | GpiQueryDefArcParams         |              |

|                             | GpiResetPS                   |              |

|                             | GpiSetDefArcParams           |              |

+-----------------------------+------------------------------+--------------+

24

+---------------------------------------------------------------------------+

| Table 2 (Page 3 of 3). Test Cases and GPI Calls - GpiArc Test Group.      |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSetDefArcParamsExh2      | GpiBeginArea                 |              |

|                             | GpiEndArea                   |              |

|                             | GpiFullArc                   |              |

|                             | GpiMove                      |              |

|                             | GpiPartialArc                |              |

|                             | GpiQueryDefArcParams         |              |

|                             | GpiResetPS                   |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetDefArcParams           |              |

|                             | GpiSetPa                     |              |

+-----------------------------+------------------------------+--------------+

| GpiFullArcExh               | GpiFullArc                   |              |

|                             | GpiMove                      |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiPartialArcExh            | GpiMove                      |              |

|                             | GpiPartialArc                |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiPointArcExh              | GpiMove                      |              |

|                             | GpiPointArc                  |              |

+-----------------------------+------------------------------+--------------+

| GpiPolyFilletExh            | GpiMove                      |              |

|                             | GpiPolyFillet                |              |

+-----------------------------+------------------------------+--------------+

| GpiPolyFilletSharpExh       | GpiMove                      |              |

|                             | GpiPolyFilletSharp           |              |

+-----------------------------+------------------------------+--------------+

| GpiPolySplineExh            | GpiMarker                    |              |

|                             | GpiMove                      |              |

|                             | GpiPolySpline                |              |

+-----------------------------+------------------------------+--------------+

GPIAREA TEST GROUP

The GpiArea test group tests area manipulation functions.

All tests include  a basic functionality test and an

exhaustive (test all return codes at the lowest level)

test.

This table lists the test cases and the functions called

by each test case in the GpiArea test group.  The GpiArc

test group is found in GPIAREA.DLL.

Printer Test Tool (PTT)  25

+---------------------------------------------------------------------------+

| Table 3 (Page 1 of 2). Test Cases and GPI Calls - GpiArea Test Group.     |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiAreaApp                  | GpiBox                       |              |

|                             | GpiMove                      |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiBeginAreaExh             | GpiBeginArea                 |              |

|                             | GpiEndArea                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryPatternExh      | GpiBox                       |              |

|                             | GpiCharStringAt              |              |

|                             | GpiEndArea                   |              |

|                             | GpiFullArc                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryPattern              |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryPatternSetExh   | GpiBeginArea                 | Direct and   |

|                             | GpiBox                       | Standard     |

|                             | GpiDeleteSetId               | or           |

__

|                             | GpiEndArea                   | Queued and   |

|                             | GpiFullArc                   | Raw          |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryBitmapParameters     |              |

|                             | GpiQueryPatternSet           |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetBitmapId               |              |

|                             | GpiSetCharSet                |              |

|                             | GpiSetPatternSet             |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryPatternRefPointE|hGpiBeginArea                 |              |

|                             | GpiBox                       |              |

|                             | GpiEndArea                   |              |

|                             | GpiFullArc                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryPatternRefPoint      |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetAttrMode               |              |

|                             | GpiSetPattern                |              |

|                             | GpiSetPatternRefPoint        |              |

+-----------------------------+------------------------------+--------------+

26

+---------------------------------------------------------------------------+

| Table 3 (Page 2 of 2). Test Cases and GPI Calls - GpiArea Test Group.     |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiPaintRegionExh           | GpiBeginPath                 |              |

|                             | GpiCreateRegion              |              |

|                             | GpiDestroyRegion             |              |

|                             | GpiEndPath                   |              |

|                             | GpiMove                      |              |

|                             | GpiPaintRegion               |              |

|                             | GpiPolyFillet                |              |

|                             | GpiPolyLine                  |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetLineWidthGeom          |              |

|                             | GpiSetPattern                |              |

|                             | GpiStrokePath                |              |

+-----------------------------+------------------------------+--------------+

GPIBITMAPS TEST GROUP

The GpiBitmaps test group tests bit map manipulation

functions.  These functions include creating, deleting,

querying, selecting, and manipulating bit maps.

All tests include  a basic functionality test and an

exhaustive (test all return codes at the lowest level)

test.

LIMITATIONS:  All hard-copy vector devices will fail on

all bit map operation since it is impossible to translate

bit maps to vectors.

This table lists the test cases and the functions called

by each test case in the GpiBitmaps test group.  The

GpiBitmaps test group is found in GPIBTMP.DLL.

+---------------------------------------------------------------------------+

| Table 4 (Page 1 of 4). Test Cases and GPI Calls - GpiBitmaps Test Group.  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiBitPre1                  | GpiDeleteBitmap              |              |

|                             | GpiDestroyPS                 |              |

|                             | GpiSetColor                  |              |

|                             | GpiWCBitBlt                  |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  27

+---------------------------------------------------------------------------+

| Table 4 (Page 2 of 4). Test Cases and GPI Calls - GpiBitmaps Test Group.  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiBitPre2                  | GpiBitBlt                    | Direct       |

|                             | GpiBitBlt                    |              |

|                             | GpiConvert                   |              |

|                             | GpiConvert                   |              |

|                             | GpiCreateBitmap              |              |

|                             | GpiCreatePS                  |              |

|                             | GpiDeleteBitmap              |              |

|                             | GpiDestroyPS                 |              |

|                             | GpiQueryDevice               |              |

|                             | GpiSetBitmap                 |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiBitApp1                  | GpoBitmapFormats             | Direct       |

|                             | GpiBeginArea                 |              |

|                             | GpiBitBlt                    |              |

|                             | GpiBox                       |              |

|                             | GpiCreateBitmap              |              |

|                             | GpiCreatePS                  |              |

|                             | GpiDeleteBitmap              |              |

|                             | GpiDestroyPS                 |              |

|                             | GpiEndArea                   |              |

|                             | GpiErase                     |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryBitmapParameters     |              |

|                             | GpiQueryDevice               |              |

|                             | GpiSetBitmap                 |              |

|                             | GpiSetBitmapBits             |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiBitApp2                  | GpiPtVisible                 |              |

|                             | GpiQueryPel                  |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetPel                    |              |

+-----------------------------+------------------------------+--------------+

28

+---------------------------------------------------------------------------+

| Table 4 (Page 3 of 4). Test Cases and GPI Calls - GpiBitmaps Test Group.  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiBitApp3                  | GpiBox                       | Direct       |

|                             | GpiCreateBitmap              |              |

|                             | GpiDeleteBitmap              |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiQueryBitmapHandle         |              |

|                             | GpiSetBackMix                |              |

|                             | GpiSetBitmapId               |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetPatternSet             |              |

|                             | GpiSetPatternSet             |              |

+-----------------------------+------------------------------+--------------+

| GpiBitApp4                  | GpiDeleteBitmap              | Direct       |

|                             | GpiLoadBitmap                |              |

|                             | GpiQueryBitmapParameters     |              |

|                             | GpiWCBitBlt                  |              |

+-----------------------------+------------------------------+--------------+

| GpiSetPelExh                | GpiPtVisible                 |              |

|                             | GpiQueryPel                  |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetPel                    |              |

+-----------------------------+------------------------------+--------------+

| GpiBitBltExh                | GpiBitBlt                    | Direct       |

|                             | GpiCreateBitmap              |              |

|                             | GpiCreatePS                  |              |

|                             | GpiDeleteBitmap              |              |

|                             | GpiDestroyPS                 |              |

|                             | GpiQueryDevice               |              |

|                             | GpiSetBitmap                 |              |

|                             | GpiSetBitmapBits             |              |

+-----------------------------+------------------------------+--------------+

| GpiSetBitmapBitsExh         | GpiBitBlt                    | Direct       |

|                             | GpiCreatePS                  |              |

|                             | GpiDeleteBitmap              |              |

|                             | GpiDestroyPS                 |              |

|                             | GpiQueryBitmapBits           |              |

|                             | GpiQueryDevice               |              |

|                             | GpiSetBitmap                 |              |

|                             | GpiSetBitmapBits             |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  29

+---------------------------------------------------------------------------+

| Table 4 (Page 4 of 4). Test Cases and GPI Calls - GpiBitmaps Test Group.  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSetBitmapIdExh           | GpiBox                       | Direct       |

|                             | GpiCreateBitmap              |              |

|                             | GpiDeleteBitmap              |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiQueryBitmapHandle         |              |

|                             | GpiSetBitmapId               |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetPatternSet             |              |

+-----------------------------+------------------------------+--------------+

| GpiWCBitBltExh              | GpiCreateBitmap              |              |

|                             | GpiDeleteBitmap              |              |

|                             | GpiSetColor                  |              |

|                             | GpiWCBitBlt                  |              |

+-----------------------------+------------------------------+--------------+

| GpiSetBitmapDimensionExh    | GpiDeleteBitmap              |              |

|                             | GpiQueryBitmapDimension      |              |

|                             | GpiSetBitmapDimension        |              |

+-----------------------------+------------------------------+--------------+

| GpiLoadBitmapExh            | GpiBitBlt                    | Direct       |

|                             | GpiCreatePS                  |              |

|                             | GpiDeleteBitmap              |              |

|                             | GpiDestroyPS                 |              |

|                             | GpiLoadBitmap                |              |

|                             | GpiQueryBitmapParameters     |              |

|                             | GpiQueryDevice               |              |

|                             | GpiSetBitmap                 |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryDeviceBitmapFormatsE|hGpiQueryDeviceBitmapFormats  |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryBitmapParametersExh | GpiCreateBitmap              |              |

|                             | GpiDeleteBitmap              |              |

|                             | GpiQueryBitmapParameters     |              |

+-----------------------------+------------------------------+--------------+

GPICHAR TEST GROUP

The GpiChar test group tests GPI text functions that:

o   Set and query the current value of the baseline

angle.

o   Set and query the current value of the character box

attribute.

o   Set and query the current value of the character

direction attribute.

30

o   Set and query the current value of the character-mode

attribute.

o   Set and query the current value of the character-

shear angle.

o   Draw a character string at the current character

position.

o   Draw a character string at a specified character

position.

o   Set and query the position at which each character in

a string is drawn with positioning controls.

o   Set and query the position at which each character in

a positioned string is drawn with positioning con-

trols.

o   Query the size of the default graphics character box.

o   Query the relative coordinates of the four corners of

a text box.

This testcase includes application tests that test the

behavior of these character functions with other GPIs.

This table lists the test cases and the functions called

by each test case in the GpiChar test group.  The GpiChar

test group is found in GPICHAR.DLL.

+---------------------------------------------------------------------------+

| Table 5 (Page 1 of 5). Test Cases and GPI Calls - GpiChar Test Group.     |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiCharTestPre1             | GpiCharStringPos             |              |

|                             | GpiCharStringPosAt           |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiSetCharAngle              |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiCharTestPre2             | GpiCharStringAt              |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  31

+---------------------------------------------------------------------------+

| Table 5 (Page 2 of 5). Test Cases and GPI Calls - GpiChar Test Group.     |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiCharTestApp1             | GpiCharString                |              |

|                             | GpiCharStringAt              |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryFontMetrics          |              |

|                             | GpiSetCharAngle              |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharDirection          |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

|                             | GpiSetCharShear              |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryCharAngleExh    | GpiCharStringAt              |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiSetCharAngle              |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryCharBoxExh      | GpiCharString                |              |

|                             | GpiCharStringAt              |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiQueryCharBox              |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryCharDirExh      | GpiCharStringAt              |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiQueryCharDirection        |              |

|                             | GpiQueryCharDirection        |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharDirection          |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryCharModeExh     | GpiCharStringAt              |              |

|                             | GpiCharStringAt              |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiQueryCharMode             |              |

|                             | GpiSetCharAngle              |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

32

+---------------------------------------------------------------------------+

| Table 5 (Page 3 of 5). Test Cases and GPI Calls - GpiChar Test Group.     |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryCharShearExh    | GpiCharStringAt              |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiQueryCharShear            |              |

|                             | GpiQueryDefCharBox           |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

|                             | GpiSetCharShear              |              |

+-----------------------------+------------------------------+--------------+

| GpiCharStringExh            | GpiCharString                |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDefCharBox           |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryFontMetrics          |              |

|                             | GpiSetCharAngle              |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiCharStringAtExh          | GpiCharString                |              |

|                             | GpiCharStringAt              |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDefCharBox           |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryFontMetrics          |              |

|                             | GpiSetCharAngle              |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiCharStringPosAndQryExh   | GpiCharString                |              |

|                             | GpiCharStringPos             |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDefCharBox           |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryFontMetrics          |              |

|                             | GpiQueryTextBox              |              |

|                             | GpiSetCharAngle              |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  33

+---------------------------------------------------------------------------+

| Table 5 (Page 4 of 5). Test Cases and GPI Calls - GpiChar Test Group.     |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiCharStringPosAndQryShadeE|hGpiCharString                |              |

|                             | GpiCharStringPos             |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDefCharBox           |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryFontMetrics          |              |

|                             | GpiQueryTextBox              |              |

|                             | GpiSetBackColor              |              |

|                             | GpiSetCharAngle              |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiCharStringPosAndQryBoxExh| GpiCharString                |              |

|                             | GpiCharStringPos             |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDefCharBox           |              |

|                             | GpiQueryFontMetrics          |              |

|                             | GpiQueryTextBox              |              |

|                             | GpiSetCharAngle              |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiCharStringPosAtAndQryExh | GpiCharString                |              |

|                             | GpiCharStringPos             |              |

|                             | GpiCharStringPosAt           |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDefCharBox           |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryFontMetrics          |              |

|                             | GpiQueryTextBox              |              |

|                             | GpiSetCharAngle              |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

34

+---------------------------------------------------------------------------+

| Table 5 (Page 5 of 5). Test Cases and GPI Calls - GpiChar Test Group.     |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiCharStringPosAtAndQryShdE|hGpiCharString                |              |

|                             | GpiCharStringPos             |              |

|                             | GpiCharStringPosAt           |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDefCharBox           |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryFontMetrics          |              |

|                             | GpiQueryTextBox              |              |

|                             | GpiSetBackColor              |              |

|                             | GpiSetCharAngle              |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiCharStringPosAtAndQryBoxE|hGpiCharString                |              |

|                             | GpiCharStringPos             |              |

|                             | GpiCharStringPosAt           |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDefCharBox           |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryFontMetrics          |              |

|                             | GpiQueryTextBox              |              |

|                             | GpiSetCharAngle              |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiQryDefCharBoxExh         | GpiCharStringAt              |              |

|                             | GpiQueryCharBox              |              |

|                             | GpiQueryDefCharBox           |              |

|                             | GpiSetCharBox                |              |

+-----------------------------+------------------------------+--------------+

| GpiQryTextBoxExh            | GpiCharStringAt              |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryTextBox              |              |

|                             | GpiSetCharAngle              |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharMode               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  35

GPICLIP TEST GROUP

The GpiClip test group tests the GPI clipping visible

APIs.  The following APIs are tested:

o   GpiExcludeClipRectangle

o   GpiIntersectClipRectangle

o   GpiOffsetClipRegion

o   GpiSetClipRegion

o   GpiSetClipPath

o   GpiQueryClipBox

o   GpiQueryClipRegion

o   GpiPtVisible

o   GpiRectVisible

This table lists the test cases and the functions called

by each test case in the GpiClip test group.  The GpiClip

test group is found in GPICLIP.DLL.

+---------------------------------------------------------------------------+

| Table 6 (Page 1 of 2). Test Cases and GPI Calls - GpiClip Test Group.     |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiClipPre                  | GpiCreateRegion              | Direct       |

|                             | GpiDestroyRegion             |              |

|                             | GpiSetClipRegion             |              |

+-----------------------------+------------------------------+--------------+

| GpiClipApp                  | GpiCreateRegion              | Direct       |

|                             | GpiDestroyRegion             |              |

|                             | GpiExcludeClipRectangle      |              |

|                             | GpiIntersectClipRectangle    |              |

|                             | GpiOffsetClipRegion          |              |

|                             | GpiPtVisible                 |              |

|                             | GpiQueryClipBox              |              |

|                             | GpiSetClipRegion             |              |

+-----------------------------+------------------------------+--------------+

| GpiPtVisibleExh             | GpiCreateRegion              | Direct       |

|                             | GpiDestroyRegion             |              |

|                             | GpiPtVisible                 |              |

|                             | GpiSetClipRegion             |              |

+-----------------------------+------------------------------+--------------+

| GpiRectVisibleExh           | GpiCreateRegion              | Direct       |

|                             | GpiDestroyRegion             |              |

|                             | GpiRectVisible               |              |

|                             | GpiSetClipRegion             |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryClipBoxExh          | GpiCreateRegion              | Direct       |

|                             | GpiDestroyRegion             |              |

|                             | GpiQueryClipBox              |              |

|                             | GpiSetClipRegion             |              |

+-----------------------------+------------------------------+--------------+

36

+---------------------------------------------------------------------------+

| Table 6 (Page 2 of 2). Test Cases and GPI Calls - GpiClip Test Group.     |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiQueryClipRegionExh       | GpiCombineRegion             | Direct       |

|                             | GpiCreateRegion              |              |

|                             | GpiDestroyRegion             |              |

|                             | GpiPaintRegion               |              |

|                             | GpiQueryClipRegion           |              |

|                             | GpiSetClipRegion             |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetClipPathExh           | GpiCreateRegion              | Queued       |

|                             | GpiDestroyRegion             |              |

|                             | GpiQueryClipBox              |              |

|                             | GpiSetClipRegion             |              |

+-----------------------------+------------------------------+--------------+

| GpiExcludeClipRectExh       | GpiCreateRegion              | Direct       |

|                             | GpiDestroyRegion             |              |

|                             | GpiExcludeClipRectangle      |              |

|                             | GpiSetClipRegion             |              |

+-----------------------------+------------------------------+--------------+

| GpiIntersectClipRectExh     | GpiCreateRegion              | Direct       |

|                             | GpiDestroyRegion             |              |

|                             | GpiIntersectClipRectangle    |              |

|                             | GpiSetClipRegion             |              |

+-----------------------------+------------------------------+--------------+

| GpiOffsetClipRegionExh      | GpiCreateRegion              | Direct       |

|                             | GpiDestroyRegion             |              |

|                             | GpiOffsetClipRegion          |              |

|                             | GpiSetClipRegion             |              |

+-----------------------------+------------------------------+--------------+

GPICOLOR TEST GROUP

The GpiColor test group tests the GPI Color API,

GpiSetColor.

This table lists the test cases and the functions called

by each test case in the GpiColor test group.  The

GpiColor test group is found in GPICOLOR.DLL.

Printer Test Tool (PTT)  37

+---------------------------------------------------------------------------+

| Table 7 (Page 1 of 4). Test Cases and GPI Calls - GpiColor Test Group.    |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiColorApp1                | GpiSetColor                  |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiColorApp2                | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiColorApp3                | GpiBeginArea                 |              |

|                             | GpiBox                       |              |

|                             | GpiEndArea                   |              |

|                             | GpiFullArc                   |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiPointArc                  |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetBackMix                |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiSetColorExh              | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiSetMixExh1               | GpiBeginArea                 |              |

|                             | GpiEndArea                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryMix                  |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetMix                    |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetMixExh2               | GpiBeginArea                 |              |

|                             | GpiEndArea                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryMix                  |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetMix                    |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

38

+---------------------------------------------------------------------------+

| Table 7 (Page 2 of 4). Test Cases and GPI Calls - GpiColor Test Group.    |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSetMixExh3               | GpiBeginArea                 |              |

|                             | GpiEndArea                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryMix                  |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetMix                    |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetMixExh4               | GpiBeginArea                 |              |

|                             | GpiEndArea                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryMix                  |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetMix                    |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetBackColorExh          | GpiBox                       |              |

|                             | GpiMove                      |              |

|                             | GpiQueryBackColor            |              |

|                             | GpiSetBackColor              |              |

|                             | GpiSetBackMix                |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetBackMixExh            | GpiBeginArea                 |              |

|                             | GpiEndArea                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryBackMix              |              |

|                             | GpiQueryDevice               |              |

|                             | GpiSetBackColor              |              |

|                             | GpiSetBackMix                |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetMixExh1                |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  39

+---------------------------------------------------------------------------+

| Table 7 (Page 3 of 4). Test Cases and GPI Calls - GpiColor Test Group.    |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiCreateLogColorTableExh1  | GpiBox                       |              |

|                             | GpiCreateLogColorTable       |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryLogColorTable        |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetPattern                |              |

|                             | GpiSetPatternSet             |              |

|                             | GpiUnrealizeColorTable       |              |

+-----------------------------+------------------------------+--------------+

| GpiCreateLogColorTableExh2  | GpiBox                       |              |

|                             | GpiCreateLogColorTable       |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDevice               |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryColorDataExh        | GpiCreateLogColorTable       |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiQueryColorData            |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryColorIndexExh       | GpiBox                       |              |

|                             | GpiCreateLogColorTable       |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiQueryColorIndex           |              |

|                             | GpiQueryDevice               |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryNearestColorExh     | GpiBox                       |              |

|                             | GpiCreateLogColorTable       |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryNearestColor         |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

40

+---------------------------------------------------------------------------+

| Table 7 (Page 4 of 4). Test Cases and GPI Calls - GpiColor Test Group.    |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiQueryRealColorsExh       | GpiBox                       |              |

|                             | GpiCreateLogColorTable       |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryRealColors           |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryRGBColorExh         | GpiBox                       |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryRGBColor             |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

GPIELEMENT TEST GROUP

This testcase is designed to test the GPI element func-

tions that:

o   Open an element bracket.

o   Close an element bracket.

o   Delete the element referenced by the current value of

the element pointer.

o   Delete a section of elements inclusively delimited by

element pointer values.

o   Delete a section of elements exclusively delimited by

labels.

o   Add a single element to the current segment.

o   Query the graphics orders for a single element.

o   Get the graphics orders for a specified segment.

o   Add a buffer of graphics orders to the current

segment.

o   Insert a label into the current segment.

o   Move the element pointer by a specified increment.

o   Query information associated with the current

element.

o   Set the current edit mode to either insert or

replace.

o   Query the current edit mode setting.

o   Set the value of the element pointer.

o   Query the value of the element pointer.

Printer Test Tool (PTT)  41

o   Set the value of the element pointer to point at a

specified label.

This testcase includes application tests that are

designed to test the behavior of these character func-

tions with other GPIs.

LIMITATIONS:  All exhaustive element functions must be

executed within a normal PS.

This table lists the test cases and the functions called

by each test case in the GpiElement test group.  The

GpiElement test group is found in GPIELMNT.DLL.

+---------------------------------------------------------------------------+

| Table 8 (Page 1 of 4). Test Cases and GPI Calls - GpiElement Test Group.  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiElementTestApp1          | GpiDrawSegment               | Normal       |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawContro             |              |

|                             | GpiSetDrawingMode            |              |

+-----------------------------+------------------------------+--------------+

| GpiElementTestApp2          | GpiDrawSegment               | Normal       |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawControl            |              |

|                             | GpiSetDrawingMode            |              |

+-----------------------------+------------------------------+--------------+

| GpiBeginAndEndElementExh    | GpiBeginElement              | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiDrawSegment               |              |

|                             | GpiEndElement                |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetDrawControl            |              |

|                             | GpiSetDrawingMode            |              |

+-----------------------------+------------------------------+--------------+

| GpiDeleteElementExh         | GpiCloseSegment              | Normal       |

|                             | GpiDeleteElement             |              |

|                             | GpiOffsetElementPointer      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawControl            |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetElementPointerAtLabel  |              |

+-----------------------------+------------------------------+--------------+

42

+---------------------------------------------------------------------------+

| Table 8 (Page 2 of 4). Test Cases and GPI Calls - GpiElement Test Group.  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiDeleteElementRangeExh    | GpiCloseSegment              | Normal       |

|                             | GpiDeleteElementRange        |              |

|                             | GpiDrawSegment               |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryElementPointer       |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawControl            |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetElementPointerAtLabel  |              |

+-----------------------------+------------------------------+--------------+

| GpiDeleteElmntsBetweenLabels|xGpiCloseSegment              | Normal       |

|                             | GpiDeleteElement             |              |

|                             | GpiDeleteElementsBetweenLabel|              |

|                             | GpiDrawSegment               |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawControl            |              |

|                             | GpiSetDrawingMode            |              |

+-----------------------------+------------------------------+--------------+

| GpiElementAndQryExh         | GpiBeginElement              | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiComment                   |              |

|                             | GpiDrawSegment               |              |

|                             | GpiElement                   |              |

|                             | GpiEndElement                |              |

|                             | GpiMove                      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryElement              |              |

|                             | GpiQueryElementType          |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawControl            |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetModelTransformMatrix   |              |

+-----------------------------+------------------------------+--------------+

| GpiGetAndPutDataExh         | GpiCloseSegment              | Normal       |

|                             | GpiDrawSegment               |              |

|                             | GpiGetData                   |              |

|                             | GpiLabel                     |              |

|                             | GpiOpenSegment               |              |

|                             | GpiPutData                   |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawControl            |              |

|                             | GpiSetDrawingMode            |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  43

+---------------------------------------------------------------------------+

| Table 8 (Page 3 of 4). Test Cases and GPI Calls - GpiElement Test Group.  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiLabelAndSetElmtPtrAtLabel|xGpiCloseSegment              | Normal       |

|                             | GpiLabel                     |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryElementPointer       |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetElementPointer         |              |

|                             | GpiSetElementPointerAtLabel  |              |

+-----------------------------+------------------------------+--------------+

| GpiOffsetElementPointerExh  | GpiCloseSegment              | Normal       |

|                             | GpiLabel                     |              |

|                             | GpiOffsetElementPointer      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryElementPointer       |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetElementPointer         |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryElementTypeExh      | GpiCallSegmentMatrix         | Normal       |

|                             | GpiCharString                |              |

|                             | GpiCharStringAt              |              |

|                             | GpiCloseSegment              |              |

|                             | GpiElement                   |              |

|                             | GpiFullArc                   |              |

|                             | GpiImage                     |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiOffsetElementPointer      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiPolyFillet                |              |

|                             | GpiPolyFilletSharp           |              |

|                             | GpiPolyMarker                |              |

|                             | GpiPolySpline                |              |

|                             | GpiQueryDefAttrs             |              |

|                             | GpiQueryElement              |              |

|                             | GpiQueryElementType          |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetAttrs                  |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetElementPointer         |              |

|                             | GpiSetSegmentAttrs           |              |

+-----------------------------+------------------------------+--------------+

44

+---------------------------------------------------------------------------+

| Table 8 (Page 4 of 4). Test Cases and GPI Calls - GpiElement Test Group.  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryEditModeExh     | GpiCloseSegment              | Normal       |

|                             | GpiDrawSegment               |              |

|                             | GpiLabel                     |              |

|                             | GpiOffsetElementPointer      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryEditMode             |              |

|                             | GpiQueryPS                   |              |

|                             | GpiRotate                    |              |

|                             | GpiSetDrawControl            |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetEditMode               |              |

|                             | GpiSetElementPointerAtLabel  |              |

|                             | GpiSetModelTransformMatrix   |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryElementPointerEx| GpiCloseSegment              | Normal       |

|                             | GpiLabel                     |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetElementPointer         |              |

+-----------------------------+------------------------------+--------------+

GPIFONT TEST GROUP

The GpiFont test group tests GPI text functions

including:

o   Determine whether a given file is a font resource

file.

o   Query the font metrics for the current logical font.

o   Query the font metrics for all available fonts, or

for the fonts specified by facename.

o   Load one or more fonts from the specified resource

file.

o   Unload fonts previously loaded from the resource

file.

o   Create a logical definition of a font that an appli-

cation requires to use.

o   Query kerning pair information for the logical font

identified by the current value of the character set

attribute.

o   Set the current value of the character-set attribute.

o   Query the character-set lcid.

o   Sets the code page ID for graphics characters for the

default font.

Printer Test Tool (PTT)  45

o   Query the currently selected graphics code page.

o   Query the fonts that have been created by

GpiCreateLogFont, and tagged bit maps.

o   Query the number of local identifiers currently in

use, referring to fonts or bit maps.

o   Delete a logical font or bit-map tag.

o   Query width table information for the logical font

identified by the value of the character-set attri-

bute.

o   Query the device characteristics.

This table lists the test cases and the functions called

by each test case in the GpiFont test group.  The GpiFont

test group is found in GPIFONT.DLL.

+---------------------------------------------------------------------------+

| Table 9 (Page 1 of 2). Test Cases and GPI Calls - GpiElement Test Group.  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiFontApp1                 | GpiCharStringAt              |              |

|                             | GpiCreateLogFont             |              |

|                             | GpiDeleteSetId               |              |

|                             | GpiQueryCharSet              |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryFontFileDescriptions |              |

|                             | GpiQueryFonts                |              |

|                             | GpiQueryNumberSetIds         |              |

|                             | GpiQueryWidthTable           |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiFontApp2                 | GpiQueryDevice               |              |

|                             | GpiQueryFontMetrics          |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryFontFileDescriptions|xGpiDeleteSetId               |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryFontFileDescriptions |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryFontMetricsExh      | GpiDeleteSetId               |              |

|                             | GpiQueryFontMetrics          |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryFontsExh            | GpiDeleteSetId               |              |

|                             | GpiLoadFonts                 |              |

|                             | GpiQueryFonts                |              |

|                             | GpiSetCharSet                |              |

|                             | GpiUnloadFonts               |              |

+-----------------------------+------------------------------+--------------+

46

+---------------------------------------------------------------------------+

| Table 9 (Page 2 of 2). Test Cases and GPI Calls - GpiElement Test Group.  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiCreateLogFontExh         | GpiDeleteSetId               |              |

|                             | GpiResetPS                   |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryKerningPairsExh     | GpiDeleteSetId               |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryFontMetrics          |              |

|                             | GpiQueryKerningPairs         |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQueryCpExh         | GpiCharString                |              |

|                             | GpiMove                      |              |

|                             | GpiQueryCp                   |              |

|                             | GpiQueryCp                   |              |

|                             | GpiSetCp                     |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQueryCharSetExh    | GpiDeleteSetId               |              |

|                             | GpiQueryCharSet              |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryNumberSetIdsExh     | GpiDeleteSetId               |              |

|                             | GpiQueryNumberSetIds         |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiDeleteSetIdExh           | GpiDeleteSetId               |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryWidthTableExh       | GpiDeleteSetId               |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryFontMetrics          |              |

|                             | GpiQueryWidthTable           |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiLoadFontsExh             | GpiLoadFonts                 |              |

|                             | GpiQueryFonts                |              |

|                             | GpiUnloadFonts               |              |

+-----------------------------+------------------------------+--------------+

| GpiUnloadFontsExh           | GpiLoadFonts                 |              |

|                             | GpiQueryFonts                |              |

|                             | GpiUnloadFonts               |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  47

GPIIMAGE TEST GROUP

This module is called by PTT to test the GPI Image APIs.

This GPI is used to display raster (bit map) images.  The

functionality of this GPI and all the error codes that

may be returned by this GPI will be tested.

LIMITATIONS:  GpiImage cannot be used to load bitmaps

that a pixel is represented by more than one bit.

This table lists the test cases and the functions called

by each test case in the GpiImage test group.  The

GpiImage test group is found in GPIIMAGE.DLL.

+---------------------------------------------------------------------------+

| Table 10. Test Cases and GPI Calls - GpiFont Test Group.                  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiImagePre                 | GpiImage                     |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiImageApp                 | GpiBox                       |              |

|                             | GpiImage                     |              |

|                             | GpiMove                      |              |

|                             | GpiRestorePS                 |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiImageExh                 | GpiImage                     |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

GPILINE TEST GROUP

The GpiLine test group tests the line and polyline APIs

with respect to line characteristics and precision.

This table lists the test cases and the functions called

by each test case in the GpiLine test group.  The GpiLine

Test Group is found in GPILINE.DLL.

48

+---------------------------------------------------------------------------+

| Table 11. Test Cases and GPI Calls - GpiLine Test Group.                  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiLinePre                  | GpiBeginPath                 |              |

|                             | GpiBox                       |              |

|                             | GpiCloseFigure               |              |

|                             | GpiEndPath                   |              |

|                             | GpiFillPath                  |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiLineApp                  | GpiBeginPath                 |              |

|                             | GpiBox                       |              |

|                             | GpiCloseFigure               |              |

|                             | GpiEndPath                   |              |

|                             | GpiFillPath                  |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiBoxExh                   | GpiBox                       |              |

|                             | GpiMove                      |              |

|                             | GpiQueryCurrentPosition      |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiLineExh                  | GpiLine                      |              |

|                             | GpiQueryLineType             |              |

|                             | GpiSetCurrentPosition        |              |

|                             | GpiSetLineType               |              |

+-----------------------------+------------------------------+--------------+

| GpiPolyLineExh              | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

+-----------------------------+------------------------------+--------------+

GPIMARKER TEST GROUP

The GpiMarker test group covers the GPI Marker APIs with

respect to precision, size, and variation.

This table lists the test cases and the functions called

by each test case in the GpiMarker test group.  The

GpiMarker test group is found in GPIMARKER.DLL.

Printer Test Tool (PTT)  49

+---------------------------------------------------------------------------+

| Table 12. Test Cases and GPI Calls - GpiMarker Test Group.                |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiMarkerPre                | GpiMarker                    |              |

|                             | GpiPolyMarker                |              |

|                             | GpiSetMarker                 |              |

+-----------------------------+------------------------------+--------------+

| GpiMarkerApp                | GpiDeleteSetId               | Direct and   |

|                             | GpiMarker                    | Standard     |

|                             | GpiPolyMarker                | or           |

|                             | GpiQueryMarker               | Queued and   |

|                             | GpiQueryMarkerBox            | Raw          |

|                             | GpiQueryMarkerSet            |              |

|                             | GpiSetMarker                 |              |

|                             | GpiSetMarkerBox              |              |

|                             | GpiSetMarkerSet              |              |

+-----------------------------+------------------------------+--------------+

| GpiMarkerExh                | GpiMarker                    | Direct and   |

|                             | GpiQueryMarker               | Standard     |

|                             | GpiQueryMarkerBox            | or           |

|                             | GpiQueryMarkerSet            | Queued and   |

|                             | GpiSetMarker                 | Raw          |

|                             | GpiSetMarkerBox              |              |

|                             | GpiSetMarkerSet              |              |

+-----------------------------+------------------------------+--------------+

| GpiPolyMarkerExh            | GpiPolyMarker                |              |

|                             | GpiQueryMarker               |              |

|                             | GpiQueryMarkerBox            |              |

|                             | GpiQueryMarkerSet            |              |

|                             | GpiSetCharDirection          |              |

|                             | GpiSetMarker                 |              |

|                             | GpiSetMarkerBox              |              |

|                             | GpiSetMarkerSet              |              |

+-----------------------------+------------------------------+--------------+

GPIMETA TEST GROUP

The GpiMeta test group covers the GPI Meta APIs and tests

the loading, copying, playing, deletion, and querying of

meta files.

This table lists the test cases and the functions called

by each test case in the GpiMeta test group.  The GpiMeta

test group is found in GPIMETA.DLL.

50

+---------------------------------------------------------------------------+

| Table 13 (Page 1 of 2). Test Cases and GPI Calls - GpiMeta Test Group.    |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiMetaApp1                 | GpiLoadMetaFile              | Raw          |

|                             | GpiPlayMetaFile              |              |

+-----------------------------+------------------------------+--------------+

| GpiMetaApp2                 | GpiLoadMetaFile              | Raw          |

|                             | GpiPlayMetaFile              |              |

+-----------------------------+------------------------------+--------------+

| GpiCopyMetaFileExh          | GpiAssociate                 | Raw          |

|                             | GpiCharString                |              |

|                             | GpiCopyMetaFile              |              |

|                             | GpiCreatePS                  |              |

|                             | GpiDestroyPS                 |              |

|                             | GpiLoadMetaFile              |              |

|                             | GpiMove                      |              |

|                             | GpiPlayMetaFile              |              |

|                             | GpiSaveMetaFile              |              |

+-----------------------------+------------------------------+--------------+

| GpiDeleteMetaFileExh        | GpiAssociate                 | Raw          |

|                             | GpiCharString                |              |

|                             | GpiCreatePS                  |              |

|                             | GpiDeleteMetaFile            |              |

|                             | GpiDestroyPS                 |              |

|                             | GpiLoadMetaFile              |              |

|                             | GpiMove                      |              |

|                             | GpiPlayMetaFile              |              |

|                             | GpiSaveMetaFile              |              |

+-----------------------------+------------------------------+--------------+

| GpiLoadPlayMetaFileExh1     | GpiLoadMetaFile              | Raw          |

|                             | GpiPlayMetaFile              |              |

+-----------------------------+------------------------------+--------------+

| GpiLoadPlayMetaFileExh2     | GpiCharString                | Raw          |

|                             | GpiLoadMetaFile              |              |

|                             | GpiMove                      |              |

|                             | GpiPlayMetaFile              |              |

+-----------------------------+------------------------------+--------------+

| GpiLoadPlayMetaFileExh3     | GpiCharString                | Raw          |

|                             | GpiLoadMetaFile              |              |

|                             | GpiMove                      |              |

|                             | GpiPlayMetaFile              |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryMetaFileLengthExh   | GpiCharString                | Raw          |

|                             | GpiLoadMetaFile              |              |

|                             | GpiMove                      |              |

|                             | GpiQueryMetaFileLength       |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  51

+---------------------------------------------------------------------------+

| Table 13 (Page 2 of 2). Test Cases and GPI Calls - GpiMeta Test Group.    |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiQuerySetMetaFileBitsExh  | GpiLoadMetaFile              | Raw          |

|                             | GpiPlayMetaFile              |              |

|                             | GpiQueryMetaFileBits         |              |

|                             | GpiQueryMetaFileLength       |              |

|                             | GpiSetMetaFileBits           |              |

+-----------------------------+------------------------------+--------------+

| GpiSaveMetaFileExh          | GpiAssociate                 | Raw          |

|                             | GpiCharString                |              |

|                             | GpiCharString                |              |

|                             | GpiCreatePS                  |              |

|                             | GpiDestroyPS                 |              |

|                             | GpiLoadMetaFile              |              |

|                             | GpiMove                      |              |

|                             | GpiSaveMetaFile              |              |

+-----------------------------+------------------------------+--------------+

GPIPATH TEST GROUP

Paths are used to draw and fill lines and non-rectangular

areas.  This testcase covers the accuracy of the path

limits, the basic functionality of path GPIs and each of

the path drawing functions in detail.

LIMITATIONS:  This testcase does not cover the use of

paths for clipping purposes.  See the test group GpiClip.

This table lists the test cases and the functions called

by each test case in the GpiPath test group.  The GpiPath

test group is found in GPIPATH.DLL.

52

+---------------------------------------------------------------------------+

| Table 14 (Page 1 of 2). Test Cases and GPI Calls - GpiPath Test Group.    |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiPathPre                  | GpiBeginPath                 |              |

|                             | GpiCloseFigure               |              |

|                             | GpiEndPath                   |              |

|                             | GpiFillPath                  |              |

|                             | GpiMove                      |              |

|                             | GpiOutlinePath               |              |

|                             | GpiPolyLine                  |              |

|                             | GpiSetLineWidthGeom          |              |

|                             | GpiSetPattern                |              |

|                             | GpiStrokePath                |              |

+-----------------------------+------------------------------+--------------+

| GpiPathApp                  | GpiBeginPath                 |              |

|                             | GpiEndPath                   |              |

|                             | GpiFillPath                  |              |

|                             | GpiFullArc                   |              |

|                             | GpiModifyPath                |              |

|                             | GpiMove                      |              |

|                             | GpiOutlinePath               |              |

|                             | GpiPointArc                  |              |

|                             | GpiPolyFillet                |              |

|                             | GpiPolyLine                  |              |

|                             | GpiSetLineEnd                |              |

|                             | GpiSetLineJoin               |              |

|                             | GpiSetLineWidthGeom          |              |

|                             | GpiSetPattern                |              |

|                             | GpiStrokePath                |              |

+-----------------------------+------------------------------+--------------+

| GpiFillPathExh              | GpiBeginPath                 |              |

|                             | GpiEndPath                   |              |

|                             | GpiFillPath                  |              |

|                             | GpiModifyPath                |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiSetLineWidthGeom          |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiOutlinePathExh           | GpiBeginPath                 |              |

|                             | GpiCloseFigure               |              |

|                             | GpiEndPath                   |              |

|                             | GpiMove                      |              |

|                             | GpiOutlinePath               |              |

|                             | GpiPolyLine                  |              |

|                             | GpiSetLineWidth              |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  53

+---------------------------------------------------------------------------+

| Table 14 (Page 2 of 2). Test Cases and GPI Calls - GpiPath Test Group.    |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiStrokePathExh            | GpiBeginPath                 |              |

|                             | GpiCloseFigure               |              |

|                             | GpiEndPath                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiSetLineWidthGeom          |              |

|                             | GpiSetPattern                |              |

|                             | GpiStrokePath                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetLineExh               | GpiBeginPath                 |              |

|                             | GpiEndPath                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryLineJoin             |              |

|                             | GpiQueryLineWidthGeom        |              |

|                             | GpiSetLineEnd                |              |

|                             | GpiSetLineJoin               |              |

|                             | GpiSetLineWidthGeom          |              |

|                             | GpiSetPattern                |              |

|                             | GpiStrokePath                |              |

+-----------------------------+------------------------------+--------------+

GPIPATHCLIP TEST GROUP

The GpiPathClip test group exhaustively tests the GPI Set

Clip Path API.

This table lists the test cases and the functions called

by each test case in the GpiPathclip test group.  The

GpiPathclip test group is found in GPIPATH.DLL.

54

+---------------------------------------------------------------------------+

| Table 15 (Page 1 of 3). Test Cases and GPI Calls - GpiPathclip Test       |

|                         Group.                                            |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSetClipPathPre           | GpiBeginPath                 |              |

|                             | GpiBox                       |              |

|                             | GpiCharStringAt              |              |

|                             | GpiEndPath                   |              |

|                             | GpiMove                      |              |

|                             | GpiSetClipPath               |              |

+-----------------------------+------------------------------+--------------+

| GpiSetClipPathApp1          | GpiBeginPath                 |              |

|                             | GpiCharStringAt              |              |

|                             | GpiCreateLogFont             |              |

|                             | GpiEndPath                   |              |

|                             | GpiQueryFonts                |              |

|                             | GpiSetCharBox                |              |

|                             | GpiSetCharSet                |              |

|                             | GpiSetClipPath               |              |

+-----------------------------+------------------------------+--------------+

| GpiSetClipPathApp2          | GpiBeginPath                 |              |

|                             | GpiBox                       |              |

|                             | GpiEndPath                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolySpline                |              |

|                             | GpiSetClipPath               |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetClipPathApp3          | GpiBeginPath                 |              |

|                             | GpiCharStringAt              |              |

|                             | GpiCreateLogFont             |              |

|                             | GpiEndPath                   |              |

|                             | GpiQueryFonts                |              |

|                             | GpiSetCharSet                |              |

|                             | GpiSetClipPath               |              |

+-----------------------------+------------------------------+--------------+

| GpiSetClipPathExh1          | GpiBeginPath                 |              |

|                             | GpiCharStringAt              |              |

|                             | GpiEndPath                   |              |

|                             | GpiSetClipPath               |              |

+-----------------------------+------------------------------+--------------+

| GpiSetClipPathExh2          | GpiBeginPath                 |              |

|                             | GpiCharStringAt              |              |

|                             | GpiEndPath                   |              |

|                             | GpiSetClipPath               |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  55

+---------------------------------------------------------------------------+

| Table 15 (Page 3 of 3). Test Cases and GPI Calls - GpiPathclip Test       |

|                         Group.                                            |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSetClipPathExh3          | GpiBeginPath                 |              |

|                             | GpiCharStringAt              |              |

|                             | GpiEndPath                   |              |

|                             | GpiSetClipPath               |              |

+-----------------------------+------------------------------+--------------+

| GpiSetClipPathExh4          | GpiBeginPath                 |              |

|                             | GpiCharStringAt              |              |

|                             | GpiEndPath                   |              |

|                             | GpiSetClipPath               |              |

+-----------------------------+------------------------------+--------------+

| GpiSetClipPathExh5          | GpiBeginPath                 |              |

|                             | GpiCharStringAt              |              |

|                             | GpiEndPath                   |              |

|                             | GpiSetClipPath               |              |

+-----------------------------+------------------------------+--------------+

| GpiSetClipPathExh6          | GpiBeginPath                 |              |

|                             | GpiCharStringAt              |              |

|                             | GpiEndPath                   |              |

|                             | GpiSetClipPath               |              |

+-----------------------------+------------------------------+--------------+

GPIREGION TEST GROUP

The GpiRegion test case tests the GPI Region APIs.

This table lists the test cases and the functions called

by each test case in the GpiRegion test group.  The

GpiRegion test group is found in GPIREGN.DLL.

56

+---------------------------------------------------------------------------+

| Table 16 (Page 1 of 2). Test Cases and GPI Calls - GpiRegion Test Group.  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiRegionApp1               | GpiBox                       | Direct       |

|                             | GpiCharStringAt              |              |

|                             | GpiCombineRegion             |              |

|                             | GpiCreateRegion              |              |

|                             | GpiMove                      |              |

|                             | GpiPaintRegion               |              |

|                             | GpiPtInRegion                |              |

|                             | GpiRectInRegion              |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiCombineRegionExh1        | GpiBox                       | Direct       |

|                             | GpiCombineRegion             |              |

|                             | GpiCreateRegion              |              |

|                             | GpiDestroyRegion             |              |

|                             | GpiMove                      |              |

|                             | GpiPaintRegion               |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiCombineRegionExh2        | GpiCombineRegion             |              |

|                             | GpiCreateRegion              |              |

|                             | GpiDestroyRegion             |              |

|                             | GpiPaintRegion               |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiCreateRegionExh          |                              |              |

+-----------------------------+------------------------------+--------------+

| GpiEqualRegionExh           | GpiCreateRegion              |              |

|                             | GpiDestroyRegion             |              |

|                             | GpiEqualRegion               |              |

|                             | GpiPaintRegion               |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiOffsetRegionExh          | GpiCreateRegion              |              |

|                             | GpiDestroyRegion             |              |

|                             | GpiOffsetRegion              |              |

|                             | GpiPaintRegion               |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiPtInRegionExh            | GpiCharStringAt              |              |

|                             | GpiCreateRegion              |              |

|                             | GpiDestroyRegion             |              |

|                             | GpiPaintRegion               |              |

|                             | GpiPtInRegion                |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  57

+---------------------------------------------------------------------------+

| Table 16 (Page 1 of 2). Test Cases and GPI Calls - GpiRegion Test Group.  |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiRectInRegionExh          | GpiBox                       |              |

|                             | GpiCreateRegion              |              |

|                             | GpiDestroyRegion             |              |

|                             | GpiMove                      |              |

|                             | GpiPaintRegion               |              |

|                             | GpiRectInRegion              |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetRegionExh             | GpiCreateRegion              |              |

|                             | GpiDestroyRegion             |              |

|                             | GpiPaintRegion               |              |

|                             | GpiSetPattern                |              |

|                             | GpiSetRegion                 |              |

+-----------------------------+------------------------------+--------------+

| GpiQueryRegionBoxExh        | GpiCreateRegion              |              |

|                             | GpiPaintRegion               |              |

|                             | GpiQueryRegionBox            |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

GPISEGMENT TEST GROUP

The GpiSegment test case tests the GPI functions associ-

ated with segments, including:  opening, drawing,

deleting, and closing.

This table lists the test cases and the functions called

by each test case in the GpiSegment test group.  The

GpiSegment test group is found in GPISEG.DLL.

58

+---------------------------------------------------------------------------+

| Table 17 (Page 1 of 5). Test Cases and GPI Calls - GpiSegment Test Group. |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiOpenSegmentExh           | GpiDeleteSegments            | Normal       |

|                             | GpiDrawChain                 |              |

|                             | GpiQueryDrawingMode          |              |

|                             | GpiQueryPS                   |              |

|                             | GpiQuerySegmentNames         |              |

|                             | GpiResetPS                   |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetDrawingMode            |              |

+-----------------------------+------------------------------+--------------+

| GpiDeleteSegmentExh         | GpiCharStringAt              | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiDeleteSegment             |              |

|                             | GpiDeleteSegments            |              |

|                             | GpiDrawChain                 |              |

|                             | GpiDrawSegment               |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiResetPS                   |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetInitialSegmentAttrs    |              |

+-----------------------------+------------------------------+--------------+

| GpiDeleteSegmentsExh        | GpiDeleteSegments            | Normal       |

|                             | GpiDrawSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawingMode            |              |

+-----------------------------+------------------------------+--------------+

| GpiDrawChainExh1            | GpiBeginArea                 | Direct and   |

|                             | GpiCallSegmentMatrix         | Normal       |

|                             | GpiCharStringAt              |              |

|                             | GpiCloseSegment              |              |

|                             | GpiDeleteSegments            |              |

|                             | GpiDrawFrom                  |              |

|                             | GpiDrawChain                 |              |

|                             | GpiEndArea                   |              |

|                             | GpiMove                      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiPolyFillet                |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiDrawChainExh2            | GpiQueryPS                   | Normal       |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  59

+---------------------------------------------------------------------------+

| Table 17 (Page 3 of 5). Test Cases and GPI Calls - GpiSegment Test Group. |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiDrawAndRemoveDynamicsExh | GpiDrawChain                 | Normal       |

|                             | GpiDrawDynamics              |              |

|                             | GpiQueryPS                   |              |

|                             | GpiRemoveDynamics            |              |

|                             | GpiResetPS                   |              |

|                             | GpiSetSegmentTransformMatrix |              |

+-----------------------------+------------------------------+--------------+

| GpiDrawFromExh1             | GpiCharStringAt              | Direct and   |

|                             | GpiCloseSegment              | Normal       |

|                             | GpiDeleteSegment             |              |

|                             | GpiDeleteSegments            |              |

|                             | GpiDrawChain                 |              |

|                             | GpiDrawForm                  |              |

|                             | GpiDrawSegment               |              |

|                             | GpiDrawSegments              |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiResetPS                   |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetDrawingMode            |              |

+-----------------------------+------------------------------+--------------+

| GpiDrawFromExh2             | GpiBeginArea                 | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiDeleteSegment             |              |

|                             | GpiDeleteSegments            |              |

|                             | GpiDrawChain                 |              |

|                             | GpiDrawSegment               |              |

|                             | GpiDrawSegments              |              |

|                             | GpiEndArea                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetAttrMode               |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetDrawControl            |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

60

+---------------------------------------------------------------------------+

| Table 17 (Page 4 of 5). Test Cases and GPI Calls - GpiSegment Test Group. |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiDrawSegmentExh           | GpiBeginArea                 | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiDeleteSegment             |              |

|                             | GpiDeleteSegments            |              |

|                             | GpiDrawSegment               |              |

|                             | GpiDrawSegments              |              |

|                             | GpiEndArea                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawControl            |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiEraseExh                 | GpiBeginPath                 | Direct,      |

|                             | GpiCloseSegment              | Standard,    |

|                             | GpiCreateRegion              | and Normal   |

|                             | GpiDrawSegment               | or           |

|                             | GpiEndPath                   | Queued, Raw, |

|                             | GpiErase                     | and Normal   |

|                             | GpiFullArc                   |              |

|                             | GpiMove                      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetClipPath               |              |

|                             | GpiSetClipRegion             |              |

|                             | GpiSetDefaultViewMatrix      |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetGraphicsField          |              |

|                             | GpiSetModelTransformMatrix   |              |

|                             | GpiSetPageViewport           |              |

|                             | GpiSetViewingLimits          |              |

+-----------------------------+------------------------------+--------------+

| GpiResetAndQryBoundaryDataEx| GpiBox                       | Normal       |

|                             | GpiCharStringAt              |              |

|                             | GpiConvert                   |              |

|                             | GpiDrawFrom                  |              |

|                             | GpiMove                      |              |

|                             | GpiQueryBoundaryData         |              |

|                             | GpiQueryPS                   |              |

|                             | GpiResetBoundaryData         |              |

|                             | GpiResetPS                   |              |

|                             | GpiSetDrawControl            |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  61

+---------------------------------------------------------------------------+

| Table 17 (Page 5 of 5). Test Cases and GPI Calls - GpiSegment Test Group. |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryInitSegmentAttrs|xGpiQueryInitialSegmentAttrs  | Normal       |

|                             | GpiQueryPS                   |              |

|                             | GpiQuerySegmentAttrs         |              |

|                             | GpiSetInitialSegmentAttrs    |              |

|                             | GpiSetSegmentAttrs           |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQrySegmentAttrsExh | GpiCloseSegment              | Normal       |

|                             | GpiDeleteSegment             |              |

|                             | GpiDrawChain                 |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiQuerySegmentAttrs         |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetSegmentAttrs           |              |

+-----------------------------+------------------------------+--------------+

| GpiQuerySegmentNamesExh     | GpiCloseSegment              | Normal       |

|                             | GpiDeleteSegments            |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiQuerySegmentNames         |              |

|                             | GpiSetDrawingMode            |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQrySegmentPriorityE|hGpiDrawChain                 | Normal       |

|                             | GpiQueryPS                   |              |

|                             | GpiQuerySegmentPriority      |              |

|                             | GpiResetPS                   |              |

|                             | GpiSetSegmentPriority        |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryStopDrawExh     | GpiQueryPS                   | Normal       |

|                             | GpiQueryStopDraw             |              |

|                             | GpiSetSegmentAttrs           |              |

|                             | GpiSetStopDraw               |              |

+-----------------------------+------------------------------+--------------+

GPISHADE TEST GROUP

The GpiShade test group tests Pel spacing and shading,

vertical box overlapping in Overpaint mode, circle

shading in Fill and Outline mode, and FM_OR shading

against the color table.

This table lists the test cases and the functions called

by each test case in the GpiShade test group.  The

GpiShade test group is found in GPISHADE.DLL.

62

+---------------------------------------------------------------------------+

| Table 18 (Page 1 of 3). Test Cases and GPI Calls - GpiShade Test Group.   |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiShadePelSpace            | GpiBox                       |              |

|                             | GpiCharString                |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiShadeOverlapBox          | GpiBox                       |              |

|                             | GpiCharString                |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiShadeOverlapCircle1      | GpiFullArc                   |              |

|                             | GpiMove                      |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiShadeORing-1             | GpiBox                       |              |

|                             | GpiCharString                |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetMix                    |              |

+-----------------------------+------------------------------+--------------+

| GpiShadeORing-2             | GpiBox                       |              |

|                             | GpiCharString                |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetMix                    |              |

+-----------------------------+------------------------------+--------------+

| GpiShadeORing-3             | GpiBox                       |              |

|                             | GpiCharString                |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetMix                    |              |

+-----------------------------+------------------------------+--------------+

| GpiShadeORing-4             | GpiBox                       |              |

|                             | GpiCharString                |              |

|                             | GpiFullArc                   |              |

|                             | GpiMove                      |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetColor                  |              |

|                             | GpiSetMix                    |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  63

+---------------------------------------------------------------------------+

| Table 18 (Page 3 of 3). Test Cases and GPI Calls - GpiShade Test Group.   |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiShadeBlue                | GpiBox                       |              |

|                             | GpiCharStringAt              |              |

|                             | GpiCreateLogColorTable       |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiShadeGreen               | GpiBox                       |              |

|                             | GpiCharStringAt              |              |

|                             | GpiCreateLogColorTable       |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiShadeRed                 | GpiBox                       |              |

|                             | GpiCharStringAt              |              |

|                             | GpiCreateLogColorTable       |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiShadeGray                | GpiBox                       |              |

|                             | GpiCharStringAt              |              |

|                             | GpiCreateLogColorTable       |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiShadeAll                 | GpiBox                       |              |

|                             | GpiCharStringAt              |              |

|                             | GpiCreateLogColorTable       |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiShadeOverlapCircle2      | GpiFullArc                   |              |

|                             | GpiMove                      |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiShadeOverlapBox2         | GpiBox                       |              |

|                             | GpiCharString                |              |

|                             | GpiMove                      |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

64

GPISTANDARD TEST GROUP

This module is called by the PTT to test GPI Standard

Precision.

This testcase consists of the drawing of a set of grids,

using GPI functions, and measuring their accuracy in

printed form.  This is a visual test; therefore, logging

is minimal.

LIMITATIONS:  The GPI APIs used to draw the grids are not

tested for functionality and exhaustive return codes in

this testcase.  They are covered in the GpiLine test

group.

This table lists the test cases and the functions called

by each test case in the GpiStandard test group.  The

GpiStandard test group is found in GPISTD.DLL.

+---------------------------------------------------------------------------+

| Table 19. Test Cases and GPI Calls - GpiStandard Test Group.              |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiStandardPre1             | GpiCharStringAt              |              |

|                             | GpiLine                      |              |

|                             | GpiSetCurrentPosition        |              |

+-----------------------------+------------------------------+--------------+

| GpiStandardPre2             | GpiLine                      |              |

|                             | GpiSetCurrentPosition        |              |

+-----------------------------+------------------------------+--------------+

| GpiStandardPre3             | GpiLine                      |              |

|                             | GpiSetCurrentPosition        |              |

+-----------------------------+------------------------------+--------------+

GPITRANSFORM TEST GROUP

The GpiTransform test group tests:

o   The Page Viewpoint functions

o   Common support functions to set up a transform matrix

o   That all transform GPI work together.

o   Manipulates the transform GPI relationships and com-

bination possibilities

o   Modal transforms

o   Segment transforms

o   The Default View Matrix

o   The Viewing transform GPIs

Printer Test Tool (PTT)  65

This table lists the test cases and the functions called

by each test case in the GpiTransform test group.  The

GpiTransform test group is found in GPIXFORM.DLL.

+---------------------------------------------------------------------------+

| Table 20 (Page 1 of 4). Test Cases and GPI Calls - GpiTransform Test      |

|                         Group.                                            |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryPageViewportPre | GpiBox                       | Direct       |

|                             | GpiMove                      |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryPageViewport         |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetPageViewport           |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryModelXformMatPre| GpiBox                       | Direct       |

|                             | GpiMove                      |              |

|                             | GpiQueryModelTransformMatrix |              |

|                             | GpiSetModelTransformMatrix   |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQrySegmentXformMatP|eGpiBox                       | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiDrawSegment               |              |

|                             | GpiMove                      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiQuerySegmentTransformMatri|              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetInitialSegmentAttrs    |              |

|                             | GpiSetPattern                |              |

|                             | GpiSetSegmentTransformMatrix |              |

+-----------------------------+------------------------------+--------------+

| GpiCallSegmentMatrixPre     | GpiBox                       | Direct and   |

|                             | GpiCallSegmentMatrix         | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiMove                      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetInitialSegmentAttrs    |              |

|                             | GpiSetModelTransformMatrix   |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

66

+---------------------------------------------------------------------------+

| Table 20 (Page 2 of 4). Test Cases and GPI Calls - GpiTransform Test      |

|                         Group.                                            |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryDefViewMatrixPre| GpiBox                       |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDefaultViewMatrix    |              |

|                             | GpiSetDefaultViewMatrix      |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryViewingXformMatP|eGpiBox                       | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiDrawSegment               |              |

|                             | GpiMove                      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiQueryViewingTransformMatri|              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetPattern                |              |

|                             | GpiSetViewingTransformMatrix |              |

+-----------------------------+------------------------------+--------------+

| GpiTransformApp             | GpiBox                       | Direct and   |

|                             | GpiCallSegmentMatrix         | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiDrawSegment               |              |

|                             | GpiMove                      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryDefaultViewMatrix    |              |

|                             | GpiQueryModelTransformMatrix |              |

|                             | GpiQueryPageViewport         |              |

|                             | GpiQueryPS                   |              |

|                             | GpiQuerySegmentTransformMatri|              |

|                             | GpiQueryViewingTransformMatri|              |

|                             | GpiSetDefaultViewMatrix      |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetInitialSegmentAttrs    |              |

|                             | GpiSetModelTransformMatrix   |              |

|                             | GpiSetPageViewport           |              |

|                             | GpiSetPattern                |              |

|                             | GpiSetSegmentTransformMatrix |              |

|                             | GpiSetViewingTransformMatrix |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryPageViewportExh | GpiBox                       | Direct       |

|                             | GpiMove                      |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryPageViewport         |              |

|                             | GpiSetPageViewport           |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  67

+---------------------------------------------------------------------------+

| Table 20 (Page 3 of 4). Test Cases and GPI Calls - GpiTransform Test      |

|                         Group.                                            |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryModelXformMatExh| GpiBox                       | Direct       |

|                             | GpiMove                      |              |

|                             | GpiQueryModelTransformMatrix |              |

|                             | GpiSetModelTransformMatrix   |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQrySegmentXformMatE|hGpiBox                       | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiDrawSegment               |              |

|                             | GpiMove                      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiQuerySegmentTransformMatri|              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetInitialSegmentAttrs    |              |

|                             | GpiSetPattern                |              |

|                             | GpiSetSegmentTransformMatrix |              |

+-----------------------------+------------------------------+--------------+

| GpiCallSegmentMatrixExh     | GpiBox                       | Direct and   |

|                             | GpiCallSegmentMatrix         | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiMove                      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetInitialSegmentAttrs    |              |

|                             | GpiSetModelTransformMatrix   |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryDefViewMatrixExh| GpiBox                       |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDefaultViewMatrix    |              |

|                             | GpiSetDefaultViewMatrix      |              |

|                             | GpiSetPattern                |              |

+-----------------------------+------------------------------+--------------+

| GpiSetAndQryViewingXformMatE|hGpiBox                       | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiDrawSegment               |              |

|                             | GpiMove                      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiQueryPS                   |              |

|                             | GpiQueryViewingTransformMatri|              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetPattern                |              |

|                             | GpiSetViewingTransformMatrix |              |

+-----------------------------+------------------------------+--------------+

68

+---------------------------------------------------------------------------+

| Table 20 (Page 4 of 4). Test Cases and GPI Calls - GpiTransform Test      |

|                         Group.                                            |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiConvertExh               | GpiConvert                   |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryPageViewport         |              |

|                             | GpiQueryPS                   |              |

|                             | GpiSetDefaultViewMatrix      |              |

|                             | GpiSetModelTransformMatrix   |              |

|                             | GpiSetPageViewport           |              |

+-----------------------------+------------------------------+--------------+

GPIVIEW TEST GROUP

The GpiView test group tests the following GPIs:

o   GpiSetDefViewingLimits

o   GpiSetGraphicsField

o   GpiSetViewingLimits

o   GpiSetPageViewport

This table lists the test cases and the functions called

by each test case in the GpiView test group.  The GpiView

Test Group is found in GPIVIEW.DLL.

+---------------------------------------------------------------------------+

| Table 21 (Page 1 of 2). Test Cases and GPI Calls - GpiView Test Group.    |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSetDefViewingLimitsPre   | GpiResetPS                   |              |

|                             | GpiSetDefViewingLimits       |              |

+-----------------------------+------------------------------+--------------+

| GpiSetGraphicsFieldPre      | GpiSetGraphicsField          |              |

+-----------------------------+------------------------------+--------------+

| GpiSetViewingLimitsPre      | GpiSetViewingLimits          |              |

+-----------------------------+------------------------------+--------------+

| GpiViewApp                  | GpiResetPS                   |              |

|                             | GpiSetDefViewingLimits       |              |

|                             | GpiSetGraphicsField          |              |

|                             | GpiSetViewingLimits          |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  69

+---------------------------------------------------------------------------+

| Table 21 (Page 2 of 2). Test Cases and GPI Calls - GpiView Test Group.    |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSetDefViewingLimitsExh1  | GpiAssociate                 | Direct and   |

|                             | GpiQueryDevice               | Normal       |

|                             | GpiSetDefViewingLimits       |              |

+-----------------------------+------------------------------+--------------+

| GpiSetDefViewingLimitsExh2  | GpiBox                       | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiDrawChain                 |              |

|                             | GpiMove                      |              |

|                             | GpiOpenSegment               |              |

|                             | GpiSetDefViewingLimits       |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetInitialSegmentAttrs    |              |

+-----------------------------+------------------------------+--------------+

| GpiSetViewingLimitsExh1     | GpiCallSegmentMatrix         | Normal       |

|                             | GpiCloseSegment              |              |

|                             | GpiOpenSegment               |              |

|                             | GpiSetDrawingMode            |              |

|                             | GpiSetInitialSegmentAttrs    |              |

|                             | GpiSetViewingLimits          |              |

+-----------------------------+------------------------------+--------------+

| GpiSetViewingLimitsExh2     | GpiSetViewingLimits          |              |

+-----------------------------+------------------------------+--------------+

| GpiSetPageViewportExh       | GpiSetPageViewport           | Direct       |

+-----------------------------+------------------------------+--------------+

STRESS TEST GROUP

The STRESS test group uses GPI test cases which use a

great deal of system resources, such as memory and

swapper file space.

This table lists the test cases and the functions called

by each test case in the STRESS test group.  The STRESS

Test Group is found in STRESS.DLL.

70

+---------------------------------------------------------------------------+

| Table 22 (Page 1 of 3). Test Cases and GPI Calls - STRESS Test Group.     |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiClipFontExh              | GpiDeleteSetId               | Raw          |

|                             | GpiResetPS                   |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiClipBitsExh              | GpiBeginPath                 | Raw          |

|                             | GpiEndPath                   |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiSetClipPath               |              |

|                             | GpiSetLineWidthGeom          |              |

|                             | GpiSetPel                    |              |

+-----------------------------+------------------------------+--------------+

| GpiClipColorExh             | GpiBeginPath                 | Raw          |

|                             | GpiEndPath                   |              |

|                             | GpiFullArc                   |              |

|                             | GpiMove                      |              |

|                             | GpiPartialArc                |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetClipPath               |              |

|                             | GpiSetLineWidthGeom          |              |

|                             | GpiStrokePath                |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

| GpiMaxPolyLinesExh          | GpiMove                      | Raw          |

|                             | GpiPolyLine                  |              |

+-----------------------------+------------------------------+--------------+

| GpiMaxPolySplinesExh        | GpiMove                      | Raw          |

|                             | GpiPolySpline                |              |

+-----------------------------+------------------------------+--------------+

| GpiCharBoxMetricsInteractive| GpiCharString                | Raw          |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryTextBox              |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiCharBoxMetricsDefault    | GpiCharString                | Raw          |

|                             | GpiMove                      |              |

|                             | GpiPolyLine                  |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryTextBox              |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

Printer Test Tool (PTT)  71

+---------------------------------------------------------------------------+

| Table 22 (Page 3 of 3). Test Cases and GPI Calls - STRESS Test Group.     |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiSpaceInteractive         | GpiCharString                | Raw          |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiQueryDevice               |              |

|                             | GpiQueryTextBox              |              |

|                             | GpiSetCharSet                |              |

+-----------------------------+------------------------------+--------------+

| GpiBitMapClipExh            | GpiBeginPath                 | Raw          |

|                             | GpiDeleteBitmap              |              |

|                             | GpiEndPath                   |              |

|                             | GpiFullArc                   |              |

|                             | GpiLoadBitmap                |              |

|                             | GpiMove                      |              |

|                             | GpiQueryBitmapParameters     |              |

|                             | GpiSetArcParams              |              |

|                             | GpiSetClipPath               |              |

|                             | GpiSetLineWidthGeom          |              |

|                             | GpiStrokePath                |              |

|                             | GpiWCBitBlt                  |              |

+-----------------------------+------------------------------+--------------+

| GpiThinLinesInPath          | GpiBeginPath                 |  Normal      |

|                             | GpiCharString                |              |

|                             | GpiEndPath                   |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiSetLineWidthGeom          |              |

|                             | GpiSetPattern                |              |

|                             | GpiStrokePath                |              |

+-----------------------------+------------------------------+--------------+

| GpiThickLinesInPath         | GpiBeginPath                 | Raw          |

|                             | GpiCharString                |              |

|                             | GpiEndPath                   |              |

|                             | GpiLine                      |              |

|                             | GpiMove                      |              |

|                             | GpiSetLineWidthGeom          |              |

|                             | GpiSetPattern                |              |

|                             | GpiStrokePath                |              |

+-----------------------------+------------------------------+--------------+

| GpiShadeCharactersInteractiv| GpiCharString                | Raw          |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiQueryTextBox              |              |

|                             | GpiSetCharSet                |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+

72

+---------------------------------------------------------------------------+

| Table 22 (Page 1 of 3). Test Cases and GPI Calls - STRESS Test Group.     |

+-----------------------------+------------------------------+--------------+

| TEST CASE                   | GPI FUNCTIONS CALLED         | CONFIGURA-   |

|                             |                              | TION         |

|                             |                              | SETTINGS     |

+-----------------------------+------------------------------+--------------+

| GpiShadeCharactersDefault   | GpiCharString                | Raw          |

|                             | GpiDeleteSetId               |              |

|                             | GpiMove                      |              |

|                             | GpiQueryTextBox              |              |

|                             | GpiSetCharSet                |              |

|                             | GpiSetColor                  |              |

+-----------------------------+------------------------------+--------------+