COOLFM.DOC File Version 2.0beta6 for

coolfm for eComStation

(c) Copyright eCo Software, 1998 - 2003.

All rights Reserved.

Contents

========

1.0  Overview

2.0  Supported hardware

3.0  Installation

3.1  Manual installation of coolfm

3.2  Additional drivers installation instructions

4.0  Usage of coolfm

4.1 Editing events

4.2 Recording transmission

4.3 Using Named Pipe

4.4 Using coolfm/VIO

5.0  Troubleshooting

6.0  Contact information

1.0  Overview

=============

coolfm is an application which allows you control fm-tuners for PC in

eComStation environment. It uses plugin mechanism so developers will create

new plugins in the future and modify existing. This is a guarantee of

your investments.

2.0  Supported hardware

=======================

coolfm supports ISA, PCI and USB fm-tuners.

USB:

# D-Link DU-R100, DSB-R100                                  (tested)

PCI, TV-onboard fm-tuners:

# LifeView Fly98/FM                                         (tested)

# AverTVPhone 98/FM                                         (tested)

# Hauppauge WinTV PCI-FM                                    (tested)

ISA:

# GemTek                                                    (tested)

# Aztech/Packard Bell/CompExpress                           (not tested)

# Mediaforte RadioMax SF16-FMX, RadioLink SF16-FMR/Typhoon  (not tested)

# Mediaforte RadioPlus SF16-FMP, RadioX SF16-FMI            (not tested)

# Mediaforte Legacy 128 SF16-FMD2, RadioLink SF16-FMR2      (not tested)

# AIMS RadioTrack, Radio Reveal RA300                       (tested)

# AIMS RadioTrack II                                        (not tested)

# Zoltrix radio plus                                        (not tested)

3.0  Installation

=================

It is recommended install coolfm using WarpIn installer. After installation

read 3.2 section and install auxiliary drivers (if necessary).

3.1 Manual installation of coolfm

---------------------------------

Remember, coolfm consists of two packs: binary data and user's data.

So, unzip coolfm binaries (you will update this directory in the future)

and directory with user's data (you can keep coolfm.key, stations info here)

to target directories. Create coolfm.cfg file in binaries directory and

add two lines:

[CooLFM-New]

userdata=%user's data directory%

coolfm will search Stations, Events, Skins in %user's data directory%.

3.2  Additional Drivers Installation Instructions

-------------------------------------------------

coolfm supports devices with different interfaces. So, you have to install

additional drivers in some cases.

USB:

a) Install basic USB support (check presence of this lines in your config.sys)

BASEDEV=USBUHCD.SYS

BASEDEV=USBD.SYS /REQ:USBUHCD$

BASEDEV=USBHID.SYS

b) Install Netlabs usbresmg.sys driver.

URL: ftp://ftp.netlabs.org/pub/usb/usbres.zip

Add a line to your config.sys:

DEVICE=%USBRESMG_path%\usbresmg.sys

c) usbcalls.dll should be located in directory mentioned in LIBPATH variable.

This library is located in coolfm's Devices\ directory.

d) Select 'D-Link DU-R100' device in coolfm settings notebook.

PCI, TV-onboard fm-tuners:

a) coolfm uses BTTV/2 subsystem. You have to download the latest BTTV/2

distribution pack, configure it following its documentation.

URL: http://www.nord-com.net/s.milcke/bttv_en.htm

For example, if you are going to use onboard AverTVPhone 98/FM fm-tuner

then add two lines to config.sys

DEVICE=%BTTV_path%\LXAPI32.sys

DEVICE=%BTTV_path%\BT32.sys /V /MODPARM:bttv:tuner=6;radio=1

b) StBTV4LX.DLL should be located in directory mentioned in LIBPATH variable.

c) Select 'BT8xx onboard' device in coolfm settings notebook.

ISA:

a) Plugins for ISA tuners use standard system TESTCFG driver.

Make sure that your config.sys contains line:

DEVICE=C:\OS2\BOOT\TESTCFG.SYS

b) Select your ISA fm-tuner in coolfm settings notebook and press

right mouse button over tuner icon to call I/O port selection menu.

4.0 Usage of coolfm

===================

This section contains tips only.

4.1 Editing events

------------------

Saving changes:

There are two methods to save changes:

a) Switch to another event or

b) Double click 'Activate' button

4.2 Recording transmission

--------------------------

Buffer indicator:

When using codec to compress audio in real-time mode, pay attention

to 'Buffer' field. This number indicates overflow. If it

increases rapidly then your computer is late with encoding and

you have cancel recording.

4.3 Using Named Pipe

--------------------

You can activate coolfm/pm pipe (look Hardware page) and then

send commands to this pipe from external program or from remote computer.

(example of pipe usage: coolfm-pipe.cmd).

The name of pipe is stored in coolfm.cfg:

npipe=\PIPE\COOLFMCONTROL

You can control coolfm from remote computer. In this case send your

commands to: \\ComputerName\PIPE\COOLFMCONTROL

4.4 Using coolfm/VIO

--------------------

VIO version of coolfm is an addition to PM version, so use it after

configuring PM version only! (VIO version should know userdata directory,

fm-tuner model, etc.)

Suspend command:

If you own Brooktree TV-tuner with onboard fm-tuner and have installed

BTTV subsystem then you must use this command. Every time you send

a command to fm-tuner add $suspend in tail else BTTV turns off your device.

Example:

First startup: coolfmvio.exe 100.5 $unmute $suspend

To change frequency you run other session of coolfmvio.exe with new command

and $suspend in the tail.

coolfmvio.exe 102.1 $beep $suspend

The first coolfvio exits by itself altough your tuner continues to work

with the aid of the second active coolfmvio.

5.0 Troubleshooting

===================

Noise

Make sure that you have connected antenna to tuner.

Silence

Make sure that cable links tuner audio output with sound card

line in. If you insert cable into Microphone socket then enable

Mic in audio Mixer.

Events & stations disappeared!

Find directory with your data and add reference to this directory

to coolfm.cfg file:

[CooLFM-New]

userdata=%user's data directory%

Traps after using lxlite

don't pack coolfm with lxlite because it was compiled with VAC4.

Reinstall coolfm if broken.

Hang on exit

Under debugging

6.0  Contact information

========================

Homepage of coolfm                  -- http://ecomstation.ru/coolfm

Send e-mail to Eugene Gorbunoff     -- eg@ecomstation.ru

Phone number                        -- +7-812-5565877

Buy end-user license in Mensys      -- http://shop.mensys.nl/catalogue/mns_coolfm.html

coolfm is a product of eCo Software -- http://ecomstation.ru/ecosoft

More eCo Software products:

http://ecomstation.ru/ecosoft/index.php?action=ga