.*;     SCCSID =

.se os2 = 'OS/2'

.se comp = 'COM01'

.*.us 1

.*.ce &comp. Component

:h3.&comp. Architecture

:h4.Problem Description/Objectives

:p.

The family1 asynchronous communications port device driver

should provide support for some of the capabilities of the new

Extended ISA (EISA) systems that are becoming available to OS/2 users.

The specific areas that need to be addressed are support for

more than two COM ports, and interrupt sharing.

:p.

The majority of the code added will be used at initialization time

to parse the EISA system's configuration information to determine

if the system has any extended capabilities that need to be supported.

There will be NO IMPACT on compatibility for AT-class machines; the

new code will never be executed when run on an AT-class machine.

:h4.Solutions/Justification

:p.

COM01.SYS was originally written to support AT-class machines only.  As

such, it only needed to support the two COM ports that were originally

designed for the AT.  EISA systems, with their expanded I/O space and an

ability to share interrupt lines, are a good deal more flexible.  The

user is allowed to configure the system to match their needs.  On many

of the EISA systems that are available, COM ports have been built

in to the system's motherboard.  If the user adds an ISA compatible

COM port, they can then re-configure one of the motherboard's

COM ports to be COM3.  OS/2 should provide support for this type of

configuration: the user should have access to COM ports beyond COM2

for OS/2 applications.

:p.

Another configurable parameter on EISA machines is the sharing of interrupt

lines.  On some of the EISA systems that provide COM ports, the COM ports can

be configured to share their interrupt line.  This is mostly useful in server

environments, where a large number of hardware adapters are added to the

system.   OS/2's COM driver should support the sharing of a COM port's

interrupt line; this will allow the user more flexibility in the

configuration of an EISA system that will be running OS/2.

:h4.Architectural Review

To be performed.

:h3.&comp. Design

:h4.Design Overview

:p.

The maximum number of COM ports supported on an EISA machine will be four.

Code will be added to parse the EISA system configuration information

at device driver install time to determine the COM port configuration.

A small amount of additional code will be added to the interrupt service

routine for support of any shared interrupt lines.

:p.

A small amount of code will be added to OS2LDR.  This code will make

sure that the return value from the DosDevConfig API contains the

correct number of asynchronous COM ports attached to an EISA system.

:h4.Data Structures Description

To be added.

:h4.Exported Interfaces

To be added.

:h4.Imported Interfaces

To be added.

:h4.Design Constraints

To be added.

:h4.Design Review

To be performed.

:h3.&comp. Implementation

:h4.Internal Interfaces

To be added.

:h4.Implementation

:p.

Two new device headers will be added to the COM01.SYS driver: COM3 and

COM4.  Minor changes will be needed in a few places to accomodate

these additional devices; this mostly consists of making the code

table driven as opposed to 'if not COM1 then assume COM2'.

:p.

A new module will be added to COM01.SYS for use at device driver

installation time.  The existing initialization code will call

a function within this module once, to set up any EISA configuration

information.  If not running on an EISA system, the new code will

simply set a flag indicating a non-EISA system and then return.  If

running on an EISA system, the EISA system's configuration information will

be searched for COM port information.  This information will be stored

in a table.

:p.

When running on an EISA system, as each of the COM device driver's

headers is initialized, this table of information will be used to

initialize the COM driver's data structure for the port.  The following

items will be saved in the COM driver's data structure: the port address,

the interrupt line, and whether or not the interrupt line is to be

shared.

:p.

A modification will be made to the interrupt service routine.

A check will be made to see if the COM port causing the interrupt

supports interrupt sharing or not.  If it does, then the UART's

Interrupt ID register is read; if it indicates that there is no pending

interrupt, then the driver immediately returns control to the kernel,

with an indication that the interrupt was not from the COM port.  Note

that this code will NOT affect the operation of the interrupt service

routine when running on an AT-class system, since interrupt sharing

is NEVER supported on AT-class systems.

:p.

The modification made to OS2LDR will be in the module DOSHLP.ASM.  The

current code is already parsing the EISA system configuration information

to determine the "sharing" status of the interrupt lines.  This code would

be modified to also count the number of asynchronous COM ports in the

system.  This value will be returned to the kernel via DHConfig.

:h4.Implementation Review

To be performed.

:h3.&comp. Appendix (included for each review)

:h4.Glossary

To be added.

:h4.Size and Performance Considerations

:p.

Interrupt sharing can have an affect on COM port throughput.  If the

user chooses to share a COM port's interrupt line with another device

that generates a high number of interrupts, COM performance can be

expected to suffer.  This is a system configuration issue, however,

not an OS/2 issue.