.*      SCCSID = @(#)kbd.doc	6.2 91/04/11

.*

.* symbol definitions

.im ..\..\equate.sci

.im ..\..\macros.sci

.*

:Bissues.kbdisu.isu

.*

.*

:h3.&vkd. Architecture

.*

.*

:h4.Problem Description/Objectives

:p.The &vkd. consists of virtual support for the

keyboard.  It allows keystrokes to be passed from the keyboard to

the &vdm..

It also allows for text to be pasted from another &pm. application

into the VDM as key strokes.

:p.This component allows extant &dosapps which access the

keyboard to operate without change.  Such applications may access

the keyboard via &cb calls, access to the &cb data area, and/or

monitoring of the keyboard interrupt and reading keystroke data

directly from the I/O ports.

:p.The &vkd. design must address the following issues:

:ul

:li.INT 21h Access

:lp.&dosapps may access the keyboard by opening the CON:

device or by default standard input access.

:li.&cb Access

:lp.&dosapps may access the keyboard by &cb INT 16h

:li.I/O Port Access

:lp.&dosapps may access the keyboard by reading and writing I/O

ports 60h and 64h.  This access is not restricted to reading scan

codes, but may include other types of access (such as LED

control).

:li.&pm Pasting

:lp.Data may be "pasted" in from other &os2 applications.

:li.Internationalization

:lp.Keystrokes are translated according to the current keyboard

translation table and code page.

:li.Performance

:lp.Key response must not be perceptably slower in the &vdm.

than in protected mode.

:li.Idle Detection

:lp.&dosapps must not waste excessive CPU time when in input

peek loops.

:eul

.*

.*

:h4.Solutions

:p.The &vkd. problems are solved from several

different perspectives:

:ul

.* DELETED :li.DOS

.* DELETED :li.CON Device Driver

:li.&cb

:li.I/O Ports

:li.Hardware Interrupts

:li.Idle Detection

:eul

.go skipit1

.* DELETED

.*

:h5.DOS Support

This support consists of keyboard access through the various INT

21h functions.  In particular, this is support for access from

the &dosmgr..  For compatibility, DOS calls the console device

driver to read keystrokes.  This allows the console device driver

to be replaced without ill effect.

.*

:h5.CON Device Driver Support

This support consists of allowing the console device driver to be

replaced.  Althought the default console device device driver

restricts keyboard access to &cb calls, user-supplied console

device drivers may access the keyboard through I/O ports or

through the &cb data area also.

...skipit1

.*

:h5.&cb Support

:p.Since the &cb INT 16h service only references the &cb data area

for keystrokes, the &cb will continue to provide this service.

:p.The &cb INT 09h interrupt service routine, however, is

emulated within the &vkd..  When the INT 09h vector

is invoked, the &vkd. passes the character to the &pdd, which

translates the character according to the current translate table.

:p.It is possible for an application or a TSR to "hook" the INT

09h vector.  Often, such applications simply monitor keystrokes

and pass them on to the previously hooked interrupt service

routine.  In this case, the &vkd. emulation code will

still run and translate scan codes according to the current

translation table.  On the other hand, if an application which

has hooked INT 09h fails to pass the scan code on, then the scan

code will not be translated nor placed into the &cb data area by

the &vkd..

.*DELETED

.go skipit2

calls on the &pdd to translate

the current scan code to a character and place the scan code and

the character into the &cb data area.  This allows all

translation code to remain in the &pdd.

...skipit2

.*

:h5.I/O Port Support

:p.The &vkd. is responsible for supporting programs that

access the keyboard ports directly.  These ports provide access

to data ports on a microprocessor that controls I/O to the

keyboard.  Primarily, this support is provided for applications

that simply read the scan code directly from the port.  Other

functions, however, are possible, such as setting the key repeat

rate.  Repeat rates affect the entire system;  per-VDM repeat

rates are not supported.  Extended scanner functions, such as providing

keyboard pre-controller translation, are not supported.  Such commands

are ignored.

.*

:h5.Hardware Interrupt Support

:p.The keyboard hardware normally issues interrupt information to

the interrupt controller when a keystroke is available.  To

assist in simulating this functionality, the &pdd will notify the

&vdd when keystrokes become available so that the &vdd can notify

the &vpic. component (VPIC).

Such notification will occur only when the current &vdm is in the

current keyboard focus.  The VPIC component is responsible for

simulating the interrupt to the &vdm..

.*

:h5.Pasting Support

:p.The &vkd. supports two levels of pasting from the clipboard into a VDM:

fast pasting and slow pasting.  In fast pasting support, characters

from the clipboard is put in the CBIOS buffer directly.  For those VDM

applications which get their key strokes from the CBIOS key buffer

(via Int 16h), fast pasting is adequate.  But for those VDM applications

which hook Int 09h or Int 15h (AH=4fh), they expect key scan sequences

which must be generated by the slow pasting mechanism.  In slow pasting,

every character from the clipboard is reverse translated into scan

sequence which may involve generating make and break scans of not only

the character itself but also the make and break of the shift keys

in order to adjust the upper/lower/control case of the character.

It may even involve generating alt-numpad scan sequences for those

characters which are not directly typable on the keyboard such as

the graphics or international characters.

The reverse translation issue is complicated by the fact that a VDM

can have its per VDM code page.  Therefore, the reverse translation

table is not a static structure for all the VDMs.  Every code page

will have its corresponding reverse translation table.  Since the

physical keyboard device driver has the access to all the code page

information, it is logical that the physical keyboard driver is

responsible for generating the reverse translation tables according

to a given code page number.  The &vkd. only requests the physical

keyboard driver to generate a reverse translation table for a

particular code page on demand.  In other words, if no VDM uses

a code page, the corresponding reverse translation table does not

exist in the &vkd..  Also, the reverse translation table is global

to all VDMs.  In other words, multiple VDMs with the same code page

number are sharing the same reverse translation table.  Therefore, the &vkd.

keeps a chain of reverse translation table on demand and each VDM

will have a pointer to the appropriate reverse translation tables according

to their code page numbers.  On the other side, if a new code page is

loaded in the physical keyboard driver which replaces existing code

page, the &vkd. is informed and the corresponding reverse translation

table is invalidated.  Then the next access to the invalid reverse

translation table will cause the &vkd. to request the reverse translation

table re-generation from the physical keyboard driver.

.*

:h5.Idle Detection Support

:p.&dosapps waste CPU time in a multitasking system when they

peek for input in a loop.

.*

:p.Idle conditions that must be detected&gml.

:ul

:li.Application is polling the keyboard using int 16h in a loop.

:li.DOS is polling in an input loop.

:li.Repeated int 28h calls from a &dosapp..

:li.Int 2fh, ax=1680h yield calls.

:li.DOS int 21h, service 0bh.

:eul

:p.VDHServices offered by &vkd..

.*

:p.The &vkd. will export two VDH services.

:ul.

:li.VDHReportPeek

:lp.Allows weighted reports of peek events.  Any VDD that detects

input peek events can report using this service.  The primary use

of the service is internal, for the &vkd. to track int 16h peek calls.

:li.VDHWakeIdle

:lp.Notes that a VDM is busy doing useful work.

If the VDM was judged to be idle, this call restores it

to normal use of the CPU.

Any VDD that uses

VDHReportPeek should also use the wake up call when input finally

arrives.  The &vkd. calls the service when keystrokes or paste

events occur.  Other VDDs report mouse button events, COM

input, printer output, and some video screen changes in windows.

:eul

.*

:p.Advanced properties.

:ul.

:li."Idle sensitivity (100%=Off)"

:lp.Idle detection judges applications

idle when they exhibit high rates of input peek events

(e.g. polling the keyboard to see if a key has been pressed).

Idle sensitivity sets the threshhold for considering a VDM idle to the

specified percent of the highest peek rate possible.

In other words, if sensitivity = 75%, then &dosapps.

issuing int 16h peeks at a rate higher than 75% of the maximum

possible are considered idle and are made to give up the CPU.

The default is 75%.

Applications that still dominate the CPU

when foreground and waiting for input should have the value

for this property decreased to lower the threshhold.

Applications that are incorrectly

judged idle when really busy should have the value increased

to increase the threshhold.

.*

:li."Idle seconds allowed"

:lp."Idle seconds allowed" forces a delay between the time

when a VDM is judged to be idle and when action is taken to

make it yield the CPU.  Some &dosapps. perform a burst of peeks

and then stop peeking after some number of attempts and

continue doing real work.

A delay

allows these applications to run.  When the delay is nonzero,

the VDM will not be considered idle for that number of seconds

after caught.  If the application stops peeking or performs

any detected work it will not be judged idle.

:eul

.*

:p.Detection techniques&gml.

:ul.

:li.Application is polling the keyboard using int 16h in a loop.

:lp.Applications in a peek loop will exhibit a high rate of peeks.

However, &dosapps may be doing real work between

peeks and some applications take a long time simply to repeat the peek

when really idle.  This makes it so we can never be certain

that an application is really idle.  An informed guess must be made

that works most of the time, and the user must be able to

control idle detection for applications not detected properly.

Whenever the weighted reports to VDHReportPeek exceed a

threshhold in a test period,

the VDM will be judged idle and will be made to sleep for a time.

VDHWaitVirrs is used to sleep allowing simulated interrupts to occur.

When busy activities like keyboard input, mouse button presses,

or non-peeking DOS calls occur, the sleep is terminated immediately.

Otherwise a timeout terminates the sleep.

The &vkd. will report int 16h peek events using a weight it callibrates

during the first few VDM creations.  Any other VDD that similarly

callibrated a weight could also add its peek reports.

Any VDD that reports peek events should also use VDHWakeIdle when

the event waited for occurs.  DOS also sets a busy flag whenever

it receives a non-peek service request or when it ends

a peek service request.   VDM's with high peek rates

are not considered idle when detected busy events are

also occuring.

.*

:li.DOS is polling in an input loop.

:lp.This is the same as when &dosapps peek at

the keyboard, but since we have control over DOS, we can

detect the condition more quickly.

DOS also has a very time consuming idle loop,

peeking at a slower rate than most applications.

This makes it more difficult to detect without aid from DOS.

DOS loops instead of blocking because applications expect

int 28h and int 2ah calls and because input can be redirected.

DOS will be modified to use an SVC service to report when

in the input loop.  The application will then be forced to sleep

for a time (with VDHWaitVIRRs).

.*

:li.Repeated int 28h calls from a &dosapp..

:lp.DOS issues int 28h in both input and output loops.

&dosapps., on the other hand, issue int 28h only when they

wish to give the spooler and other idle time code an

opportunity to execute.  Applications that do repeated

int 28h calls can be considered idle for a time.  They

should, however, run periodically since they may be

waiting for input that is not reported

by a VDD.  They also may be running useful int 28h code.

DOS will use an SVC call to report int 28h whenever a fixed

number of int 28h's in a row come from an application (not from

DOS).

Idle detection will

arrange to yield when this notification is made

(either by sleeping with VDHWaitVIRRs or by

reducing priority to idle class and yielding).

:li.Int 2fh, ax=1680h yield calls.

:lp.Newer applications use this call to yield their time slice.

When other applications are not ready to run, these cooperative

DOS applications should not be made to lose too much CPU time.

DOS will hook int 2fh and use an SVC call to report

when the yield service is called.

Idle detection will

arrange to yield when this notification is made

(either by sleeping with VDHWaitVIRRs or by

reducing priority to idle class and yielding).

:li.DOS int 21h, service 0bh.

:lp.DOS executes so much code and

causes so many traps to ring 0 when executing an int 21h

peek that it cannot be detected simply by looking at

the int 16h rate.  DOS produces a rate so low that it

could easily be produced by an application actually

working between int 16h peeks.

Therefore, peek loops using int 21h, service 0bh need

special handling.

DOS will increment a counter each time service 0bh is called.

Idle detection will use this counter in addition to

peek reports to determine when a high rate of service 0b requests

is being made.

:eul

.*

:p.The role of DOS in idle detection.

As noted above, DOS reports when in an input loop, when

repeated int 28h's occur from the application, and when

an int 2fh yield occurs.  DOS also increments a counter

of int 21h 0bh service calls and clears the count when

it gets a nonpeek request or gets input in a peek request.

In addition, DOS uses an SVC call to notify the &vkd. when

it is completing initialization.  At this time, DOS performs

a set of int 16h peeks if requested to, for callibration

of the maximum peek rate.

All of these activities could be moved outside DOS.  Since we

have control of the DOS code it is simpler to have DOS

perform the work.  However, in order to allow switching

to a design that requires less (or no) DOS modification,

DOS should not be given

any role that is not easy to move out of DOS.

.*

.*

:h4.Architectural Review

To be performed.

.*

.*

.*

:h3.&vkd. Design

.*

.*

:h4.Design Overview

:p.The execution control of the &vkd. is divided into

six sub-components:

:ol

:li.&vdm Creation

:li.Virtual I/O Port Access

:li.&pdd Notification

:li.INT 09h Processing

:li.&vdm Termination

:li.Idle Detection

:eol

.*

:h5.&vdm Creation

:p.Upon creation of the &vdm., the &vkd. establishes

communication with the keyboard &pdd. and initializes the

portions of the &cb data area associated with the keyboard.

Subsequently, the &pdd notifies the &vdd for each scan code that

is bound for the &vdm..

:p.To allow monitoring of I/O activity, the &vkd. instructs the

&v86mgr to notify it when a &vdm program accesses a virtual

keyboard port.

.*

:h5.Virtual I/O Port Access

:p.The &vkd. supports two virtual I/O ports:

:ul

:li.Port 64h - Controller Status/Command

:li.Port 60h - Controller Input/Output Buffer

:eul

:p.These two ports respond in a large variety of ways, depending

on the state of the controller.  For a detailed description of

the structures and interfaces, see :hdref refid=VKDH002..  The

remainder of this section discusses the I/O port interface at the

most general level.

:p.Read Output Buffer - An I/O read to port 0x60 reads the

contents of the controller output buffer.  If the

output-buffer-full status was set before the read, then a timer,

T1, is started.  The output-buffer-full status is cleared.  When

the T1 timer expires, the &vkd. determines if another

byte is ready to placed into the output buffer.  If there is, the

byte is placed into the output buffer and the output-buffer-full

status is set.  The value of the T1 timer will be determined

empirically during implementation.

:p.Write Output Buffer - An I/O write to port 0x60 writes the

contents to the controller input buffer.  The previous contents

of the input buffer are lost.  The port number (0x60) is saved

and the byte written is processed, depending on the state of the

keyboard controller.  The input-buffer-empty status is never set.

:p.Status Read - An I/O read to port 0x64 simply returns the

contents of the controller internal status register.  Reading

this port has no effect on the state of the virtual keyboard

hardware.

:p.Write Controller Command - An I/O write to port 0x64, like a

write to port 0x60, writes the specified byte to the controller

input buffer.  Since the port number (0x64) is saved here, the

system distingushes between a command byte and a data byte.  As

above, the byte written is processed, depending on the state of

the controller, and the input-buffer-empty status is never set.

.*

:h5.&pdd Notification

:p.Since keystrokes are, ultimately, external events, it is the

responsibility of the &pdd.  to notify the &vkd. when

keystrokes are available.  In particular, the &pdd.  calls the

&vdd.  when hardware scan codes arrive, and the &pdd.  passes

each scan code received to the &vdd..  This occurs whenever the

keyboard's current focus is the &vdd..

:p.When called, the &vdd.  places the scan code in a queue.  If

the queue was previously empty, the controller output-buffer-full

status condition is set and, if interrupts are enabled, the &vpic.

is called to simulate the interrupt to the &vdm..  If

the queue was not previously empty, the scan code is added

without any other processing.  If the queue is full, the speaker

is sounded with a "beep".

.*

:h5.INT 09h Processing

:p.In a real PC, the IRQ1 interrupt request is translated by the

interrupt controller to cause the INT 09h interrupt service

routine to be invoked.  This interrupt vector normally points to

a routine in the &cb..  In the &vdm., this is not desired, since

the &cb. only performs U.S.  key translation.  Instead, the

&vkd. simulates this &cb.  function.  Thus, the

&vkd. may use whatever key translation is appropriate

for the current country and code page.

:p.The INT 09h emulation code will perform all the functions that

the &cb.  would normally perform.  This includes calls to INT 15h

for scan code monitoring and SysRq key processing, update of

keyboard LED's, INT 1Bh (CTRL-BREAK) processing, PAUSE key

processing, INT 05h (PRINT-SCRN) processing, key translation, and

&cb. data area status update and key and scan code enqueueing.

.*

:h5.&vdm. Termination

Upon termination, the &vkd. reliquishes access to the

keyboard.

:h5.Idle Detection

:p.Idle detection and VVIDEO peek notification.

:p.When the peek rate is high, but below threshhold,

a flag will be set to stop the &vkd. from performing

optional video peek notification.  This optional activity

during peeks slows the peek rate down and

can prevent the threshhhold from being reached

when the application really is idle.

Video notification will only be prevented when the

peek rate is high, but below threshhold, when video

notification has not been prevented for a time, and

when no busy activity has occurred for a time.  Video

notification is only prevented for a single VDHQuerySysValue

clock tick.

:p.Idle detection and DOS

:p.Idle detection will use VDHRegisterVDD to register

an entry point for the kernel to use.  This entry

will return two function addresses, one to be called

when the DOS callibration SVC call is made and

a second to be called when the DOS sleep SVC call

is made.

DOS makes its SVC idle callibration call when it is

about to run command.com.  DOS supplies the address

of a word it will increment at each int 21h 0bh service

request; DOS clears this work whenever it receives

non-peek requests.  Idle detection sets the variable

to 1 each time it starts a new test period.

DOS receives a count of the number of int 16h peek calls

to make as a return value from the callibration call.

DOS performs a loop of int 16h peeks and then calls

the callibration SVC again when finished.

Idle detection uses the maximum peek rate

detected in this loop to set the peek weight for

keyboard peek event reporting.  This value

is chosen so that reports at the maximum rate

exceed 64K in one tick of the VDHQuerySysValue

millisecond clock.

:p.Idle peek report handling

:p.A new test period begins every VDHQuerySysValue

millisecond clock tick.  When the threshhold is

exceeded in a test period, a timeout is set up

and then the VDM is made to sleep with VDHWaitVIRRs.

If the VDM wakes to simulate interrupts, it

is put back to sleep after interrupt simulation

if no busy activity has been detected.

The timeout also ends the sleep if it has not already ended due to a

busy notification.

Whenever a VDM is caught being idle it goes on probation.

A busy event or being caught in another kind of idle loop ends probation.

While on probation for peeking too

much, the VDM is allowed to run periodically, but is

put back to sleep after only a small number of peeks.

This allows int 28h hooks to run and also allows

the application to proceed if it receives input

from some source that does not call VDHWakeIdle.

It also allows the application to continue if the application

times out when looking for input (it stops looking after some

number of seconds).

When a high rate of int 21h service 0bh

requests were made, the initial peek count of the next

period is set to a high value so that the application will

be caught if it continues peeking.

.*

:p.Idle handling of DOS sleep requests

:p.When DOS reports it is in its input loop, the application

is made to sleep for a time (using VDHWaitVIRRs to allow

simulated interrupts).

After sleeping, if still in the loop, DOS will report again and

will go back to sleep.

This allows int 28h and int 2ah to be run periodically while

DOS is waiting for input.  It also allows periodic peeking

in the case that stdin has been redirected.

After remaining on probation in the idle

loop for some time, the amount of time slept will increase.

If the event waited

for is one that a VDD reports as busy activity (keystroke,

COM input, mouse button press), the VDM will wake immediately

when it occurs.

Otherwise, it will have to wait for its timeout.

The timeout is short

until the wait has continued for 15 minutes with no detetable busy

events, and then is at most only 2 seconds.

:p.When int 2fh or int 28h reports occur, idle detection

waits for the second report in the test period to make

sure the apps entire loop has run at least once and that

at least a fixed number of int 28h's have occurred in

the time slice.

When the second call is made a timeout

is arranged and priority is set to the idle class.

Any time idle detection is entered while still in the

idle class, idle detection will yield and return.

.*

:p.Idle Probation

:p.Idle probation is useful to change idle detection behavior

when a loop has continued for a long time.

For calls from

DOS, the sleep time increases after some number of minutes.

In peek detection, the number of peeks needed to

cross the threshhold decreases when it is

believed the application is in a peek loop.

It is not always possible to know when a peek loop ends.

Probation ends whenever busy events are reported.

Also, whenever an idle event is detected that is different from

the one that caused probation, a new probation period starts.

Probation also ends

if a number of successive test periods fail to find the

VDM idle or if some time has passed since the VDM was last

judged idle.

.*

.*

:h4 ID=VKDH001.Data Structures Description

.*

:h5 ID=VKDH002.Hardware Description

:p.This section describes the keyboard hardware as it is

implemented by the &vkd..  Portions which are not

implemented are not described here, or are specifically noted as

unsupported.

:p.The keyboard hardware consists of a micro-controller both on

the system board and an additional micro-controller in the

keyboard itself.  In order to more easily distinguish between the

two micro-controllers, the one on the system board is referred to

as the "controller" and the one in the keyboard is referred to as

the "scanner".

:p.Virtualization of the auxiliary port, as provided in the &ps2., is

not supported.

.*

:h6.Keyboard Controller

:p.The keyboard controller directly responds to system accesses

to I/O ports 0x60 and 0x64.  It is responsible for communicating

with the scanner, reporting the status of external logical states

and providing external logical control.  There are internal

status and control mechanisms which control and monitor the

behaviour of the controller.

:p.Controller Internal Status Register - This register is used to

report internal status of the controller and the status of the

bit-serial link to the scanner.  The bits are numbered from 0 to

7, with 0 representing the low-order bit:

:ul

:li.Bit 0 - Output Buffer Full

:lp.A "1" in this bit position indicates that the controller

output buffer has data which has not yet been read by the system.

If keyboard interrupts have been enabled, then this bit

represents the state of the keyboard interrupt request line

(IRQ1).

:li.Bit 1 - Input Buffer Empty

:lp.A "1" in this bit position indicates that the controller has

not yet read the command in the input buffer.  A write to the

input buffer while this bit is set may or may not destroy the

contents of the buffer.  In virtualizing the operation of this

bit, it always remains reset.

:li.Bit 2 - System Flag

:lp.This bit may be set or reset under system control via a

controller command.  It is reset when the system is reset and is

set by the controller after the controller completes its self

test successfully.  In virtualizing this bit, it is initial set,

but may be reset by a &dosapp..

:li.Bit 3 - Controller Command/Data Indicator

:lp.This bit indicates how the last byte written to the input

buffer is interpreted.  A "1" in this bit position indicates that

the byte in the input buffer is interpreted as a command for the

controller (as a result of writing to I/O port 0x64).  A "0" in

this bit position indicates that the data in the input buffer is

interpreted as data or a scanner command (as a result of writing

to I/O port 0x60).

:li.Bit 4 - Keyboard Enable

:lp.A "1" in this bit indicates that the keyboard is enabled.  A

"0" indicates that keyboard data is inhibited by the controller.

This bit is updated as data is placed into the controller output

buffer.  In virtualizing this function, data from the keyboard

will not be forwarded to the controller output buffer unless this

bit is set.

:li.Bit 5 - Transmit Time Out

:lp.A "1" in this bit position indicates that a byte being sent

to the scanner was not clocked out within a particular amount of

time, or that the response to a commmand was not received from

the scanner, or was received in error.  This bit is cleared after

the internal status register is read.  When virtualizing this

bit, it always remains reset.

:li.Bit 6 - Receive Time Out

:lp.A "1" in this bit position indicates that a byte being

received from the scanner was not completely received within a

particular amount of time.  This bit is cleared after the

internal status register is read.  In virtualizing this bit, it

always remains reset.

:li.Bit 7 - Receive Parity Error

:lp.A "1" in this bit position indicates that the parity

associated with a byte being received from the scanner was

incorrect.  This bit is cleared after the internal status

register is read.  In virtualizing this bit, it always remains

reset.

:eul

:p.Controller External Status Register - This register reports

the status of logic which is external to the controller.  The

bits are numbered from 0 to 7, with 0 representing the low-order

bit:

:ul

:li.Bits 0 through 3 - Reserved

:lp.These bits are not used and always return 1's.

:li.Bit 4 - System Board RAM Size

:lp.A "1" in this bit indicates that 256K of system board RAM is

enabled.  A "0" indicates that 512K of system board RAM is

enabled.  In virtualizing this bit, a "0" is always returned.

:li.Bit 5 - Manufacturing Jumper

:lp.A "1" in this bit position indicates that the manufacturing

jumper is NOT installed.  In virtualizing this bit, a "1" is

always returned.

:li.Bit 6 - CGA/MONO Indicator

:lp.A "0" indicates that a CGA is attached to the primary display

adapter.  A "1" indicates that a MONO adapter is attached.  In

virtualizing this bit, a "0" is always returned.

:li.Bit 7 - Security Lock

:lp.A "0" indicates that the security lock is in the locked

position.  A "1" indicates that the lock is in the unlocked

position.

:eul

:p.Controller Internal Control Latch - this latch affects certain

internal states of the controller and the serial link to the

scanner.  The bits are numbered from 0 to 7, with 0 representing

the low-order bit:

:ul

:li.Bit 0 - Interrupt Enable

:lp.A "1" in this bit position indicates that the keyboard

interrupt request line (IRQ1) follows the output buffer full

status.  A "0" in this bit indicates that the keyboard interrupt

request line is always negated.  Logically, IRQ1 = IE*OBF.  When

virtualizing this function, the &vpic. is called to

signal a virtual interrupt only when this bit is set and a byte

is placed into the controller output buffer.

:li.Bit 1 - Reserved

:lp.In virtualizing this bit, it may be read or written by &dosapps., but

otherwise provides no functionality.

:li.Bit 2 - System Flag

:lp.The contents of this bit are copied to the System Flag bit of

the controller internal status register when this register is

written.

:li.Bit 3 - Inhibit Override

:lp.A "1" in this bit position disables the keyboard inhibit

function.  In virtualizing this bit, it may be read or written by

&dosapps., but otherwise provides no functionality.

:li.Bit 4 - Disable Scanner Data

:lp.A "1" in this bit position drives the scanner clock line low,

thereby stopping data from being transferred between the

controller and the scanner.  When virtualizing this bit, when it

is set, the &vkd. will not forward any keystrokes to

the controller output buffer.

:li.Bit 5 (&ps2.) - Auxiliary Disable

:lp.A "1" in this bit position drives the auxiliary device clock

line low, thereby stopping data from being transferred between

the controller and the auxiliary device.  When virtualizing this

bit, it is initially "0"; if a &dosapp attempts to set this bit,

the request is ignored.

:li.Bit 5 (PC/AT) - PC Mode

:lp.A "1" in this bit position allows access to old PC keyboards.

When virtualizing this bit, it is initially "0"; if a &dosapp

attempts to set this bit, the request is ignored.

:li.Bit 6 - Translate Scan Codes

:lp.A "1" in this bit position indicates that scan codes from the

scanner are to be translated into conventional scan codes.  When

it is "0", codes are passed from the scanner without translation.

In virtualizing this function, this bit is always set.  If a

&dosapp attempts to reset this bit, the request is ignored.

:li.Bit 7 - Reserved

:lp.This bit is always zero.  In virtualizing this bit, if a

&dosapp attempts to set this bit, the request is ignored.

:eul

:p.Controller External Control Latch - this latch affects certain

states of the system allows a &dosapp to set certain controller

internal states.  The bits are numbered from 0 to 7, with 0

representing the low-order bit:

:ul

:li.Bit 0 - System RESET

:lp.A "0" in this position asserts the System RESET signal.  If

this bit is reset, the system is placed into the reset state.

Unless the controller toggle command is used, resetting this bit

will cause the system to remain in a reset state until power is

removed from the system.  By use of the controller "toggle"

command (described below), this bit will be reset momementarily,

allowing a full hardware reset of the computer

system.  In virtualizing this bit, if a &dosapp attempts to reset

this bit, the request is ignored.

:li.Bit 1 - Address Line 20 Control

:lp.A "1" in this position allows the the system bus address

line 20 (A20) to operate normally as any other address line.  A

"0" in this position forces the A20 line to a low (0) state,

causing references to the second megabyte of memory to be

redirected to the first megabyte of memory.  This is for

compatibility with the older PC's which had no A20 address line.

Older applications could address low memory through a high

segment, invoking the wrap-around feature of the 8088 hardware.

The 80286 hardware does not wrap-around, so the function is

simulated externally via this method.  Note that resetting this

bit also maps the third megabyte to the fourth, the fifth to the

sixth, and so on.  This bit is normally reset when running real

mode applications.  This bit is not virtualized in the &vkd..

When this bit is accessed, an error popup will be issued giving

the user the option of killing the VDM or ignoring the error.

:li.Bits 2 and 3

:lp.These bits are reserved.  In virtualizing these bits, a

&dosapp may set, reset or examine these bits without any

additional functionality.

:li.Bit 4 - Output Buffer Full

:lp.This bit serves the status function described above in the

internal status register description.  In virtualizing this bit,

if a &dosapp sets this bit, it is treated as if the controller

were writing a byte to the output buffer and an interrupt may be

generated, depended on the state of the internal control register

interrupt enable bit.

:li.Bit 5 - Input Buffer Empty

:lp.This bit serves inverse of the status function described

above in the internal status register description.  When written,

its inverse is copied to the corresponding bit of the internal

status register.

:li.Bits 6 and 7 - Keyboard Clock and Data

:lp.These bits are used to drive the keyboard clock and data

signals.  In virtualizing these bits, a &dosapp may set, reset or

examine these bits without any additional functionality.

:eul

:p.Controller Commands - the controller accepts a variety of

commands.  Some of the commands listed here are only supported on

the &ps2..  The remaining commands are

supported on all types of PC hardware.  Unless indicated

otherwise, the virtual operation of these commands is the same as

their real operation.  The numbers listed are in hexadecimal.

:ul

:li.20 - Read Controller Internal Control Latch

:lp.The controller places the latch in the output buffer and sets

the output buffer full indication.

:li.21-3F (&ps2.) - Read Controller Memory Byte

:lp.The controller places the specified memory byte in the output

buffer and sets the output buffer full indication.  In

virtualizing this command, if a &dosapp issues one of these

commands, it is ignored.

:li.60 - Write Controller Control Latch

:lp.The controller waits for a byte to be written into the

controller input buffer and places it into the controller control

latch.  Any changes in the latch are processed according to their

functions.

:li.61-7F - Write Controller Memory Byte

:lp.The controller waits for a byte to be written into the

controller input buffer and places it into the corresponding

controller memory location.  In virtualizing this command, if a

&dosapp issues one of these commands, the request is ignored.

:li.A4 (&ps2.) - Test Password Installed

:lp.A result code is placed in the output buffer.  A value of

0xFA indicates that a password is installed.  A value of 0xF1

indicates that one is not.  In the virtual environment, if a

&dosapp attempts to issue this command, the request is ignored.

:li.A5 (&ps2.) - Load Security

:lp.The controller reads data from the input buffer until a 0x00

is read.  In the virtual environment, if a &dosapp attempts to

issue this command, the request is ignored.

:li.A6 (&ps2.) - Enable Security

:lp.If a password has been loaded, security mode is enabled with

this command.  In the virtual environment, if a &dosapp attempts

to issue this command, the request is ignored.

:li.A7-A9 - Auxiliary Device Commands

:lp.These are commands related to the auxiliary device, which is

not supported in the virtual environment.  In the virtual

environment, if a &dosapp attempts to issue one of these

commands, the request is ignored.

:li.AA - Perform Self-Test

:lp.The controller performs diagostics tests and places 0x55 into

the output buffer if no errors are detected.  In the virtual

environment, the &vkd. places 0x55 into the output

buffer.

:li.AB - Perform Interface Test

:lp.The controller tests the clock and data lines and places a result

code on the output buffer.  The result codes are:

:sl

:li.00h - No error detected

:li.01h - The clock line is stuck low

:li.02h - The clock line is stuck high

:li.03h - The data line is stuck low

:li.04h - The data line is stuck high

:esl

:pc.In the virtual environment, this function always returns a

result code of 0x00.

:li.AC - Diagnostic Dump

:lp.Sends 16-bytes of diagnostic information via the output

buffer.  In the virtual environment, if a &dosapp attempts to

issue this command, the request is ignored.

:li.AD - Disable Keyboard

:lp.Sets bit 4 of the controller internal control latch, which

has the effect of preventing scanner data from being placed into

the controller output buffer.

:li.AE - Enable Keyboard

:lp.Sets bit 4 of the controller internal control latch.  This

resumes the flow of scanner data into the controller output

buffer.

:li.C0 - Read External Status Register.

:lp.Reads the external status register into the output buffer.

:li.C1-C2 (&ps2.) - Port Input Port High/Low.

:lp.In the virtual environment, if a &dosapp attempts to issue

either of these commands, the request is ignored.

:li.D0 - Read External Control Register

:lp.Reads the external control register into the output buffer.

:li.D1 - Write External Control Register

:lp.The next byte written to the input buffer is transferred to

the external control register.

:li.D3-D4 (&ps2.) - Write Auxilary Device/Output Buffer

:lp.In the virtual environment, if a &dosapp attempts to issue

either of these commands, the request is ignored.

:li.E0 - Read test inputs

:lp.The test inputs are read into the output buffer.  Test input

T0 is written to bit 0, and test input T1 is written to bit T1.

In the virtual environment, the test inputs are initially cleared

(0), and are toggled each time the test input is read.

:li.F0-FE - Pulse External Control Latch Bit(s)

:lp.The specified bits of the controller external control latch

are pulsed low for approximately six microseconds.  Bits 0

through 3 may be specified individually or in combination by the

lower three bits of the command.  A zero (0) in a bit position

pulses the corresponding bit of the external control port.  A one

(1) leaves the bit unmodified.  Pulsing bit 0 of the external

control port performs a system reset sequence.  In the virtual

environment, this command has no effect.

:li.FF - Null Command

:lp.This command is a no-op and is ignored by the controller.  It

is used by &dosapps that need to wait for A20 address line

control to take effect.  After issuing the command to change the

A20 address line control, such applications issue a null command.

When the application sees that the controller has accepted the

null command, the application is assured that the controller has

processed the A20 address line command.

:eul

.*

:h6.Keyboard Scanner

:p.The keyboard scanner is connected to the controller via a

bit-serial link.  It is the responsibility of the controller to

communicate with the scanner on behalf of the system.  The system

can only communicate with the scanner through the use of certain

controller commands.

:p.In addition to sending scan codes, the scanner also processes

commands and provides responses to the commands.

:p.Scanner Commands - All of the following scanner functions are

supported on the extended keyboard.  The older version of the

keyboard only supports a subset of the following commands.  Such

commands are denoted by "STD" in parentheses next to the command

code.

:p.Unless stated otherwise, the scanner replies to these commands

with an ACK scan code (FA).

:p.The following command codes are shown in hexadecimal:

:ul

:li.ED (STD) - Set Keyboard LED's

:lp.The scanner sets the Keyboard LED's to the indicated states.

After replying to the command with an ACK, the scanner awaits a

bit-encoded byte indicating the new LED states:

:sl.

:li.Bit 0 - Scroll Lock Indicator

:li.Bit 1 - Num Lock Indicator

:li.Bit 2 - Caps Lock Indicator

:esl.

:li.EE - Echo

:lp.This command is a no-op.  The scanner replies to this command

with an EE.

:li.F2 (STD) - Report Identification

:lp.The scanner reports its version.  The reply consists of an

ACK followed by two bytes.  For the standard keyboard, the reply

is [?? ??].  For the extended keyboard, the reply is AB 41.

:li.F3 - Set Auto-Repeat Rate/Delay

:lp.The scanner adjusts the delay and rate at which auto-repeat codes

are sent.  The delay specifies the amount of time to wait between

depression of the key and the beginning of auto-repeat.  The rate

specifies the amount of time between subsequent auto-repeat scan

codes.  After replying to the command with ACK, the scanner

awaits a bit-packed byte indicating the rate and delay, to which

the scanner again replies with ACK.

:lp.Bit 7 must be zero.  Bits 6 and 5 form a number from 0 to 3.

The delay is this number plus one times 250 milliseconds, plus or

minus 20%.  Bits 4 and 3 form a number: call it A; bits 2, 1, and

0 form another number: call this number B.  The rate is the

inverse of the period specified by:

:fig place=inline

P = (8 + A)*(2^B)*(4.17) milliseconds

:efig

:pc.This may vary by as much as 20 percent.

:li.F4 - Enable

:lp.This enables the scanner (starts scanning).  In the virtual

environment, if this command is received, scan code data is not

placed into the output buffer, although other controller and

scanner data may be placed there.

:li.F5 - Default Disable

:lp.The scanner resets internal states and stops scanning.  In

the virtual environment, if this command is received, scan code

data is not placed into the output buffer, although other

controller and scanner data may be placed there.

:li.F6 - Default Enable

:lp.The scanner resets internal states and continues scanning.

This negates the effect of the F4 and F5 commands.

:li.F7 - Set All Keys Auto-Repeat

:lp.This command causes the scanner to periodically re-send down

codes for the last key that is held down.  Up codes are not sent

when the keys are released.  This command is acknowledged and

otherwise ignored in virtual mode.

:li.F8 - Set All Keys Make/Break

:lp.This command causes the scanner to send a down code only when

the key is pressed and to send an up code when the keys are

released.  This command is acknowledged and otherwise ignored in

virtual mode.

:li.F9 - Set All Keys Make

:lp.This command causes the scanner to send a down code only when

the key is pressed.  Up codes are not sent.  This command is

acknowledged and otherwise ignored in virtual mode.

:li.FA -  Set All Keys Auto-Repeat/Make/Break

:lp.This command causes the scanner to periodically re-send down

codes for the last key that is held down and send an up code when

keys are released.  This is the default mode.  This command is

acknowledged and otherwise ignored in virtual mode.

:li.FE - Resend

:lp.The scanner re-sends the last byte sent unless it was a "resend"

command, in which case it resends the previous byte.

:li.FF - Reset

:lp.The scanner performs a hardware reset and performs an

internal diagnostic self-test.  Before resetting, the scanner

sends an ACK reply, and waits for the system to raise the clock

and data signals for a minimum of 500 microseconds.  After

completing the self test, the scanner returns a completion code.

In virtual mode, the &vkd. acknowledges the command and

returns a successful reply.

:eul

.*

:h5.&cb. Data Area

:p.The &cb. data area is initialized upon creation of the &vdd. and

is referenced and modified during execution.

:figref REFID=LPTF003 page=no.  lists each &cb. data item

associated with the &vkd..  The offsets shown are

relative to the &cb. data segment (0040:0000):

:fig id=LPTF003 frame=none place=inline

.ti > 09

.tb 2 11 23

.bx 1 10 22 62

>      >Initial

>      >Value

>Offset>(Hex)    >Description

.bx

.in 23

.ir 5

.un 21

.*            |23456789x123456789x123456789x123456789x1|

0017 >   20    >KB_FLAG (BYTE) - State flags.  Keeps

.un 21

>         >track of the shift and toggle states.

.bx

.un 21

0018 >   00    >KB_FLAG_1 (BYTE) - Down-state flags.

.un 21

>         >Keeps track of toggle keys which are

.un 21

>         >currently depressed.

.bx

.un 21

0019 >   00    >ALT_INPUT (BYTE) - Keeps track of ALT-

.un 21

>         >NUMPAD character being built.

.bx

.un 21

001C > 001E    >BUFFER_TAIL (WORD) - Points to the

.un 21

>         >tail of the circular keystroke buffer.

.un 21

>         >This is updated by the INT 16h funct-

.un 21

>         >ion.

.bx

.un 21

001E >         >KB_BUFFER (16 WORDS) - The circular

.un 21

>         >keystroke buffer.  This buffer is mov-

.un 21

>         >able within segment 0040.

.bx

.un 21

0080 > 001E    >BUFFER_START (WORD) - Points to the

.un 21

>         >lowest address of the circular key-

.un 21

>         >stroke buffer.  This word is not

.un 21

>         >changed after initialization.

.bx

.un 21

0082 > 003E    >BUFFER_END (WORD) - Points to the

.un 21

>         >byte immediately after the highest

.un 21

>         >address of the circular keystroke buf-

.un 21

>         >fer.  This word is not changed after

.un 21

>         >initialization.

.bx

.un 21

0096 >   00    >KB_FLAG_3 (WORD) - State data.  This

.un 21

>         >byte keeps track of the keyboard

.un 21

>         >protocol state.

.bx

.*            |23456789x123456789x123456789x123456789x1|

.un 21

0097 >   02    >KB_FLAG_2 (WORD) - LED State.  This

.un 21

>         >byte contains the keyboard LED State.

.bx off

.in

.ir

:figcap.&vkd. &cb. Data Items

.ti 3E 3E

:efig

.*

.*

:h4.Exported Interfaces

.*

:h5.Exported data declarations

.im1 kbdext.sin

.*

:h5.Exported procedure interface flag definitions

.im1 kbdexk.sin

.*

:h5.Exported procedure headers

.im1 kbdexp.sin

.*

.*

:h4.Imported Interfaces

.im1 kbdimp.sin

.*

.*

:h4.Design Constraints

To be added.

.*

.*

:h4.Design Review

To be performed.

.*

.*

.*

:h3.&vkd. Implementation

.*

.*

:h4.Internal Interfaces

.*

:h5.Local data declarations

.im1 kbdlot.sin

.*

:h5.Local procedure interface flag definitions

.im1 kbdlok.sin

.*

:h5.Local procedure headers

.im1 kbdlop.sin

.*

.*

:h4.Implementation

To be added.

.*

.*

:h4.Implementation Review

To be performed.

.*

.*

.*

:h3.&vkd. Appendix

.*

.*

:h4.Glossary

To be added.

.*

.*

:h4.Size and Performance Considerations

To be added.

.*

.*

:h4.Implementation Estimates

To be added.

.*

.*

:h4.Design History

.im kbdhis.sci

.*

:Eissues.kbdisu.isu