****************************************************************************

NCR VGA Drivers

SoftSet.Exe and Soft_Res.Sys

Versions 2.xI and 2.xP

Copyright (c) NCR Corporation, 1992

****************************************************************************

Contents:

---------

- Files Provided.

- Soft_res vs  older SoftMode

- Why Should You Use Soft_Res.SYS ?

- Setting up Soft_Res.sys with SoftSet.exe.

- Setting up Soft_Res.sys to support VESA calls.

- Dot Clock Frequencies of Soft_Res 2.xI and 2.xP.

- Setting up 132x25, 132x50, 80x25, and 80x50 text.

- Using Soft_Res.Sys without SoftSet.exe.

- CONFIG.SYS Example.

- Setting a Mode.

- Setting up Soft_Res.sys for use with NCR OS/2 2.1 drivers.

- Setting up Soft_Res.sys for 8-bit wide memory.

- Problem List.

Files Provided

--------------

- Soft_Res.sys       version 2.51Pab with dual VESA window.

The following optional .SYS files must be renamed to

soft_res.sys before using. See Dot Clock Tables and

Problem List below for usage.

- SRP.sys             version 2.51Pa with single VESA window.

- SRPD.sys            backup of above soft_res.sys

- SRI.sys             version 2.51Ia with single VESA window.

- SRID.sys            version 2.51Ia with dual VESA window.

- SoftSet.exe        allows you to setup Soft_Res.sys and test each

mode timing.

- SetVid.exe         allows you to set a mode from DOS. Both NCR

and VESA mode numbers are supported.

- SetMode.exe        allows special text mode setup.

- SetMode.doc        documentation for SetMode.

- Soft_Res.doc       this file.

Soft_Res vs older SoftMode

---------------------------

- Soft_Res.sys and the older SoftMode.sys are basically the same.

Soft_Res, however, has been updated to be compatible with the

NCR OS2 2.1 drivers and the 77c32BLT chip.

Why Should You Use Soft_Res.sys?

--------------------------------

The reasons you should use Soft_Res.sys:

- Extended Modes:

For the older, NCR-Award VGA BIOS, there is no support

for modes beyond 1024x768 256 color (mode 62h).  Soft_Res.sys

provides the ability to set modes beyond mode 62h including

the new HiColor 32k and 64k color modes.  The NCR

Windows drivers rely on Soft_Res.sys to properly set these

extended modes.  In addition, the NCR AutoCad 11 drivers

require the use of Soft_Res.sys.

- WIN-OS/2 support:

To properly set up the modes for WIN-OS/2 sessions, Soft_Res.sys

version 2.5 or greater is required.

- Mode Timing Selection:

There is no way you can tell ROM BIOS which timing to use

for a given mode number.  However, Soft_Res.sys supports a variety

of timing sets for certain modes.  This enables you to select the

timing which best suits your monitor.

- Image Centering:

There is no way you can center the video image using

BIOS.  By running SoftSet to initially select the mode

timings for Soft_Res.sys, you can also center each mode for

your monitor.  This way you won't need to touch the monitor

each time a different mode is set by applications.

- VESA BIOS Extensions:

There are no VESA extensions built into the ROM BIOS.

Soft_Res.sys supports VESA version 1.2 BIOS extensions.

Setting up Soft_Res.sys with SoftSet.exe

----------------------------------------

It is recommended that you run SOFTSET to setup Soft_Res.sys.

SoftSet.exe provides the user with an easy way to setup

Soft_Res.Sys.  To run SoftSet make sure that both files,

soft_res.sys and softset.exe, are in the same directory.

Go to that directory and type SOFTSET [drive] <return>.

The optional "drive" parameter signals SoftSet to find

your config.sys file on a drive other than the boot drive.

The defualt drive is the boot drive if you boot from C or A drive.

Otherwise, the default is the drive on which SoftSet is located.

You can overide the default by providing the "drive" parameter.

This is often the case when setting up for OS2 2.1.

Softset allows you to test and observe each of the mode settings

provided in Soft_Res.sys.  You may then "Use" or "Not Use" each

setting. All of the used settings are assembled in a list which

is added to a command line in the CONFIG.SYS file.  Users may

edit the CONFIG.SYS file themselves or allow SoftSet to add the

command line automatically.

Setting up Soft_Res to support VESA calls

-----------------------------------------

Soft_Res 2.x supports VESA BIOS extensions.  To enable VESA

functions, include the /V switch on the command line.  This

can be done with SoftSet.exe version 1.55 or later.

Dot Clock Frequencies of Soft_Res

---------------------------------

Soft_Res supports two types of dot clock chips.

Soft_Res.sys 2.xI assumes that the dot clocks of Table 1a are selectable.

Soft_Res.sys 2.xP assumes that the dot clocks of Table 1b are selectable.

Table 1a: Dot Clock Frequencies for the ICS2494-253 and ICS1394-073.

Files SRI.sys and SRID.sys are built using this table.

Hex Address      Frequency in Mhz

0              25.175

1              28.322

2              40.000

3              65.000

4              44.900

5              50.000

6              56.000

7              75.000

8              25.175

9              28.322

A              00.000

B              00.000

C              60.000

D              80.000

E              00.000

F              00.000

Table 1b: Dot Clock Frequencies for the AV9194-46 and ICS2494-277.

Files soft_res.sys (default) and SRP.sys are built using this table.

Hex Address      Frequency in Mhz

0               25.175

1               28.322

2               36.000

3               65.000

4               44.900

5               50.000

6               80.000

7               75.000

8               56.644

9               63.000

A               72.000

B              130.000

C               90.000

D              100.000

E              110.000

F              120.000

Setting up 132x25, 132x50, 80x25, and 80x50 text.

------------------------------------------------

There is a new version of SETMODE.EXE to replace the old SETMODE.COM.

The old setmode.com was a TSR which provided high resolution text.

Now, this feature is built into Soft_Res.  Use SETMODE.EXE to set up

the resolution required the same way the old TSR was used.

See SETMODE.DOC for details.

Using Soft_Res.Sys Without SoftSet.exe

--------------------------------------

Soft_Res's purpose is to allow access to extended graphics modes

without up-grading BIOS.  In addition, the user may specify

which timings apply when a specific mode is called for by an

application. This usually depends on the monitor.

For example, if the monitor is an NEC 4d, then the user would

want this driver to allow a non-interlaced timing for the

1024x768 256 color mode. At the same time, the 1280x1024 mode

must be limited to an interlaced timing.

To accomplish the above, it is necessary to supply command

line parameters following the DEVICE=soft_res.sys statement.

The following is a list of rules required to set up the parameters:

- The parameter list must be preceded by the /M: switch.

- Each parameter must be a decimal number taken from the

list of supported setting numbers of Table 2 below.

- Multiple parameters must be separated by commas.

Table 2: Graphics Modes with Frequency Info and Setting Numbers.

Frequencies:

Vertical   Horizonatal    Dot          Type           Setting Number

--------   -----------    ---          ----           --------------

12h: 640x480,16 colors

72.874  Hz  37.967 KHz  32.500 MHz  Non-Interlaced   ( Setting # 13 )

60.004  Hz  35.403 KHz  28.322 MHz  Non-Interlaced   ( Setting # 17 )

5fh: 640x480,256 colors,  VESA Mode 101

72.814  Hz  37.791 KHz  32.500 MHz  Non-Interlaced   ( Setting # 12 )

59.940  Hz  31.469 KHz  25.175 MHz  Non-Interlaced   ( Setting # 2 )

58h: 800x600,16 colors,  VESA Mode 102

71.971  Hz  48.077 KHz  50.000 MHz  Non-Interlaced   ( Setting # 8 )

60.317  Hz  37.879 KHz  40.000 MHz  Non-Interlaced   ( Setting # 0 )

56.021  Hz  35.461 KHz  40.000 MHz  Non-Interlaced   ( Setting # 15 )

5ch: 800x600,256 colors,  VESA Mode 103

71.971  Hz  48.077 KHz  50.000 MHz  Non-Interlaced   ( Setting # 10 )

60.317  Hz  37.879 KHz  40.000 MHz  Non-Interlaced   ( Setting # 4 )

56.021  Hz  35.461 KHz  40.000 MHz  Non-Interlaced   ( Setting # 16 )

5dh: 1024x768,16 colors,  VESA Mode 104

70.069  Hz  56.476 KHz  75.000 MHz  Non-Interlaced   ( Setting # 14 )

70.059  Hz  56.818 KHz  75.000 MHz  Non-Interlaced   ( Setting # 1 )

60.530  Hz  48.363 KHz  65.000 MHz  Non-Interlaced   ( Setting # 9 )

87.064  Hz  35.522 KHz  44.900 MHz  Interlaced       ( Setting # 3 )

62h: 1024x768,256 colors,  VESA Mode 105

70.099  Hz  56.991 KHz  75.000 MHz  Non-Interlaced   ( Setting # 11 )

60.004  Hz  48.363 KHz  65.000 MHz  Non-Interlaced   ( Setting # 6 )

87.064  Hz  35.522 KHz  44.900 MHz  Interlaced       ( Setting # 5 )

6ah: 1280x1024,256 colors,  VESA Mode 107

60.024  Hz  63.025 KHz  90.000 MHz  Non-Interlaced   ( Setting # 107 )

86.938  Hz  48.077 KHz  75.000 MHz  Interlaced       ( Setting # 7 )

5eh: 640x400,256 colors,  VESA Mode 100

60.055  Hz  31.469 KHz  28.322 MHz  Non-Interlaced   ( Setting # 20 )

6ch: 1280x1024,16 colors,  VESA Mode 106

86.938  Hz  48.077 KHz  75.000 MHz  Interlaced       ( Setting # 67 )

5ah: 1024x768,2 colors

87.064  Hz  35.522 KHz  44.900 MHz  Interlaced       ( Setting # 19 )

70h: 640x480,32k colors,  VESA Mode 110

59.974  Hz  31.486 KHz  50.000 MHz  Non-Interlaced   ( Setting # 100 )

78h: 640x480,64k colors,  VESA Mode 111

59.974  Hz  31.486 KHz  50.000 MHz  Non-Interlaced   ( Setting # 110 )

71h: 800x600,32k colors,  VESA Mode 113

59.979  Hz  38.986 KHz  80.000 MHz  Non-Interlaced   ( Setting # 102 )

56.230  Hz  36.550 KHz  75.000 MHz  Non-Interlaced   ( Setting # 101 )

79h: 800x600,64k colors,  VESA Mode 114

59.979  Hz  38.986 KHz  80.000 MHz  Non-Interlaced   ( Setting # 112 )

56.230  Hz  36.550 KHz  75.000 MHz  Non-Interlaced   ( Setting # 111 )

72h: 1024x768,32k colors,  VESA Mode 116

84.902  Hz  35.489 KHz  90.000 MHz  Interlaced       ( Setting # 108 )

75.468  Hz  31.546 KHz  80.000 MHz  Interlaced       ( Setting # 105 )

7ah: 1024x768,64k colors,  VESA Mode 117

84.902  Hz  35.489 KHz  90.000 MHz  Interlaced       ( Setting # 109 )

75.468  Hz  31.546 KHz  80.000 MHz  Interlaced       ( Setting # 106 )

50h: 640x480,16M colors,  VESA Mode 112

60.024  Hz  31.513 KHz  75.000 MHz  Non-Interlaced   ( Setting # 103 )

51h: 800x600,16M colors,  VESA Mode 115

97.777  Hz  33.733 KHz  90.000 MHz  Interlaced       ( Setting # 114 )

86.913  Hz  29.985 KHz  80.000 MHz  Interlaced       ( Setting # 113 )

The following text modes are also included in Soft_Res:

setting#   mode

30      3

54     54

55     55

CONFIG.SYS Example

------------------

To tell Soft_Res to enable non-interlaced timings for mode 5d,

but interlaced timings for mode 62, the user must create the following line

in his or her config.sys file:

DEVICE = path\SOFT_RES.SYS /M:1,5

where path is the path to the directory location of the Soft_res.sys file.

After modifying the config.sys file, the system must be re-booted.

Upon loading at boot time, Soft_Res will respond by displaying:

Modes and settings enabled:

5d.....1

62.....5

Note that the above example produces the following results:

- Modes 5d and 62 will be set by Soft_Res.sys.

- Any standard modes (57 hex or below) will be set by BIOS.

- Any modes above 57 hex, other than 5d and 62, will not be set.

If the setting number 30 was specified with the /M:30 switch, then

mode 3 would be set by Soft_Res and not by BIOS.

If no parameters are given, Soft_Res will not be loaded and an error

message will appear on the screen.

Setting a Mode

--------------

Once the system is re-booted and Soft_Res is loaded successfully, the

video mode will be, as always, mode 3 set by BIOS. To activate one of the

modes that you have enabled in Soft_Res, you can use the SETVID.EXE

utility as follows:

In the above example, modes 5d and 62 are the only possible mode choices

for Soft_Res to set.

To activate mode 5d, simply type:

SETVID 5d  <return>

Then, mode 5d will be selected using the timings of number one, i.e. non-

interlaced 70Hz timings.

To return to mode 3 set by BIOS, type SETVID 3  <return>.

Setting up Soft_Res for use with NCR OS/2 2.1 drivers.

------------------------------------------------------

The NCR OS/2 drivers require Soft_Res to set up the WIN-OS/2 sessions

a special way.  To tell Soft_Res you are running under OS/2,

use the /O switch on the command line.

Setting up Soft_Res for 8-bit wide memory

-----------------------------------------

A conflict with some adapter cards may require the VGA to

use 8-bit wide memory.  To force the VGA into 8-bit mode, include

the /8 switch on the command line.

The Softset program may not allow the user to add the /8

switch automatically.  Use an editor to add the /8 switch after

using SoftSet.

Problem List

------------

- Soft_Res.sys version 2.54 was modified to work with

Microsoft Flight Simulator 5.

- Soft_Res.Sys does not currently support any functions other

than the setting of a mode.  Therefore, all other INT 10 calls

are routed to BIOS.  If the user has BIOS which

lacks, for example, the write tty function for the mode

selected,  the screen may not be legible when the mode is

set with SETVID.  The mode is, however, useful when set

by an application driver, such as a Windows 3.0 driver.

- Versions prior to 2.48 had problems with

VESA functions 6 and 7 (scan line and display start).

- Version 2.48 fixes VESA functions 6 and 7 for 16 and

256 color modes.  Hicolor and true color modes may

not yield correct results.

- Versions 2.xxxab allows dual window support for VESA.

This should work with most applications.  However,

not all applictions check for this support and

assume single window support.  When this is true, you

may find that the screen is not correct or mouse droppings

occur.  If this is the case, you will want to use the alternate

SRI.sys or SRP.sys file which is also provided.  To use one of

the alternate files, you must rename it to SOFT_RES.sys.  Otherwise,

SoftSet.exe will not find it.