SERIAL RADIO LINK

The serial radio link uses a pair of inexpensive walkie-talkies

operating on the 49-mHz "toy radio device" band.  Range is

typically 50 to 100 feet.  Data format is 8-bit, odd-parity, one

stop bit, FSK modulated, 2400 baud, half duplex.  Half duplex is

required, since only one side may be transmitting at a time.

The FSK (frequency-shift keyed) modulation selects the output

signal from either a 4X-baud or 8X-baud frequency generator.

###################################