STEPPER MOTOR CONTROL

Each motor is a four-phase, permanent-magnet, uni-polar stepper

motor.  This means that there are four single-polarity coils to

energize.  In the normal full-step mode, two coils are energized

at once.  Wave-drive mode energizes only one coil at a time.

Half-step mode alternates between one and two coils every other

step.  Each coil is controlled by one output bit, so each motor

requires four bits.  Each eight-bit output port controls two

motors.  Eight ports are provided for stepper motors, therefore

sixteen motors may be controlled.

An all-software approach is taken for stepper motor control.

This approach offers the advantages of less power consumption,

less expense, and a less complex hardware design.  In addition,

it provides greater flexibility in step modes and step rates. The

alternative would be a sequencer chip for each motor.  The

software would not be any simpler, since sixteen motors must be

controlled individually.  The only dedicated hardware in the

final design is the Darlington power amplifier for each coil.

The stepper motors are controlled by a timer-interrupt driven

routine.  The timer interrupts every milli-second.  The routine

scans the stepper motor work areas to determine if a motor is

ready for its next step.  Each work area has a counter that is

set to the number of milli-seconds between steps.  The routine

decrements the counters for all motors that are in "active"

status.  If a counter reaches zero, it is reset to its initial

value, and the next sequential bit pattern is selected from a

table.  This pattern is written to the output port for the proper

motor.

The pattern table contains the bit patterns required to energize

the coils in the proper sequence.  There are table entries for

forward/reverse, full-step, half-step, and wave drive sequences.

The direction and step-type bits specified in the start-up

request are used to index to the proper table entry.  The start-

up request also specifies the step rate in milli-seconds.

If a motor is stepped too fast, it will miss steps.  If it is

stepped much slower than its rated speed, it will buzz.  This is

because the rotor has time to come to a stop after each step, and

must start up again for the next step.  This start-stop action

causes vibrations which are annoying and often harmful to the

power train (gears, bearings, etc.).  A step-rate initialization

table specifies the optimum step rate for each motor.

To allow for smoothe starts and stops, an accelleration and

decelleration mechanism is included.  If the accellerate flag is

set by the requestor, the initial step rate is set slower than

normal.  Each succeeding step is speeded up by one milli-second,

until normal speed is reached.  If the flag is set for

decelleration, the step time starts increasing several steps from

the stopping point.

Provision is made for continuous-run mode, where no step count is

requested.  A limit switch interrupt will reset the motor if the

requestor does not.  This is useful for positioning all motors to

a known point after a system reset.  In order to provide

additional holding torque, the requestor may specify that the

coil(s) remain energized after the motor reaches its stopping

point.

The only drawback to the all-software design approach is that the

CPU may not be fast enough to keep all of the motors running at

once.  Indeed, if all the motors required a new bit pattern

during the same milli-second, the update routine would not be

finished before the next timer interrupt occurred.  If the update

routine ran disabled for interrupts, succeeding timer interrupts

would be lost or delayed.  In actual operation, it would be rare

to find more than three or four motors active at any given time.

It would be VERY rare for all of them to be stepping at exactly

the same time.  A 4mHz Z80 can only handle two updates per milli-

second.  The only way to prevent lost timer interrupts (which

would slow down timer services), is to run the stepper motor

updates under the stepper motor control task, which is enabled

for interrupts.  Then if things get busy, some stepper motors

slow down a little, but the timer stays accurate.

################################